3GPP TSG RAN WG1 Meeting #109-e	R1-2204622

Online, 9 – 20 May, 2022

Agenda Item:	8.12.1

Source: 	LG Electronics Inc.

Title: 	Mechanisms to improve Reliability of Broadcast/Multicast service

Document for:	Discussion and decision

Introduction

In this contribution, we discuss remaining issues on support of multicast HARQ-ACK.

TP-1 on more than one NACK-only HARQ-ACKs

TS 38.213 v17.1.0 introduced multiplexing more than one NACK-only HARQ-ACKs as follows:

For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable when a number of HARQ-ACK information bits is more than four, or for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is 2, 3, or 4, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1.

The highlighted sentences was specified based on the following agreements:

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability

As agreed in RAN1#108-e, Alt4 is limited to the case when a number of HARQ-ACK information bits is 2, 3, or 4. However, Alt1 is not limited to the case. Based on the agreement, we understand that Alt1 can support more than 4 HARQ-ACK information bits.

In addition, it is unclear whether to send HARQ-ACK when the number of HARQ-ACK information bits is more than 4 in Alt4. In our view, no HARQ-ACK information is transmitted when the number of HARQ-ACK information bits is more than 4 in Alt4.

Therefore, we propose to adopt TP-1 for Section 18 of TS 38.213 as follows to clarify that Alt1 can support more than 4 HARQ-ACK information bits and no HARQ-ACK information is transmitted when the number of HARQ-ACK information bits is more than 4 in Alt4.

Proposal 1: Adopt TP-1 to clarify the previous agreement that Alt1 can support more than 4 HARQ-ACK information bits.

- Further clarify that no HARQ-ACK information is transmitted when the number of HARQ-ACK information bits is more than 4 in Alt4, as in TP-1.

TP-1 for TS 38.213 ----------------------------------------------------Text proposal starts---------------------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is more than one, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. When a number of HARQ-ACK information bits is more than four, the UE does not transmit the PUCCH that would include HARQ-ACK information unless the HARQ-ACK information is transmitted according to the first HARQ-ACK report mode. < Unchanged parts are omitted > ----------------------------------------------------Text proposal ends-----------------------------------------------

TP-2 on multiplexing NACK-only HARQ-ACK with other UCI

As shown in the agreed CR to 38.213 below, the previous sentence in 38.213 v17.0.0 was replaced by the new sentences in 38.213 v17.1.0.

If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with other UCI in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are ‘ACK’. For the second HARQ-ACK reporting mode, if a UE would multiplex HARQ-ACK information associated with multicast DCI formats and HARQ-ACK information associated with unicast DCI formats in a PUCCH, or with P/SP-CSI in a PUCCH, or in a PUSCH, the UE provides HARQ-ACK information according to the first HARQ-ACK reporting mode.

TS 38.213 v17.0.0 specified the case when multicast NACK only based HARQ-ACK is multiplexed with unicast HARQ-ACK, while TS 38.213 v17.1.0 specified the additional cases when multicast NACK only based HARQ-ACK is multiplexed with other UCI including unicast HARQ-ACK.

We understand the single sentence in TS 38.213 v17.1.0 addresses the following agreement:

Agreement When PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority, support UE multiplexing the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit. - Note: When the TB configured with NACK-only feedback is correctly decoded, the ACK will be transmitted and multiplexed with others.

In addition, the single sentence in TS 38.213 v17.1.0 can address the case when multicast NACK only based HARQ-ACK is multiplexed with SR/CSI.

However, the current sentence in TS 38.213 v17.1.0 can be misleading due to the following reasons:

- The term ‘other UCI’ can include the other NACK-only HARQ-ACK in the first PUCCH. However, the other UCI shouldn’t be the other NACK-only HARQ-ACK, if a number of HARQ-ACK information bits is 2, 3, or 4 and the UE is indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1.

- It is not clear whether CG-UCI can be multiplexed with NACK-only based HARQ-ACKs. If CG-UCI can be multiplexed with multicast HARQ-ACKs, we need to discuss how to support such multiplexing before implementing CR.

Therefore, we propose to adopt TP-2 for Section 18 of TS 38.213 as follows to clarify that the other UCI include neither the other NACK-only HARQ-ACK nor CG-UCI.

Proposal 2: Adopt TP-2 to clarify that the other UCI include neither the other NACK-only HARQ-ACK nor CG-UCI.

TP-2 for TS 38.213 ----------------------------------------------------Text proposal starts---------------------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with HARQ-ACK information according to the first HARQ-ACK reporting mode, HARQ-ACK information associated with unicast DCI formats, SR or CSI report(s) in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are ‘ACK’. < Unchanged parts are omitted > ----------------------------------------------------Text proposal ends-----------------------------------------------

TP-3 on NACK-only PUCCH overlapped with PUSCH

NACK-only based PUCCH could be overlapped with PUSCH in a slot. NACK-only based PUCCH could corresponds to one HARQ-ACK bit. In this case, it seems natural that the NACK-only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK for multiplexing NACK-only based PUCCH and PUSCH in a same slot.

In addition, NACK-only based PUCCH could corresponds to multiple HARQ-ACK bits that are multiplexed based on Alt4. For example, a resultant PUCCH resource selected for multiplexing four NACK-only based HARQ-ACK bits according to Alt4 could be overlapped with PUSCH in time. In this case, it is not clear whether the NACK-only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK for multiplexing NACK-only based PUCCH and PUSCH in a same slot, because Alt4 is configured for multiplexing NACK-only based HARQ-ACKs. In our view, it seems natural that the multiple NACK-only based HARQ-ACKs are also transformed to ACK/NACK based HARQ-ACK for multiplexing the resultant NACK-only based PUCCH and PUSCH in a same slot. If transformation to ACK/NACK based HARQ-ACK is not supported, UE could transmit the PUSCH and drops the PUCCH.

Proposal 3: NACK-only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK for NACK-only based PUCCH overlapped with PUSCH.

Proposal 4: If the resultant NACK-only based PUCCH selected for multiplexing multiple NACK-only HARQ-ACK bits in Alt4 is overlapped with PUSCH, NACK-only based HARQ-ACKs are transformed to ACK/NACK based HARQ-ACK for multiplexing the NACK-only HARQ-ACK bits in the PUSCH.

Proposal 5: Adopt TP-3 to clarify that multiple NACK-only based HARQ-ACKs are also transformed to ACK/NACK based HARQ-ACK for NACK-only based PUCCH overlapped with PUSCH.

- NACK-only based PUCCH can correspond to either one HARQ-ACK bit or the resultant NACK-only based PUCCH selected for multiplexing multiple NACK-only HARQ-ACK bits in Alt4.

TP-3 for TS 38.213 ----------------------------------------------------Text proposal starts---------------------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with other UCI in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. If a PUCCH carrying HARQ-ACK information associated with multicast DCI formats according to the second HARQ-ACK reporting mode is overlapped with a PUSCH in a slot, the UE provides the HARQ-ACK information in the PUSCH according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are ‘ACK’. < Unchanged parts are omitted > ----------------------------------------------------Text proposal ends-----------------------------------------------

TP-4 on multicast PUCCH overlapping with MSG3 PUSCH

When multiple UEs in the same group send ACK/NACK or NACK-only based HARQ-ACK(s) to multicast PDSCH, one of the UEs may perform RACH e.g. for Scheduling Request, Beam Failure Report or Timing Alignment. In this case, PUCCH for multicast HARQ-ACK could overlap with MSG3 PUSCH in a same slot.

For PUCCH overlapping with MSG3 PUSCH, we think that UE may be able to simultaneously transmit PUCCH carrying multicast HARQ-ACK and MSG3 PUSCH in a same slot, considering that the TA timer will start upon receiving MSG2. However, considering the simultaneous transmissions of PUCCH and MSG3 PUSCH do not currently occur, it seems simple that UE receiving multicast PDSCH does not transmit multicast HARQ-ACK in the slot where MSG3 PUSCH is transmitted. That is, UE does not need to support simultaneous transmissions of PUCCH and MSG3 PUSCH. This can be also applied to PUCCH overlapping with MSGA PUSCH

Proposal 6: Adopt TP-4 to clarify that UE does not transmit PUCCH carrying multicast HARQ-ACK overlapping with MSG3/MSGA PUSCH.

TP-4 for TS 38.213 ----------------------------------------------------Text proposal starts---------------------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a PUCCH carrying a HARQ-ACK information associated with multicast DCI formats overlaps with MSG3 PUSCH or MSGA PUSCH in a slot, UE does not transmit the PUCCH. < Unchanged parts are omitted > ----------------------------------------------------Text proposal ends-----------------------------------------------

Multiplexing SR and HARQ-ACK bits for unicast and multicast

It seems not discussed yet how to multiplex positive SR, 1 HARQ-ACK bit for unicast and 1 HARQ-ACK bit for multicast for PUCCH format 0/1. Considering the existing multiplexing rules, for a same priority, we propose to multiplex a positive SR and two HARQ-ACK bits for unicast and multicast respectively as follows:

Proposal 7: For multiplexing a positive SR and two HARQ-ACK bits for unicast and multicast respectively of a same priority,

- For PUCCH format 0 for unicast HARQ-ACK,

  - Three bits are multiplexed on the PUCCH resource with PUCCH format 0 allocated for unicast HARQ-ACK according to the existing rules.

- For PUCCH format 1 for unicast HARQ-ACK and SR,

  - Two HARQ-ACK bits are multiplexed on the SR PUCCH resource according to the existing rules

- For PUCCH format 1 for unicast HARQ-ACK and PUCCH format 0 for SR

  - SR is dropped and Two HARQ-ACK bits are multiplexed on the PUCCH resource with PUCCH format 1 allocated for unicast HARQ-ACK according to the existing rules.

In addition, for Rel-17 intra-UE multiplexing, RAN1 agreed that for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH or a PUSCH in R17, separate coding for the two HARQ-ACKs are supported. Considering this agreement, we propose that HARQ-ACKs for unicast and HARQ-ACKs for multicast are multiplexed per each priority before applying R17 intra-UE multiplexing rules. Afterwards, R17 intra-UE multiplexing rules can be applied to multiplexing PUCCH for HP and PUCCH for LP, assuming that the multiplexed unicast/multicast HARQ-ACKs of different priorities are considered HP/LP HARQ-ACKs respectively in the rules.

Proposal 8: For multiplexing HARQ-ACK information for unicast and multicast respectively and other UCIs of both LP and HP,

- HARQ-ACKs for unicast and HARQ-ACKs for multicast are multiplexed per each priority before applying R17 intra-UE multiplexing rules.

- Afterwards, R17 intra-UE multiplexing rules are applied to multiplexing PUCCH for HP and PUCCH for LP, assuming that the multiplexed unicast/multicast HARQ-ACKs of different priorities are considered HP/LP HARQ-ACKs respectively in the rules.

Type-3 codebook

In RAN1#108-e, the following proposal was discussed.

Proposal 2.1.3-2 When UE is scheduled to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide Type-3 codebook in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the Type-3 codebook that consists of HARQ-ACK bits for all HARQ processes including the ones used for multicast.

If pdsch-HARQ-ACK-OneShotFeedback is configured, the DCI_format 1_1 can request the UE to report A/N for all HARQ processes and all CCs configured in the PUCCH group. Since Type-3 codebook is currently supporting unicast HARQ-ACKs only, the Type-3 codebook could consist of HARQ-ACK bits to the last unicast PDSCHs for all HARQ processes. In this case, the HARQ-ACK bit for a HARQ process in Type-3 codebook cannot be the HARQ-ACK to multicast PDSCH.

Alternatively, the Type-3 codebook could consist of HARQ-ACK bits to the last PDSCHs for all HARQ processes, regardless of whether the last PDSCH corresponds to unicast or multicast. In this case, the HARQ-ACK bit for a HARQ process in Type-3 codebook can be the HARQ-ACK to multicast PDSCH.

Meanwhile, if pdsch-HARQ-ACK-OneShotFeedbackNDI is configured as well as pdsch-HARQ-ACK-OneShotFeedback, the DCI_format 1_1 can request the UE to include NDI for each A/N reported in Rel-16. We should clarify whether NDI values indicated by last multicast DCIs can be included in Type-3 codebook if HARQ-ACK bits in Type-3 codebook can be HARQ-ACK to multicast PDSCH.

Accordingly, the following proposal can be discussed for Type-3 codebook:

Proposal 9: When UE is scheduled to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide Type-3 codebook in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the Type-3 codebook that consists of HARQ-ACK bits (and NDI values if pdsch-HARQ-ACK-OneShotFeedbackNDI is configured) based on the existing rules in 9.1.4 of 38.213:

- Alt 1: the Type-3 codebook consists of HARQ-ACK bits to the last unicast PDSCHs and NDI values indicated by the last unicast DCI format for all HARQ processes.

- Alt 2: the Type-3 codebook consists of HARQ-ACK bits to the last PDSCHs and NDI values indicated by the last DCI formats for all HARQ processes (i.e. the last one can be either unicast or multicast).

Deferring HARQ-ACK to multicast SPS PDSCH

As agreed in Rel-17 URLLC, SPS HARQ-ACK deferral can be configured for a SPS configuration. According to ASN.1 structure in TS 38.331 v17.0.0 that sps-HARQ-Deferral-r17 can be configured for multicast SPS as well as unicast SPS.

We think that if ACK/NACK based HARQ-ACK is configured for multicast SPS, the rules of deferring HARQ-ACK to unicast SPS PDSCH could be easily reused for deferring HARQ-ACK to multicast SPS PDSCH. However, we wonder if deferring NACK-only based HARQ-ACK to multicast SPS can always work well, considering NACK-only based HARQ-ACK can be shared by multiple UEs.

In our view, the rules of deferring HARQ-ACK to unicast SPS PDSCH could be reused for deferring HARQ-ACK to multicast SPS PDSCH, if deferred NACK-only based feedback is transformed to ACK/NACK based feedback. Alternatively, we could avoid deferring NACK-only based HARQ-ACK to multicast SPS PDSCH by invalidating simultaneous configuration of NACK-only based HARQ-ACK and SPS HARQ-ACK deferral for a SPS configuration.

Observation: In Rel-17, SPS HARQ-ACK deferral can be configured for a SPS configuration. According to ASN.1 structure in TS 38.331 v17.0.0 that sps-HARQ-Deferral-r17 can be configured for multicast SPS as well as unicast SPS.

Proposal 10: For ACK/NACK based HARQ-ACK, SPS HARQ-ACK deferral can be configured for a SPS configuration of multicast SPS. If configured, the rules of deferring HARQ-ACK to unicast SPS PDSCH are reused for deferring HARQ-ACK to multicast SPS PDSCH.

Proposal 11: For NACK-only based feedback to multicast SPS PDSCH,

- Alt 1: SPS HARQ-ACK deferral can be configured for multicast SPS as in 38.331. If configured, the rules of deferring HARQ-ACK to unicast SPS PDSCH are reused for deferring HARQ-ACK to multicast SPS PDSCH by transforming NACK-only based feedback to ACK/NACK based feedback.

- Alt 2: Clarify in 38.331 that SPS HARQ-ACK deferral cannot be configured for a SPS configuration configured with NACK-only based feedback.

Multicast HARQ-ACK with PUCCH Cell Switching

In our view, for PUCCH for ACK/NACK based feedback for multicast, if pucch-sSCellPattern is configured in Rel-17, the rules of PUCCH cell switching can be easily applied to PUCCH for ACK/NACK based feedback for multicast. Thus, we propose that PUCCH cell switching can be applicable for ACK/NACK based feedback for multicast.

Meanwhile, we wonder if PUCCH cell switching can also work with PUCCH for NACK-only based HARQ-ACK which can be shared by multiple UEs. For simplicity, we could consider that PUCCH cell switching is only applicable for ACK/NACK based feedback for multicast.

Proposal 12: For PUCCH for ACK/NACK based feedback for multicast, if pucch-sSCellPattern is configured, the rules of PUCCH cell switching is also applied to PUCCH for ACK/NACK based feedback for multicast

Proposal 13: For NACK-only based feedback for multicast, if pucch-sSCellPattern is configured,

- Alt 1: the rules of PUCCH cell switching is also applied to PUCCH for NACK-only based feedback for multicast

- Alt 2: the rules of PUCCH cell switching is not applied to PUCCH for NACK-only based feedback for multicast

Conclusion

In conclusion, we propose to agree the following TPs and proposals:

Proposal 1: Adopt TP-1 to clarify the previous agreement that Alt1 can support more than 4 HARQ-ACK information bits.

- Further clarify that no HARQ-ACK information is transmitted when the number of HARQ-ACK information bits is more than 4 in Alt4, as in TP-1.

TP-1 for TS 38.213 ----------------------------------------------------Text proposal starts---------------------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is more than one, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. When a number of HARQ-ACK information bits is more than four, the UE does not transmit the PUCCH that would include HARQ-ACK information unless the HARQ-ACK information is transmitted according to the first HARQ-ACK report mode. < Unchanged parts are omitted > ----------------------------------------------------Text proposal ends-----------------------------------------------

Proposal 2: Adopt TP-2 to clarify that the other UCI include neither the other NACK-only HARQ-ACK nor CG-UCI.

TP-2 for TS 38.213 ----------------------------------------------------Text proposal starts---------------------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with HARQ-ACK information according to the first HARQ-ACK reporting mode, HARQ-ACK information associated with unicast DCI formats, SR or CSI report(s) in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are ‘ACK’. < Unchanged parts are omitted > ----------------------------------------------------Text proposal ends-----------------------------------------------

Proposal 3: NACK-only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK for NACK-only based PUCCH overlapped with PUSCH.

Proposal 4: If the resultant NACK-only based PUCCH selected for multiplexing multiple NACK-only HARQ-ACK bits in Alt4 is overlapped with PUSCH, NACK-only based HARQ-ACKs are transformed to ACK/NACK based HARQ-ACK for multiplexing the NACK-only HARQ-ACK bits in the PUSCH.

Proposal 5: Adopt TP-3 to clarify that multiple NACK-only based HARQ-ACKs are also transformed to ACK/NACK based HARQ-ACK for NACK-only based PUCCH overlapped with PUSCH.

- NACK-only based PUCCH can correspond to either one HARQ-ACK bit or the resultant NACK-only based PUCCH selected for multiplexing multiple NACK-only HARQ-ACK bits in Alt4.

TP-3 for TS 38.213 ----------------------------------------------------Text proposal starts---------------------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with other UCI in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. If a PUCCH carrying HARQ-ACK information associated with multicast DCI formats according to the second HARQ-ACK reporting mode is overlapped with a PUSCH in a slot, the UE provides the HARQ-ACK information in the PUSCH according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are ‘ACK’. < Unchanged parts are omitted > ----------------------------------------------------Text proposal ends-----------------------------------------------

Proposal 6: Adopt TP-4 to clarify that UE does not transmit PUCCH carrying multicast HARQ-ACK overlapping with MSG3/MSGA PUSCH.

TP-4 for TS 38.213 ----------------------------------------------------Text proposal starts---------------------------------------------- # 18 Multicast Broadcast Services < Unchanged parts are omitted > If a PUCCH carrying a HARQ-ACK information associated with multicast DCI formats overlaps with MSG3 PUSCH or MSGA PUSCH in a slot, UE does not transmit the PUCCH. < Unchanged parts are omitted > ----------------------------------------------------Text proposal ends-----------------------------------------------

Proposal 7: For multiplexing a positive SR and two HARQ-ACK bits for unicast and multicast respectively of a same priority,

- For PUCCH format 0 for unicast HARQ-ACK,

  - Three bits are multiplexed on the PUCCH resource with PUCCH format 0 allocated for unicast HARQ-ACK according to the existing rules.

- For PUCCH format 1 for unicast HARQ-ACK and SR,

  - Two HARQ-ACK bits are multiplexed on the SR PUCCH resource according to the existing rules

- For PUCCH format 1 for unicast HARQ-ACK and PUCCH format 0 for SR

  - SR is dropped and Two HARQ-ACK bits are multiplexed on the PUCCH resource with PUCCH format 1 allocated for unicast HARQ-ACK according to the existing rules.

Proposal 8: For multiplexing HARQ-ACK information for unicast and multicast respectively and other UCIs of both LP and HP,

- HARQ-ACKs for unicast and HARQ-ACKs for multicast are multiplexed per each priority before applying R17 intra-UE multiplexing rules.

- Afterwards, R17 intra-UE multiplexing rules are applied to multiplexing PUCCH for HP and PUCCH for LP, assuming that the multiplexed unicast/multicast HARQ-ACKs of different priorities are considered HP/LP HARQ-ACKs respectively in the rules.

Proposal 9: When UE is scheduled to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide Type-3 codebook in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the Type-3 codebook that consists of HARQ-ACK bits (and NDI values if pdsch-HARQ-ACK-OneShotFeedbackNDI is configured) based on the existing rules in 9.1.4 of 38.213:

- Alt 1: the Type-3 codebook consists of HARQ-ACK bits to the last unicast PDSCHs and NDI values indicated by the last unicast DCI format for all HARQ processes.

- Alt 2: the Type-3 codebook consists of HARQ-ACK bits to the last PDSCHs and NDI values indicated by the last DCI formats for all HARQ processes (i.e. the last one can be either unicast or multicast).

Observation: In Rel-17, SPS HARQ-ACK deferral can be configured for a SPS configuration. According to ASN.1 structure in TS 38.331 v17.0.0 that sps-HARQ-Deferral-r17 can be configured for multicast SPS as well as unicast SPS.

Proposal 10: For ACK/NACK based HARQ-ACK, SPS HARQ-ACK deferral can be configured for a SPS configuration of multicast SPS. If configured, the rules of deferring HARQ-ACK to unicast SPS PDSCH are reused for deferring HARQ-ACK to multicast SPS PDSCH.

Proposal 11: For NACK-only based feedback to multicast SPS PDSCH,

- Alt 1: SPS HARQ-ACK deferral can be configured for multicast SPS as in 38.331. If configured, the rules of deferring HARQ-ACK to unicast SPS PDSCH are reused for deferring HARQ-ACK to multicast SPS PDSCH by transforming NACK-only based feedback to ACK/NACK based feedback.

- Alt 2: Clarify in 38.331 that SPS HARQ-ACK deferral cannot be configured for a SPS configuration configured with NACK-only based feedback.

Proposal 12: For PUCCH for ACK/NACK based feedback for multicast, if pucch-sSCellPattern is configured, the rules of PUCCH cell switching is also applied to PUCCH for ACK/NACK based feedback for multicast

Proposal 13: For NACK-only based feedback for multicast, if pucch-sSCellPattern is configured,

- Alt 1: the rules of PUCCH cell switching is also applied to PUCCH for NACK-only based feedback for multicast

- Alt 2: the rules of PUCCH cell switching is not applied to PUCCH for NACK-only based feedback for multicast