3GPP TSG RAN WG1 #109-e		                         R1-2204624

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.13

Source: 	LG Electronics

Title: 	Remaining issues on Cross-carrier scheduling from Scell to Pcell

Document for:	Discussion and decision

- Discussion

Agreement (RAN1#108-e) - When UE is configured for CCS from sSCell to P(S)SCell, and if SS set (x_p) of P(S)Cell and SS set (x_s) of sSCell are configured with same searchSpaceId value - x_s is used for CCS from sSCell to P(S)Cell (Note: already agreed) - x_s can be used for sSCell self-scheduling - x_p is not used for P(S)Cell self-scheduling and parameters other than searchSpaceId and nrofCandidates are not configured for that SS set - Note: RAN2 spec may need some update, but it depends on RAN2 decision.

In RAN1#108-e meeting, the rule to determine whether SS set of P(S)Cell is used for P(S)Cell self-scheduling or for cross-carrier scheduling from sSCell to P(S)Cell, was agreed. However, it should be clarified if the rule is applied to common SS set of P(S)Cell as well. In our view, a common SS set of P(S)Cell should be excluded from this rule and the common SS set shall be used only for self-scheduling. Therefore, we propose to clarify the earlier agreement as follows.

Proposal 1: The following agreement made in RAN1#108-e applies only for UE-specific SS set (x_p) of P(S)Cell. If a common SS set (x_p) of P(S)Cell and SS set (x_s) of sSCell are configured with same searchSpaceId value, then x_s is not used for CCS from sSCell to P(S)Cell and used for self-carrier scheduling.

Agreement (RAN1#108-e) - When UE is configured for CCS from sSCell to P(S)SCell, and if SS set (x_p) of P(S)Cell and SS set (x_s) of sSCell are configured with same searchSpaceId value - x_s is used for CCS from sSCell to P(S)Cell (Note: already agreed) - x_s can be used for sSCell self-scheduling - x_p is not used for P(S)Cell self-scheduling and parameters other than searchSpaceId and nrofCandidates are not configured for that SS set - Note: RAN2 spec may need some update, but it depends on RAN2 decision.

The following two agreements were made to align a non-fallback DCI format on the P(S)Cell with the corresponding DCI format on the sSCell for cross-carrier scheduling the P(S)Cell.

Agreement (RAN1#108-e) - If a DCI format on the P(S)Cell for self-scheduling the P(S)Cell includes X bits and the corresponding DCI format on the sSCell for cross-carrier scheduling the P(S)Cell includes Y bits, ¦X-Y¦ bits are padded to the DCI format with the smaller size Agreement (RAN1#107-e) Confirm the WA from RAN1#106bis-e with addition of below Note (shown in blue) Working Assumption - When CIF for sSCell to Pcell cross-carrier scheduling is configured, non-fallback DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling - Note: per RAN1#102-e agreement, when sSCell to P(S)Cell scheduling is configured for the UE, cross-carrier scheduling from P(S)Cell to another cell is not allowed. The CIF bits included in non-fallback DCI formats on P(S)Cell are considered reserved.

It can be observed that the agreement in RAN1#108-e has a generalized form to match two DCI formats while the agreement in RAN1#107-e focuses only on a single field (i.e., CIF) specifically. This may lead to unnecessarily increased DCI payload size, which degrades PDCCH decoding performance. Therefore, the agreement in RAN1#108-e should supersede the agreement in RAN1#107-e and this also simplifies the current specification.

Proposal 2: Adopt the following TP in TS 38.212.

##### 7.3.1.1.2 Format 0_1 DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE. The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Carrier indicator – 0 or 3 bits, as defined in Clause 10.1 of [5, TS38.213]. <Unchanged parts are omitted> ##### 7.3.1.1.3 Format 0_2 DCI format 0_2 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Carrier indicator – 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeDCI-0-2, as defined in Clause 10.1 of [5, TS38.213]. <Unchanged parts are omitted> ##### 7.3.1.2.2 Format 1_1 DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell. The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Carrier indicator – 0 or 3 bits as defined in Clause 10.1 of [5, TS 38.213]. <Unchanged parts are omitted> ##### 7.3.1.2.3 Format 1_2 DCI format 1_2 is used for the scheduling of PDSCH in one cell. The following information is transmitted by means of the DCI format 1_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format. - Carrier indicator – 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeDCI-1-2, as defined in Clause 10.1 of [5, TS38.213].

Next, regarding the aspects of PDCCH monitoring and BD/CCE limit handling, [based on Option C] was adopted in RAN1#106bis-e. One of remaining issues is whether to support other s1/s2 values than agreed ones (i.e., s1=1, s2=0). It would be beneficial to introduce other s1/s2 values (e.g., s1=0.5, s2=0.5), in terms of PDCCH offloading from P(S)Cell to sSCell.

Proposal 3: For BD/CCE limit handling at least for Type B UE, introduce additional s1 and s2 values (e.g., s1=0.5, s2=0.5) based on RRC configuration.

Furthermore, it may need to consider the case when the sSCell configured as scheduling cell for P(S)Cell can be deactivated (or dormant). UE is supposed to stop any PDCCH monitoring on a deactivated SCell. However, if the deactivated SCell was configured for cross-carrier scheduling from sSCell to P(S)Cell and PDCCH monitoring on the sSCell was turned off, gNB may not have sufficient resource for PDCCH scheduling PDSCH/PUSCH on P(S)Cell. To figure out this issue, the following three options can be considered and Option 1 is preferred since it doesn’t require additional signalling.

- Option 1: Fallback to P(S)Cell self-carrier scheduling as if cross-carrier scheduling from sSCell to P(S)Cell is not configured

- Option 2: New sSCell indication via DCI or MAC CE

- Option 3: Separate USS set configurations for each of activated sSCell case and deactivated sSCell case

Proposal 4: After cross-carrier scheduling from sSCell to P(S)Cell is configured, if the sSCell is deactivated or in dormant state, the following options can be considered to provide sufficient PDCCH resource and Option 1 is preferred since it doesn’t require additional signalling.

- Option 1: Fallback to P(S)Cell self-carrier scheduling as if cross-carrier scheduling from sSCell to P(S)Cell is not configured

- Option 2: New sSCell indication via DCI or MAC CE

- Option 3: Separate USS set configurations for each of activated sSCell case and deactivated sSCell case

- Conclusion

In this contribution, we discussed and provided our views on the support of cross-carrier scheduling from sSCell to P(S)Cell, and the proposals are as follows.

Proposal 1: The following agreement made in RAN1#108-e applies only for UE-specific SS set (x_p) of P(S)Cell. If a common SS set (x_p) of P(S)Cell and SS set (x_s) of sSCell are configured with same searchSpaceId value, then x_s is not used for CCS from sSCell to P(S)Cell and used for self-carrier scheduling.

Agreement (RAN1#108-e) - When UE is configured for CCS from sSCell to P(S)SCell, and if SS set (x_p) of P(S)Cell and SS set (x_s) of sSCell are configured with same searchSpaceId value - x_s is used for CCS from sSCell to P(S)Cell (Note: already agreed) - x_s can be used for sSCell self-scheduling - x_p is not used for P(S)Cell self-scheduling and parameters other than searchSpaceId and nrofCandidates are not configured for that SS set - Note: RAN2 spec may need some update, but it depends on RAN2 decision.

Proposal 2: Adopt the following TP in TS 38.212.

##### 7.3.1.1.2 Format 0_1 DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE. The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Carrier indicator – 0 or 3 bits, as defined in Clause 10.1 of [5, TS38.213]. <Unchanged parts are omitted> ##### 7.3.1.1.3 Format 0_2 DCI format 0_2 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Carrier indicator – 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeDCI-0-2, as defined in Clause 10.1 of [5, TS38.213]. <Unchanged parts are omitted> ##### 7.3.1.2.2 Format 1_1 DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell. The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Carrier indicator – 0 or 3 bits as defined in Clause 10.1 of [5, TS 38.213]. <Unchanged parts are omitted> ##### 7.3.1.2.3 Format 1_2 DCI format 1_2 is used for the scheduling of PDSCH in one cell. The following information is transmitted by means of the DCI format 1_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format. - Carrier indicator – 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeDCI-1-2, as defined in Clause 10.1 of [5, TS38.213].

Proposal 3: For BD/CCE limit handling at least for Type B UE, introduce additional s1 and s2 values (e.g., s1=0.5, s2=0.5) based on RRC configuration.

Proposal 4: After cross-carrier scheduling from sSCell to P(S)Cell is configured, if the sSCell is deactivated or in dormant state, the following options can be considered to provide sufficient PDCCH resource and Option 1 is preferred since it doesn’t require additional signalling.

- Option 1: Fallback to P(S)Cell self-carrier scheduling as if cross-carrier scheduling from sSCell to P(S)Cell is not configured

- Option 2: New sSCell indication via DCI or MAC CE

- Option 3: Separate USS set configurations for each of activated sSCell case and deactivated sSCell case