3GPP TSG RAN WG1 #109-e                                 R1-2204628

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.7.1

Source: 	LG Electronics

Title: 	Discussion on performance evaluation for network energy savings

Document for:	Discussion and decision

- Introduction

The following SID objectives related to the network energy saving for NR were approved in RAN#94-e meeting [1]:

The objectives of the study are the following: - Definition of a base station energy consumption model [RAN1] - Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations. - Definition of an evaluation methodology and KPIs [RAN1] - The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. Note: WGs will decide KPIs to evaluate and how. - Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed. The following example scenarios (mapping between scenarios and network loads is left to the study) including single-carrier and multi-carrier deployments are used as the starting point for discussion on prioritized scenarios for the study. The following example scenarios are listed in no particular order. - Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells) - FR2 beam-based scenarios (note: this scenario can also model small cells) - Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS) - EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments. Note 2: the study of energy savings specifically for IAB is not part of the scope. The study should coordinate with RAN4 as needed.

In this contribution, we discuss and provide views on the gNB power consumption model and KPI for network energy savings (NES).

- Discussion

- gNB power consumption model

gNB power model which supports the multiple sleep modes based on sub-component deactivation is proposed in [2]. The presented power model provides the power consumption values for sleep modes 1 to 4 for various types of gNB as well as the transition time between the sleep modes. gNB’s hardware remains active even when there is no loaded data (i.e., in an idle state), the partial deactivation of gNB would additionally reduce the power consumption. Therefore, as defined in UE power saving study in [3], it is possible to consider introducing multiple sleep modes for NES. In addition, the framework of the UE power consumption evaluation in [3] such as the decomposing power state for data transmission and RS transmission, the reference configuration, the scaling for adaptation, and the power consumption during the state transition can be reused.

Proposal #1: Introducing multiple sleep modes can be considered for gNB power consumption model similar to the UE power saving model in TR 38.840, and the framework of the UE power consumption evaluation in TR 38.840 can be used as the starting point for that of gNB power consumption evaluation.

Based on the power model in [2], four sleep modes for gNB, the characteristics for each sleep mode, and relative power consumption are presented in Table 1. gNB is assumed as 4x4 macro base station which radiates 49 dBm transmit power with a bandwidth of 100 MHz. The relative power given in Table 1 is a value obtained by normalizing based on the power consumption value of Sleep mode 4 of Table V in [2]. As gNB enters a deeper sleep mode, the relative power decreases while latency (i.e., a large activation/deactivation time) increases.

Meanwhile, gNB power state machine and the additional assumption of [4] can be considered to implement the proposed power model to perform system-level simulation. The assumptions for each power state in [4] can be summarized as follows: Active mode and Sleep mode 1 can be considered as normal operation, so gNB is able to transmit and receive signals/channels without additional delay. Some of sub-components (e.g., power amplifier) of gNB can be deactivated during Sleep mode 1. Sleep modes 2 to 4 are the power saving modes with different power consumption and deactivation/activation times. In these modes, gNB doesn’t transmit or receive anything but reacts to incoming DL user traffic by waking up. In addition, it also periodically wakes up to transmit synchronization signals (SS) and to listen on random access channel (RACH).

Proposal #2: At least the relative power consumed in each of sleep modes to be defined for gNB and the power consumed in the active state for PDCCH+PDSCH and RS transmissions can be included for gNB power consumption model.

Reference configuration for FR1

- Base station type: 4x4 MIMO macro

- Number of carrier: 1 CC

- System bandwidth: 100 MHz

- TX power: 49 dBm

Table 1. gNB power consumption model for FR1

| Power state |  | Characteristics | Relative power |
| --- | --- | --- | --- |
| Sleep | Sleep mode 1 | Immediate transition is assumed for power saving study purpose from or to a non-sleep state. | [14] |
|  | Sleep mode 2 | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [2] |
|  | Sleep mode 3 | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [1.2] |
|  | Sleep mode 4 | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [1] |
| Active | RS transmission | SSB or CSI-RS/TRS transmission | [120] |
|  | PDCCH + PDSCH | PDCCH+PDSCH transmission | [300] |
| IDLE |  | There is no loaded data but gNB’s hardware remains active. | [22] |

Proposal #3: The following reference configuration and gNB power consumption model in Table 1 can be considered for FR1.

Reference configuration for FR1

- Base station type: 4x4 MIMO macro

- Number of carrier: 1 CC

- System bandwidth: 100 MHz

- TX power: 49 dBm

Table 1. gNB power consumption model for FR1

| Power state |  | Characteristics | Relative power |
| --- | --- | --- | --- |
| Sleep | Sleep mode 1 | Immediate transition is assumed for power saving study purpose from or to a non-sleep state. | [14] |
|  | Sleep mode 2 | A sleep state with lower power consumption than Sleep mode 1 and time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [2] |
|  | Sleep mode 3 | A sleep state with lower power consumption than Sleep mode 2 and time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [1.2] |
|  | Sleep mode 4 | A sleep state with lower power consumption than Sleep mode 3 and time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [1] |
| Active | RS transmission | SSB or CSI-RS/TRS transmission | [120] |
|  | PDCCH + PDSCH | PDCCH+PDSCH transmission | [300] |
| IDLE |  | There is no loaded data but gNB’s hardware remains active. | [22] |

For each sleep mode, the transition time and additional transition energy are required during the state transition as shown in Table 2. Four sleep modes are defined in the model [2], corresponding to a total transition time (deactivation plus reactivation) of 71 μs (1 OFDM symbol for 15 kHz SCS), 1 ms (sub-frame), 10 ms (frame) and 1 s (long-term sleep). However, since the transition time to Sleep mode 1 is small enough to ignore, immediate transition can be assumed for power saving study purpose. In Table 2, it is assumed that the relative transition power is calculated as the arithmetic average of the relative power values of the source sleep mode and the target sleep mode.

Besides, since gNB should transit to active mode periodically in order to transmit the periodic signal such as SSB, some sleep modes may not be able to be reached depending on the periodicity of SSB. Therefore, among the four sleep modes, it is needed to determine how many sleep modes should be considered for gNB power consumption model. For example, since Sleep mode 4 cannot be reached at any simulated SSB periodicity according to simulation results in [4], only sleep modes 1 to 3 can be considered for gNB power states.

Table 2. gNB power consumption during the state transition

| Sleep mode | Transition time | Relative transition power | Additional transition energy (Relative power x ms) |
| --- | --- | --- | --- |
| Sleep mode 1 | 0 ms | 18 | 0 |
| Sleep mode 2 | 1 ms | 8 | 8 |
| Sleep mode 3 | 10 ms | 1.6 | 16 |
| Sleep mode 4 | 1000 ms | 1.1 | 1100 |

Proposal #4: In addition to the relative power for each sleep mode, the transition time and additional transition energy should be considered and Table 2 can be considered if four sleep modes are considered. FFS: how many sleep modes should be considered for gNB power consumption model.

Table 2. gNB power consumption during the state transition

| Sleep mode | Transition time | Relative transition power | Additional transition energy (Relative power x ms) |
| --- | --- | --- | --- |
| Sleep mode 1 | 0 ms | 18 | 0 |
| Sleep mode 2 | 1 ms | 8 | 8 |
| Sleep mode 3 | 10 ms | 1.6 | 16 |
| Sleep mode 4 | 1000 ms | 1.1 | 1100 |

In the above gNB power consumption model, the relative power consumption of each operation is based on the reference configuration and reference base station type. The relative power consumption value for the different configurations such as resource utilization, number of carriers, and number of antennas can be obtained by scaling on the values of the reference configuration.

Table 3 shows the antenna scaling formula of UE power consumption scaling for adaptation in TR 38.840 [3], and it is necessary to discuss whether these antenna scaling formulas can be reused for gNB’s antenna. It can be reused if the PA (power amplifier) structure of gNB is implemented per antenna element but otherwise, the different formula may be required depending on the implemented PA structure of gNB. Therefore, in scaling the number of antennas compared to the reference configuration, it is necessary to clarify whether the implementation of gNB PA per antenna element can be assumed.

Table 3: UE power consumption scaling for adaptation

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| Antenna scaling (DL) | 2Rx power is 0.7x 4Rx power for FR1 1Rx power is 0.7x 2Rx power for FR2 | Assume same number of antenna elements per Rx chain |
| Antenna scaling (UL) | 2Tx power is 1.4x 1Tx power at 0dBm. 1.2x.at 23dBm FR1 only | 2Tx support is not considered for FR2. |

Proposal #5: Discuss whether the formula for linear scaling with the number of antenna elements can be applied to gNB power consumption model, by clarifying how gNB implementation for power amplifier can be assumed for evaluation purpose.

- KPI

In RAN#95-e meeting, the LS on energy efficiency (EE) was sent from SA to RAN [5]. It requested the recipients including RAN to consider EE as a guiding principle when developing new solutions and evolving the 3GPP systems specification. With this regard, EE should be also included as one of KPIs in evaluation methodology for NES.

Proposal #6: Energy efficiency should be included as one of KPIs in evaluation methodology for network energy savings.

The definition of EE from TR 38.913 Clause 7.19 [8] is as follows:

EEglobal=scenario KbKEEscenario K

- where bK refers to the weights of every deployment scenario where the network energy efficiency is evaluated and,

- EEScenario=load level 1a1V1EC1

  - where V1= Refers to the traffic per second served by a base station (in bits/s),

  - EC1 = Refers to the power consumed by a base station to serve V1 (in Watt = Joule/s), and

  - a1 = Refers to the weight for each traffic load level.

According to the above equation captured from TR 38.913 [8], the absolute power value is required to calculate EE. However, according to the objective in [1] or the UE power saving model in [3], the relative power can be used to define gNB power consumption model. Therefore, the definition of EE may need to be modified so that it can also be calculated based on a relative power value. A simple solution could be to replace the absolute power in EE formula with the relative power (i.e., new EE = ratio of throughput to reference power consumed by gNB). Given the relationship between absolute power and relative power, the original EE value calculated by the absolute power can be derived from the new EE value calculated by the relative power. Therefore, the EE value calculated by relative power can also be considered as a valuable KPI for evaluation methodology.

Proposal #7: New energy efficiency is defined as the ratio of throughput to reference power consumed by gNB, and the new energy efficiency is considered as one of KPIs for study on network energy savings.

In addition, we need to consider the trade-off relationship between the performance gain of gNB and the performance loss of UE. To be specific, gNB’s performance gain and UE’s performance loss compared to the reference scenario (i.e. scenario without NES techniques) can be reported at the same time. Moreover, a lower bound for UE’s performance loss can be defined to guarantee that UE’s performance cannot be degraded under the lower bound.

Proposal #8: Considering the trade-off between the performance gain of gNB and the performance loss of UE, both of gNB’s performance gain and UE’s performance loss compared to the reference scenario (i.e. scenarios without network energy saving techniques) can be reported.

- Conclusions

In this contribution, the definition of gNB power consumption model and KPI to study on network energy saving for NR was discussed, and the followings are proposed.

Proposal #1: Introducing multiple sleep modes can be considered for gNB power consumption model similar to the UE power saving model in TR 38.840, and the framework of the UE power consumption evaluation in TR 38.840 can be used as the starting point for that of gNB power consumption evaluation.

Proposal #2: At least the relative power consumed in each of sleep modes to be defined for gNB and the power consumed in the active state for PDCCH+PDSCH and RS transmissions can be included for gNB power consumption model.

Proposal #3: The following reference configuration and gNB power consumption model in Table 1 can be considered for FR1.

Reference configuration for FR1

- Base station type: 4x4 MIMO macro

- Number of carrier: 1 CC

- System bandwidth: 100 MHz

- TX power: 49 dBm

Table 1. gNB power consumption model for FR1

| Power state |  | Characteristics | Relative power |
| --- | --- | --- | --- |
| Sleep | Sleep mode 1 | Immediate transition is assumed for power saving study purpose from or to a non-sleep state. | [14] |
|  | Sleep mode 2 | A sleep state with lower power consumption than Sleep mode 1 and time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [2] |
|  | Sleep mode 3 | A sleep state with lower power consumption than Sleep mode 2 and time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [1.2] |
|  | Sleep mode 4 | A sleep state with lower power consumption than Sleep mode 3 and time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [1] |
| Active | RS transmission | SSB or CSI-RS/TRS transmission | [120] |
|  | PDCCH + PDSCH | PDCCH+PDSCH transmission | [300] |
| IDLE |  | There is no loaded data but gNB’s hardware remains active. | [22] |

Proposal #4: In addition to the relative power for each sleep mode, the transition time and additional transition energy should be considered and Table 2 can be considered if four sleep modes are considered. FFS: how many sleep modes should be considered for gNB power consumption model.

Table 2. gNB power consumption during the state transition

| Sleep mode | Transition time | Relative transition power | Additional transition energy (Relative power x ms) |
| --- | --- | --- | --- |
| Sleep mode 1 | 0 ms | 18 | 0 |
| Sleep mode 2 | 1 ms | 8 | 8 |
| Sleep mode 3 | 10 ms | 1.6 | 16 |
| Sleep mode 4 | 1000 ms | 1.1 | 1100 |

Proposal #5: Discuss whether the formula for linear scaling with the number of antenna elements can be applied to gNB power consumption model, by clarifying how gNB implementation for power amplifier can be assumed for evaluation purpose.

Proposal #6: Energy efficiency should be included as one of KPIs in evaluation methodology for network energy savings.

Proposal #7: New energy efficiency is defined as the ratio of throughput to reference power consumed by gNB, and the new energy efficiency is considered as one of KPIs for study on network energy savings.

Proposal #8: Considering the trade-off between the performance gain of gNB and the performance loss of UE, both of gNB’s performance gain and UE’s performance loss compared to the reference scenario (i.e. scenarios without network energy saving techniques) can be reported.

- References

- RP-213554, “Study on network energy savings for NR”, RAN#94-e, Huawei.

- B. Debaillie, C. Desset, and F. Louagie, “A flexible and future-proof power model for cellular base stations,” IEEE Vehicular Technology Conference, May 2015.

- 3GPP TR 38.840 V16.0.0 (2019-06), Study on User Equipment (UE) power saving in NR, Release 16

- P. Lähdekorpi, M. Hronec, P. Jolma and J. Moilanen, “Energy efficiency of 5G mobile networks with base station sleep modes,” 2017 IEEE Conference on Standards for Communications and Networking (CSCN), 2017

- RP-220025, “LS on Energy Efficiency as guiding principle for new solutions,” RAN#95-e, TSG SA.

- RP-220036, “Reply LS on energy efficiency as guiding principle for new solutions,” RAN#95-e, 3GPP SA5.

- RP-220037, “LS on 3GPP SA5 work on 5G network energy efficiency and energy saving,” RAN#95-e, 3GPP SA5

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”.