3GPP TSG RAN WG1 #109-e			R1-2204629

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.7.2

Source: 	LG Electronics

Title: 	Discussion on physical layer techniques for network energy savings

Document for:	Discussion and decision

- Introduction

In RAN#94-e, a new SI for study on network energy savings in Rel-18 was approved with following objective in SID [1].

- Definition of a base station energy consumption model [RAN1] - Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations. - Definition of an evaluation methodology and KPIs [RAN1] - The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. Note: WGs will decide KPIs to evaluate and how. - Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

In this contribution, we discuss and provide our views on physical layer techniques for network energy savings (NES) in spatial, time, frequency, and power domains.

- Physical layer techniques for NES

  - NES techniques in spatial domain

As MIMO technology evolves, a gNB can typically be equipped with dozens of antenna elements (or even more than 100) at least for massive MIMO in TDD or mmWave bands. Therefore, it is obvious that when the traffic load is medium or low, gNB can reduce power consumption by switching off some of deployed antenna elements.

Proposal #1: It is beneficial to dynamically adjust the number of gNB’s activated antenna elements, in terms of network energy savings.

In order to support the dynamic change of the number of gNB’s transmit antenna elements based on the traffic load, it seems necessary to study how UE can recognize the change and behave appropriately depending on gNB’s antenna on/off status. To be specific, gNB may inform a UE that some of antenna ports (or TRPs) configured to the UE will be turned off to improve energy saving gain (e.g., via (group-common) DCI or MAC CE). For instance, UE can be indicated to stop receiving a CSI-RS resource (set) with 32 antenna ports while start receiving another CSI-RS resource (set) with 16 antenna ports, or to stop receiving 16 antenna ports configured for the CSI-RS resource (set), for which CSI enhancement can be required (e.g., how to construct MIMO codebook). For this case, it can be also considered how to address beam management issue when UE’s current reception beam is no longer suitable after gNB changes the number of activated transmit antenna elements.

Proposal #2: Study how to efficiently support changing the number of gNB’s transmit antenna elements (e.g., by deactivating a NZP CSI-RS with 32 antenna ports while activating another NZP CSI-RS with 16 antenna ports, or turning off 16 antenna ports out of 32 antenna ports configured for the NZP CSI-RS).

In addition, it can be considered to adjust the number of gNB’s receive antenna elements. For this case, similar to CSI-RS, gNB can control SRS resource (set) or SRS antenna port to be turned on or off depending on which of gNB’s receive antenna elements are activated. However, we should first discuss the necessity on UL signal/channel enhancement given that the energy saving gains from reducing the number of receive antennas are expected to be relatively small compared to reducing the number of transmit antennas.

Proposal #3: Discuss whether any enhancements for UL signal/channel (e.g., SRS) transmission are needed depending on the number of gNB’s receive antenna elements.

  - NES techniques in time domain

Even if gNB has no data to transmit, gNB’s power may be consumed to transmit (also to receive) periodic/semi-persistent signals. Therefore, temporarily turning off periodic/semi-persistent signals can help gNB reduce power consumption, at least when gNB does not need to transmit data to the UE.

To support a mechanism to turn off gNB’s transmission for a certain period, gNB can simply indicate off-period (e.g., via (group-common) DCI or MAC CE) during which periodic/semi-persistent RRC-configured DL signals will not be transmitted. Alternatively, the periodicity of periodic/semi-persistent signal can be modified with a dynamic indication method.

Proposal #4: It is beneficial to switch off gNB’s periodic/semi-persistent transmission (and/or reception) at least when gNB does not need to transmit data to the UE, in terms of network energy savings.

Proposal #5: Study how to support efficient mechanisms to switch off gNB’s transmission (and/or reception) for a specific period of time.

On the other hand, if the off-period or gNB’s power save mode lasts long, the performance of UE can be degraded due to the effect of increased latency. To alleviate this latency increase issue, UE can be allowed to transmit UL signal/channel (e.g., SR) to wake up gNB or to inform the updated buffer status e.g., within a time domain window. Considering that gNB needs to transmit at least SSB and TRS periodically, the time domain window configured for UE to inform UE’s data arrival can be configured close to the location of SSB or TRS. In addition, it could be useful for UE to report zero-buffer status to aid gNB’s decision on whether to go into power save mode or not.

Proposal #6: Study how to support a mechanism for waking gNB from power save mode when new data arrives at UE.

  - NES techniques in frequency domain

One of the main motivations for NES is that gNB must occupy a lot of frequency domain resources for high peak data rate, but most of them are useless when the traffic load is quite low or empty. However, it seems that there are several legacy NR mechanisms (e.g., SCell (de)activation, BWP switching, or SCell dormancy indication) that can be reused for NES techniques in frequency domain.

Observation: Legacy mechanisms such as SCell (de)activation, BWP switching, and SCell dormancy indication, can be reused for the purpose of network energy savings in frequency domain.

Nevertheless, we can consider enhancement schemes for more efficient deactivation of frequency domain resources. When gNB goes into power save mode and deactivates a serving cell (or a BWP) for all of UEs associated with the serving cell (or the BWP), currently gNB has to transmit SCell deactivation (or BWP switching) message to each UE individually. Therefore, it would be beneficial to use group-common DCI (or MAC CE) for indicating SCell (de)activation or BWP switching. In addition, considering BWP switching delay and limited number of BWPs that can be configured per a serving cell, it can be taken into account to reduce gNB’s power consumption by dynamically adjusting the bandwidth of a given BWP instead of BWP switching (e.g., by turning on/off RB set(s) configured as in RB-Set-Configuration).

Proposal #7: Consider to enhance indication methods for deactivating frequency domain resources (e.g., SCell (de)activation or BWP switching via group-common DCI or MAC CE) or for adjusting the bandwidth of a given BWP.

  - NES techniques in power domain

gNB may reduce power consumption by controlling transmission power of SSB, NZP CSI-RS, or PDSCH/DM-RS. As to PDSCH/DM-RS (and potentially PT-RS), no additional UE behaviour is foreseen even if gNB changes transmit power of those DL signals/channels since UE performs channel estimation and decodes PDSCH data based on UE-specific reference signals.

However, as to SSB and NZP CSI-RS, if gNB dynamically changes their transmission power, it seems necessary to inform UE changed transmit power values with a new signaling method. It is noted that according to current NR specification, SSB power is configured as ss-PBCH-BlockPower parameter and NZP CSI-RS power is configured with powerControlOffsetSS parameter per CSI-RS resource as the offset from SSB power. Furthermore, at least following impacts need to be investigated if it is introduced to dynamically change transmit power for SSB or NZP CSI-RS.

- Update of path-loss estimation

- Cell identification/coverage

- RRM/CSI measurement

Proposal #8: Investigate impacts of power adaptation for SSB and/or NZP CSI-RS if transmit power for SSB and/or NZP CSI-RS can be dynamically changed.

- Conclusions

In this contribution, physical layer techniques for network energy savings were discussed, and the followings were proposed.

NES techniques in spatial domain

Proposal #1: It is beneficial to dynamically adjust the number of gNB’s activated antenna elements, in terms of network energy savings.

Proposal #2: Study how to efficiently support changing the number of gNB’s transmit antenna elements (e.g., by deactivating a NZP CSI-RS with 32 antenna ports while activating another NZP CSI-RS with 16 antenna ports, or turning off 16 antenna ports out of 32 antenna ports configured for the NZP CSI-RS).

Proposal #3: Discuss whether any enhancements for UL signal/channel (e.g., SRS) transmission are needed depending on the number of gNB’s receive antenna elements.

NES techniques in time domain

Proposal #4: It is beneficial to switch off gNB’s periodic/semi-persistent transmission (and/or reception) at least when gNB does not need to transmit data to the UE, in terms of network energy savings.

Proposal #5: Study how to support efficient mechanisms to switch off gNB’s transmission (and/or reception) for a specific period of time.

Proposal #6: Study how to support a mechanism for waking gNB from power save mode when new data arrives at UE.

NES techniques in frequency domain

Observation: Legacy mechanisms such as SCell (de)activation, BWP switching, and SCell dormancy indication, can be reused for the purpose of network energy savings in frequency domain.

Proposal #7: Consider to enhance indication methods for deactivating frequency domain resources (e.g., SCell (de)activation or BWP switching via group-common DCI or MAC CE) or for adjusting the bandwidth of a given BWP.

NES techniques in power domain

Proposal #8: Investigate impacts of power adaptation for SSB and/or NZP CSI-RS if transmit power for SSB and/or NZP CSI-RS can be dynamically changed.

- References

RP-213554, “New SI: Study on network energy savings for NR,” Huawei, RAN#94-e, Dec. 2021