3GPP TSG RAN WG1 Meeting #109-e			R1-2204633

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.11.1

Source: 	LG Electronics

Title: 	Discussion on XR-specific power saving techniques

Document for:	Discussion and decision

- Introduction

The Re1-18 SI titled “Study on XR Enhancements for NR” was approved in RAN#94-e meeting [1]. The study is mainly on the techniques for enhancements for XR from RAN1 perspective. The SI objectives are copied below for reference from the latest version of the SID [2].

## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

- Discussion

In this contribution, we present our views on the potential enhancements of power saving techniques for XR.

  - Characteristics of XR traffic relevant for XR-specific PS enhancements

In this section, we briefly discuss the XR traffic characteristics that are relevant for XR-specific power saving enhancements. As captured in the SID objectives, the following service characteristics needs to be taken into account for XR-specific PS enhancements.

- Periodicity (non-integer)

- Jitter

- Latency

- Reliability

- Multiple flows

The first four are considered as generic XR traffic characteristics that apply to a single stream/flow as well as to each of the streams if an XR service consists of multiple streams/flows. For the fifth, the evaluation of the multiple flows has been optional in the Rel-18 XR SI [3], but we agree that the multiple flows are very XR-specific and should be taken into account for XR-specific enhancements for both power saving and capacity.

Meanwhile, we think the high throughput and capacity required for typical XR services should also be taken into account for XR-specific enhancements. For most of the pre-Rel-18 PS techniques, sacrifice has been unavoidable to a great extent from this aspect, but not sacrificing that much is one of major goals for XR-specific power saving as we understand it.

Observation 1: High throughput and capacity should also be taken into account for XR-specific PS enhancements.

  - Pre-Rel-18 NR PS techniques

In this section, we briefly summarize the NR PS techniques that have been developed up to and including Rel-17 NR PS techniques. The following techniques can be a starting point for XR-specific power saving discussion.

- DRX

A UE configured with DRX monitors PDCCH intermittently. In idle mode DRX, a UE periodically wakes up to monitor paging PDCCH and goes back to sleep if paging message is not intended for the UE. In connected mode DRX (CDRX), a UE monitors PDCCH only in DRX Active Time. A UE enters DRX Active Time by starting drx-onDurationTimer periodically and goes back to sleep when it expires. A UE can save the power consumption by turning off the RF chains and going to sleep not to monitor PDCCH. Outside DRX Active Time, a UE sleeps and monitors DCI format 2_6 with CRC scrambled by PS-RNTI only at occasions located at a configured offset before the start of DRX OnDuration. Wake-up signal (WUS) tells a UE whether or not to start drx-onDurationTimer for the next DRX cycle.

- Cross-slot scheduling

Cross-slot scheduling means that a PDCCH received by a UE and the PDSCH/PUSCH scheduled by the PDCCH can be in different slots. In Rel-15, multiple K0 (K2) are configured by RRC, and a UE can know which value is used for scheduling by decoding a DCI. If RRC configured K0 (K2) values are greater than 0, a PDSCH/PUSCH is always cross-slot scheduled for the UE. In Rel-16, the minimum scheduling offset restriction K0min (K2min) is introduced. Consequently, a UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH (PUSCH) with K0 (K2) smaller than the minimum scheduling offset restriction K0min (K2min). A UE can decrease power consumption by fast decoding and sleep during at least K0min (K2min) slots, or relaxed PDCCH decoding over K0min (K2min) slots.

- Max MIMO layer adaptation (by BWP switching)

In Rel-15, the maximum number of DL MIMO layers is configured per serving cell which is common to all the DL BWPs of the serving cell. In Rel-16, the maximum number of DL MIMO layers can be separately configured for each DL BWP. The maximum number of DL MIMO layers can be changed by BWP switching. A UE can reduce power consumption by adaptation to less number of Rx antenna at UE side.

- SCell dormancy indication

SCell can be transitioned to a ‘dormant’ state where a UE does not monitor a PDCCH. In Rel-15, gNB can transition SCell to a dormant state by MAC signaling. In Rel-16, the dormancy behavior can be implemented at BWP level. DCI format 2_6 outside DRX Active Time conveys SCell dormancy indication and scheduling DCI (i.e. DCI format 0_1 and 1_1) conveys SCell dormancy indication inside DRX Active Time.

- PDCCH monitoring adaptation for connected mode

In Rel-17, PDCCH monitoring adaptation during DRX Active Time for an active BWP was specified. One of the motivation is reducing PDCCH monitoring of a UE by DCI indication. It was agreed that SS set group (SSSG) switching and PDCCH skipping are introduced. Scheduling DCI carries PDCCH monitoring adaptation indication field of at most 2 bits.

Rel-17 SSSG switching for power saving is an enhancement of Rel-16 SSSG switching introduced for NR operation with shared spectrum channel access (i.e. NR-U). A UE can be provided a group index for SS sets for PDCCH monitoring and a timer value. At most three groups per BWP can be configured. Each of SSSG may be configured with different monitoring periodicities, for example, a SSSG for dense monitoring and another SSSG for sparse monitoring. UE’s PDCCH monitoring can be adapted by switching SSSGs.

PDCCH monitoring skipping means a UE stops monitoring PDCCH and sleeps for an indicated time duration. In this way, a gNB can indicate that the UE may not monitor PDCCH for the indicated time duration during which no data traffic will be scheduled for the UE.

  - CDRX enhancements

Based on Rel-17 XR study, XR DL traffic is modelled as a sequence of video frames arriving at gNB according to the considered video frame rates and random jitter. The size of each frame is also random according to a certain distribution.

Figure 1: Single stream DL traffic model

In this model, the packet arrival rate is determined by the frame generation rate, e.g., 60fps. Accordingly, the average packet arrival periodicity is given by the inverse of the frame rate, e.g., 16.6667ms = 1/60fps.

As the traffic is periodic, CDRX may be configured for UE power saving. But, with the current CDRX mechanism from which the cycle can only be configured with some integer values, there would be increasing discrepancy b/w the timing of the gNB being ready to transmit the IP packets corresponding to a frame and the time at which the drx-onDurationTimer starts. This increasing gap would cause frequent reconfigurations of the CDRX and/or increase the overall latency.

Observation 2: Upon use for XR service, current CDRX mechanism would cause frequent reconfigurations of the CDRX and/or increase the overall latency.

Enhancements on the CDRX to tackle the non-integer periodicity of XR traffic can be discussed during the Rel-18 SI phase for XR enhancements. To the best of our knowledge, adjustment of the DRX OnDuration start time or the Active Time is a relevant technique that can be pursued toward this aspect. For the adjustment, either predefined rules for sporadic adjustment (e.g., every X ms), or dynamic adjustment using L1/L2 signaling can be considered, but details can be further discussed throughout the SI phase.

Proposal 1: Study PS enhancement techniques to tackle the non-integer periodicity of XR traffic.

On top of the periodic nature of the XR traffic arrival time, in a real system, there is a jitter in the arrival time for the packets arriving at the gNB. The jitter mainly comes from the varying frame encoding delay and network transfer time. In [3], distribution of the jitter was modelled as truncated Gaussian with the following statistical parameters.

Table 1: Statistical parameters for jitter

| Parameter | unit | Baseline value for evaluation | Optional value for evaluation |
| --- | --- | --- | --- |
| Mean | ms | 0 |  |
| STD | ms | 2 |  |
| Truncation range | ms | [-4, 4] | [-5, 5] |

Moving around the DRX OnDuration or Active Time per se cannot handle the jitter efficiently if it is a reasonable assumption that gNB cannot predict the jitter. Instead, a simple approach to cope with the jitter problem would be to extend OnDuration time or the Active Time. To minimize the additional power consumption with this approach, the extension may need to be conditioned on the event that the UE does not receive any data during the periodic DRX OnDuration, or the UE receives a triggering indication.

Proposal 2: Study PS enhancement techniques to tackle the jitter in arrival time of XR packets.

XR traffic is characterized by high throughput in a short time occurring periodically. So, CA is relevant for XR service. Another potential enhancements coming from this aspect would be to the secondary DRX in the XR context. Currently, the start of second DRX OnDuration is synchronized to that of the base DRX, but just ends earlier to reduce power consumption from the carriers belonging to the second DRX group. Now if we adjust/extend the DRX OnDuration or the Active Time that would also affect the second DRX. Hence similar approaches that we would make to the base DRX would eventually need to apply to the second DRX.

Proposal 3: Study enhancements to the secondary DRX group taking into account the XR traffic characteristics such as non-integer periodicity and jitter.

WUS for CDRX has been introduced in Rel-16. Joint operation with WUS can also be considered for potential enhancements. WUS currently has a very limited functionality. One could imagine that if the WUS could indicate PDCCH skipping or SSSG switching for some time at the beginning of the DRX OnDuration, it would be beneficial for UE power saving to some extent. Or if the WUS could indicate e.g., delayed/early start of the DRX OnDuration, ideally that would also be helpful for UE power saving.

Proposal 4: Study enhancements to WUS for joint operation with CDRX for XR-specific power saving.

Lastly, before delving into details of new potential enhancements, we need to discuss if and how the current CDRX mechanism (long DRX with additional and optional short DRX) can be reused for UE power saving for XR services. Can the long DRX alone be the baseline DRX scheme based on which we build up enhancements for XR? Or should the optional short DRX be the baseline DRX scheme for XR services? Or, is the optional short DRX conditioned on the long DRX still useful for XR services? Trying to answer those questions related to the current CDRX mechanism for use in XR would also help clarify the baseline CDRX schemes upon which the proposed potential enhancement techniques are based.

Observation 3: Discussion on the baseline CDRX scheme that can serve as the baseline for potential enhancements would be useful.

  - PDCCH monitoring enhancements

Firstly, among the pre-Rel-18 PS techniques, Rel-17 PDCCH monitoring adaptation in connected mode can be readily applied for XR-specific power saving regardless of whether the UE is configured with the DRX  or not.

Taking into account the jitter explained in Section 2.3, DRX OnDuration may be configured long enough in order not to miss the XR packet. In this case, PDCCH monitoring adaptation indication can be used to adapt UE’s PDCCH monitoring before and/or after receiving XR packet. As briefly mentioned in Section 2.3, WUS outside DRX Active Time as well as the DCI inside DRX Active Time can be a good candidate for indicating the PDCCH monitoring adaptation. As DCI format 2_6 does not convey PDCCH monitoring adaptation indication field, we can consider introducing monitoring adaptation indication field into DCI format 2_6 or allowing PDCCH monitoring outside DRX Active Time for scheduling DCI conveying monitoring adaptation indication. In order not to violate DRX functionality, we can introduce new DCI format and new RNTI which can be monitored outside DRX Active Time and carries XR-specific PDCCH monitoring adaptation field.

For PDCCH monitoring enhancements, we also need to consider a UE not being configured with the CDRX for XR. In this case, the PDCCH monitoring enhancements should be made in such a way that the UE’s PDCCH monitoring and skipping behavior mimics the DRX functionality. For example, after a UE monitors PDCCH for XR packet reception and then successfully receives the XR packet, the UE can sleep as it is outside DRX Active Time by being indicated to skip PDCCH monitoring until next XR packet reception. The indication can be carried by a scheduling DCI for XR packet or some other DCI format. Furthermore, if gNB plans to transmit other packets than the XR packets, the UE is not indicated to stop monitoring all PDCCHs, but may be indicate to switch SSSG for monitoring PDCCH with low power. In this way, we can consider enhancements on PDCCH monitoring adaptation in connected mode to mimic the CDRX function for a UE not being configured with the CDRX.

Observation 4: Rel-17 PDCCH monitoring adaptation in connected mode can also be applied to the UE not being configured with the CDRX, with enhancements if necessary.

Proposal 5: Consider WUS outside DRX Active Time as well as the DCI inside DRX Active Time as a candidate for indicating the PDCCH monitoring adaptation.

Proposal 6: Consider enhancements on PDCCH monitoring adaptation in connected mode to mimic the CDRX function for a UE not being configured with the CDRX.

As mentioned in Section 2.2, periodicity and jitter are two important XR traffic characteristics to be considered for XR-specific power saving. In addition to that, latency should also be taken into account to meet the XR service requirement with the power saving techniques ON. The latency requirement of XR traffic in RAN side is modelled as packet delay budget (PDB), where the PDB is a limited time budget for a packet to be transmitted over the air from a gNB to a UE. It means that within the PDB the XR packet must be successfully delivered to the UE via the air interface. As there is no need to further monitor PDCCH for the packets that would violate the PDB requirement, the information on PDB margin or remaining PDB would be useful for UE power saving if it is available to a scheduler and/or to a UE. Therefore, for PDCCH monitoring adaptation indication, we should consider periodicity and jitter distribution of the XR traffic arrival time, and PDB margin.

Proposal 7: For enhancements on PDCCH monitoring adaptation in connected mode, consider PDB margin as well as periodicity and jitter distribution of the XR traffic arrival time.

- Conclusion

In this contribution, we shared our views on the potential enhancements of power saving techniques for XR.

Observation 1: High throughput and capacity should also be taken into account for XR-specific PS enhancements.

Observation 2: Upon use for XR service, current CDRX mechanism would cause frequent reconfigurations of the CDRX and/or increase the overall latency.

Observation 3: Discussion on the baseline CDRX scheme that can serve as the baseline for potential enhancements would be useful.

Observation 4: Rel-17 PDCCH monitoring adaptation in connected mode can also be applied to the UE not being configured with the CDRX, with enhancements if necessary.

Proposal 1: Study PS enhancement techniques to tackle the non-integer periodicity of XR traffic.

Proposal 2: Study PS enhancement techniques to tackle the jitter in arrival time of XR packets.

Proposal 3: Study enhancements to the secondary DRX group taking into account the XR traffic characteristics such as non-integer periodicity and jitter.

Proposal 4: Study enhancements to WUS for joint operation with CDRX for XR-specific power saving.

Proposal 5: Consider WUS outside DRX Active Time as well as the DCI inside DRX Active Time as a candidate for indicating the PDCCH monitoring adaptation.

Proposal 6: Consider enhancements on PDCCH monitoring adaptation in connected mode to mimic the CDRX function for a UE not being configured with the CDRX.

Proposal 7: For enhancements on PDCCH monitoring adaptation in connected mode, consider PDB margin as well as periodicity and jitter distribution of the XR traffic arrival time.

- References

RP-213587	New SID “Study on XR Enhancements for NR”, 		Nokia (Rapporteur)

RP-220285	Revised SID : Study on XR Enhancements for NR, 	Nokia (Rapporteur)

3GPP TR 38.838 V17.0.0 (2021-12) Study on XR (Extended Reality) Evaluations for NR (Release 17)