3GPP TSG RAN WG1 Meeting #109-e			R1-2204635

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.11.3

Source: 	LG Electronics

Title: 	Other aspects of XR enhancements for NR

Document for:	Discussion and decision

- Introduction

The Re1-18 SI titled “Study on XR Enhancements for NR” was approved in RAN#94-e meeting [1]. The study is mainly on the techniques for enhancements for XR from RAN1 perspective. The SI objectives are copied below for reference from the latest version of the SID [2].

## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

- Discussion

In this contribution, we discuss other aspects of XR enhancements for NR.

  - XR-specific higher layer information

XR-specific enhancements in terms of capacity and power saving will be facilitated if relevant XR-specific higher layer information is available to XR UEs. Related to the XR-specific higher layer information, SA2 started a study item titled “Study on architecture enhancement for XR and media services” to identify the system architecture aspects related to better support advanced media services, e.g., High Data Rate Low Latency (HDRLL) services, AR/VR/XR services, and tactile/multi-modality communication services [3]. In SA2#149 meeting in February (the first meeting of this SI), 9 key issues were agreed and captured in the TR [4]. These are important because the solutions to each of the Key Issues will be subsequently discussed in the follow-on meetings. Among the 9 key issues listed below,

- Policy control enhancements to support multi-modality flows coordinated transmission for single UE

- Support the Application Synchronization and QoS Policy Coordination for Multi-modal Traffic among Multiple UEs

- 5GS information exposure for XR/media Enhancements

- PDU Set integrated packet handling

- Differentiated PDU Set Handling

- Uplink-downlink transmission coordination to meet Round-Trip latency requirements

- Policy enhancements for jitter minimization

- Enhancements to power savings for XR services

- Trade-off of QoE and Power Saving Requirements

5 key issues (Key issues #3/4/5/8/9, highlighted in yellow) were identified to have RAN impacts or need coordination with RAN. And the other 4 key issues were identified to have possible coordination with SA4. Detailed description on each of the 9 key issues can be found in [4].

XR-specific higher layer information would be helpful for XR-specific capacity and power saving enhancements. Especially for the Key issues #4 and #5, the awareness of the higher layer information would affect the availability of some of the potential enhancement schemes.

Observation 1: Providing XR-specific higher layer information to the gNB and potentially to XR UEs would facilitate XR-specific enhancements on capacity and power saving.

Proposal 1: Discuss whether we could assume the higher layer information related to Key issue #4 and #5 above is available to XR UEs during the SI phase.

Not sure if it belongs to one of the Key issues above, but, information on remaining PDB or PDB margin would also be useful for XR-specific power saving and capacity enhancements. Discussion on the , there may be more aspects   Any other information that is deemed useful for XR-specific enhancements?

Observation 2: Information on remaining PDB or PDB margin from the scheduler to XR UEs and vice versa can be used to enhance power saving and/or capacity for XR.

Proposal 2: Discuss which XR-specific higher layer information would be potentially useful for XR-specific capacity and/or power saving enhancements in RAN1.

- Conclusion

In this contribution, we shared our views on the XR-specific higher layer information that would be useful for XR-specific power saving and capacity enhancements.

Observation 1: Providing XR-specific higher layer information to the gNB and potentially to XR UEs would facilitate XR-specific enhancements on capacity and power saving.

Observation 2: Information on remaining PDB or PDB margin from the scheduler to XR UEs and vice versa can be used to enhance power saving and/or capacity for XR.

Proposal 1: Discuss whether we could assume the higher layer information related to Key issue #4 and #5 above is available to XR UEs during the SI phase.

Proposal 2: Discuss which XR-specific higher layer information would be potentially useful for XR-specific capacity and/or power saving enhancements in RAN1.

- References

RP-213587	New SID “Study on XR Enhancements for NR”, 		Nokia (Rapporteur)

RP-220285	Revised SID : Study on XR Enhancements for NR, 	Nokia (Rapporteur)

SP-211646	New SID on Study on architecture enhancement for XR and media services

3GPP TR 23.700-60	Study on XR (Extended Reality) and media services