3GPP TSG-RAN WG1 Meeting #109-e	R1-2204641

Online, 9th May – 20th May 2022

Agenda Item:	8.16.10

Source:	Ericsson

Title:	UE features for enhanced IAB

Document for:	Discussion, Decision

# Introduction

This contribution addresses the reporting of multiplexing capability and multiplexing conditions to support simultaneous operation in enhanced IAB.

# Discussion

In the RAN1#108-e meeting, an updated list of UE features for Rel-17 eIAB was agreed on and summarized in [1]. In this contribution we address the remaining issues.

## FG 31-4 Case 6 timing alignment

In RAN1#107-e, the following agreements on Case 6 timing alignment was achieved [2]:

Agreement: A child IAB-MT can inform a parent node via MAC-CE whether Case 6 timing is required for simultaneous operation.

To reflect the above RAN1 agreement, in the RAN1#108-e meeting, there was a proposal [3] to introduce a second FG sub-component, namely “Support signaling to the parent-node that Case 6 Timing Mode is required for simultaneous transmission”. The proposal was supported by 4 companies and there was no concern raised by any other company. However, this consensus was neither captured in the agreements by the First Check Point, nor by the Final Check Point. Based on the above observation, the components in FG 31-4 Case 6 timing alignment should include a component 2) that is marked in red below:

- Support Case 6 timing alignment indication reception

- Support signalling to the parent-node that Case 6 Timing Mode is required for simultaneous transmission

## FG 31-10 Updated T_delta range

In RAN1#108-e, a new FG 31-10 was agreed on to be used by the IAB-node to report support of updated T_delta range reception. However, since the previous agreed list has only 8 FGs, we propose to change the numbering from FG 31-10 to FG 31-9.

Based on the above discussions on the feature groups FG31-4 to FG31-10, we propose the following update to the UE features for Rel-17 eIAB:

RAN1 to agree on the updated UE features for Rel-17 eIAB as presented in the Appendix.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to agree on the updated UE features for Rel-17 eIAB as presented in the Appendix.

# References

R1-2202854 “Summary of UE features for IAB enhancements,” Moderators (AT&T), RAN1 #108-e, March 2022

Chair’s Notes, Qualcomm, RAN1 #107-e, November 2021

R1-2202405 “UE features for enhanced IAB” Ericson, RAN1 #108-e, March 2022.

# Appendix

| - Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support Rel-17 DesiredGuardSymbols reporting 2) Support Rel-17 ProvidedGuardSymbols reception | one or more of {31-4, 31-5} | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-2 | IAB-DU beam restriction indication | Support restricted IAB-DU Beam Indication reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-3 | IAB-MT beam recommendation indication | Support recommended IAB-MT Beam Indication transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended IAB-MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-4 | Case 6 timing alignment | 1) Support Case 6 timing alignment indication reception 2) Support signalling to the parent-node that Case 6 Timing Mode is required for simultaneous transmission |  | Yes | N/A | Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. When to perform Case 6 timing at the IAB-node cannot be controlled by the parent node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-5 | Case 7 timing alignment | 1.) Support Case7 timing offset indication reception 2.) Support Case 7 timing at parent-node indication reception |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | 1.) Support Desired DL TX Power Adjustment reporting 2.) Support DL TX Power Adjustment reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting and reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-7 | Desired UL TX power adjustment | Support Desired IAB-MT PSD range reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-8 | Dynamic indication of FDM soft resource availability | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for FDM Soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-109 | Updated T_delta range | Support updated T_delta range reception |  | Yes | 31-4 | The updated T_delta range for an IAB-node operating solely in Case 6 timing is not supported. | Per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |