3GPP TSG-RAN WG1 Meeting #109-e	R1-2204643

Online, May 9th – 20th 2022

Agenda Item:	9.8.2

Source:	Ericsson

Title:	Signaling of control information for NW-controlled repeaters

Document for:	Discussion, Decision

# Introduction

RAN #94 agreed to study network-controlled repeaters with the following assumptions and scenarios [1]:

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters.

Specifically, RAN1 was tasked with the following objectives:

Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signalling (including its configuration) to carry the side control information [RAN1]

In addition to this contribution, we discuss the required control information in our accompanying contribution [2], and we discuss terminology in the specification work with network-controlled repeaters [1].

# Discussion

## General considerations

The network-controlled repeater needs to at least perform random-access and maintain a data channel to the controlling gNB. This is managed by a UE function, the repeater-MT, similar to the IAB-MT, and it is likely required to support most NR signals and channels.

The repeater-MT is likely required to support most NR signals and channels.

We do not see any reason why the existing data channel and control information framework is not sufficient for the repeater. New control and system information for the repeater is required but can be transported over existing transport and physical channels. Therefore, the existing control information framework based on SI, UCI, DCI, MAC CE etc. provided by SSB, PDCCH, PDSCH, PUCCH, PUSCH etc. should be re-used.

Reuse existing control information framework based on SI, UCI, DCI, MAC CE etc. provided by SSB, PDCCH, PDSCH, PUCCH, PUSCH.

Specify new repeater-specific information elements in existing signals and channels.

## Beamforming signaling

Broadcasted NR signals such as SSB, CORESET0 and PRACH are transmitted periodically and can hence be configured semi-statically. There is no need for repeater to have any dynamic configuration for the purpose of their forwarding and that also includes the beam configuration on the UE-side. It is very different for dedicated or UE-specific signals/channels, e.g., PDCCH, PDSCH, PUCCH, PUSCH or UE-specific reference signals for channel quality or interference measurements. These signals are transmitted in random directions, subject to individual scheduling decisions and therefore require dynamic beamforming. Consequently, both semi-static and dynamic signaling of beamforming should be studied

Semi-static beamforming is beneficial for broadcast signals such as SSB, CORESET0 and PRACH.

Dynamic beamforming is beneficial for dedicated signaling, e.g., UE-specific PxyCH as well as SSB and CSI-RS for UE measurements.

For efficient resource utilization, both semi-static and dynamic signaling of beamforming should be studied.

## Timing signaling

We expect that the existing timing framework for the repeater-MT, including the repeater-MT DL Rx timing and repeater-MT UL Tx timing, can be re-used and is in principle sufficient for repeater to set or determine signal timing and coordinate beam switching. Since timing advance is originally specified for UE transmission timing control, it might require improvements for repeater purposes.

No additional timing information is specified unless RAN1 agrees that improved synchronization (TA) is required.

## TDD configuration signaling

It is reasonable to assume that all UEs in the serving cell have in principle the same semi-static TDD configuration. From that it follows that also the transmission direction (i.e., UL or DL) of the repeater-Fwd function can be principally derived from the repeater-MT configuration and there is no need for additional TDD configuration provided the repeater node.

There is no directional conflict between the repeater’s operation on BS and UE sides. Both sides follow the same TDD configuration.

There is no need for additional TDD configuration provided the repeater follows the gNB TDD pattern.

## ON/OFF signaling

The repeater node has, from an external point of view, two active functions, the repeater-MT and the repeater-Fwd functions, that are transmitting (and receiving). For reasons of power saving (and interference management), it should be possible to put either one to sleep or shut down (and wake up, of course). That is, any ON/OFF related signaling should consider both repeater-MT and repeater-Fwd, whereas for the repeater-MT most/all existing power saving functionality can be re-used.

It may be beneficial to allow for individual ON/OFF signaling for both repeater-Fwd and repeater-MT.

The repeater-MT can use existing power saving functionality.

OFF signaling is mutually exclusive from beam indication, in that no beam needs to be configured if repeater-Fwd is configured OFF and vice versa. In that sense, OFF can be viewed as an extra beam, or a special state of the beam configuration, and will benefit from both semi-static and dynamic signaling.

The requirements for repeater-Fwd ON/OFF is very similar to the repeater-Fwd beamforming requirements in its need for both semi-static and dynamic signaling and can be studied together with that.

## Power control

The main purpose of a repeater is to amplify signals that would be otherwise too low, such as in coverage critical areas. For that it is also desirable for the repeater to operate with a gain as high as possible most, probably all the time. This will make dynamic gain control unnecessary, and semi-static power control is sufficient. The receive power of the UE signals is usually gNB controlled (TPC); UE signals are ideally received with very similar power levels. Because of that, it can also be assumed that UE signals received by the repeater can be at very similar power level and make UE-specific repeater gain control unnecessary. In fact, it is very questionable whether a UE-specific gain control can be even (practically) realized, considering gain setting transition times in real amplifiers.

Similar to Rel-17 repeaters, the repeater will operate on a preferred high amplification unless that is not possible. For that reason, semi-static power control is sufficient.

UE-specific repeater power control is neither desirable, nor feasible.

Too high repeater gains can lead to oscillations (self-interference). This problem can be effectively handled by reducing the amplification gain and, correspondingly, the repeater notifying the gNB about its new state of operation. Contrary, network interference by the repeater should be avoided by proper repeater deployment and configuration. Power control for this reason should be down-prioritized.

Prioritize power control due to self-interference before power control for interference management.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The repeater-MT is likely required to support most NR signals and channels.

Observation 2	Semi-static beamforming is beneficial for broadcast signals such as SSB, CORESET0 and PRACH.

Observation 3	Dynamic beamforming is beneficial for dedicated signaling, e.g., UE-specific PxyCH as well as SSB and CSI-RS for UE measurements.

Observation 4	There is no directional conflict between the repeater’s operation on BS and UE sides. Both sides follow the same TDD configuration.

Observation 5	It may be beneficial to allow for individual ON/OFF signaling for both repeater-Fwd and repeater-MT.

Observation 6	The repeater-MT can use existing power saving functionality.

Observation 7	The requirements for repeater-Fwd ON/OFF is very similar to the repeater-Fwd beamforming requirements in its need for both semi-static and dynamic signaling and can be studied together with that.

Observation 8	Similar to Rel-17 repeaters, the repeater will operate on a preferred high amplification unless that is not possible. For that reason, semi-static power control is sufficient.

Observation 9	UE-specific repeater power control is neither desirable, nor feasible.

Observation 10	Prioritize power control due to self-interference before power control for interference management.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reuse existing control information framework based on SI, UCI, DCI, MAC CE etc. provided by SSB, PDCCH, PDSCH, PUCCH, PUSCH.

Proposal 2	Specify new repeater-specific information elements in existing signals and channels.

Proposal 3	For efficient resource utilization, both semi-static and dynamic signaling of beamforming should be studied.

Proposal 4	No additional timing information is specified unless RAN1 agrees that improved synchronization (TA) is required.

Proposal 5	There is no need for additional TDD configuration provided the repeater follows the gNB TDD pattern.

# References

RP-213700, “Study on NR Network-controlled Repeaters,” ZTE, RAN #94-e, December 2021.

R1-2204642, “Control information for enabling NW-controlled repeaters,” Ericsson, RAN1 #109-e, May 2022.

R1-2204644, “Terminology for NW-controlled repeaters,” Ericsson, RAN1 #109-e, May 2022.