3GPP TSG-RAN WG1 Meeting #109-e	R1-2204644

Online, May 9th – 20th 2022

Agenda Item:	9.8.3

Source:	Ericsson

Title:	Terminology for NW-controlled repeaters

Document for:	Discussion, Decision

# Introduction

The Study-Item for Network-controlled Repeaters has the following objectives [1]:

Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

Below we present our views on the architecture and propose a terminology for the specification work of network-controlled repeaters (NCR).

# Discussion

## Repeater links

The repeater is a node in-between a gNB on the one side and a UE on the other side. As such, it is simultaneously operating two links, one link between the gNB and the repeater and another link between the repeater and a UE. Analog to Rel-17 repeaters (and partly also NTN), these links are referred to as donor link and service link, respectively, see Figure 1.

Figure 1: Nodes connected to the network-controlled repeater.

## Network controlled repeater architecture

Different from the Rel-17 repeater, the Rel-18 network-controlled repeater will need to actively communicate with the network, most likely including both receive and transmit functionality, in order to properly decode repeater control signaling. This is a function that is related to the mobile termination (MT) function in an IAB-node and for that reason could be referred to as the repeater-MT or NCR-MT. In addition to the modem function, the repeater-MT could also include a controller function that controls certain repeater operation, see Figure 2.

Figure 2: Assumed architecture and proposed terminology of the network-controlled repeater.

Additionally, the repeater will include a forwarding (Fwd) function (deliberately not named repeater to avoid confusion of what is referred to), comprising donor link antennas, likely shared with the repeater-MT, an amplification function and service link antennas. The UE-side antennas for the service link are controlled by the controller as instructed by the gNB whereas the BS-side antennas for the donor link can be controlled by the MT, with few if any changes compared to normal UE operation.

Adopt the following terminology in the specification work with network-controlled repeaters:

  - The network-controlled repeater is abbreviated NCR.

  - The link between the gNB and the repeater is referred to as the donor link and is managed by the repeater’s BS-side antennas.

  - The link between repeater and UE is referred to as the service link and is managed by the repeater’s UE-side antennas.

  - The repeater function of communicating and controlling the repeater configuration is referred to as the repeater Mobile Termination, repeater-MT, or NCR-MT.

  - The repeater function of receiving, amplifying and transmitting between gNB and UE is referred to as repeater forwarding, repeater-Fwd, or NCR-Fwd.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the following terminology in the specification work with network-controlled repeaters:

a.	The network-controlled repeater is abbreviated NCR.

b.	The link between the gNB and the repeater is referred to as the donor link and is managed by the repeater’s BS-side antennas.

c.	The link between repeater and UE is referred to as the service link and is managed by the repeater’s UE-side antennas.

d.	The repeater function of communicating and controlling the repeater configuration is referred to as the repeater Mobile Termination, repeater-MT, or NCR-MT.

e.	The repeater function of receiving, amplifying and transmitting between gNB and UE is referred to as repeater forwarding, repeater-Fwd, or NCR-Fwd.

# References

RP-213700, “Study on NR Network-controlled Repeaters,” ZTE, RAN #94-e, December 2021.