3GPP TSG RAN WG1 #109-e	R1-2204646

e-Meeting, May 9th – May 20th, 2022

Source:	Sharp

Title:	Discussions on Disabling of HARQ feedback for IoT NTN

Agenda Item:	9.12.2

Document for:	Discussion and Decision

# Introduction

In this document, we share our views on disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates according to the WID on IoT NTN enhancements [1].

# Discussion

In NR NTN, HARQ feedback disabling mechanism was introduced to reduce the packet delivery latency and increase data rate, since NTN network does not need to wait a long RTT for the HARQ-ACK feedback from devices before it can schedule the same HARQ processes again. Similarly, some benefits from HARQ feedback disabling are also applicable to IoT NTN.

## Benefits of HARQ feedback disabling for IoT NTN

Comparing to NR NTN, IoT NTN share the same large propagation latency. Moreover, there is a much smaller number of HARQ processes for IoT NTN. Thus, HARQ stalling could be an issue. Disabling of HARQ feedback can avoid the large packet delivery latency and increase data rate.

Additionally, in IoT NTN, HARQ-ACK feedback always requires a huge number of repetitions in the uplink, which occupy lots of uplink resources. If the HARQ-ACK feedback is disabled, the corresponding uplink resources are not needed any more and the saved resource can be used for other uplink transmissions. Potentially, disabling HARQ feedback for a downlink transmission can improve uplink throughput in IoT NTN.

Another benefit of HARQ feedback disabling is power saving since UE does not need to consume power to feedback on a disabled HARQ process any more. A long battery life is always an important KPI for IoT devices.

To better utilize all the benefits from HARQ feedback disabling, we have the following proposal:

Proposal 1:

Disabling HARQ feedback for IoT NTN is supported in Rel-18.

## Flexibility of HARQ feedback disabling/enabling configuration

In NR NTN, HARQ feedback disabling is configured per HARQ process via UE specific RRC signalling so that the network has the flexibility to disable or enable a specific HARQ process. For IoT NTN, we would like to propose the same flexibility of HARQ feedback disabling/enabling configuration.

Proposal 2:

In IoT NTN, HARQ feedback disabling/enabling is configured per HARQ process via UE specific RRC signalling.

# Conclusion

In this contribution, for the HARQ enhancements of IoT NTN, we propose the following.

Proposal 1:

Disabling HARQ feedback for IoT NTN is supported in Rel-18.

Proposal 2:

In IoT NTN, HARQ feedback disabling/enabling is configured per HARQ process via UE specific RRC signalling.

# References

- RP-220979, Revised WID on IoT NTN enhancements, 3GPP RAN #95e, March 17 - 23, 2022