3GPP TSG RAN WG1 Meeting #109-e  			    			             R1- 2204649

e-Meeting, May 9th –20th, 2022

# Source:	ETRI

# Title:		Remaining issues on inter-UE coordination for Mode 2 enhancements

# Agenda item:	8.11.2

# Document for:	Discussion/Decision

Introduction

In the last plenary meeting, Rel-17 sidelink enhancements WI was concluded as completed with the following remaining open issues [1].

- Finalization of UE-B’s behavior when it receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As - Finalization of relationship between start/end slots of resource selection window used for sidelink transmission carrying inter-UE coordination information and start/end slots of resource selection window for determining the set of resources

In this contribution, we introduce our views for the remaining issues.

Remaining issues on inter-UE coordination

All the remaining issues are related with Scheme 1 in which the coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission. The followings are the agreements related to the remaining issues [2].

Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As, - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As. - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline) - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection Agreement - Notations: - (n+T_1) – Start slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement - (n+T_2) – End slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered - For inter-UE coordination information triggered by UE-B’s explicit request - Alt 1-1: - X1 ≤ (n’+T’_1) - (n’+T’_2) ≤ X2 - For inter-UE coordination information triggered by a condition other than explicit request reception, - Alt 2-2: - (n’+T’_2) < X3 - FFS: Values for X1, X2, X3

Regarding UE-B’s behavior, it was finalized UE-B’s behavior when it receives preferred resource set or non-preferred resource set from the same UE-A or different UE-As in the last RAN1 meeting. However, in case of receiving both preferred resource set and non-preferred resource set from the same UE-A or different UE-As, there are many things to be considered, for example whether both sets are from the same UE-A, whether preferred set is from one UE-A and non-preferred set is from another UE-A, and so on especially for the case of reception of both sets from different UE-As. It might be very difficult to finalize UE-B’s behavior in consideration of various different situations at this maintenance phase. Therefore, in order to avoid standard hole, we prefer to leave UE-B’s behavior for this case as UE implementation, and focus on stabilization of Rel-17 specifications.

Proposal 1: For UE-B’s behavior when it receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As,

- It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Regarding the resource selection window used for sidelink transmission carrying inter-UE coordination information, there are several alternatives proposed in the last RAN1 meeting. For both request-based and condition-based, we think the inter-UE coordination information should be available before the starting of resource selection window for determining the set of resources in order for UE-B to use the information for resource selection as much as possible. Therefore, we propose to support the following relationships between start/end slots of resource selection window used for sidelink transmission carrying inter-UE coordination information and start/end slots of resource selection window for determining the set of resources.

Proposal 2: For the relationship between two selection windows, it is proposed to support the followings:

- For inter-UE coordination information triggered by UE-B’s explicit request,

  - (n’+T’_2) < (n+T_1)

- For inter-UE coordination information triggered by a condition other than explicit request reception,

  - (n’+T’_2) < (n+T_1)

Summary

In this contribution, we made the following proposals for inter-UE coordination for Mode 2 enhancements.

Proposal 1: For UE-B’s behavior when it receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As,

- It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Proposal 2: For the relationship between two selection windows, it is proposed to support the followings:

- For inter-UE coordination information triggered by UE-B’s explicit request,

  - (n’+T’_2) < (n+T_1)

- For inter-UE coordination information triggered by a condition other than explicit request reception,

  - (n’+T’_2) < (n+T_1)

References

- RP-220945, “Status report of WI NR sidelink enhancement; rapporteur: LG Electronics,” RAN1

- Chair’s notes, RAN1#108-e, February 2022.