3GPP TSG RAN WG1 Meeting #109-e  			    			             R1- 2204652

e-Meeting, May 9th –20th, 2022

# Source:	ETRI

# Title:		Discussion on co-channel coexistence for LTE sidelink and NR sidelink

# Agenda item:	9.4.2

# Document for:	Discussion

Introduction

The WI on NR sidelink evolution was approved in RAN#94-e meeting. In the new WI, there are four objectives related to RAN1 as follows [1]:

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#97) - Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Study and specify enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] (This part of the work is put on hold until further checking in RAN#97) - Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible

Among the objectives, the co-channel coexistence of LTE sidelink and NR sidelink was proposed by automotive industry(5GAA) as one of the most important topics for NR sidelink evolution [2]. Considering limited frequency resources in ITS bandwidth, and also smooth migration and re-farming of frequency resources on ITS carriers from LTE to NR, it will be necessary to support co-channel coexistence of LTE sidelink and NR sidelink. In this contribution, we introduce our views on the co-channel coexistence of LTE sidelink and NR sidelink.

Co-channel coexistence of LTE sidelink and NR sidelink

In Rel-16, NR sidelink supports in-device coexistence framework assuming a UE supporting both LTE sidelink and NR sidelink. Even though it should be reused the existing in-device coexistence framework as much as possible for co-channel coexistence between two RATs, we think the existing framework is insufficient since it only focused on internal UE operation. Regarding the approach for supporting the co-channel coexistence, the following two alternatives were discussed [3].

- Alt 1: Resource pool separation between the two RATs.

- Alt 2: Dynamic resource sharing using overlapping resource pools between the two RATs.

Without network control, LTE or NR sidelink UE will use its default configuration for resource allocation. In the initial stage of V2X service, the default configuration of LTE sidelink would be the entire ITS bandwidth. Once LTE sidelink UEs with such a configuration are deployed, it is most likely impossible to change or update the default configuration. After that, if NR sidelink UEs are also deployed in the same carrier, it would be difficult to avoid the collision between two RATs. In order to prevent such a situation, one simplest solution could be resource pool separation between two RATs in a static manner i.e., Alt 1. However, it will cause resource inefficiency, and prevent smooth migration from LTE sidelink to NR sidelink since the default resource configuration of initial LTE V2X UE could not be changed or updated as mentioned in the above. Therefore, we propose to support Alt 2 for co-channel coexistence of LTE sidelink and NR sidelink.

Proposal 1: For co-channel coexistence of LTE sidelink and NR sidelink, Alt 2 should be supported.

- Alt 2: Dynamic resource sharing using overlapping resource pools between the two RATs.

For dynamic resource sharing using overlapping resource pools between two RATs, it should be guaranteed no impact to LTE sidelink specification, and also ensured backward compatibility of Rel-16/17 NR sidelink.

Proposal 2: It is proposed to guarantee no impact to LTE sidelink specification, and ensure backward compatibility of Rel-16/17 NR sidelink.

For V2X operation, there are two modes in each RAT, which are mode 3 & 4 and mode 1 & 2 for LTE and NR respectively. In case of LTE mode 3 and NR mode 1, it is not necessary to handle the case since network will configure proper resource allocations for both RATs taking the actual traffic into account. In case of mixed situations, for example LTE SL mode 3 and NR mode 2, or vice versa, it seems a very rare case. Therefore, we propose to focus LTE SL mode 4 and NR SL mode 2 only.

Proposal 3: It is proposed to focus on scenarios where LTE mode 4 and NR mode 2 coexist.

Considering the above proposals, the possible simple solution seems that a UE that supports both LTE and NR sidelinks, passes LTE sensing result to NR and uses it for NR resource selection procedure. In order to support the solution, a UE supporting both LTE sidelink and NR sidelink should be a basic assumption.

Proposal 4: For dynamic resource sharing using overlapping resource pools between two RATs, it should be assumed a UE supporting both LTE sidelink and NR sidelink operations.

In order to support the solution in the above, it would be better to align resource grids between LTE and NR sidelinks, for example same subchannel size or integer multiple relationship between LTE and NR considering a ratio of different subcarrier spacings (N*LTE subchannel size = NR subchannel size or LTE subchannel size = M*NR subchannel size). If not aligned between two resource grids, sidelink resources would be wasted.

Figure 1. Example of misaligned resource grids between LTE sidelink and NR sidelink

Figure 1 shows an example of misaligned resource grids between LTE and NR sidelinks i.e., NR subchannel size > LTE subchannel size and not integer multiple relationship. In the figure, both Case #1 and Case #2 shows NR sidelink resources overlapped with used LTE sidelink resources. In order to avoid resource collisions, NR sidelink will not use the overlapped regions. If the resource grids between two RATs are misaligned, unnecessary resource waste will be occurred, especially in Case #2, NR SL resources will be wasted more (e.g., “wasted NR resources” as shown in  Figure 1).

If the sensing results are passed from LTE sidelink module to NR sidelink module, during a resource (re-)selection procedure of NR sidelink, different subcarrier spacings and/or different resource grids if any should be considered. If the subcarrier spacings between LTE and NR are different, the candidate single-slot resource between LTE and NR will be different. If the resource grids between LTE and NR are also different, the candidate single-slot resources will be also different. When NR sidelink module uses the passed LTE sensing results for NR sidelink resource selection, the different size between candidate single-slot resources of two RATs should be considered.

In order to prevent the passed sensing results to be outdated and/or ensure the passed sensing results to be used as much as possible for NR sidelink resource selection, the timeline to pass the sensing results from LTE sidelink module to NR sidelink module should be also carefully studied.

Figure 2. Example of timeline of sensing and resource (re-)selection procedure

Figure 2 shows an example of timeline of sensing and resource (re-)selection procedure for Rel-16 NR sidelink. Considering the timeline, for example, it will be beneficial for the sensing results from LTE module to be available before NR module starts the resource selection procedure i.e., < n+T1.

In addition to the above mentioned aspects, other aspects such as handling priority of LTE sensing results should be also studied.

Proposal 5: It is proposed to study the following aspects to support dynamic resource sharing using overlapping resource pools between two RATs:

- Resource configuration (aligned resource grids between two RATs), resource (re-)selection procedure taking LTE sensing results into account, timeline to pass the sensing results from LTE module to NR module, and so on.

Summary

In this contribution, we made the following proposals to support co-channel coexistence between LTE sidelink and NR sidelink.

Proposal 1: For co-channel coexistence of LTE sidelink and NR sidelink, Alt 2 should be supported.

- Alt 2: Dynamic resource sharing using overlapping resource pools between the two RATs.

Proposal 2: It is proposed to guarantee no impact to LTE sidelink specification, and ensure backward compatibility of Rel-16/17 NR sidelink.

Proposal 3: It is proposed to focus on scenarios where LTE mode 4 and NR mode 2 coexist.

Proposal 4: For dynamic resource sharing using overlapping resource pools between two RATs, it should be assumed a UE supporting both LTE sidelink and NR sidelink operations.

Proposal 5: It is proposed to study the following aspects to support dynamic resource sharing using overlapping resource pools between two RATs:

- Resource configuration (aligned resource grids between two RATs), resource (re-)selection procedure taking LTE sensing results into account, timeline to pass the sensing results from LTE module to NR module, and so on.

References

- RP-220300, “WID revision: NR sidelink evolution,” OPPO

- RWS-210360, “5GAA input to 3GPP Rel. 18 Workshop,” 5GAA.

- RP-212503, “Moderator's summary for discussion [RAN93e-R18Prep-06] Sidelink enhancements (excluding positioning),” LG Electronics.