3GPP TSG RAN WG1 #109-e	R1-2204653

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.8.1

Source: 	ETRI

Title:	Discussion on side control information for network-controlled repeater

Document for:	Discussion/Decision

Introduction

As per the continuous efforts to extend the coverage of wireless network with better cost-efficiency, the SID on evolution of NR network-controlled (NC) repeater was agreed with the following objectives [1]:

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

This contribution presents ETRI’s views on NC repeater focusing on the following topics:

- Operation scenarios with high priority.

- System-level simulation (SLS) results with or without NC repeaters.

- Potential specification supports to enable NC repeater.

Discussion

Operation scenarios for network-controlled repeater

During the development phase of the SID on NC repeater [1], there had been abundant discussions on the preferred operation scenarios for NC repeaters. From our perspectives, the most interesting operation scenario for NC repeater is to support indoor UEs by NC repeater in FR2 frequency band. It has been widely reported that due to the large penetration loss at high frequency band, the received signal qualities (e.g., RSRP or RSRQ in the specifications) at the indoor UE (or outdoor UE) are significantly low in general, if the signal is transmitted from the outdoor transmission point (or from the indoor transmission point). Detailed information can be found in the coverage map report of several operators who have on-going commercial services at FR2 frequency band.

Figure 1 depicts examples of of wireless networks (a) with gNB only, (b) with gNB + Rel-17 RF repeater, and (c) with gNB + Rel-18 NC repeater. In the first example where the network only comprises gNBs, the indoor UEs usually suffer from high penetration loss due to wall, glass, or wood of the building. To tackle this issue, Rel-17 RF repeater has been specified [2] with some requirements. Since Rel-17 RF repeater does not support any gNB-repeater interface, the potential flexibility of the RF repeater should be limited and, e.g., uncontrollable fixed beam for both repeater-MT and repeater-RU should be assumed/utilized. Therefore, it is expected that in FR2 the performance gain from RF repeater will be far below compared with that from NC repeater, which can adjust its beam direction(s) per the gNB indication.

Figure 1. Examples of of wireless networks (a) with gNB only, (b) with gNB + Rel-17 RF repeater, and (c) with gNB + Rel-18 NC repeater.

In such scenarios (i.e., O2I at FR2), some set of assumptions for both this study item and the follow-up normative work might be helpful to clarify the work scope considering the limited TUs for NC repeater in Rel-18. For instance, we may assume that repeater-MT and repeater-RU within a repeater are not co-located with each other, where gNB and repeater-MT constitute a gNB-repeater link and repeater-RU and UE build a repeater-UE link. In another example, it can be assumed that some properties (e.g., the boresight of repeater-MT antenna array/panel(s) and/or Tx/Rx beam direction of repeater-MT antenna array/panel(s)) of repeater-MT can be pre-optimized/configured for the serving gNB. Under the assumptions above, it is proposed to have first priority to study beam management/control for repeater-UE link, while second priority would be preferable for beam management/control or gNB-repeater-UE.

Proposal 1. For Rel-18, assume that repeater-MT and repeater-RU within a repeater are not co-located.

- gNB and Repeater-MT constitute a gNB-repeater link.

- Repeater-RU and UE constitute a repeater-UE link.

Proposal 2. For Rel-18, assume that some properties of repeater-MT can be pre-optimized/configured for the serving gNB:

- E.g., the boresight of repeater-MT antenna array/panel(s).

- E.g., Tx/Rx beam direction of repeater-MT antenna array/panel(s).

Proposal 3. For Rel-18, study beam management/control for repeater-UE link with high priority considering the limited TU.

- Beam management/control for gNB-repeater-UE link can be studied with low priority.

SLS evaluations on network-controlled repeater

In this section, SLS evaluation results with or without NC repeater are provided to study the necessity of specification supports on side control information (SCI) for NC repeater.

Figure 2 shows a simplified flowchart of the SLS for NC repeater. As in Figure 2, the number of buildings with a repeater per sector (for either of Rel-17 RF repeater or Rel-18 NC repeater) can be configured as a system parameter (see Table 1 for more details). Since only a subset of UEs among the entire indoor UEs will be serviced by the repeater in this simulation (e.g., an indoor UE who is on outside of the building with repeater will have low RSRP for repeater-UE links in general), we have the following three types of UEs; 1) UE type #1: Indoor UE served by gNB, 2) Outdoor UE served by gNB, 3) Indoor UE served by repeater. The experimental ratios of UE types can be found in Figure 2. In the following evaluations, the number of buildings with repeater is configured as 3 per sector.

More detailed evaluation assumptions and parameters are given in section 4.1.

Figure 2. A simplified flowchart of SLS for NC repeater.

Figure 3, Figure 4, Figure 5, and Figure 6 provide wideband SINR CDF curves of wireless networks with gNB only (blue line), with gNB + Rel-17 RF repeater (orange line), with gNB + Rel-18 NC repeater (other lines) according to various repeater antenna configurations and repeater gain control assumptions.

Specifically, Figure 3 and Figure 5 represent evaluation results with smaller repeater-RU antenna configuration of (M, N, P) = (4, 4, 2) with total 32 antenna elements in repeater-RU side. On the other hand, Figure 4 and Figure 6 depict evaluation results with larger repeater-RU antenna configuration of (M, N, P) = (8, 8, 2) with total 128 antenna elements in repeater-RU side. By comparison between the two sets of evaluation results, it is observed that large (e.g., ≥8 bits per beam) SCI payload for beam control may provide additional SINR gains for large (e.g., (M, N, P) = (8, 8, 2)) repeater-RU antenna configurations, while small (e.g., 4 bits per beam) SCI payload for beam control would be enough for small (e.g., (M, N, P) = (4, 4, 2)) repeater-RU antenna configurations.

From gain/power control aspects, Figure 3 and Figure 4 utilized ideal repeater gain control so the repeater can utilize the maximum transmission power at repeater-RU. On the other hand, since a fixed repeater gain of 50 dB was assumed in Figure 5 and Figure 6, each repeater may or may not be able to use the maximum transmission power at repeater-RU per the channel condition of gNB-repeater link. From the two sets of evaluations, it is observed that Dynamic repeater gain/power control can provide additional SINR gain over semi-static repeater gain/power configuration.

Observation 1. The required SCI payload for beam control can be varied according to the repeater-RU antenna configuration:

- Small (e.g., 4 bits per beam) SCI payload for beam control would be enough for small (e.g., (M, N, P) = (4, 4, 2)) repeater-RU antenna configurations.

- Large (e.g., ≥8 bits per beam) SCI payload for beam control may provide additional SINR gains (e.g., ≥5 dB) for large (e.g., (M, N, P) = (8, 8, 2)) repeater-RU antenna configurations.

Observation 2. Dynamic repeater gain/power control can provide additional SINR gain over semi-static repeater gain/power configuration.

Proposal 4. Capture the evaluation results in the TR.

Figure 3. SINR CDF curves of wireless networks with gNB only, with gNB + Rel-17 RF repeater, or with gNB + Rel-18 NC repeater, where the repeater-RU antenna configuration is (M, N, P) = (4, 4, 2) and ideal repeater gain control is assumed; (a) considering all UE types, (b) considering UE type 3 only.

Figure 4. SINR CDF curves of wireless networks with gNB only, with gNB + Rel-17 RF repeater, or with gNB + Rel-18 NC repeater, where the repeater-RU antenna configuration is (M, N, P) = (8, 8, 2) and ideal repeater gain control is assumed; (a) considering all UE types, (b) considering UE type 3 only.

Figure 5. SINR CDF curves of wireless networks with gNB only, with gNB + Rel-17 RF repeater, or with gNB + Rel-18 NC repeater, where the repeater-RU antenna configuration is (M, N, P) = (4, 4, 2) and 50 dB fixed repeater gain is assumed; (a) considering all UE types, (b) considering UE type 3 only.

Figure 6. SINR CDF curves of wireless networks with gNB only, with gNB + Rel-17 RF repeater, or with gNB + Rel-18 NC repeater, where the repeater-RU antenna configuration is (M, N, P) = (8, 8, 2) and 50 dB fixed repeater gain is assumed; (a) considering all UE types, (b) considering UE type 3 only.

Table 1. SINR gains over gNB only cases, where the repeater-RU antenna configuration is (M, N, P) = (4, 4, 2) and ideal repeater gain control is assumed (associated with Figure 3).

| gNB / repeater layout | SINR gain (over gNB only) [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 %-tile | 50 %-tile | 95 %-tile | Average |
| gNB + RF Repeater (SCI payload = 0 bit) | 0.12 | 4.70 | 1.53 | 2.77 |
| gNB + NC Repeater (SCI payload = 4 bits) | -0.98 | 8.15 | 9.20 | 6.21 |
| gNB + NC Repeater (SCI payload = 8 bits) | -0.15 | 7.10 | 12.51 | 6.48 |
| gNB + NC Repeater (SCI payload = 12 bits) | 0.27 | 7.36 | 13.10 | 6.46 |
| gNB + NC Repeater (SCI payload = ∞ bits) | 6.46 | 11.20 | 43.85 | 16.11 |

Table 2. SINR gains over gNB only cases, where the repeater-RU antenna configuration is (M, N, P) = (8, 8, 2) and ideal repeater gain control is assumed (associated with Figure 4).

| gNB / repeater layout | SINR gain (over gNB only) [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 %-tile | 50 %-tile | 95 %-tile | Average |
| gNB + RF Repeater (SCI payload = 0 bit) | 0.12 | 3.84 | 1.72 | 2.47 |
| gNB + NC Repeater (SCI payload = 4 bits) | -0.74 | 8.02 | 10.64 | 6.44 |
| gNB + NC Repeater (SCI payload = 8 bits) | 0.18 | 7.32 | 14.63 | 7.41 |
| gNB + NC Repeater (SCI payload = 12 bits) | 0.37 | 7.82 | 16.77 | 7.57 |
| gNB + NC Repeater (SCI payload = ∞ bits) | 7.84 | 11.44 | 49.70 | 18.05 |

Table 3. SINR gains over gNB only cases, where the repeater-RU antenna configuration is (M, N, P) = (4, 4, 2) and 50 dB fixed repeater gain control is assumed (associated with Figure 5).

| gNB / repeater layout | SINR gain (over gNB only) [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 %-tile | 50 %-tile | 95 %-tile | Average |
| gNB + RF Repeater (SCI payload = 0 bit) | -0.48 | -0.08 | 0.26 | 0.69 |
| gNB + NC Repeater (SCI payload = 4 bits) | 0.77 | 4.16 | 0.30 | 2.26 |
| gNB + NC Repeater (SCI payload = 8 bits) | 0.06 | 4.01 | 1.16 | 2.37 |
| gNB + NC Repeater (SCI payload = 12 bits) | -0.75 | 3.89 | 1.32 | 2.43 |
| gNB + NC Repeater (SCI payload = ∞ bits) | 1.50 | 7.83 | 18.63 | 8.37 |

Table 4. SINR gains over gNB only cases, where the repeater-RU antenna configuration is (M, N, P) = (8, 8, 2) and 50 dB fixed repeater gain control is assumed (associated with Figure 6).

| gNB / repeater layout | SINR gain (over gNB only) [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 %-tile | 50 %-tile | 95 %-tile | Average |
| gNB + RF Repeater (SCI payload = 0 bit) | -0.06 | -0.14 | 0.31 | 0.68 |
| gNB + NC Repeater (SCI payload = 4 bits) | -0.06 | 4.23 | 0.55 | 2.52 |
| gNB + NC Repeater (SCI payload = 8 bits) | -0.06 | 5.91 | 1.88 | 3.13 |
| gNB + NC Repeater (SCI payload = 12 bits) | -0.06 | 5.91 | 2.98 | 3.34 |
| gNB + NC Repeater (SCI payload = ∞ bits) | 1.97 | 8.26 | 24.42 | 9.67 |

Potential specification supports for network-controlled repeater

According to the observations in section 2.2, the following two proposals can be made:

Proposal 5. RAN1 to support SCI to indicate one or more beams among 16 repeater-RU beams, at least.

Proposal 6. RAN1 to support dynamic repeater gain/power control via SCI.

In addition to the proposals above, beam management/control procedures considering the associated gNB-repeater and repeater-UE links.

Figure 6 shows an example of Tx/Rx beam sweeping to determine beam pair for a gNB-UE link. As represented by Figure 6, to perform either of Tx beam sweeping or Rx beam sweeping from downlink perspective, one among the gNB Tx beam or the UE Rx beam should be fixed. Therefore, it can be summarized that the current specifications support the following two beam sweeping types, at least for downlink beam management:

- Type #A: gNB Tx beam sweeping + fixed UE Rx beam

- Type #B: Fixed gNB Tx beam + UE Rx beam sweeping

Since we have two more possible antenna panels (or two groups of antenna panels), i.e., repeater-MT panel(s) and repeater-RU panel(s), extensions of the above beam sweeping types would be helpful for the future discussions on how to enable beam sweeping to determine the beam pair(s) for gNB-repeater-UE links. For instance, the possible additional beam sweeping types would be:

- Type #C: gNB Tx beam sweeping + fixed {repeater-MT Rx, repeater-RU Tx, UE Rx} beams

- Type #D: Repeater-MT Rx beam sweeping + fixed {gNB Tx, repeater-RU Tx, UE Rx} beams

- Type #E: Repeater-RU Tx beam sweeping + fixed { gNB Tx, repeater-MT Rx, , UE Rx} beams

- Type #F: UE Rx beam sweeping + fixed {gNB Tx, repeater-MT Rx, repeater-RU Tx} beams

Figure 7. An example of Tx/Rx beam sweeping to determine beam pair for a gNB-UE link.

Observation 3. The current specification supports the following beam sweeping types:

- Type #A: gNB Tx beam sweeping + fixed UE Rx beam

- Type #B: Fixed gNB Tx beam + UE Rx beam sweeping

Proposal 7. RAN1 to consider the following additional beam sweeping types for NC repeater:

- Type #C: gNB Tx beam sweeping + fixed {repeater-MT Rx, repeater-RU Tx, UE Rx} beams

- Type #D: Repeater-MT Rx beam sweeping + fixed {gNB Tx, repeater-RU Tx, UE Rx} beams

- Type #E: Repeater-RU Tx beam sweeping + fixed { gNB Tx, repeater-MT Rx, , UE Rx} beams

- Type #F: UE Rx beam sweeping + fixed {gNB Tx, repeater-MT Rx, repeater-RU Tx} beams

Conclusion

In this contribution, ETRI’s views on NC repeater were shown and the following observations and proposals were made:

Observation 1. The required SCI payload for beam control can be varied according to the repeater-RU antenna configuration:

- Small (e.g., 4 bits per beam) SCI payload for beam control would be enough for small (e.g., (M, N, P) = (4, 4, 2)) repeater-RU antenna configurations.

- Large (e.g., ≥8 bits per beam) SCI payload for beam control may provide additional SINR gains (e.g., ≥5 dB) for large (e.g., (M, N, P) = (8, 8, 2)) repeater-RU antenna configurations.

Observation 2. Dynamic repeater gain/power control can provide additional SINR gain over semi-static repeater gain/power configuration.

Observation 3. The current specification supports the following beam sweeping types:

- Type #A: gNB Tx beam sweeping + fixed UE Rx beam

- Type #B: Fixed gNB Tx beam + UE Rx beam sweeping

Proposal 1. For Rel-18, assume that repeater-MT and repeater-RU within a repeater are not co-located.

- gNB and Repeater-MT constitute a gNB-repeater link.

- Repeater-RU and UE constitute a repeater-UE link.

Proposal 2. For Rel-18, assume that some properties of repeater-MT can be pre-optimized/configured for the serving gNB:

- E.g., the boresight of repeater-MT antenna array/panel(s).

- E.g., Tx/Rx beam direction of repeater-MT antenna array/panel(s).

Proposal 3. For Rel-18, study beam management/control for repeater-UE link with high priority considering the limited TU.

- Beam management/control for gNB-repeater-UE link can be studied with low priority.

Proposal 4. Capture the evaluation results in the TR.

Proposal 5. RAN1 to support SCI to indicate one or more beams among 16 repeater-RU beams, at least.

Proposal 6. RAN1 to support dynamic repeater gain/power control via SCI.

Proposal 7. RAN1 to consider the following additional beam sweeping types for NC repeater:

- Type #C: gNB Tx beam sweeping + fixed {repeater-MT Rx, repeater-RU Tx, UE Rx} beams

- Type #D: Repeater-MT Rx beam sweeping + fixed {gNB Tx, repeater-RU Tx, UE Rx} beams

- Type #E: Repeater-RU Tx beam sweeping + fixed { gNB Tx, repeater-MT Rx, , UE Rx} beams

- Type #F: UE Rx beam sweeping + fixed {gNB Tx, repeater-MT Rx, repeater-RU Tx} beams

Appendix

Evaluation assumptions

Table 5. Evaluation assumptions for network configuration parameters

| Parameter | Values / Descriptions |
| --- | --- |
| Scenarios | UMa, UMi-Street Canyon (for gNB-repeater link or gNB-UE link) Indoor-office (for repeater-UE link) |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees |
| Carrier frequency | 5 GHz, 28 GHz |
| System bandwidth | 100 MHz for 5GHz, 400 MHz for 28 GHz |
| Wrapping method | Geographical distance based wrapping |
| Region of interest | 19 site (57 sectors) |
| Number of repeaters (buildings) per sector | 3 |
| Building width | 120 m |
| Building length | 50 m |
| The number of floors of each building | 1 |
| O2I penetration loss | According to subclause 7.4.3.1 in TR38.901 |
| UE attachment | RSRP based on single gNB antenna port |

Table 6. Evaluation assumptions for gNB configuration parameters

| Parameter | Values / Descriptions |
| --- | --- |
| gNB antenna configurations | Mg = Ng = 1, dv = 0.5λ, (M, N, P) = (8, 8, 2) |
| gNB antenna port mapping | All elements with the same polarization are mapped to a single port |
| Antenna virtualization (analog beam forming) | DFT vector based |
| gNB Tx power | 46 dBm for UMa 41 dBm for UMi |

Table 7. Evaluation assumptions for repeater configuration parameters

| Parameter | Values / Descriptions |
| --- | --- |
| Repeater-MT antenna configurations | Mg = Ng = 1, dv = 0.5λ, (M, N, P) = (4, 4, 2) Boresight direction: To the serving gNB |
| Repeater-RU antenna configurations | Mg = Ng = 1, dv = 0.5λ, (M, N, P) = (4, 4, 2) or (8, 8, 2) Boresight direction: From ceiling to floor, i.e., α, β,γ=0o,90o,0o |
| Repeater antenna port mapping | All elements with the same polarization are mapped to a single port |
| Antenna virtualization (analog beam forming) | DFT vector based |
| Repeater-RU Tx power | 30 dBm |
| Repeater gain | Ideal (), fixed (30 dB, 50 dB) |
| Repeater-MT height | 4.5 m (rooftop of one-story building) |
| Repeater-RU height | 3 m (ceiling of the first floor) |

Table 8. Evaluation assumptions for UE configuration parameters

| Parameter | Values / Descriptions |
| --- | --- |
| UE antenna configurations | Mg = Ng = 1, dv = 0.5λ, (M, N, P) = (1, 4, 2) Two panels, one for front side and the other for rear side. |
| UE antenna port mapping | All elements with the same polarization are mapped to a single port |
| Antenna virtualization (analog beam forming) | DFT vector based |
| UE speed | 3 km/h |

Calibration results

For the calibration purpose, the evaluation assumptions from subclause 7.8.1 in [3] were utilized.

Figure 1 and Figure 2 show large scale parameter calibration results for UMa and UMi scenarios, respectively. In Figure 1 and Figure 2, “Ref. avg”, “Ref. min”, and “Ref. max” represent “the averaged calibration results”, “the CDF curves with lowest values”, and “the CDF curves with highest values” in each scenario, respectively, which can be found in [4].

Figure 8. Large scale parameter calibration results for UMa at 30 GHz carrier frequency.

Figure 9. Large scale parameter calibration results for UMi at 30 GHz carrier frequency.

References

- RP-213700, New SI: Study on NR Network-controlled Repeaters, ZTE

- RP-212129, Revised WID on NR Repeaters, Qualcomm

- TR38.901 V17.0.0, Study on channel model for frequencies from 0.5 to 100 GHz, 3GPP

- R1-165974, Large-scale calibration results, Samsung