3GPP TSG RAN WG1 #109-e	R1-2204654

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.8.2

Source: 	ETRI

Title:	Discussion on L1/L2 signaling for side control information

Document for:	Discussion/Decision

Introduction

T As per the continuous efforts to extend the coverage of wireless network with better cost-efficiency, the SID on evolution of NR network-controlled (NC) repeater was agreed with the following objectives [1]:

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

This contribution presents ETRI’s views on NC repeater focusing on the following topics:

- L1/L2 signaling (including its configuration) to carry the side control information (SCI).

Discussion

From our companion contribution [2], potential gains from NC repeater have been investigated via system-level evaluations according to various assumptions, e.g., on repeater antenna configurations, beam control, repeater gain control, etc. Given that dynamic and flexible beam/gain controls provide higher SINR gain than semi-static and inflexible ones, we prefer to have L1 signaling (DCI) as the baseline for SCI design.

For the potential (explicit and/or implicit) contents of the L1 signaling to carry the SCI, we think the following options can be considered as a starting point:

- Regarding beamforming information,

  - Indexes of repeater-RU beams for a given time/frequency resources.

  - Number of available repeater-RU beams for a given time/frequency resources.

  - Indexes of repeater-MT beams for a given time/frequency resources.

  - Number of available repeater-MT beams for a given time/frequency resources.

  - Indexes of repeater beams for a given time/frequency resources.

  - Number of available repeater beams for a given time/frequency resources.

  - (Non-)Preferences on a beam set.

  - Indexes of the time resources (e.g., application timing) and/or frequency resources (e.g., applicable band or CC list) associated with the beam indication(s).

- Regarding information on UL-DL TDD configuration,

  - UL-DL direction(s) for a given time/frequency resources.

  - Time duration (frequency band) where the indicated UL-DL direction(s) shall be applied.

  - Time duration (frequency band) where the UL-DL direction(s) can be determined by repeater implementation.

- Regarding, ON-OFF information or power control information

  - Range/value of repeater gain for a given time/frequency resources.

  - Time duration (frequency band) where the indicated repeater gain shall be applied.

  - Time duration (frequency band) where the repeater gain can be determined by repeater implementation.

Note that it should be possible to contain only a subset of above options for the SCI considering various repeater implementations.

To minimize repeater implementation burden, the monitoring occasion (MO) of SCI should be carefully designed. For instance, some of NC repeaters with low capability may be allowed to monitor the MO of SCI less frequently than normal UEs. One of possible relaxations above is to restrict the MO of SCI as the PDCCH MO configured by MIB. Such clarifications would help the network to build a strategy to have an aligned UL-DL directions for all UEs and repeaters within a certain time duration, regardless of various repeater capabilities.

Conclusion

In this contribution, ETRI’s views on L1/L2 signaling to carry the SCI were shown and the following proposals were made:

Proposal 1. RAN1 to support L1 signaling to carry the SCI.

Proposal 2. RAN1 to consider the following potential (explicit and/or implicit) contents of the SCI:

- Regarding beamforming information,

  - Indexes of repeater-RU beams for a given time/frequency resources.

  - Number of available repeater-RU beams for a given time/frequency resources.

  - Indexes of repeater-MT beams for a given time/frequency resources.

  - Number of available repeater-MT beams for a given time/frequency resources.

  - Indexes of repeater beams for a given time/frequency resources.

  - Number of available repeater beams for a given time/frequency resources.

  - (Non-)Preferences on a beam set.

  - Indexes of the time resources (e.g., application timing) and/or frequency resources (e.g., applicable band or CC list) associated with the beam indication(s).

- Regarding information on UL-DL TDD configuration,

  - UL-DL direction(s) for a given time/frequency resources.

  - Time duration (frequency band) where the indicated UL-DL direction(s) shall be applied.

  - Time duration (frequency band) where the UL-DL direction(s) can be determined by repeater implementation.

- Regarding, ON-OFF information or power control information

  - Range/value of repeater gain for a given time/frequency resources.

  - Time duration (frequency band) where the indicated repeater gain shall be applied.

  - Time duration (frequency band) where the repeater gain can be determined by repeater implementation.

Proposal 3. RAN1 to study the monitoring occasion (MO) of the SCI for low-cost repeaters.

- References

- RP-213700, New SI: Study on NR Network-controlled Repeaters, ZTE

- R1-2204653, Discussion on side control information for network-controlled repeater, ETRI