3GPP TSG RAN WG1 #109-e                                        R1-2204655

e-Meeting, May 9th – 20th, 2022

Source: 	ETRI

Title:	Discussion on power saving techniques for XR

Agenda Item:	9.11.1   XR specific power saving techniques

Document for:	Discussion/Decision

Introduction

While XR and CG are attractive use cases for future mobile systems, they also impose a set of challenges for NR that need to be studied [1]. For example, during the Rel-17 SID on XR evaluations, it was identified that the current DRX configuration may not be enough to accommodate the XR traffic characteristics. Based on such observations, a new SID on XR enhancements has been approved for Rel-18 to further investigate feasibility of potential enhancement schemes including the following [1]:

Objectives on XR-specific Power Saving (RAN1, RAN2):

- Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques:

  - C-DRX enhancement.

  - PDCCH monitoring enhancement.

In this contribution, we present our initial views on power saving techniques for improving C-DRX and PDCCH monitoring operations for XR.

Discussions

Issue 1: Non-integer XR traffic periodicity

In the Rel-17 SI evaluation, 60 fps and 120 fps frame rates were typically considered for the XR video traffic [2]. Since those values correspond to non-integer traffic periodicities of 16.67 ms and 8.33 ms, respectively, they do not fit exactly to the NR C-DRX cycles that are currently supported. Therefore, mismatch error occurs between the traffic arrivals and the DRX cycles, and as time passes it would be accumulated to be larger than a tolerable latency threshold. Then, to meet the QoS of the XR traffic, gNB should reconfigure the DRX cycles to adjust its start offset whenever the error exceeds the threshold.

One potential solution is to extend the DRX configuration such that one DRX cycle is constituted by multiple time patterns. The time patterns can have their own periodicities, and the on-duration may start at the front of each time pattern. Fig. 1 illustrates one example of the extension of the DRX configuration. Noting that the sum of three traffic arrivals with arrival rate 8.33 ms makes an integer value of 25 ms, a DRX cycle of 25 ms can be configured by concatenating three time patterns with periodicity 9 ms, 8 ms, and 8 ms, respectively. This seems similar to the approach used in the NR slot format configuration where pattern 1 and pattern 2 can be concatenated. Alternatively, each time pattern can be considered as one DRX cycle. Based on those approaches, one DRX cycle or a set of DRX cycles can be exactly aligned with N(=3) times the arrival rate. Therefore, the aforementioned issue can be resolved and the UE can minimize its wake-up durations.

Proposal 1: To efficiently serve XR traffics having non-integer periodicities, study an extension of C-DRX configuration to guarantee that one DRX cycle or a bunch of DRX cycles is aligned with multiple traffic arrivals.

Fig. 1. An example of enhanced C-DRX operation

Issue 2: Multi-flow XR traffic

A UE that is being provided with the XR service may have multiple traffic flows. For example, the XR traffic may comprise multiple streams, e.g., I-frame + P-frame, video + audio/data. The XR UE may also have a normal broadcast and unicast traffic not related to the XR service. Each traffic flow may have its own arrival rate. In that case, one DRX cycle configuration would not be enough to serve the whole traffics while minimizing the UE’s wake up durations.

To improve the DRX operation toward the multi-flow traffic, in can be considered to introduce multiple DRX layers based on multiple DRX configurations within a DRX group. Each DRX layer can be configured to serve each service traffic with a proper DRX cycle. By performing the DRX operation for each DRX payer, it would be possible for UE to wake-up only when the traffic exists.

To support multi-DRX layers, some issues may need to be addressed. For example, whether some of UE behaviours, e.g., PDCCH monitoring, are applied per DRX layer or per serving cell may need to be clarified. The WUS monitoring may also be a discussion point. Currently, UE is allowed to monitor the WUS only outside the active time. However, if multiple DRX layers with different periodicity and offset are configured, it may be difficult to avoid collision of some WUS monitoring occasions with a DRX active time of a different DRX layer. Therefore, it may be beneficial to allow the WUS monitoring within the active time at least across the DRX layers.

Proposal 2: To efficiently serve multi-flow XR traffic, study feasibility and benefit of DRX operation based on multiple DRX layers (i.e., multiple DRX configurations) within a DRX group.

Issue 3: Quasi-periodic XR traffic periodicity (jitter)

Due to varying frame encoding delay, jitter may be introduced to the XR packet arrivals. In the Rel-17 SI evaluation, jitter was modelled as truncated Gaussian with a truncation range [-4 ms, 4 ms] or optionally [-5 ms, 5 ms]. Due to this nature, the packet may arrive earlier or later than expected (i.e., mean arrival time) by up to several slots. To keep the same latency budget under the timing uncertainty, DRX active time (i.e., DX on-duration timer and/or inactivity timer) needs to be configured much wider and it would increase the amount of UE power consumption.

One basic idea for enhancement is to dynamically control the DRX start offset at each DRX cycle. WUS can be considered as the L1 signalling to indicate the start offset. As depicted in Fig. 2, UE can monitor multiple WUS monitoring occasions (MOs). Each WUS MO, or each set of MOs, can be associated with a different DRX start offset in a one-to-one manner. Based on the WUS reception timing and the association information, UE may determine the start of the on-duration to wake up. Related issues may include configuration of WUS SS sets, WUS contents, definition/configuration of the DRX start offset, etc. Those details can be discussed together during the study item to materialize the feature.

Proposal 3: To efficiently handle jitter in XR packet arrival time, study dynamic DRX start offset indication based on WUS.

Fig. 2. An example of WUS-based DRX start offset indication

Conclusion

In this contribution, we discuss our initial views on power saving techniques focusing on C-DRX and PDCCH monitoring enhancements for XR. The following proposals are drawn:

Proposal 1: To efficiently serve XR traffics having non-integer periodicities, study an extension of C-DRX configuration to guarantee that one DRX cycle or a bunch of DRX cycles is aligned with multiple traffic arrivals.

Proposal 2: To efficiently serve multi-flow XR traffic, study feasibility and benefit of DRX operation based on multiple DRX layers (i.e., multiple DRX configurations) within a DRX group.

Proposal 3: To efficiently handle jitter in XR packet arrival time, study dynamic DRX start offset indication based on WUS.

References

[1] RP-220285, “Revised SID: Study on XR Enhancements for NR,” Nokia (Rapporteur), RAN #95e.

[2] TR 38.838 V17.0.0, “Study on XR (Extended Reality) Evaluations for NR (Release 17).”