3GPP TSG RAN WG1 #109e		R1-2204657

e-Meeting, May 9th – 20th, 2022

Source:	Panasonic

Title: 	Discussion on remaining issues on PUSCH repetition Type A enhancements

Agenda Item:		8.8.1

Document for:	Discussion

Introduction

This document provides our view on the remaining issues related to PUSCH repetition Type A enhancements.

Discussion

Issue 1: The slot indicated by K2 offset for the available slot counting (DG-PUSCH with K > 1 scheduled by DCI format 0-1 or 0-2)

In RAN1#108e, following was agreed.

Agreement:

- For DG-PUSCH repetition Type A scheduled by DCI format 0-1 or 0-2, the legacy counting method applies when K = 1, regardless of whether AvailableSlotCounting is enabled or not.

  - Note: The legacy assumption on the K2 offset is applied, i.e., no RAN1 specification impact on the K2 offset is expected.

- For CG-PUSCH repetition Type A, the legacy counting method applies when K = 1, regardless of whether AvailableSlotCounting is enabled or not.

  - Note: No RAN1 specification impact is expected.

  - Note: No additional restriction on CG periodicity configurations, compared to Rel.16, is considered for CG-PUSCH with K = 1.

- For CG-PUSCH repetition Type A, when AvailableSlotCounting is enabled, and for K > 1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in TS 38.321 Section 5.8.2 can be the slot which is not counted in K available slots.

  - Note: No RAN1 specification impact is expected.

The remaining issue is whether the slot indicated by K2 offset for the available slot counting shall be an available slot or not for K > 1. Following UE behaviours / assumptions were identified.

- Option 1: A UE does not assume that the slot indicated by K2 offset is the slot which is not counted in K available slot(s).

- Option 2: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

In our understanding, in Rel.15/16, there is no gNB scheduling restriction that the slot for UL transmission can’t overlap with a DL slot. It is handled by UE and the collision handling rule between DG-PUSCH and DL/SSB symbols is described in TS 38.213 Section 11.1 as follows.

For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, the UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE in the set of symbols of the slot. For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

In Rel.17, in order to avoid the dropping due to the overlap with DL slot, available slot counting is introduced. Since the collision between PUSCH and a DL slot can be avoided by available slot counting, we think there is no need to introduce any additional restriction on gNB scheduling. Therefore, Option 2 should be taken for UE behaviour.

Proposal 1: For DG-PUSCH repetition Type A scheduled by DCI format 0-1 or 0-2, when AvailableSlotCounting is enables and for K > 1, a UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

Issue 2: Clarification of out-of-order handling

The description on out-of-order handling in the specification is as follows. The yellow marked part is especially related to out-of-order handling.

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2 with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. When the UE is scheduled with multiple PUSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time among the two configured PDCCH candidates is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI.

In RAN1#108e, [1] raised the following issue on out-of-order scheduling due to available slot counting.

Considering a scenario where PDCCH received in Slot 0 schedules PUSCH in Slot 3 (using K2 offset of 3) and another PDCCH schedules a second PUSCH in Slot 4 (K2 offset of 3), this is a valid scheduling for Rel-15/16. For Rel-17, on the other hand, if Slot 3 is not an available slot for the first PUSCH transmission and the first PUSCH transmission gets deferred to Slot 4 with starting symbol later than that of the second PUSCH, then this leads to Out-of-Order. Figure 1 of R1-2202151

Based on the description of the out-of-order handling in TS 38.213, there are two check points: 1) “the end of the first PUSCH”, 2) “scheduled to transmit a (second) PUSCH”. For PUSCH repetition Type A with available slot counting, it is necessary to clarify which of the following options is applied for out-of-order determination.

- For the determination of “the end of the first PUSCH”

  - Option A1: based on available slot (i.e., end of the last possible repetition, regardless of actual transmission or not)

  - Option A2: based on the actual transmitted PUSCH (i.e., end of the last actual transmitted repetition)

- For the determination of “scheduled to transmit a (second) PUSCH

  - Option B1: based on the slot indicated by K2 value

  - Option B2: based on available slot (i.e., start of the first possible repetition, regardless of actual transmission or not)

  - Option B3: based on the actual transmitted PUSCH (i.e., start of the first actual transmitted repetition)

PUSCH repetition Type A with available slot counting is based on the 2-step procedure. In Step 1, available slots for K repetitions are determined based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH. Therefore, the slots for possible PUSCH repetition (or potential PUSCH transmission occasion) in Step 1 are determined at the time to receive/decode PDCCH. In Step 2, the UE determines whether to drop a PUSCH repetition or not according to PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. The actual PUSCH transmission in Step 2 is determined in each slot.

For the determination of “the end of the first PUSCH”, if Option A1 of available slot is taken, the out-of-order handling is deterministic at the time to receive/decode PDCCH. UE does not need to prepare PUSCH transmission to be transmitted before the end of the last possible repetition (e.g., Slot#8 in Fig.1). On the other hand, if Option A2 of actual transmission is taken, the end of the first PUSCH can be changed depending on whether to drop a PUSCH repetition or not according to PUSCH dropping rules. In this case, UE needs to prepare the case that the situation is not out-of-order, i.e., possibility to transmit PUSCH before the end of the last actual repetition, which can be dynamically changed after the time to receive/decode PDCCH scheduling first PUSCH. This is different from the design principle of PUSCH repetition Type A and it requires complex UE implementation. Therefore, we propose to take Option A1 for the determination of “the end of the first PUSCH”.

Fig.1 Determination of “the end of the first PUSCH2 for Option A1 and Option A2

For the determination of “schedule to transmit a (second) PUSCH”, if Option B1 of K2 indicated slot is taken, UE does not need to perform available slot determination overlapped in time. On the other hand, if Option B2 of available slot is taken, there might be possibility that UE needs to perform available slot determination overlapped in time as shown in Fig.2. In Fig.2, Slot#7 is available slot for PUSCH#A, while unavailable slot for PUSCH#B. Then, PUSCH#B is deferred to the next available slot. Since timeline checking is based on the first available slot in Option B2, the example in Fig.2 is not handled as out-of-order. Note that if the decision of Issue 1 is Option 1, there is no difference between Option B1 and Option B2. If Option B3 of actual transmission is taken, similar to Option A2, the start of the actual transmission can be changed depending on whether to drop a PUSCH repetition or not according to PUSCH dropping rules. In order to avoid complex UE implementation, we propose to take Option B1 for the determination of “schedule to transmit a (second) PUSCH”.

Fig.2 Example of Option B2

Proposal 2: Out-of-order handling for PUSCH repetition A with available slot counting is clarified as follows.

- For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol j.

  - “The end of the first PUSCH” for out-of-order handling is determined based on the available slot regardless of potential drop or not.

  - The start of the PUSCH scheduled by a PDCCH that ends later than symbol j, for out-of-order handling, is based on the slot indicated by K2 value.

Proposal 3: Adapt the following text proposal for clarify the out-of-order handling for PUSCH repetition Type A with available slot counting for the end of the first PUSCH. The text proposal of the start of the PUSCH is after the decision of issue #1.

TS 38.3214 Section 6.1 UE procedure for transmitting the physical uplink shared channel [Omitted] A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2 with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. When the UE is scheduled with multiple PUSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. When AvailableSlotCounting is enabled, the end of the first PUSCH is the PUSCH transmission in the last slot determined in Clause 6.1.2.1 regardless UE does not transmit the PUSCH or not. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time among the two configured PDCCH candidates is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI. [Omitted]

Conclusion

In this contribution, we provide our view on the remaining issue related to PUSCH repetition Type A enhancement. We made following proposals.

Proposal 1: For DG-PUSCH repetition Type A scheduled by DCI format 0-1 or 0-2, when AvailableSlotCounting is enables and for K > 1, a UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

Proposal 2: Out-of-order handling for PUSCH repetition A with available slot counting is clarified as follows.

- For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol j.

  - “The end of the first PUSCH” for out-of-order handling is determined based on the available slot regardless of potential drop or not.

  - The start of the PUSCH scheduled by a PDCCH that ends later than symbol j, for out-of-order handling, is based on the slot indicated by K2 value.

Proposal 3: Adapt the following text proposal for clarify the out-of-order handling for PUSCH repetition Type A with available slot counting for the end of the first PUSCH. The text proposal of the start of the PUSCH is after the decision of issue #1.

TS 38.3214 Section 6.1 UE procedure for transmitting the physical uplink shared channel [Omitted] A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2 with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. When the UE is scheduled with multiple PUSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. When AvailableSlotCounting is enabled, the end of the first PUSCH is the PUSCH transmission in the last slot determined in Clause 6.1.2.1 regardless UE does not transmit the PUSCH or not. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time among the two configured PDCCH candidates is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI. [Omitted]

- Reference

[1]	R1-2202151	“Enhancements on PUSCH Repetition Type A,”	Qualcomm Incorporated, RAN1#108e.