3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2204660

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.4

Source:	Ericsson

Title:	On NR NTN maintenance issues

Document for:	Discussion

# 1	Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the work item description is updated in [1]. An overview of 3GPP non-terrestrial networks can be found in [5] and an overview of state of the art in LEO satellite access can be found in [6].

In the status report to RAN#95-e [2], the following open issues are listed for RAN1:

The following topics may require minor corrections and hence may be treated as part of Rel-17 maintenance activity:

- Details on UE behaviour w.r.t Validity timer expiry

- Support of negative TACommonDriftVariation values for GEO

- Resolving the ambiguity in interpretation of SFN indicating Epoch time

Topics related to the third bullet are discussed in section 2.1 while a discussion around the third bullet can be found in section 2.2. The second bullet is discussed in section 2.3. In addition, a TP for the definition of UE-specific and Common TA in 38.213 is provided in section 2.4.

# 2	Discussion

## 2.1	Interpretation of SFN indicating Epoch time

At RAN1#108-e, there was discussion about the interpretation of the SFN indicating explicit Epoch time. Specifically, since the SFN wraps around every 1024 frames (10.24 seconds), which radio frame with the indicated SFN that defines the Epoch time. Candidate solutions included the last frame, the next frame, or the nearest frame with the given SFN.

According to the following observation from Thales [3], the network (NCC) can predict the satellite orbit with high precision several minutes in advance:

"Observation 11: Typical Precision Orbit Determination (initial 3D Position RMS Error = 0.5 m and 3D Velocity RMS Error = 0.5 mm/s) allows Satellite position prediction 60 seconds ahead with max error of 1.47m and 5 minutes ahead with max error of 3.87m"

On the other hand, a UE cannot be required to implement such accurate prediction algorithms due to complexity limitations. E.g., simulations in the same contribution [3] show prediction 45 seconds ahead with a prediction error of 43 meters by the UE.

Therefore, setting the Epoch time in the future can significantly reduce the need for frequent reacquisition of the ephemeris. As illustrated in the upper part of Figure 1, if the network indicates ephemeris data with Epoch time in the past, part of the validity time will have passed before the ephemeris can be used. On the other hand, as shown in the lower part of Figure 1, if the network predicts the ephemeris ahead in time and indicates this ephemeris with an Epoch time in the future, the UE can propagate the satellite orbit both backward and forward from this point, and the useful period of the received ephemeris will be significantly longer. This benefits both network and UE without any significant cost.

Figure 1: Illustration of Epoch time in the past and in the future, and the impact on the useful period of the ephemeris.

If the network indicates ephemeris with an Epoch time in the future, the UE can propagate the satellite orbit both backward and forward from this point, and the useful period of the received ephemeris will be significantly longer than with an Epoch time in the past. This benefits both network and UE without significant cost.

Support indication of explicit Epoch time through the SFN of a future radio frame.

As mentioned above, the use of SFN limits the range of the explicit Epoch time to 10.24 seconds. This means that the network has to update its broadcast ephemeris data and common TA parameters at least as often. For GEO, the ephemeris can be valid for a significantly longer time. E.g., at RAN1#108-e, a validity duration value of 900 seconds was introduced for use in GEO. Frequent updates of the ephemeris/common TA put an unnecessary burden on the NTN.

The limited range of the SFN (10.24 seconds) forces the network to frequently update the broadcast ephemeris data, which is an unnecessary burden on the network in e.g. GEO where the ephemeris can be valid for a significantly longer time.

To avoid this, it should be possible to indicate an Epoch time with a larger range. In NR Rel-17, a hyper system frame number (H-SFN) was introduced in the context of eDRX. The H-SFN is signaled in SIB1. To extend the range of the Epoch time, a few LSBs of the H-SFN can be indicated as part of the explicit Epoch time.

To extend the range of explicit Epoch time, indicate a few LSBs of the H-SFN in addition to SFN and subframe number.

## 2.2	UE behaviour related to validity duration and Epoch time

In section 2.1, we propose to support indication of Epoch time at a future point in time. For this to be meaningful, it is necessary to allow the UE to use the assistance information prior to the Epoch time (i.e., to propagate it backwards in time from the Epoch time). This topic was also discussed at RAN1#108-e without conclusion. The final proposal from the moderator (see [4]) was as follows:

Updated Proposal 6 (Rev 2)

- The UE shall re-acquire new assistance information before expiry of UL validity timer

- If a UE has obtained new serving satellite ephemeris and Common TA related parameters prior to the time of the validity timer expiring, the UE is allowed to maintain its UL synchronization until the new Epoch time is reached. For this, the time interval from the expiration of the validity timer until the new Epoch time must not be larger than the new validity duration. In this case:

  - The UE suspends the timer during this period such that it does not expire, and restarts the validity timer at the new Epoch time.

Note : UE shall always apply new assistance information obtained within uplink sync validity duration

Regarding the first bullet, it is already captured in the specifications through the following note in TS 38.331 [7]:

NOTE:      UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

Regarding the second bullet, for reasons explained in section 2.1, we believe it should be agreed in principle. We therefore put forth the following proposals.

Assistance information with an Epoch time at a future point in time is also valid for a period P before the indicated Epoch time (in addition to a period P after the indicated Epoch time), where P is given by the validity duration parameter.

If a UE has obtained new assistance information prior to the time of the validity timer of old assistance information expires, the UE is allowed to maintain its UL synchronization until the new Epoch time is reached, under condition that the validity periods of the old and new assistance information overlap. In this case, the UE applies the new assistance information as soon as it is valid, suspends the validity timer during this period such that it does not expire, and restarts the validity timer at the new Epoch time.

The solution is illustrated in Figure 2.

Figure 2: The new assistance information is valid prior to the new Epoch time when the time period until the new Epoch time is not larger than the validity duration of the new assistance information.

We also propose to inform RAN2 about this solution. Since the RAN2#118-e meeting is parallel to RAN1#109-e and the last meeting in which RAN2 can make corresponding decisions, we propose that an LS be sent as soon as possible during the RAN1#109-e meeting.

Send an LS to RAN2 to ask them take into account the solution above (assuming it is agreed by RAN1). Due to parallel RAN1/RAN2 meetings, the LS should be sent as soon as possible during the RAN1 meeting.

## 2.3	Support of negative TACommonDriftVariation values for GEO

The following value range, granularity and bits allocation for the TACommonDriftVariation parameter were agreed at RAN1#107-e:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
| TACommonDriftVariation | 0…0.60 µs/s2 | 0.2×10-4 µs/s2 | 15 bits |

At RAN1#108-e, it was noted that this parameter range was designed for LEO and did not consider that TACommonDriftVariation can be negative for GEO. It was therefore proposed to modify the encoding of TACommonDriftVariation as follows:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
| TACommonDriftVariation | -0.60 µs/s2 … 0.60 µs/s2 | 0.2×10-4 µs/s2 | 16 bits |

However, this was not agreed.

In this section, we look closer at the TACommonDriftVariation parameter for GEO. An analysis of the TACommonDriftVariation for GEO with various assumptions on inclination angle and GW latitude shows that a suitable range for the parameter is (-2×10-4 µs/s2 … 2×10-4 µs/s2), as shown in Figure 3. This confirms the need for negative drift variation values, but one can also note that the initially proposed value range is unnecessarily large.

Figure 3: TACommonDriftVariation range for GEO with different gateway latitudes and satellite inclination angles.

The common TA parameter TACommonDriftVariation can have negative values down to approximately -2×10-4 µs/s2 for GEO with large inclination angles.

Next, we consider the granularity of TACommonDriftVariation for GEO. Figure 4 shows the 99th percentile common TA approximation error vs prediction time with different assumptions on the coding of the TACommonDriftVariation. The scenario is a GEO with 10° inclination angle and a gateway at latitude 60° N. The black solid curve shows reference performance if the 2nd order parameter would be signaled without limitations in range or resolution. The red solid curve shows performance with the current agreement, i.e., that negative TACommonDriftVariation values are not allowed, in which case the parameter would have to be set to zero to minimize the error. The dashed curves show performance with different assumptions on granularity of TACommonDriftVariation. It can be seen that with granularity = 2×10-7 µs/s2, the quantization loss is insignificant.

Figure 4: Common TA approximation error with different assumptions on TACommonDriftVariation granularity.

This is summarized in the following proposal:

For GEO, the common TA parameter TACommonDriftVariation should have a value range of at least (-2×10-4 µs/s2 … 2×10-4 µs/s2) and a granularity of at least 2×10-7 µs/s2.

## 2.4	TP for 38.213 on definitions of UE-specific TA and Common TA

At RAN1#108-e, text proposals for 38.213 on definitions of UE-specific TA and Common TA were discussed but not agreed. These definitions are based on agreements from RAN1#107-e and it is necessary to capture them in the specifications. Therefore, we propose to adopt the latest TP from the final FL summary [4] from RAN1#108-e (there denoted Modified Proposal 12 (rev 1)).

Adopt the following TP for 3GPP TS 38.213:

--------------------------------- Start of TP for 3GPP TS 38.213 ----------------------------------

## 4.2  Transmission timing adjustments

<Unchanged Text Omitted>

Using higher-layer ephemeris parameters for the serving satellite, if configured, the UE calculates NTA,adjUE, using serving satellite position and its own position, to pre-compensate the two-way transmission delay on the service link.

To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the satellite, NTA,adjcommon is derived by the UE based on Delaycommont ,which can be obtained as:

Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariation2×t-tepoch2

where tepoch is the Epoch time of the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation.

This Delaycommon(t) gives the distance at time t between the satellite and the uplink time synchronization reference point divided by the speed of light.

The uplink time synchronization reference point is the point at which DL and UL are frame aligned with an offset given by NTA,offset.

---------------------------------- End of TP for 3GPP TS 38.213 ---------------------------------

# Conclusion

In the previous sections, we discuss maintenance issues for NR NTN.We made the following observations:

Observation 1	If the network indicates ephemeris with an Epoch time in the future, the UE can propagate the satellite orbit both backward and forward from this point, and the useful period of the received ephemeris will be significantly longer than with an Epoch time in the past. This benefits both network and UE without significant cost.

Observation 2	The limited range of the SFN (10.24 seconds) forces the network to frequently update the broadcast ephemeris data, which is an unnecessary burden on the network in e.g. GEO where the ephemeris can be valid for a significantly longer time.

Observation 3	The common TA parameter TACommonDriftVariation can have negative values down to approximately -2×10-4 µs/s2 for GEO with large inclination angles.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support indication of explicit Epoch time through the SFN of a future radio frame.

Proposal 2	To extend the range of explicit Epoch time, indicate a few LSBs of the H-SFN in addition to SFN and subframe number.

Proposal 3	Assistance information with an Epoch time at a future point in time is also valid for a period P before the indicated Epoch time (in addition to a period P after the indicated Epoch time), where P is given by the validity duration parameter.

Proposal 4	If a UE has obtained new assistance information prior to the time of the validity timer of old assistance information expires, the UE is allowed to maintain its UL synchronization until the new Epoch time is reached, under condition that the validity periods of the old and new assistance information overlap. In this case, the UE applies the new assistance information as soon as it is valid, suspends the validity timer during this period such that it does not expire, and restarts the validity timer at the new Epoch time.

Proposal 5	Send an LS to RAN2 to ask them take into account the solution above (assuming it is agreed by RAN1). Due to parallel RAN1/RAN2 meetings, the LS should be sent as soon as possible during the RAN1 meeting.

Proposal 6	For GEO, the common TA parameter TACommonDriftVariation should have a value range of at least (-2×10-4 µs/s2 … 2×10-4 µs/s2) and a granularity of at least 2×10-7 µs/s2.

Proposal 7	Adopt the following TP for 3GPP TS 38.213:

--------------------------------- Start of TP for 3GPP TS 38.213 ----------------------------------

## 4.2  Transmission timing adjustments

<Unchanged Text Omitted>

Using higher-layer ephemeris parameters for the serving satellite, if configured, the UE calculates NTA,adjUE, using serving satellite position and its own position, to pre-compensate the two-way transmission delay on the service link.

To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the satellite, NTA,adjcommon is derived by the UE based on Delaycommont ,which can be obtained as:

Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariation2×t-tepoch2

where tepoch is the Epoch time of the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation.

This Delaycommon(t) gives the distance at time t between the satellite and the uplink time synchronization reference point divided by the speed of light.

The uplink time synchronization reference point is the point at which DL and UL are frame aligned with an offset given by NTA,offset.

---------------------------------- End of TP for 3GPP TS 38.213 ---------------------------------

# References

RP-220208, “Solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#95-e, March 2022.

RP-220952, "Status report for WI Solutions for NR to support NTN; rapporteur: Thales", RAN2, RAN#95-e, March 2022.

R1-2111122, "Considerations on UL timing and frequency synchronization in NTN", Thales, RAN1#107-e, November 2021.

R1-2202908, "FL Summary #4: Maintenance on UL time and frequency synchronization for NR NTN ", Moderator (Thales), RAN1#108-e, February 2022

X. Lin, S. Rommer, S. Euler, E. A. Yavuz, and R. S. Karlsson, “5G from space: An overview of 3GPP non-terrestrial networks,” to appear in IEEE Communications Standards Magazine, March 2021. Available at arXiv preprint arXiv:2103.09156.

X. Lin, S. Cioni, G. Charbit, N. Chuberre, S. Hellsten, and J.-F. Boutillon, “On the path to 6G: Embracing the next wave of low Earth orbit satellite access,” to appear in IEEE Communications Magazine, April 2021. Available at arXiv preprint arXiv:2104.10533.

3GPP TS 38.331, "NR; Radio Resource Control (RRC) protocol specification", v17.0.0, March 2022.