3GPP TSG-RAN WG1 Meeting #108-e	R1- 2204661

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.4

Source:	Ericsson

Title:	On UE features for NR NTN

Document for:	Discussion

# 1	Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the latest work item description is available in [1]. An overview of 3GPP non-terrestrial networks can be found in [2] and an overview of state of the art in LEO satellite access can be found in [3].

In this contribution, we provide comments on the latest version of the endorsed list of UE features for NR NTN [4].

# 2	Guidelines for UE capability definitions

The discussion on defining UE features parameters for Rel-17 is proceeding. It is beneficial to recall RAN2’s guidelines for UE capability definitions, as documented in the RAN2 LS [5] and copied below:

1	Avoid defining “incapability” bits as they may cause interpretation issues

The definition of the capability should not say that “a UE setting the bit does not support Rel-16 feature X”. Such statements caused a lot of problems in Rel-15. One example was the pucch-F0-2WithoutFH that indicates that “the UE does not support PUCCH formats 0 and 2 without frequency hopping”.

2	Absence of a capability bit shall not imply support for something that is not yet defined

One example is “absence of channelBW means that the UE supports all BWs”. Since new BWs might be added later (and in a release independent manner) and maybe in some other specification, “all BWs” can become ambiguous. Any legacy UE would suddenly be expected to support the new bandwidth as well.

Another example is the default capability for UE power class and UL duty cycle. From signalling point of view, reducing the overhead is marginal (unless it is reported per-BandCombination or Per-Band-of-a-BandCombination as discussed in bullet 4 below). Instead, gNB faces difficulty in comprehending the implicit capabilities amongst UEs compliant with different versions in a release.

3 	Define baseline feature bits that covers the minimum parts of a feature and use additional bits for parts that can be used together with the baseline feature

If a feature has a baseline feature, the baseline feature must be clearly defined. It is not enough to write that “if the UE does not set the bit it does not support feature group x-y”.  If a capability bit for a certain “feature X” has been defined, all UEs that support “feature X” shall set the bit, and additional parts of the feature may not be supported by all UEs.

Capability signalling and rules to determine whether a UE supports a feature should not be mixed. For example, avoid to say

“In TDD band combinations, only UEs that set the featureX capability bit support feature. In FDD-only band combinations all UEs support feature X even if they do not set the bit.”

If later FDD-only band combinations are defined, where UEs are not required to support the feature, band combinations will not be release independent. An example where this caused problem in Rel-15 was the capability simultaneousRx-Tx for EN-DC.

4 	Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination”

The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account.

It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered.

5 	Avoid defining functionality that has no RRC configuration but is dependent on capability bits.

The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting  the reported capabilities.

A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

6	UE capabilities defined in specifications must be self-contained

RAN2 would also like to point out that once the “snapshot” of defined Rel-16 capabilities have been captured in RAN2 TR 38.822, RAN2 does not plan to update this TR. Discussions (on any WG capabilities) should be based on the actual specification text. RAN2 will document the capabilities in TS 3x.306, preferably with references to RAN1/RAN4 specifications and to corresponding RRC configurations (fields and information elements).

RAN2 understands that RAN1 uses the “feature IDs”, since they are the only available definitions during the initial stage of discussions. RAN2 would like to request that once the RRC signalling is in place and can be mapped to each “feature ID”, the actual field names used in the signalling are also utilized in the RAN1/RAN4 capability discussions (to avoid inter-WG confusion e.g. in LSs).

7	Rationale for necessity of both xDD and FRx differentiations for per-UE capability

RAN2 did not discuss the RAN1 LS on XDD-FRX Differentiation (R1-1913579/R2-2000013) at the RAN2#109-e meeting, but RAN2 would anyway appreciate to be provided with rationale in case Per-UE capability with both FDD/TDD and FR1/FR2 differentiations is deemed as necessary for a feature.

# 3	Detailed comments on the initial list of UE features for NR NTN

Our detailed comments on the endorsed list of UE features for NR NTN are in the table below (with Track Changes).

Adopt the changes proposed in the column “Ericsson comments”.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional | Ericsson comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - Support of common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) - For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - Support of pre-compensation of the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - Support of receiving cell-specific K_offset/K_mac in system information |  | No | No | Release 17 NR UE cannot communicate via satellite | per band | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported. | Align wording by consistent use of (e.g.) "support of". Editorial change (one comma added) |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information related to TA pre-compensation | 1. Support UE reporting of information related to TA pre-compensation | 26-1 | Yes | No | UE does not support reporting of information related to TA pre-compensation for NR communication via satellite | Per band | No | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling | Editorial change (one space added) |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | Per band | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling | Editorial change (deleted "supported]" |
| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook enhancement | - Support of type-2 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |  |
| 26. NR_NTN_solutions | 26-6a | Type-1 HARQ codebook enhancement | - Support of Type-1 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |  |
| 26. NR_NTN_solutions | 26-6b | Type-3 HARQ codebook enhancement | - Support of Type-3 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |  |
| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | UE is not able to take the advantage of polarization information to save power | Per band | No | No | No | [For UE supports NR communication via satellite, UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |  |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Support reception of UE-specific K_offset via MAC-CE - Support determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific K_offset | 26-1, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling | Align wording by consistent use of (e.g.) "support of". |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the changes proposed in the column “Ericsson comments”.

# References

RP-220208, “Solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#95-e, March 2022.

X. Lin, S. Rommer, S. Euler, E. A. Yavuz, and R. S. Karlsson, “5G from space: An overview of 3GPP non-terrestrial networks,” to appear in IEEE Communications Standards Magazine, March 2021. Available at arXiv preprint arXiv:2103.09156.

X. Lin, S. Cioni, G. Charbit, N. Chuberre, S. Hellsten, and J.-F. Boutillon, “On the path to 6G: Embracing the next wave of low Earth orbit satellite access,” to appear in IEEE Communications Magazine, April 2021. Available at arXiv preprint arXiv:2104.10533.

R1-2202928, "Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e", Moderators (AT&T, NTT DOCOMO, INC.), RAN1#108-e, February 2022

R1-2001513, Guidelines for UE capability definitions.

R1-1909953, LS on ambiguity of UE L1 FDD/TDD FR1/FR2 capabilities.

R1-1911741, Reply LS on ambiguity of UE L1 FDD/TDD FR1/FR2 capabilities.