3GPP TSG RAN WG1 #109-e	R1-2204664

e-Meeting, May 9th – 20th, 2022

Source:	Sharp

Title:	PUSCH enhancements for Rel-17 CovEnh

Agenda Item:	8.8.1

Document for:	Discussion and Decision

Background

In this contribution, we present our views on the necessary corrections related to the PUSCH enhancements for CovEnh, including enhancements on PUSCH repetition type A, TB processing over multi-slot PUSCH, and Type A PUSCH repetitions for Msg3.

Discussions

  - Enhancements on PUSCH repetition type A

The slot indicated by K2 offset for the available slot counting (DG-PUSCH with K>1 scheduled by DCI format 0_1 or 0_2)

For DG-PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, it was agreed in RAN1#108-e that the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not. It is widely understood that, when K=1, the reasonable legacy gNB schedulers do not set the K2 offset value that indicates an invalid slot for UL transmission. Therefore, the same assumption could be applied to Rel-17 PUSCH with K=1 when AvailebleSlotCounting is enabled.

It was proposed that such a restriction should apply to the available slot counting with K>1, too. One of the motivation to do so is that the change of the 1st symbol of the scheduled PUSCH transmission would lead to the uncertainty on whether the Out-of-Order (OoO) check has to be done before or after the determination of available slots. In RAN1#108-e, some companies preferred the OoO check to be done after the determination of available slots, while some other companies thought the OoO check should be based only on the K2 offset value. In order to avoid the creation of a new issue, we recommend adopting the restriction on K2 offset value for the PUSCH with K>1 and with available slot counting.

On the other hand, for K>1, the large number of companies expressed their views that Rel-15/16 UEs need to assume that overlapping between PUSCH symbols in the slot indicated by K2 offset and DL symbols may occur, while some companies assumed that there was no reason for Rel-15/16 gNB scheduler to use such a K2 offset value even for K>1. Therefore, if it is preferable to have the restriction that the UE may assume that the slot indicated by K2 to be always counted as an available slot for the scheduled PUSCH, it should be clarified in the group.

Proposal 1:

- For DG-PUSCH with K>1 scheduled by DCI format 0_1 or 0_2 and when AvailebleSlotCounting is enabled, the UE may assume that the slot indicated by K2 to be always counted as an available slot for the scheduled PUSCH.

Use of SSBs across inter-cell multiple TRPs for the available slot determination

In NR Rel-17 feMIMO WI, it was agreed to support different SSB sets with different PCIs transmitted from mTRPs in a same serving cell. In RAN1#107-e and RAN1#107bis-e, we discussed whether to consider more than one SSB sets for available slot counting for CovEnh, considering the following 3 options.

- Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

- Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

- Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

However, it was pointed out that there had not been much discussion on the collision between UL transmission and SSBs in feMIMO session yet, and as such, it was concluded with the remark that this issue would be discussed after achieving a progress in feMIMO discussion.

In RAN1#108-e, the following agreement was made in feMIMO session.

Agreement For inter-cell mTRP, UE does not transmit PUCCH/PUSCH/PRACH in a slot or SRS in the symbols if in time domain the PUCCH/PUSCH/PRACH/SRS overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI.

The agreed procedure is already captured in the current TS38.213 section 11.1 as follows.

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE for reception of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

In our view, the same principle should be applied to the available slot determination, that is to say, for inter-cell mTRP, UE does not count a slot for PUSCH repetitions if in time domain the PUSCH overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI. The associated TP can be seen in Appendix.

Proposal 2:

- For inter-cell mTRP, UE does not count a slot for PUSCH repetitions if in time domain the PUSCH overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI.

Frequency hopping procedure for PUSCH scheduled by RAR UL grant or by DCI format 0_0

In RAN1#108-e, it was discussed whether/how TS38.214 section 6.3.1 needs to be updated such that the frequency hopping procedure for PUSCH scheduled by RAR UL grant or by DCI format 0_0 is clarified. During the several rounds of discussions, no consensus on any particular TP was reached. On the other hand, people were mostly fine with the following conclusion proposed by FL. However, the conclusion was not formally taken during the GTW session, because of the lack of time.

The latest version of Proposed conclusion on Issue#2-1: - It is RAN1’s common understanding that: - For PUSCH scheduled (with or without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE is not required to read frequencyHopping in pusch-Config for the determination of frequency hopping type or for the determination of whether the frequency hopping is configured for the PUSCH and is required to read the frequency hopping flag information field of the RAR UL grant or the DCI format 0_0 with CRC scrambled by TC-RNTI for the determination of whether the frequency hopping is configured for the PUSCH or not. - For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the conclusion for R1-2003594 in RAN1#101-e is applied.

We still see the benefit to make it as a conclusion so as to avoid repeated discussions in the future.

Proposal 3:

- Take the following as a conclusion:

  - It is RAN1’s common understanding that:

    - For PUSCH scheduled (with or without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE is not required to read frequencyHopping in pusch-Config for the determination of frequency hopping type or for the determination of whether the frequency hopping is configured for the PUSCH and is required to read the frequency hopping flag information field of the RAR UL grant or the DCI format 0_0 with CRC scrambled by TC-RNTI for the determination of whether the frequency hopping is configured for the PUSCH or not.

  - For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the conclusion for R1-2003594 in RAN1#101-e is applied.

Editorial issues

For TBoMS, the available slot counting is used, irrespective of whether AvailableSlotCounting is enabled. However, in the current specification, the available slot counting procedure for HD-RedCap UE is with the condition of “AvailableSlotCounting is enabled”. In addition, in RAN1#108-e, it was agreed that the legacy counting is used for K=1, which has not been captured for HD-RedCap UE cases in paired spectrum and SUL band. To resolve these issues, the following TP is proposed.

Proposal 4:

- Adopt Text proposal #1.

##### Text proposal#1 for TS38.214 v17.1.0 - 6.1.2.1 Resource allocation in time domain [Omitted] For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - For the case of a reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 with K>1 when AvailableSlotCounting is enabled, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. [Omitted]

| Reason for change | - In the current specification, the available slot counting procedure for HD-RedCap UE is always with the condition of “AvailableSlotCounting is enabled”. In addition, although it was agreed that the legacy counting is used for K=1, that has not been captured for HD-RedCap UE cases in paired spectrum and SUL band |
| --- | --- |
| - Summary of change | - For paired spectrum and SUL band and for the case of a reduced capability half-duplex UE, the position of the condition “when AvailableSlotCounting is enabled” is changed such that it applies only to PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 but not to a PUSCH transmission of TB processing over multiple slots. “with K>1” is added in the same sentence such that the available slot counting does not apply to K=1 case. |
| - Consequences if not approved | - For HD-RedCap UE with TBoMS, the available slot counting is applied only when AvailableSlotCounting is enabled. Besides, the available slot counting applies to HD-RedCap UE with PUSCH repetition Type A with K=1 when AvailableSlotCounting is enabled. |

  - TB processing over multi-slot PUSCH

Editorial issues

Although it was agreed that the slots for TBoMS is counted with the available-slot basis, available slot determination procedure for TBoMS with configured grant has not been captured in the specification yet. Therefore, we propose the following text proposal.

Proposal 5:

- Adopt Text proposal #2.

##### Text proposal#2 for TS38.214 v17.1.0 ##### 6.1.2.3.3 Transport Block repetition for uplink transmissions of TB processing over multiple slots with a configured grant The procedures described in this clause apply to PUSCH transmissions of TB processing over multiple slots with a Type 2 configured grant. The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig, the nth transmission occasion among N∙K transmissions occasions, n=0,1, …, N∙K -1, is associated with (mod((n-mod(n, N))/N,4)+1)th value in the configured RV sequence. When K=1, or when K>1 and the configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of the transport block may only start at the first transmission ocasion of the N∙K transmission occasions. Otherwise, the initial transmission of the transport block may start at - The first transmission occasion of the N∙K transmission occasions if the configured RV sequence is {0,2,3,1}. - Any transmission occasion n associated with RV=0, and for which n mod N =0, if the configured RV sequence is {0,3,0,3} or {0,0,0,0}. The UE is not expected to be configured with the time duration for N∙K transmissions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available in a slot for the PUSCH transmission of TB processing over multiple slots is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion. For unpaired spectrum: - The UE determines N∙K slots for a PUSCH transmission of a TB processing over multiple slots, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst. - A slot is not counted in the number of N∙K slots for PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a TB processing over multiple slots. - For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a TB processing over multiple slots. A slot is not counted in the number of N∙K slots for a PUSCH transmission of a TB processing over multiple slots, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots, the UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. A Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots is omitted in a slot according to the conditions in clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213].

| Reason for change | - The available slot counting procedures for CG-PUSCH with TBoMS have not been captured. |
| --- | --- |
| - Summary of change | - For CG-PUSCH with TBoMS, capture the available slot counting procedures which is similar to those for CG-PUSCH repetition Type A. |
| - Consequences if not approved | - The CG-PUSCH with TBoMS follows the legacy counting, which is not aligned with the agreement that the available slot counting is used for TBoMS. |

Conclusion

In this contribution, we have the following observations and proposals:

Proposal 1:

- For DG-PUSCH with K>1 scheduled by DCI format 0_1 or 0_2 and when AvailebleSlotCounting is enabled, the UE may assume that the slot indicated by K2 to be always counted as an available slot for the scheduled PUSCH.

Proposal 2:

- For inter-cell mTRP, UE does not count a slot for PUSCH repetitions if in time domain the PUSCH overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI.

Proposal 3:

- Take the following as a conclusion:

  - It is RAN1’s common understanding that:

    - For PUSCH scheduled (with or without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE is not required to read frequencyHopping in pusch-Config for the determination of frequency hopping type or for the determination of whether the frequency hopping is configured for the PUSCH and is required to read the frequency hopping flag information field of the RAR UL grant or the DCI format 0_0 with CRC scrambled by TC-RNTI for the determination of whether the frequency hopping is configured for the PUSCH or not.

  - For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the conclusion for R1-2003594 in RAN1#101-e is applied.

Proposal 4:

- Adopt Text proposal #1.

Proposal 5:

- Adopt Text proposal #2.

References

- 3GPP TS38.211V17.1.0, March 2022

- 3GPP TS38.212V17.1.0, March 2022

- 3GPP TS38.213V17.1.0, March 2022

- 3GPP TS38.214V17.1.0, March 2022

- R1-2202569, “FL Summary #4 on Enhancements on PUSCH repetition type A,” Moderator (Sharp), February 2022.

Appendix

  - TP for the use of SSBs across inter-cell multiple TRPs for the available slot determination

##### Text proposal#3 for TS38.214 v17.1.0 - 6.1.2.1 Resource allocation in time domain [Omitted] For unpaired spectrum: - When AvailableSlotCounting is enabled, and in case K>1, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2. - A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol for reception of an SS/PBCH block with index provided by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states. - Otherwise, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - The UE determines N∙K slots for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2. - A slot is not counted in the number of N∙K slots for a PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol for reception of an SS/PBCH block with index provided by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states. [Omitted]

| Reason for change | - The collision between PUSCH and an SS/PBCH block associated with an additional PCI for inter-cell mTRP has not been considered for the available slot counting for PUSCH repetition Type A and TBoMS. |
| --- | --- |
| - Summary of change | - Capture that if a PUSCH would overlap in symbols with an SS/PBCH block associated with an additional PCI for inter-cell mTRP in a slot, the UE does not count the slot as available for the PUSCH repetitions or the TBoMS. |
| - Consequences if not approved | - When a PUSCH would overlap in symbols with an SS/PBCH block associated with an additional PCI for inter-cell mTRP in a slot, the UE counts the slot as available for the PUSCH repetitions of the TBoMS even though the PUSCH in the slot is anyway dropped because of the collision. |