3GPP TSG RAN WG1 #109-e	R1-2204665

e-Meeting, May 9th – 20th, 2022

Source:	Sharp

Title:	UE feature for Rel-17 coverage enhancement

Agenda Item:	8.16.8

Document for:	Discussion and Decision

Background

In [1], the updated list of UE features for Rel-17 has been provided. In this contribution, we discuss remaining issues on UE features for Rel-17 coverage enhancement.

Discussion

  - Enhancements on PUSCH repetition type A

| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17, [11-6, 30-1]} | Yes | N/A | UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In RAN1#108-e, FGs for PUSCH Type A repetitions were refined. However, there is still one remaining thing to be determined, which is the prerequisite FGs for FG 30-2. In our view, the UE supporting the available slot counting should be capable of some kind of PUSCH repetitions, i.e., at least one of Rel-15, 16 or 17 PUSCH repetitions, because it has been agreed that the available slot counting is applicable only for K>1. In this sense, “One of {5-14, 5-16, 5-17, 11-6, 30-1}” would be an appropriate prerequisite FGs for FG 30-2. At the same time, it is also true that UEs capable of FG 11-6 or 30-1 need to be capable of “one of {5-16, 5-17}” or “one of {5-14, 5-16, 5-17}”, respectively, anyway. Therefore, just putting “One of {5-14, 5-16, 5-17}” as prerequisite for FG 30-2 should also work.

Proposal 1:

For the prerequisite FGs for FG 30-2, “One of {5-14, 5-16, 5-17, 11-6, 30-1}” is slightly preferred. We can also live with “One of {5-14, 5-16, 5-17}”.

  - TB processing over multi-slot PUSCH

| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For TBoMS, there are still a couple of remaining things to be determined.

One of them is the prerequisite FGs for FG 30-3. In RAN1#108-e, it was concluded that TBoMS with Type-1 CG would not be supported. As the number of allocated slot for a single TBoMS needs to be indicated by TDRA field value of the DCI format 0_1 or 0_2, TBoMS can be considered to require the support of the dynamic repetition factor indication, i.e., FG 11-6.

Proposal 2:

The prerequisite FGs for FG 30-3 should be “11-6”.

| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Another remaining issue related to TBoMS is the prerequisite FGs for FG 30-3a. As FG 30-3a is about repetitions of TBoMS, it is straightforward that FG 30-3a requires the single-TBoMS feature, i.e. FG 30-3, as well as the dynamic repetition factor indication feature, i.e. FG 11-6.

Proposal 3:

The prerequisite FGs for FG 30-3a should be “11-6 and 30-3”.

  - Type A PUSCH repetitions for Msg3

| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI] |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | [No] | [No] | [N/A ] |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For Type A PUSCH repetitions for Msg3, the only remaining issue is whether the FG 30-6 is per UE or per band/scenario. In our view, per-band granularity is not necessary for this feature, because we do not believe Msg 3 repetitions would lead to any significant difference in different bands. Per-UE with differentiation of NTN or unlicensed would be a reasonable option, considering the IODT for those scenarios.

Proposal 4:

FG 30-6 should be per UE with differentiation of NTN or unlicensed.

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

For the prerequisite FGs for FG 30-2, “One of {5-14, 5-16, 5-17, 11-6, 30-1}” is slightly preferred. We can also live with “One of {5-14, 5-16, 5-17}”.

Proposal 2:

The prerequisite FGs for FG 30-3 should be “11-6”.

Proposal 3:

The prerequisite FGs for FG 30-3a should be “11-6 and 30-3”.

Proposal 4:

FG 30-6 should be per UE with differentiation of NTN or unlicensed.

References

- R1-2202928, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e”, Moderators (AT&T, NTT DOCOMO, INC.), February 2022.

- R1-2201517, “Summary on UE features for NR coverage enhancement,” Moderator (NTT DOCOMO, INC.), February 2022.