3GPP TSG RAN WG1 #109-e	R1-2204666

e-Meeting, May 9th – 20th, 2022

Source:	Sharp

Title:	Views on SL positioning scenarios and requirements

Agenda Item:	9.5.1.1

Document for:	Discussion and Decision

Introduction

In RAN#94e, a new study item on “expanded and improved NR positioning” was approved.

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we share the view on SL positioning scenarios and requirements.

Discussions

As described in SID of Rel-18 Positioning [1], for sidelink positioning, 4 use cases (V2X, public safety, commercial with ranging, IIoT) can be mainly considered and the required positioning methods and physical structure are needed to be identified to satisfy each requirement for each use case.

In Rel-17, scenarios and requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases was studied [2] and the requirement for V2X case and public safety were captured in TR38.845 [3].

V2X

Requirement

In TR38.845, for V2X use case, following three sets of categorisations were captured:

- Set 1: 10 – 50 m with 68 – 95 % confidence level. This includes Group 1 in 5GAA TRs I & II [4] and Service level 1 in TS22.261 [5].

- Set 2: 1 – 3 m with 95 – 99 % confidence level. This includes Group 2 in 5GAA TRs I & II [4], Service level 2, 3, 4 in TS22.261 [5].

- Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level. This includes Group 3 in in 5GAA TRs I & II [4], Service level 5, 6, 7 in TS22.261 [5].

Therefore, for this case, this categorization and the requirement in 38.845 should be the baseline.

Coverage scenario

In this scenario, the positioning is mainly used for autonomous driving aligned with a road lane and for collision avoidance. In such use case, the position of the UE should not depend on the cell coverage. Therefore, for V2X scenario, all coverage scenario (i.e., in-coverage, partial-coverage and out-of-coverage) should be covered.

Spectrum

For V2X, the usage of sidelink positioning may be for safety and security. Therefore, we assume that dedicated spectrum has first priority to consider, and it can be licensed spectrum. On the other hand, unlicensed band should be deprioritized.

Proposal 1:

For V2X scenario, following should be considered:

- Requirement: following three sets defined in 38.845

  - Set 1: 10 – 50 m with 68 – 95 % confidence level. This includes Group 1 in 5GAA TRs I & II and Service level 1 in TS22.261.

  - Set 2: 1 – 3 m with 95 – 99 % confidence level. This includes Group 2 in 5GAA TRs I & II, Service level 2, 3, 4 in TS22.261.

  - Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level. This includes Group 3 in in 5GAA TRs I & II, Service level 5, 6, 7 in TS22.261.

- Coverage scenarios: in-coverage, partial-coverage, out-of-coverage

- Spectrum: dedicated (ITS) (1st priority), licensed (2nd priority)

Public safety

Requirement

As same as V2X, for public safety use case, following requirement was captured in TR38.845.

1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability

Therefore, as same as V2X, this requirement should be the baseline.

Coverage scenario

For public safety use case, the coverage should not be limited only for in-coverage but also for partial-coverage and out-of-coverage.

Spectrum

Regarding spectrum, at least dedicated spectrum should be considered, and also licensed spectrum can be covered.

Proposal 2:

For public scenario, following should be considered:

- Requirement: following requirement defined in 38.845

- 1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability

- Coverage scenarios: in-coverage, partial-coverage, out-of-coverage

- Spectrum: dedicated or licensed

Commercial with ranging

Regarding commercial with ranging scenario, absolute positioning may not be needed but relative positioning can be mainly focused. Moreover, for the ranging purpose, it may not need to cover measurement with such a long distance.

For this scenario, the application to use the positioning is still not clear. Therefore, the detail of the requirement should be studied after the clarification of the application.

Proposal 3:

For commercial with ranging scenario, the more concrete applications for the positioning should be firstly clarified.

IIoT

For IIoT scenario, the main use can be the positioning in factories including distribution warehouse. In 22.104 [6], high accuracy positioning use case for the factories is captured in Table 5.7.1-1 (as shown in Appendix). If the IIoT scenario is considered, whether each of the use cases of the table should be realized by sidelink positioning or not should be clarified. If some of the use cases are decided to be covered by sidelink positioning, then the corresponding requirements in the table would be applied as a baseline.

Proposal 4:

For IIoT scenario, whether each of the industrial use cases captured in TS 22.104 should be realized by sidelink positioning or not should be clarified.

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

For V2X scenario, following should be considered:

- Requirement: following three sets defined in 38.845

  - Set 1: 10 – 50 m with 68 – 95 % confidence level. This includes Group 1 in 5GAA TRs I & II and Service level 1 in TS22.261.

  - Set 2: 1 – 3 m with 95 – 99 % confidence level. This includes Group 2 in 5GAA TRs I & II, Service level 2, 3, 4 in TS22.261.

  - Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level. This includes Group 3 in in 5GAA TRs I & II, Service level 5, 6, 7 in TS22.261.

- Coverage scenarios: in-coverage, partial-coverage, out-of-coverage

- Spectrum: dedicated (ITS) (1st priority), licensed (2nd priority)

Proposal 2:

For public scenario, following should be considered:

- Requirement: following requirement defined in 38.845

- 1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability

- Coverage scenarios: in-coverage, partial-coverage, out-of-coverage

- Spectrum: dedicated or licensed

Proposal 3:

For commercial with ranging scenario, the more concrete applications for the positioning should be firstly clarified.

Proposal 4:

For IIoT scenario, whether each of the industrial use cases captured in TS 22.104 should be realized by sidelink positioning or not should be clarified.

References

RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, Dec. 2021

RP-201518, Revised SID on Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases, LG Electronics, Sep. 2020

TR 38.845, V17.0.0 (2021-12)

5GAA TR, C-V2X Use Cases and Service Level Requirements Volume I&II

TS 22.261 V18.5.0 (2021-12)

TS 22.104, V18.3.0 (2021-12)

- Appendix

- TS 22.104 Table 5.7.1-1: Positioning performance requirements

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | Heading | Latency for position estimation of UE | UE speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mobile control panels with safety functions (non-danger zones) | < 5 m | < 3 m | 90 % | n/a | < 5 s | n/a | Service Level 2 |
| Process automation – plant asset management | < 1 m | < 3 m | 90 % | n/a | < 2 s | < 30 km/h | Service Level 3 |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1 m (relative positioning) | n/a | 99 % | n/a | 1 s | < 30 km/h | Service Level 3 |
| Augmented reality in smart factories | < 1 m | < 3 m | 99 % | < 0.17 rad | < 15 ms | < 10 km/h | Service Level 4 |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | < 3 m | 99.9 % | < 0.54 rad | < 1 s | n/a | Service Level 4 |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) | < 50 cm | < 3 m | 99 % | n/a | 1 s | < 30 km/h | Service Level 5 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | < 3 m | 99.9 % | n/a | 10 ms | < 30 km/h | Service Level 6 |
| Inbound logistics for manufacturing (for storage of goods) | < 20 cm | < 20 cm | 99 % | n/a | < 1 s | < 30 km/h | Service Level 7 |