3GPP TSG RAN WG1 #109-e	R1-2204667

e-Meeting, May 9th – 20th, 2022

Source:	Sharp

Title:	Views on potential solutions for SL positioning

Agenda Item:	9.5.1.3

Document for:	Discussion and Decision

Introduction

In RAN#94e, a new study item on “expanded and improved NR positioning” was approved [1].

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we share the view on potential solutions for SL positioning.

Discussions

Positioning methods

In Rel-16, various location technologies to support regulatory as well as commercial use cases. Then, to address the higher accuracy, lower latency, high integrity and reliability requirements, the enhanced location technologies were introduced in Rel-17 [2]. Moreover, In Rel-18, the additional technologies to support improved positioning accuracy, integrity, and power efficiency (i.e. agenda item 9.5.2) will be considered.

For sidelink positioning, the target scenario/use cases will be discussed in agenda item 9.5.1.1. Once the scenario/use cases are determined, the requirement for Rel-18 sidelink positioning would be clarified. Then, based on the requirement, at least whether/which of the Rel-16/17 positioning technologies are reused for sidelink positioning should be discussed. It can be considered whether Rel-18 enhancements discussed under 9.5.2 are also applicable to sidelink, if necessary.

Proposal 1: As the potential positioning methods for sidelink, whether/which of the Rel-16/17 positioning technologies are reused for sidelink positioning should be discussed.

Sidelink reference signals for positioning purpose

In Rel-16/17, as reference signals for positioning, PRS for DL positioning measurement and SRS for positioning for UL positioning measurement have been adopted. For sidelink positioning, the starting point for the discussion of the required reference signal should be these PRS and SRS for positioning structure.

For the detailed structure of the reference signals, the situation to apply the reference signals may be different with Rel-16/17 positioning cases. For example, when the PRS is applied on sidelink positioning, each sidelink UE may transmit the PRS and the occupancy by the signalled PRSs may be drastically increased compared with the case of DL PRS measurement. On the other hand, the required transmission power for sidelink positioning may be lower than the DL PRS. When the required resources/periodicity for each reference signal is studied, the consideration such differences is necessary.

At the same time, the achievable performance of latency and/or accuracy and the overhead of the reference signals are in a trade-off relationship. When the reference signal for sidelink positioning is designed, the trade-off between the performance of sidelink positioning and the overhead of the reference signals should be considered.

Proposal 2: For reference signals for sidelink positioning, the starting point for the discussion of the structure is PRS and SRS for positioning in Rel-16/17.

- The difference of the SL environment from DL/UL should be considered

- The trade-off between the performance of sidelink positioning and the overhead of the reference signals should be considered

Conclusion

In this contribution, we have the following proposals:

Proposal 1: As the potential positioning methods for sidelink, whether/which of the Rel-16/17 positioning technologies are reused for sidelink positioning should be discussed.

Proposal 2: For reference signals for sidelink positioning, the starting point for the discussion of the structure is PRS and SRS for positioning in Rel-16/17.

- The difference of the SL environment from DL/UL should be considered

- The trade-off between the performance of sidelink positioning and the overhead of the reference signals should be considered

References

RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, Dec. 2021

RP-210903, Revised WID on NR Positioning Enhancements, Intel, CATT, Mar. 2021