3GPP TSG RAN WG1 #109-e	R1-2204670

e-Meeting, May 9th – 20th, 2022

Source:	Sharp

Title:	Views on low power high accuracy positioning

Agenda Item:	9.5.2.3

Document for:	Discussion and Decision

Introduction

In RAN#94e, a new study item on “expanded and improved NR positioning” was approved [1].

- Improved accuracy, integrity, and power efficiency: - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible. - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4]: - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this contribution, we share the view on low power high accuracy positioning.

Discussions

Although Rel-17 positioning has mainly focused on improvement of positioning accuracy, latency and network efficiency, many scenarios require low power consumption with enough positioning accuracy and/or integrity. In SA, the requirements of low power high accuracy positioning have been discussed and captured in TS 22.104 [2] (as shown in appendix). In our view, the use case 6 (Flexible modular assembly area: Tracking of workpiece (in- and outdoor) in assembly area and warehouse) is the one of the reasonable use cases and the requirements of the use case 6 as shown in the following table can be used as is, for the LPHAP study.

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |

Proposal 1: The requirements of the use case 6 as defined TS 22.104 can be used as is, for the LPHAP study.

When low power high accuracy positioning is studied, two aspects should be considered. How high accuracy can be achieved depends on how much resources are used in frequency domain and/or time domain. On the other hand, how much power consumption reduction can be achieved depends on how long duration without any transmission are kept.

For balancing of the two aspects, it is important how to allocate the reference signals for positioning to the resources in time and/or frequency domain. For example, the duration where UEs perform measurements for positioning should be limited as much as possible, so that the UEs can sleep outside the duration.

As the power saving technology for inactive/idle mode, DRX is the one of the effective approaches. Therefore, by fitting the configuration of DRX and positioning procedure (i.e. resource allocation of reference signals), the optimization achieving both accuracy and low power consumption should be realized.

For the consideration, there may be two approaches:

1) firstly, how much frequency/time resources are necessary to satisfy the required accuracy (e.g. 1m) is identified. Then, under the required resource condition, whether the existing DRX cycle can satisfy the low power consumption requirement or not is studied.

2) firstly, the DRX cycle which achieves the low power consumption requirement is identified. Then, with the identified DRX cycle, whether the frequency/time resources can satisfy the accuracy requirement (e.g. 1m) or not is studied.

In both approaches, if necessary, the enhancement of DRX and/or positioning procedures should be considered.

Proposal 2: To achieve both accuracy and low power consumption, the required frequency/time resources for the accuracy requirement and the DRX cycle for the low power consumption requirement are jointly considered.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: The requirements of the use case 6 as defined TS 22.104 can be used as is, for the LPHAP study.

Proposal 2: To achieve both accuracy and low power consumption, the required frequency/time resources for the accuracy requirement and the DRX cycle for the low power consumption requirement are jointly considered.

References

RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, Dec. 2021

TS 22.104, V18.3.0 (2021-12)

- Appendix

- TS 22.104

Table A.7.2-1: Low power high accuracy positioning use cases

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

Use case one

Process automation: Dolly tracking (outdoor).

Use case two

Process automation: Asset tracking.

Use case three

Flexible modulare assembly area: Tool tracking in flexible, modular assembly areas in smart factories.

Use case four

Process automation: Sequence container (Intralogistics).

Use case five

Process automation: Palette tracking (e.g. in turbine construction).

Use case six

Flexible modulare assembly area: Tracking of workpiece (in- and outdoor) in assembly area and warehouse.

Use case seven

Flexible modulare assembly area: Tool assignment (assign tool to vehicles in a production line, left/right) in flexible, modular assembly area in smart factories.

Use case eight

Flexible modulare assembly area: Positioning of autonomous vehicles for monitoring purposes (vehicles in line, distance 1.5 meter).

Use case nine

- (Intra-)logistics: Asset tracking