3GPP TSG RAN WG1 #109-e	R1-2204671

e-Meeting, May 9th – 20th, 2022

Source:	Sharp

Title:	Views on positioning for RedCap UEs

Agenda Item:	9.5.3

Document for:	Discussion and Decision

Introduction

In RAN#94e, a new study item on “expanded and improved NR positioning” was approved [1].

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review. - Improved accuracy, integrity, and power efficiency: - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible. - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4]: - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state - Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In this contribution, we share the view on positioning for RedCap UEs.

Discussions

In Rel-17, RedCap devices have been standardized for 3 use cases, massive industrial wireless sensor network (IWSN), video surveillance and wearables [2]. Among these use cases, the target for RedCap positioning may be IWSN for IIoT (and can be wearables), where the requirement should be based on following (as described in Rel-17 RedCap WID):

- Industrial wireless sensors: Reference use cases and requirements are described in TR 22.832 and TS 22.104: Communication service availability is 99.99% and end-to-end latency less than 100 ms. The reference bit rate is less than 2 Mbps (potentially asymmetric e.g. UL heavy traffic) for all use cases and the device is stationary. The battery should last at least few years. For safety related sensors, latency requirement is lower, 5-10 ms (TR 22.804)

Proposal 1: Among three Rel-17 RedCap use cases, the target for positioning should be IWSN for IIoT.

For IWSN use case, one of the key elements would be the battery life. When considering the positioning with RedCap UEs which have to support the longer battery life, the positioning schemes with low power consumption are necessary. In TS 22.104 [3], the low power high accuracy positioning use cases with various combination of accuracy levels, duty cycles and battery life lengths have been captured in the Table A.7.2-1. The categorization in this table is not limited for non-RedCap devices but is also applicable for RedCap devices. Therefore, the use case of the RedCap positioning should be selected among the use cases categorized in TS22.104.

Proposal 2: The use case of the RedCap positioning should be selected among the use cases categorized in TS22.104.

Considering reasonable positioning accuracy and the requirement for IWSN which requires the battery life with few years, the positioning use case that overlaps with the RedCap IWSN would be use case #4 (Process automation: Sequence container (Intralogistics): < 1 m of horizontal accuracy, 6-8 years of batter life time) in that table.

Observation: Given that the positioning is applied for RedCap devices with IWSN use case, the positioning use case that overlaps with the RedCap IWSN would be use case #4 (Process automation: Sequence container (Intralogistics) in 22.104.

Once the use case for RedCap positioning is selected, the corresponding set of requirements is identified. It should be discussed whether the identified requirements can be satisfied for the IWSN devices or not. The biggest issue to be resolved is the how the accuracy requirement for positioning is realized with the basic RedCap assumption. More specifically, to achieve high accuracy requirement such as < 1m, whether existing positioning technology is enough for the Rel-17-based RedCap UE devices with maximum UE bandwidth limitation (20MHz in FR1 and 100MHz in FR2) should be firstly studied. Then, if not achievable, what kind of enhancement to achieve the requirement (e.g. frequency and/or time domain) need to be further studied.

Proposal 3: For RedCap IWSN, how the clarified requirements for positioning use case can be achievable should be studied.

Conclusion

In this contribution, we have the following proposals and observations:

Proposal 1: Among three Rel-17 RedCap use cases, the target for positioning should be IWSN for IIoT.

Proposal 2: The use case of the RedCap positioning should be selected among the use cases categorized in TS22.104.

Observation: Given that the positioning is applied for RedCap devices with IWSN use case, the positioning use case that overlaps with the RedCap IWSN would be use case #4 (Process automation: Sequence container (Intralogistics) in 22.104.

Proposal 3: For RedCap IWSN, how the clarified requirements for positioning use case can be achievable should be studied.

References

RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, Dec. 2021

RP-211574, Revised WID on support of reduced capability NR devices, Ericsson, June 2021

TS 22.104, V18.3.0 (2021-12)

- Appendix

TS 22.104

Table A.7.2-1: Low power high accuracy positioning use cases

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

TS 22.261

Table 7.3.2.2-1 Performance requirements for Horizontal and Vertical positioning service levels

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |