3GPP TSG RAN WG1 #109-e	R1-2204674

e-Meeting, May 09th – May 20th, 2022

Agenda item:		9.11.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on XR-specific power saving enhancements

Document for:		Discussion and Decision

# Introduction

RAN#95 approved a Revised SID: Study on XR Enhancements for NR [1] with the following objectives:

### 4.1 Objective of SI or Core part WI or Testing part WI The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements. - Dynamic scheduling/grant enhancements.

In this contribution we present our views on the power saving techniques to be used for XR devices and services.

# XR Power Saving Features

## Simulation setup

During Release 17 SI on XR a comprehensive discussion held place on the applicable evaluation methodology for power saving (PS) evaluations. This discussion led to the definition of the system-level evaluation methodology, as summarized in [2]. The methodology particularly contrasts the achievable performance gains introduced by a certain PS scheme to the corresponding losses in XR capacity.

The said methodology was proven to be illustrative and useful, so it is proposed to use the PS evaluation methodology from [2] as a starting point when developing the evaluation methodology for PS studies for Release 18 XR work.

Proposal 1: Use the PS evaluation methodology from TR 38.838 as a starting point, when developing the methodology for PS studies in Release 18 XR work.

At the same time, Release 17 XR PS studies were reported for an extensive set of heterogeneous setups (deployments, radio configurations, etc.), but primarily showing similar major trends in those. It is also important to note that the anticipated scope of potential PS enhancements to be proposed for Release 18 XR work is relatively large. Hence, to facilitate a productive work with respect to the limited duration of the Release 18 SI, it is proposed to notably reduce the scope of the considered scenarios and focus primarily on the common subset as detailed below.

Proposal 2: To facilitate efficient study of applicable PS schemes, narrow the scope of the considered deployments and setups to a minimal sufficient set.

The following particular set (see Table 1) is proposed to meet the said criteria for traffic model parameters.

Proposal 3: The proposed unified set of setups for PS evaluations is summarized in Table 1.

Table 1 – Proposed set of evaluation setups for XR PS studies in Release 18.

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

### Traffic models for system-level evaluation

Below, we discuss the parameters for the statistical model for XR traffic we used in the performance evaluation of our proposed power saving enhancements. The parameters have been set according to the discussion in [2]. Table 2 illustrates the parameters of the traffic models used for generating DL traffic. Table 3 illustrates the traffic models of XR applications as well as their QoS constraints used to evaluate user satisfaction and system capacity. Note that the maximum packet size of the transport layer is fixed to the maximum frame size to avoid segmentation at transport layer.

Table 2 – Parameters of DL traffic models for XR applications.

| Parameter | Value |  |
| --- | --- | --- |
| Type of content | Video (DL) | Video (DL) |
| Traffic model | Single stream for dual-eye buffer | Single stream for dual-eye buffer |
| Number of streams | 1 | 1 |
| Packet size distribution | Truncated Gaussian |  |
| Bitrate | 30 Mbps | 45 Mbps |
| Packet size (avg) | 62500 Bytes | 93750 Bytes |
| Packet size (std) | 6563 Bytes (10.5% of mean value) | 9844 Bytes (10.5% of mean value) |
| Packet size (max) | 93750 Bytes (150% of mean value) | 140625 Bytes (150% of mean value) |
| Packet size (min) | 31250 Bytes (50% of mean value) | 46875 Bytes (50% of mean value) |
| Transport protocol | UDP |  |
| Maximum transport packet size | 93750 Bytes 1 packet = 1 PDU | 140625Bytes 1 packet = 1 PDU |
| Packet Rate | 60 fps | 60 fps |
| Inter-arrival time distribution | Truncated Gaussian |  |
| Interarrival time (avg) | 16.67 ms |  |
| Interarrival time (std) | 2 ms |  |
| Interarrival time (max) | 20.67 ms |  |
| Interarrival time (min) | 12.67 ms |  |

Table 3 – Traffic models and QoS constraints used to evaluate XR applications in DL direction.

| Application | CG | VR | AR |
| --- | --- | --- | --- |
| Traffic model | Video single-stream | Video single-stream | Video single-stream |
| Bitrate | 30 Mbps | 30 Mbps | 30 Mbps |
| Packet rate | 60 fps | 60 fps | 60 fps |
| Packet Delay Budget (PDB) | 15 ms | 10 ms | 10 ms |
| Packet Error Rate (PER) | 1% | 1% | 1% |
| Number of streams | 1 | 1 | 1 |

### KPIs for power saving evaluation

As detailed in [2], power saving schemes are evaluated in terms of system capacity and energy efficiency. The system capacity is measured in terms on “number of satisfied UEs”, while energy efficiency is measured both in terms of power saving gain and capacity loss with respect to the baseline in order to capture the capacity and UE power saving trade-off. The baseline in Rel-17 studies, which was used to measure the reference capacity and power consumption, corresponds to the case where UEs are “always ON” (i.e., any UE power saving scheme is disabled). More specifically, a UE is marked as “satisfied UE” if more than X% of packets are successfully transmitted over the air interface within the PDB. The PDB is applied to the whole application layer frame. The system capacity is defined as the maximum number of UEs per cell with at least Y% of UEs marked as satisfied UEs. According to [2] in our evaluation we adopt X=99% as the percentage of frames to be successfully received within the PDB to identify satisfied UEs, while Y=90% as the percentage of satisfied UEs to measure the system capacity.

The capacity loss and power saving gain of any UE power saving scheme are measured with respect to the baseline where UEs are “always ON”. Specifically, once the system capacity for the case where UEs are “always ON” has been identified, the evaluation is performed again with the power saving scheme enabled for all UEs. The capacity loss is measured as follows:

η=1-CPSCB

where CPS is the ratio/percentage of satisfied UEs with power saving scheme enabled and CB is the ratio/percentage of satisfied UEs “always ON” (baseline). Similarly, the power saving gain is computed as follows:

γ=1-PPSPB

where PPS is the average power consumed by all UEs with power saving scheme enabled and PPS is the average power consumed by all UEs “always ON”.

While the “always ON” is a relevant baseline from capacity perspective, it would not appear as a proper baseline for UE power saving perspective. When moving forward in Rel-18, identifying possible enhancements for UE power saving, it would seem important to also account the existing functionalities for UE power saving since Rel-15, including Rel-17 features. These functionalities can also offer opportunities for UE power saving e.g. via reducing unnecessary PDCCH monitoring with limited impact to the capacity. Hence, the proposed power saving enhancements should be compared to a proper baseline of Rel-15/16/17 functionalities. This would ensure that considered enhancements would offer incremental gains in terms of UE power saving. Therefore, wherever possible we propose that the power saving gain of any enhancements is evaluated against existing Rel-15/16/17 power saving schemes in two values: (i) the power saving gain of the Rel-15/16/17 power saving scheme without the evaluated enhancement (over the “always ON”), and (ii) the extra power saving gain of the evaluated enhancement.

Observation 1: KPIs of further UE power saving schemes should be compared to the achievable performance with existing Rel-15/16/17 functionalities.

Proposal 4: In addition to the overall power saving gain of the evaluated enhancement, we propose to report the (i) power saving gain of the Rel-15/16/17 power saving scheme without the evaluated enhancement, and (ii) the extra power saving gain of the evaluated enhancement.

## Rel 17 UE power saving features

Prior releases (including Release 16 and Release 17) contain a set of configurable PS mechanisms that could be applied for XR services as well. This includes but is not limited to DRX, PDCCH skipping, sparse PDCCH monitoring.

During the Release 17 XR SI, the simplistic “always ON” scheme was used as a baseline for the PS evaluations [2]:

### 8.3.1 Baseline power evaluation results This clause includes the baseline power consumption results. PS schemes considered in this clause includes Always On, R15/16/17 power saving schemes such as CDRX, cross slot scheduling and MIMO layer adaptation by BWP switching, PDCCH monitoring adaptation. - Always On: In this scheme, UE is always available for scheduling (i.e., no DRX off period). When UE is not receiving/transmitting DL/UL data, UE is assumed to keep monitoring PDCCH. …

This “always ON” scheme was defined as a baseline during RAN1#104-e (see the agreement text below) and used as reference, when comparing the performance gains by various PS schemes, both Release 17-compliant, as in Section 8.3.2 of [2] and potential enhancements by individual companies, as summarized in Section 8.3.3 of [2].

Agreements To facilitate further discussion on evaluation of power saving effect of different power saving schemes, the following references are defined. Case 1 (baseline): UE power consumption assuming UE is always ON, i.e., UE is always available for gNB scheduling.

Such a simple common baseline was sufficient for the Release 17 SI, as Section 8 of [2] primarily reports just the relative performance gains of different PS schemes. However, as discussed in previous section, for Release 18, this may be insufficient, as the actual specification changes need to be motivated through incremental gains. For this purpose, it would seem important that the proposed enhancements are compared against the existing methods for UE power saving. Hence, there is need to define a set of Release 17-compliant UE and gNB behaviours that would be accounted in Rel-18 XR work for PS evaluations.

Proposal 5: For Rel-18 XR PS evaluations, define a Release 17-compliant baseline setup that includes all the applicable power saving features, including but not limited to DRX and PDCCH monitoring adaptation that should be accounted in evaluation of possible benefits.

As considered in Rel-17 XR study, Rel-15 Connected mode DRX (CDRX) operation, accounting both short and long DRX cycles and MAC-CE commands, would offer good reference for the UE power consumption. In addition, in Rel-15 DCI-based BWP switching could be considered to adapt different UE configurations. In context of XR, CDRX configuration would evidently seem as necessary component to reduce the UE power consumption in durations when there is no data activity.

In Rel-16 further mechanisms were introduced to enable UE power saving. First, method to reduce PDCCH monitoring during unnecessary drx-onDurationTimer’s was introduced with DCI (format 2_6 scrambled with P-RNTI, DCP) based indication. In context of XR this would allow to mitigate the power consumption impact of frequent unnecessary drx-onDurationTimer so that the configured longDRX-cycle value could be set to be shorter, thereby mitigating negative impact of CDRX to capacity KPIs. In addition, to enable more frequent micro sleep for the UE by assuming cross-slot scheduling, minimum scheduling offset restriction was introduced. In context of XR, this could help reduce the cost of frequent PDCCH monitoring, by allowing UE to sleep the remainder of slot when PDSCH has not been scheduled (in earlier slot). Method to configure the maximum number of DL MIMO layers as a BWP specific parameter was also added in Rel-16, which could be accounted if power saving via BWP switching is seen relevant. While CA has not been accounted in the evaluated scenarios, dormant BWP behaviour to reduce the unnecessary PDCCH monitoring in Scells was introduced as well. Also, UE assistance information power saving was introduced to allow UE to indicate its preference e.g. on DRX cycle length(s), inactivity timer etc.

Finally, in Rel-17 UE power saving enhancements in CONNECTED mode provided further methods to reduce PDCCH monitoring during DRX Active time. DCI (and timer) -based PDCCH monitoring adaptation methods was introduced to allow UE to skip PDCCH monitoring for a duration or to adjust the ‘active’ SS sets through SSSG switching. In context of XR, these methods would further allow to reduce the unnecessary PDCCH monitoring e.g. through skipping over a duration when traffic is not expected or through monitoring in reduced periodicity when the XR frame has not yet arrived.

Observation 2: Earlier releases contain multiple functionalities that should be accounted when further UE power saving enhancements are considered for XR, such as:

- CDRX operation with short- and longDRX-cycle and MAC-CEs commands

- DCP

- Minimum scheduling offset restriction

- PDCCH monitoring adaptation for skipping PDCCH monitoring and SSSG switching

## Possible enhancements

In this section we present potential enhancements to improve the trade-off between capacity and UE power saving and the results of their system level evaluation.

To further improve UE power saving, in addition to R15/16/17 power saving schemes, we can investigate the following enhancements:

- Adaptive DRX (ADRX): a new signaling scheme for fast adaptation of CDRX parameters. ADRX enables the implementation of ML policies to adapt DRX configuration according to system status (traffic and RAN status).

- Traffic-Aware Wake-Up Signal (TA-WUS): WUS sent by gNB to skip a consecutive number of consecutive On Durations triggered by the identification of the successful reception of the application-layer frame.

- PDCCH sparse monitoring: monitoring of the PDCCH is made discontinuous and controlled by a parameter that indicates the slot frequency of the monitoring. Therefore, the UE monitors only a subset of PDCCH slots.

### Drift correction for DRX

The current DRX specification allows only a single configuration for the discontinuous reception cycle with an integer value for the definition of the duration of the DRX cycle. In contrast, XR traffic shows a non-integer and non-recurrent periodicity due to source that generate the 3D video content. Typical frame rate is 30, 60, 90, and 120 frame per second that corresponds to the period between two consecutive frame arrivals of 33.33ms, 16.67ms, 11.11ms, and 8.33ms [2]. The non-integer and non-recurrent periodicity of XR traffic results in time drift between the start of the DRX cycle and the XR traffic arrival. The time drift accumulates over time and leads to the misalignment between the DRX cycle and the XR traffic. This misalignment can eventually result in the loss of an XR frame since the accumulated gap can become as large as the PDB. An example of the problem is illustrated in Figure 1, where we assume that the XR frame rate is equal to 60 fps, which corresponds to an average interarrival time (or periodicity) of video frames equal to 16.67 ms. We further assume that the network has configured the DRX cycle with drx-onDurationTimer =5, drx-InactivityTimer =0, drx-LongCycle =10, and drx-StartOffset =6 (short DRX cycle is not configured). This means that every 16ms the UE monitors PDCCH for at least 5ms. If no control message is received before drx-onDurationTimer expires, then the UE goes to sleep mode. As illustrated in the Figure 1, in case the UE gets scheduled and it remains active every DRX cycle for 5ms (i.e., assuming that drx-InactivityTimer  is triggered only once every DRX cycle), after 8 frames the UE will not detect the control message for a new video frame transmission (i.e., the ninth frame) since the accumulated gap between the non-integer XR periodicity and the DRX cycle is 5.33ms (0.667*8), which is larger than 5ms.

Figure 1 – The drift caused by integer periodicity of DRX cycle and non-integer periodicity of XR frame generation accumulates over time. Therefore, the DRX cycle drifts from the frame arrival. In case of 60fps, the ninth frame (third I-frame in the time plot) is lost or delayed.

The gNB can detect this mismatch and trigger a RRC reconfiguration, but periodic RRC reconfiguration is necessary to realign the DRX cycle with the XR traffic. If the realignment between XR traffic and DRX does not take place, the drift keeps accumulating creating a gap up to the point when the accumulated gap exceeds the PDB of the XR application. Figure 2 shows an example of this problem. Assume that the duration of the DRX cycle can be configured to approximate the XR periodicity to the closed integer. For 60fps, the two durations can be used: either 16ms (DRX 1) or 17 ms (DRX 2), which correspond to drifts of 0.67ms and 0.33ms. The blue and green lines in the figure represent the evolution of the drift over the DRX cycles for DRX 1 and DRX 2, respectively. We can observe that after 16 cycles for DRX 1 and 31 cycles for DRX 2, the accumulated gap due to the drift exceeds the PDB, which is represent as yellow lines in the figure. Therefore, the network must periodically send a RRC reconfiguration to keep the accumulated gap under control.

Figure 2 – Example of drift evolution assuming two DRX cycles of duration equal to 16ms (DRX 1) and 17ms (DRX 2). Yellow lines represent the PDB of VR/AR applications. After 16 and 31 cycles, the accumulated gap of configurations DRX 1 and DRX 2 have exceeded the PDB of VR/AR traffic.

However, we observe that the RRC reconfiguration is a slow process that may introduce extra-delay in the transmission. Therefore, we propose to introduce new Layer 1 (L1 like DCI) or Layer 2 (L2 like MAC CE) signalling and new parameters communicated to the UE during the RRC connection setup through a new Information Element (IE) to realign the DRX cycle with the XR traffic arrival process. L1 and L2 signalling can be used for asynchronous realignment, while the new IE can be used for synchronous (i.e., periodic) realignment. An example is illustrated in Figure 3, where the UE and the network realign the DRX cycle with the XR traffic by extending the duration every 4 DRX cycles (in red the DRX cycle whose duration is extended).

Figure 3 – The UE and the network realign the DRX cycle with the XR traffic by extending the duration every four DRX cycles.

Observation 3: The non-integer and recurrent periodicity of XR traffic and the integer periodicity of the DRX cycle results in the misalignment between XR traffic and DRX cycle. This misalignment causes a time drift between the XR frame arrivals and the start of the DRX cycles. The drift accumulates over time and eventually the start of the DRX cycle falls outside expected arrival interval of the XR traffic.

Proposal 6: We propose to introduce L1/L2 signalling and new IEs to realign the DRX cycle with the XR traffic and control the drift between the DRX cycle and XR traffic. The realignment can be execute synchronously using the parameters of the new IE or asynchronously by the network using the new L1/L2 signalling mechanism.

### Adaptive DRX (ADRX)

We observe that the current specifications of CDRX allows only for a fixed configuration for the discontinuous reception (DRX) cycle. This may result in inefficient configuration of the CDRX cycle that hardly fits the XR traffic pattern, resulting in wasted energy due to the random traffic arrival. Furthermore, changes in the traffic arrival process, like the increase/decrease of framerate and quality, may require a quick reconfiguration of the CDRX cycle. This operation may simply involve scaling up/down all parameters to match a new framerate (i.e., a different inter-arrival time between frames). Therefore, the ON duration (drx-OnDuration) and inactivity period (drx-InactivityTimer), which may be set based on the worst expected inter-arrival arrival time, would result too long. Since frames are expected to arrive most of the time within a subinterval of the jitter window, a fixed CDRX configuration results in a loss of energy for most of the transmissions. On the other hand, reconfiguring the CDRX cycle according to the XR frame pattern might be a rather slow process since such change requires RRC signalling/procedures. Therefore, to fully enable the configuration and dynamic adjustment of CDRX cycle according to traffic pattern, we propose Adaptive DRX (ADRX), which extends the standard CDRX power saving scheme. ADRX is based on new Information Elements (IEs) exchanged through RRC connection reconfiguration messages and L1/L2 signalling for fast adjustment of the CDRX cycle parameters. In our system-level evaluation we couple ADRX with a ML policy to dynamically adapt some of the CDRX parameters according to the UE satisfaction (i.e., number of frames received within PDB) and UE power consumption. The ML uses past observations and future predictions of the system status, including frame arrivals, UE satisfaction, UE power consumption, and channel status. CDRX configuration parameters are modified accordingly to the decision of the ML policy, which is communicated to the UE by the network using fast L1/L2 signalling. In particular, the ML policy we designed selects the best On Duration based on UE satisfaction and power consumption. ADRX needs the following additional parameters with respect to CDRX:

- MinOnDuration: minimum value of the On Duration parameter

- MaxOnDuration: maximum value of the On Duration parameter, i.e., the policy will choose the appropriate value for On Duration Timer in the range [MinOnDuration, MaxOnDuration] ms.

In our evaluation, we compare the following three power saving schemes:

- CDRX: (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8)

- ADRX: (DRX long cycle, inactivity timer value, MinOnDuration, MaxOnDuration)=(16, 0, 1, 16)

- Baseline used in [2]: UEs “always ON” (i.e., any power saving scheme disabled)

We first present the capacity of the power saving schemes measured in terms of ratio of satisfied UEs. Figure 4(a) and Figure 4(b) show the ratio of satisfied UEs for CG and AR/VR services as a function of the system load, which corresponds to the number of UEs per cell deployed in the system. We can observe that when DRX is disabled and UEs are always active, the ratio of satisfied users remains above 90% when each cell serves up to 8 and 6 CG and AR/VR users per cell, respectively. When CDRX is enabled with the configuration (16, 8, 8), the ratio of satisfied users drops steeply. In particular, by using that CDRX configuration only up to 4~CG users per cell can be served, whereas no AR/VR user can be served since the ratio of satisfied users drops quickly to zero. In contrast, with our ADRX scheme the system can serve both CG and AR/VR services. Specifically, the ratio of satisfied users remains above 90% when each cell serves up to 6 and 4 CG and AR/VR users per cell, respectively.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) AR/VR in FR1 at 30Mbps with X=99%. |
| --- | --- |

Figure 4 – DL capacity of ADRX versus CDRX for CG traffic (Video Single-Stream) and AR/VR (Video Single-Stream) in Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded.

Figure 5 shows the power saving gain measured with respect to users always active as a function of the system load. It can be observed that ADRX reduces power consumption up to 12% with respect to users always in active mode. However, CDRX cannot support any user for AR/VR services with the evaluated configuration (it can support only CG users, which have larger delay budget). Therefore, CDRX has no power saving gain for AR/VR users.

Figure 5 – Power saving gain of ADRX versus CDRX for CG traffic (Video Single-Stream) and AR/VR (Video Single-Stream) in Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded.

Table 3 and Table 4 summarize the PS gain and system capacity loss of CDRX and ADRX with respect to “always ON” policy for CG and AR/VR applications, respectively. In both tables, PS gains are computed considering all UEs in the system (option 1). Both tables shows that there is a clear trade-off between power saving and capacity loss. However, ADRX can limit the capacity loss and still achieving a power saving by adapting the OnDuration to any changing system or traffic characteristics like the PDB. In contrast, a single fixed configuration for CDRX is inefficient since it can result in huge capacity loss as illustrated in our evaluation: when PDB decreases from 15ms (CG) to 10ms (AR/VR) the CDRX configuration (16,8,8) cannot support any AR/VR user.

Table 4 – Evaluation of enhanced ADRX enhancement for {Dense Urban, CG, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 8 | 8 | - |
| CDRX(16,8,8) | 13.6 | 16.8 | 14.4 | 8.3 | 4 / 8 | 6 | 50% |
| ADRX(16,1,16) | 7.5 | 8.1 | 7.2 | 3.1 | 5.6 / 8 | 4 | 70% |

Table 5 – Evaluation of enhanced ADRX enhancement for {Dense Urban, AR/VR, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |  |  |
| Always ON | - | - | - | - | - / 6 | 6 | - |
| CDRX(16,8,8) | - | - | - | - | 0 / 6 | 0 | 0% |
| ADRX(16,1,16) | 7.5 | 8.1 | 7.2 | 3.1 | 4 / 6 | 4 | 66% |

Observation 4: A single CDRX configuration for multiple XR applications, which have different QoS characteristics, results in suboptimal performance. For example, the CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) cannot satisfy any UE when PDB changes from 15ms to 10ms.

Observation 5: The possibility to quickly reconfigure the parameters of the CDRX configuration or selecting a CDRX configuration among a set of pre-configured CDRX configurations like in Adaptive DRX (ADRX) reduces the capacity loss while maintaining a 10% power saving gain with respect to the baseline “always ON”.

Proposal 7: We propose to extend DRX configuration to have multiple adjustable parameters and/or configurations as well as to introduce L1/L2 signalling to quickly change the CDRX configuration.

Observation 6: When proposing power saving enhancements, capacity loss should be minimized.

### Traffic Aware Wake-Up Signal (TA-WUS)

The evaluation of XR application done in Release 17 [1] showed that DRX configurations with short and dense On Durations, namely DRX configurations with multiple cycles within the inter-arrival time of two consecutive XR frames, reduce the capacity loss while achieving comparable power saving gain of DRX configurations with long and sparse OnDurations. Example of short and dense DRX configurations are (DRX long cycle, inactivity timer value, On duration timer value)=(8,0,4) and (4,0,2), where on average 2 and 4 cycles are contained in an interval between two frames, respectively, as illustrated in Figure 6. CDRX configurations with short cycle periods that make OnDurations dense within the interval of two consecutive XR frames and DCP can also address some of the problems described above like the misalignment between XR traffic and DRX cycle and the large jitter of XR frame arrivals. Therefore, different kind of CDRX configurations should be considered for the power saving evaluation of evaluated power saving enhancements.

With this type of short and dense DRX configurations, we observe that some of the OnDurations may be skipped by the UE when the XR frame has been fully received and no traffic is expected to arrive. Furthermore, the short amount of inactive period when the UE is allowed to sleep prevents the UE entering light sleep mode. As discussed in [3], the UE requires at least 6 ms and 20 ms of continuous inactivity time to enter light and deep sleep modes, respectively. With DRX configurations like (8,0,4) and (4,0,2), the UE can enter only micro sleep mode. In order to improve the power saving gain, we propose to use traffic awareness to trigger the Wake-Up Signal (WUS) to allows the UE to skip the OnDurations where no traffic is expected to be (re)transmitted. This enhancement requires to be able to detect the last packet of the frame to be sure that the frame has been successfully delivered to the UE.

Figure 6 – Three CDRX configurations: short and dense configurations like (8,0,4) and (4,0,2) have multiple cycles within the interval of two consecutive XR frames. When the frame is fully received some of the On Duration where no traffic arrival is expected can be canceled using WUS.

Table 5 and Table 6 summarize the PS gain and system capacity loss of CDRX configurations with and without WUS triggered by the reception of the acknowledgment of the last fragment of XR frame. Power saving gain is computed with respect to the baseline of Release-17 (UEs “always ON”) for both CG and AR/VR applications. In both tables, PS gains are computed considering all UEs in the system (option 1). Both tables shows that there is a clear trade-off between power saving and capacity loss.

Table 6 – Evaluation of enhanced TA-WUS enhancement for {Dense Urban, CG, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- |
|  | Mean PS gain |  |  |  |
| Always ON | - | - / 8 | 8 | 96% |
| CDRX(8,0,4) | 16.7 | 6.72 / 8 | 7 | 84% |
| CDRX(8,0,4) + 1 WUS | 17 | 5.6 / 8 | 6 | 70% |
| CDRX(4,0,2) | 18 | 7.04 / 8 | 7 | 88% |
| CDRX(4,0,2) + 1 WUS | 20 | 6.32 / 8 | 7 | 79% |
| CDRX(4,0,2) + 2 WUS | 22.5 | 6.08 / 8 | 7 | 76% |

Table 7 – Evaluation of enhanced TA-WUS enhancement for {Dense Urban, AR/VR, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs.

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS | Capacity with PS [#satisfied UEs/cell with PS] | Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS |
| --- | --- | --- | --- | --- |
|  | Mean PS gain |  |  |  |
| Always ON | - | - / 6 | 6 | - |
| CDRX(8,0,4) | 18 | 3.66 / 6 | 5 | 61% |
| CDRX(8,0,4) + 1 WUS | 20 | 2.7 / 6 | 4 | 45% |
| CDRX(4,0,2) | 20 | 4.98 / 6 | 5 | 83% |
| CDRX(4,0,2) + 1 WUS | 22.5 | 3.9 / 6 | 5 | 65% |
| CDRX(4,0,2) + 2 WUS | 25.3 | 3.9 / 6 | 5 | 64% |

Observation 7: Dense and short DRX configurations reduce capacity loss with respect to R17 baseline of XR evaluation (UE “always ON”). Therefore, dense and short DRX configurations represent good candidates as baseline for power saving evaluation.

Observation 8: While dense and short DRX configurations reduce capacity loss with respect to R17 baseline of XR evaluation (UE “always ON”), they also limit the power saving gain since the UE can enter only micro sleep mode.

Observation 9: When a frame is successfully delivered to the UE, future On Durations of short and dense DRX cycles can be skipped when no traffic is expected.

Observation 10: Traffic awareness, like knowledge of the boundaries of the XR frame (i.e., first and last PDU in a PDU Set), is beneficial to use the Wake-Up signal to skip the On Durations when no traffic is expected to be transmitted.

Proposal 8: We propose to evaluate enhancements, which might be on traffic awareness ,to skip the On Durations when no traffic is expected to be transmitted and the XR frame has been successfully delivered.

### PDCCH Sparse Monitoring

Introduced in 5G NR Release 15, PDCCH sparse monitoring allows the UE to determine in every slot if there may be a PDCCH monitoring occasion prior to decoding the PDCCH. The setting for said sparse monitoring is preconfigured by the NW and signalled to the UE as part of the search space sets configuration per bandwidth part (BWP); up to 10 search space sets can be configured per UE per BWP (including common search spaces and UE-specific search spaces). For each search space set s, a single configuration for PDCCH monitoring is provided comprising a PDCCH monitoring frequency of ks slots, offset of os slots and pattern within a slot. For every configured search space set s on an active DL BWP, a UE determines if there is a PDCCH monitoring occasion in slot number ns, fμ in frame with number nf if:

nf∙Nslotframe, μ+ns, fμ-os mod ks=0

where μ is the SCS configuration and Nslotframe, μ represents the number of slots per frame.

PDCCH decoding is one of the main contributors to UE power consumption, therefore adjusting the PDCCH monitoring periodicity allows for UE power saving. However, for applications like XR with a tight PDB and a main KPI depending directly on packet delay, having a PDCCH monitoring periodicity ks>1 during data activity can lead to significant capacity loss [5]. Adapting the PDCCH monitoring frequency according to XR data traffic behaviour to have e.g.,  ks=1 during the activity periods and  ks>1 during the inactivity periods (as shown in Figure 7), could offer less capacity loss and still provide UE power saving. However, since the UE looks for PDCCH monitoring occasions over all configured search space sets, the lowest value of ks among all the configured search space sets would set the granularity for PDCCH monitoring periodicity. This means that as long as there is a search space set configured with, e.g.,  ks= 1 (to be used during the data activity periods and avoid the high-capacity loss), the UE will perform PDCCH monitoring in every slot.

Figure 7 – Example of adaptive PDCCH sparse monitoring for values ks = 1 and ks > 1

To change the minimum value of  ks, e.g., to have  ks=2 as the lowest value instead of  ks= 1 during the inactivity periods, a reconfiguration or switch of the search space sets would be needed, which would introduce extra delays and could overshadow the benefits of PDCCH sparse monitoring. Current specifications allow at least two ways to modify the search space sets to look for PDCCH monitoring occasions; one approach is through a BWP switch and the other is through search space set group switching.

With the first approach, since the set of search space sets configured for a UE is done per BWP, switching the active BWP would allow changing the PDCCH monitoring frequency without needing to change any of the other BWP related parameters (e.g., SCS, location and bandwidth). The delay associated to the BWP switch would depend on the applied method, RRC-based, or DCI-based and also on the UE capabilities and the value of μ, as shown in Table 7. Furthermore, during BWP switch, both UE reception and transmission are assumed to be interrupted.

Table 8 BWP maximum switching delay in FR1

|  | NR Slot length | BWP switch delay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

The second approach, introduced in Rel. 16 for NR-U and in Rel-17 for power saving, allows grouping the search space sets so that during a period only part of all the configured search space sets in the active BWP of a UE would be considered for monitoring the PDCCH; current specifications allow having up to three search space set groups (SSSG), i.e., SSSG#0, SSSG#1, SSSG#2. The switching between one search space set group and another can be signalled via DCI in an explicit way by using a PDCCH monitoring adaptation field (of 1 or 2 bits). A switching timer can also be configured, to autonomously fall-back to SSSG#0 when UE has not been scheduled for the duration. Switching the search space sets also comes with a switching delay which depends on the UE processing capabilities and the value of μ, as shown in Table 8.

Table 9 Minimum value of search space sets group switching delay in FR1

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] | Minimum Pswitch value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 (≈ 1.79 ms) | 10 (≈ 0.71 ms) |
| 1 | 25 (≈ 0.9 ms) | 12 (≈ 0.43 ms) |
| 2 | 25 (≈ 0.45 ms) | 22 (≈ 0.4 ms) |

Regardless of the approach used to modify the PDCCH monitoring periodicity, the delays associated to it should be considered to better assess the impact they could have in the system capacity. Alternatives such as PDCCH skipping can also be used to quickly stop the PDCCH monitoring for a certain period, however the flexibility of this approach might be more limited and a higher accuracy for setting the duration of the skipping period is needed. Therefore, further solutions than can allow for a switch of the value of  ks in an even more seamless way might be considered (refer to results in [5]), especially knowing that XR services could have variable packet rates and characterize by a non-integer inter-arrival time.

Observation 11: XR services could benefit from an adaptive PDCCH monitoring approach, with monitoring periodicity changing depending on the traffic behaviour.

Proposal 9: Study the influence of adapting the PDCCH monitoring periodicity in XR services according to traffic characteristics and behaviour, focusing also on the impact the delays/interruptions associated with changing the PDCCH monitoring periodicity could have on the system capacity.

# Answer to LS on UE Power Saving for XR and Media Services

SA WG2 send a LS to RAN1 and RAN2 in [4], with following question to RAN1 (and RAN2):

ACTION: SA2 kindly asks RAN1, RAN2 to take the above information into account and clarify which type of information should be provided to the RAN for power saving enhancements for XR applications.

In the R17 study item on XR evaluation for NR, the traffic models that were considered for the performance analysis had the following key properties:

- Video traffic is quasi-periodic with the period composed of (i) a constant/deterministic scalar, which is determined by the rate at which the application generate the application-layer data units, (ii) and a random and bounded jitter.

- Application-layer data units of the video traffic stream have random size but the range is bounded. The size of an application-layer data unit for video traffic is modeled as a random variable with bounded range.

- Pose information (as known as pose updates) is periodic with the period between two application-layer data units defined by a constant/deterministic scalar.

- Application-layer data units of the pose information stream have constant size.

- Delay budget and maximum error rate are applied to application-layer data units. To simplify the performance evaluation the application-layer data unit model (called frame-based model in Rel-17) has been considered as mandatory in Rel-17, with the optional evaluation of the packet-based model where application-layer data units are fragmented into smaller network-layer packets.

According to the analysis done in R17 on XR evaluation for NR [2], additional information will enable the optimal configuration and the design of enhancements to R15/R16/R17 UE power saving schemes. In particular, we see the following information as essential:

- Identification of the boundaries of the application-layer data unit (e.g., an XR frame). Since application-layer data units may be fragmented into multiple PDUs, RAN shall be able to distinguish PDUs that belong to the same XR frame, the first PDU of a new XR frame and the last PDU of an XR frame.

- Whether PDUs in which the application-layer data units is fragmented are delivered in order or out of order.

- Indication of the delay budget and maximum error rate for application-layer data units.

Furthermore, we see the following information as auxiliary information, namely information that would assist RAN to optimally configure and improve UE power saving schemes, yet not essential for enabling the enhancements:

- Periodicity of the application layer traffic. Information like the rate at which application-layer data units such as XR frames and pose information packets are generated would assist the RAN to configure and adjust UE power saving schemes.

- Jitter statistics like mean and variance as well as the range of the jitter (minimum and maximum value of the jitter) of the application layer stream would assist RAN to configure and adjust UE power saving schemes that can tolerate the variability of the expected arrivals in order to fulfill the delay budget and error rate of XR applications.

- Number of PDUs in which an application-layer data unit is fragmented would assist the RAN to decide whether and when to trigger a UE power saving enhancements.

- Indication of priorities, dependencies and validity of application-layer data units would assist the RAN to decide whether and when to trigger a UE power saving enhancements.

We observe the auxiliary assistance information requires further evaluation to understand whether is essential for the right configuration of RAN power saving functionalities and to improve KPIs of XR applications as defined in Rel-17.

Therefore, we would propose to provide SA2 the following answer:

RAN WG1 would like to thank SA WG2 for their LS. RAN WG1 discussed the question raised by SA WG2 and felt that, to improve UE power saving following information would be highly beneficial: - Identification of the boundaries of an application-layer data units (e.g., XR frames). - Whether PDUs of fragmented application-layer data units are delivered out of order. - Indication of the delay budget and maximum error rate for application-layer data units. RAN WG1 also felt that to enable proper configuration of UE power saving related functionality following information would be useful but further evaluation is needed: - Periodicity of the application layer traffic, and statistics of jitter - Number of PDUs to which an application layer data unit (e.g., an XR frame) is fragmented. - Indication of priorities, dependencies and validity of application-layer data units.

Proposal 10: Provide reply to SA2 question in their LS as follows:

RAN WG1 would like to thank SA WG2 for their LS. RAN WG1 discussed the question raised by SA WG2 and felt that, to improve UE power saving following information would be highly beneficial: - Identification of the boundaries of an XR frame (application layer packet). - Whether PDUs of fragmented XR frame are delivered out of order. RAN WG1 also felt that to enable proper configuration of UE power saving related functionality following information would be useful but further evaluation is needed: - Periodicity of the application layer traffic, and statistics of jitter - Number of PDUs to which an XR frame is fragmented. - Indication of priorities, dependencies and validity of application-layer data units.

# Conclusion

In this contribution we have discussed and evaluated various possible enhancements for R15/R15/R17 power saving schemes that may further help improve the XR performance of 5G NR. Those naturally require further studies and discussions. In summary, those are captured by the following observations:

Observation 1: KPIs of further UE power saving schemes should be compared to the achievable performance with existing Rel-15/16/17 functionalities.

Observation 2: Earlier releases contain multiple functionalities that should be accounted when further UE power saving enhancements are considered for XR, such as:

- CDRX operation with short- and longDRX-cycle and MAC-CEs commands

- DCP

- Minimum scheduling offset restriction

- PDCCH monitoring adaptation for skipping PDCCH monitoring and SSSG switching

Observation 3: The non-integer and recurrent periodicity of XR traffic and the integer periodicity of the DRX cycle results in the misalignment between XR traffic and DRX cycle. This misalignment causes a time drift between the XR frame arrivals and the start of the DRX cycles. The drift accumulates over time and eventually the start of the DRX cycle falls outside expected arrival interval of the XR traffic.

Observation 4: A single CDRX configuration for multiple XR applications, which have different QoS characteristics, results in suboptimal performance. For example, the CDRX configuration (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8) cannot satisfy any UE when PDB changes from 15ms to 10ms.

Observation 5: The possibility to quickly reconfigure the parameters of the CDRX configuration or selecting a CDRX configuration among a set of pre-configured CDRX configurations like in Adaptive DRX (ADRX) reduces the capacity loss while maintaining a 10% power saving gain with respect to the baseline “always ON”.

Observation 6: When proposing power saving enhancements, capacity loss should be minimized.

Observation 7: Dense and short DRX configurations reduce capacity loss with respect to R17 baseline of XR evaluation (UE “always ON”). Therefore, dense and short DRX configurations represent good candidates as baseline for power saving evaluation.

Observation 8: While dense and short DRX configurations reduce capacity loss with respect to R17 baseline of XR evaluation (UE “always ON”), they also limit the power saving gain since the UE can enter only micro sleep mode.

Observation 9: When a frame is successfully delivered to the UE, future On Durations of short and dense DRX cycles can be skipped when no traffic is expected.

Observation 10: Traffic awareness, like knowledge of the boundaries of the XR frame (i.e., first and last PDU in a PDU Set), is beneficial to use the Wake-Up signal to skip the On Durations when no traffic is expected to be transmitted.

Observation 11: XR services could benefit from an adaptive PDCCH monitoring approach, with monitoring periodicity changing depending on the traffic behaviour.

According to the above discussion and observations, the following main proposals have been made for analysis of possible power saving enhancements:

Proposal 1: Use the PS evaluation methodology from TR 38.838 as a starting point, when developing the methodology for PS studies in Release 18 XR work.

Proposal 2: To facilitate efficient study of applicable PS schemes, narrow the scope of the considered deployments and setups to a minimal sufficient set.

Proposal 3: The proposed unified set of setups for PS evaluations is summarized in the following Table:

Proposed set of evaluation setups for XR PS studies in Release 18.

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

Proposal 4: In addition to the overall power saving gain of the evaluated enhancement, we propose to report the (i) power saving gain of the Rel-15/16/17 power saving scheme without the evaluated enhancement, and (ii) the extra power saving gain of the evaluated enhancement.

Proposal 5: For Rel-18 XR PS evaluations, define a Release 17-compliant baseline setup that includes all the applicable power saving features, including but not limited to DRX and PDCCH monitoring adaptation that should be accounted in evaluation of possible benefits.

Proposal 6: We propose to introduce L1/L2 signalling and new IEs to realign the DRX cycle with the XR traffic and control the drift between the DRX cycle and XR traffic. The realignment can be execute synchronously using the parameters of the new IE or asynchronously by the network using the new L1/L2 signalling mechanism.

Proposal 7: We propose to extend DRX configuration to have multiple adjustable parameters and/or configurations as well as to introduce L1/L2 signalling to quickly change the CDRX configuration.

Proposal 8: We propose to evaluate enhancements, which might be on traffic awareness ,to skip the On Durations when no traffic is expected to be transmitted and the XR frame has been successfully delivered.

Proposal 9: Study the influence of adapting the PDCCH monitoring periodicity in XR services according to traffic characteristics and behaviour, focusing also on the impact the delays/interruptions associated with changing the PDCCH monitoring periodicity could have on the system capacity.

Finally, we propose the following answer to the LS sent by SA WG2:

Proposal 10: Provide reply to SA2 question in their LS as follows:

RAN WG1 would like to thank SA WG2 for their LS. RAN WG1 discussed the question raised by SA WG2 and felt that, to improve UE power saving following information would be highly beneficial: - Identification of the boundaries of an XR frame (application layer packet). - Whether PDUs of fragmented XR frame are delivered out of order. RAN WG1 also felt that to enable proper configuration of UE power saving related functionality following information would be useful but further evaluation is needed: - Periodicity of the application layer traffic, and statistics of jitter. - Number of PDUs to which an XR frame is fragmented. - Indication of priorities, dependencies and validity of application-layer data units.

# References

- RP-220285, Revised SID: Study on XR Enhancements for NR, RAN#95.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17)

- TR38.840, Study on User Equipment (UE) power saving in NR (Release 16)

- R1-2203219, LS on UE Power Saving for XR and Media Services, SA2

- R1-2204676, Performance results of XR-related enhancements

# Appendix A – Simulation settings

In this Appendix A, we summarize the main simulation settings used for the evaluation of power saving enhancements. We observe that in the study item of R17 on XR (Extended Reality) Evaluations for NR [2] the baseline for power saving evaluation was UE always ON (i.e., any power saving scheme disabled). The use of three parameters for DRX, namely DRX long duration cycle, inactivity timer value, On duration timer value, was agreed but no specific value for these parameters was agreed.

## Indoor Hotspot (InH) scenario

The carrier frequency is set to 4 GHz (FR1). System bandwidth is assumed to be 100 MHz. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 kHz. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set as follows: 31dBm with 100MHz (24dBm per 20MHz) in FR1. Table 10 lists the main parameters of the Indoor Hotspot (InH) deployment that are considered in this study.

Table 10 – Main parameters for Indoor Hotspot (InH) deployment

| Parameter | Value |
| --- | --- |
| Layout | - 120m x 50m, Single layer (indoor floor, open office) - 12 cells/TRPs - ISD: 20m |
| Channel model | InH |
| Carrier frequency | FR1: 4 GHz |
| Subcarrier spacing | FR1: 30 kHz |
| System bandwidth | FR1: 100 MHz |
| BS height | 3 m |
| UE height | 1.5 m |
| BS noise figure | FR1: 5 dB |
| UE noise figure | FR1: 9 dB |
| BS receiver | LMMSE-IRC |
| UE receiver | LMMSE-IRC |
| Channel estimation | Realistic (with ideal CSI) |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS Tx power | FR1: 31dBm (24dBm per 20MHz) |
| UE Tx max power | 23 dBm |
| TDD Frame structure | DDDSU |
| Cell Selection | RSRP Slow Fading |
| BS antenna configuration | - Pattern: Ceiling-mount antenna radiation pattern - Gain: 5 dBi - Downtilt: 90° |
|  | Configuration in FR1: - 32 TxRU - (M, N, P, Mg, Ng) = (4,4,2,1,1) - (dH, dV) = (0.5, 0.5)λ - (Mp, Np) = (4,4) |
| UE antenna configuration | - Pattern : Omni-directional, - Gain : 0 dBi, - Configuration : 2T/4R (M, N, P, Mg, Ng) = (1,2,2,1,1) (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,2) |
| Scheduler | SU-MIMO, Proportional Fairness |
| CSI acquisition | Periodic CQI on 2 ms period |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |
| PDCCH overhead | Modelled |
| Target BLER | 10% for first transmission |
| Max HARQ transmission | 3 |
| HARQ scheme | Chase combining |

## Dense Urban (DU) Scenario

The carrier frequency is set to 4 GHz (FR1) and the system bandwidth has been fixed to 100MHz. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRB) of 12 subcarriers, each of 30kHz. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmissions before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set to 51 dBm (i.e., 44dBm for 20MHz). Table 11 lists the main parameters of the Dense Urban deployment that are considered in this study.

Table 11 – Main parameters for Dense Urban (UMa) deployment

| Parameter | Value |  |
| --- | --- | --- |
| Layout | 21 cells with wraparound (ISD: 200m) |  |
| Channel model | UMa |  |
| Carrier frequency | FR1: 4 GHz |  |
| Subcarrier spacing | FR1: 30 kHz |  |
| System bandwidth | FR1: Option 1: 100 MHz |  |
| BS height | 25m |  |
| UE height | hUT = 3(nfl – 1) + 1.5 |  |
|  | Outdoor: nfl = 1 | Indoor: - nfl ~ uniform(1,Nfl) - Nfl ~ uniform(4,8) |
| BS noise figure | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic (with ideal CSI) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS Tx power | 44 dBm per 20 MHz 51 dBm per 100 MHz |  |
| UE Tx max power | FR1: 23 dBm |  |
| TDD Frame structure | Option 1: DDDSU |  |
| Mechanical Downtilt | Baseline: 12 degrees |  |
| Cell Selection | RSRP Slow Fading |  |
| BS antenna configuration | FR1: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2), (dH, dV) = (0.5λ, 0.5λ) |  |
| UE antenna configuration | FR1: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, -N/Aλ) |  |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |
| Scheduler | SU-MIMO, Proportional Fairness |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |
| PDCCH overhead | Modelled |  |
| Target BLER | 10% for first transmission |  |
| Max HARQ transmission | 3 |  |
| HARQ scheme | Chase combining |  |