3GPP TSG RAN WG1 #109-e	R1-2204675

e-Meeting, May 09th – May 20th, 2022

Agenda item:		9.11.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on XR-specific capacity enhancements

Document for:		Discussion and Decision

# Introduction

RAN#95 approved a Revised SID: Study on XR Enhancements for NR [1] with the following objectives:

### 4.1 Objective of SI or Core part WI or Testing part WI The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In this contribution we present our views on the evaluation methodology, main parameters, and possible capacity enhancements that will further support XR services.

# Evaluation methodology

During Release 17 XR over NR SI, companies evaluated multiple deployment scenarios and various applications with different parameters. Augmented reality (AR), virtual reality (VR), and cloud gaming (CG) applications were identified and confirmed as the applications of interest in the study. An extensive simulation campaign allowed to comprehensively study how well can XR be supported with state-of-the-art NR.

Baseline traffic model for DL and UL video was agreed with the following main characteristics:

- Video frame consists of multiple IP packets and it has the size following Truncated Gaussian Distribution with mean R×1e6 / F / 8, where R is data rate of the flow in Mbps and F is frame generation rate of the flow in fps;

- Inter-arrival time of 1/fps with jitter that follows Truncated Gaussian Distribution with truncation range [-4, 4] ms.

Baseline traffic model for UL pose/control was agreed with the following main characteristics:

- Packet with size of 100 bytes;

- Average data rate of 0.2 Mbit/s.

The outcome of Release 17 XR SI is presented in TR 38.838 [2].

One of the objectives for Rel18 XR SI, is to study XR-specific capacity improvements related to more efficient resource allocation and scheduling for XR service characteristics. The main focus is to study SPS/CG and dynamic grant improvements.

In order to provide an evaluation and comparison among different solutions, it is proposed to employ the methodology and baseline parameters agreed in Rel 17 TR 38.838 with some further down scoping to focus more on proposed solutions.

Proposal 1: Use the capacity evaluation methodology from TR 38.838 as a starting point, when developing the methodology for capacity enhancements studies in Release 18 Study on XR Enhancements for NR .

During Rel17 XR SI, a number of baseline options for deployment scenario, frequency range, TDD structure, bandwidth, BS and UE antenna patterns were agreed. A summary of main simulation parameters and deployment scenarios are given in Appendix B. Such broad range of baselines or different options limits the possibility of thorough comparison of various enhancements among companies. Thereby, we propose to identify the minimum set of simulation parameters and deployments to be submitted by companies.

Proposal 2: To facilitate efficient study of applicable capacity enhancements schemes, identify the minimum set of simulation parameters and deployments. Example areas for potential down scoping: e.g., deployment scenario, frequency range.

The traffic model parameters summarized in Table 1 were agreed to be the minimum set to be submitted by companies during Rel 17 XR SI. We propose to employ those set of parameters to be a baseline for Rel 18 XR SI.

Table 1 - XR traffic model parameters for capacity enhancements evaluation.

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

Proposal 3: The proposed unified set of traffic model parameters for capacity enhancements evaluations is summarized in Table 1.

### KPIs for capacity evaluation

As detailed in [2], the system capacity is measured in terms of “number of satisfied UEs”. More specifically, a UE is marked as “satisfied UE” if more than X% of packets are successfully transmitted over the air interface within the PDB. The PDB is applied to the whole application layer frame. The system capacity is defined as the maximum number of UEs per cell with at least Y% of UEs marked as satisfied UEs.

From agreements of Rel17 XR SI, the following baseline parameters for X and Y were chosen:

- the maximum number of users per cell with at least Y = 90 % of UEs being satisfied.

- a UE is declared a satisfied UE if more than X = 99 (%) of packets are successfully delivered within a given air interface PDB.

Proposal 4: Use the baseline KPIs for capacity evaluation from TR 38.838 and Rel17 XR over NR SI.

### Traffic models for system-level evaluation

Below, we discuss the parameters for the statistical model for XR traffic we used in the performance evaluation of our proposed capacity enhancements. The parameters have been set according to the discussion in [2]. Table 2 illustrates the parameters of the traffic models used for generating DL traffic. Table 3 illustrates the traffic models of XR applications as well as their QoS constraints used to evaluate user satisfaction and system capacity. Note that the maximum packet size of the transport layer is fixed to the maximum frame size to avoid segmentation at transport layer.

Table 2 – Parameters of DL traffic models for XR applications.

| Parameter | Value |  |
| --- | --- | --- |
| Type of content | Video (DL) | Video (DL) |
| Traffic model | Single stream for dual-eye buffer | Single stream for dual-eye buffer |
| Number of streams | 1 | 1 |
| Packet size distribution | Truncated Gaussian |  |
| Bitrate | 30 Mbps | 45 Mbps |
| Packet size (avg) | 62500 Bytes | 93750 Bytes |
| Packet size (std) | 6563 Bytes (10.5% of mean value) | 9844 Bytes (10.5% of mean value) |
| Packet size (max) | 93750 Bytes (150% of mean value) | 140625 Bytes (150% of mean value) |
| Packet size (min) | 31250 Bytes (50% of mean value) | 46875 Bytes (50% of mean value) |
| Transport protocol | UDP |  |
| Maximum transport packet size | 93750 Bytes 1 packet = 1 PDU | 140625Bytes 1 packet = 1 PDU |
| Packet Rate | 60 fps | 60 fps |
| Inter-arrival time distribution | Truncated Gaussian |  |
| Interarrival time (avg) | 16.67 ms |  |
| Interarrival time (std) | 2 ms |  |
| Interarrival time (max) | 20.67 ms |  |
| Interarrival time (min) | 12.67 ms |  |

Table 3 – Traffic models and QoS constraints used to evaluate XR applications in DL direction.

| Application | CG | VR | AR |
| --- | --- | --- | --- |
| Traffic model | Video single-stream | Video single-stream | Video single-stream |
| Bitrate | 30 Mbps | 30 Mbps | 30 Mbps |
| Packet rate | 60 fps | 60 fps | 60 fps |
| Packet Delay Budget (PDB) | 15 ms | 10 ms | 10 ms |
| Packet Error Rate (PER) | 1% | 1% | 1% |
| Number of streams | 1 | 1 | 1 |

### Mixed traffic scenario

In this part, we discuss other scenarios that not only include XR UEs in the cells, but also have other types of traffic such as enhanced Mobile Broadband (eMBB). Most of the studies for supporting XR services over NR are done considering a single type of traffic. This assumption is indeed sensible for evaluating the possibility of supporting the XR services over NR. As observed in R1-2107657 [5], NR can support up to 5-6 XR users (at 30Mbps and PDB=15ms), with the objective to increase this even further in Rel-18.

From another perspective, more realistic scenarios should also be considered to reflect the existing conditions of the networks. One of these scenarios is to consider coexistence of other traffic types such as eMBB or massive Machine Type Communication (mMTC). Several evaluation criteria can be defined to measure the performance of the solutions while supporting such mixed traffic models. As a first step, we suggest to allow studying cases with several XR UEs plus some background best effort eMBB users in each cell. If we model the eMBB best effort users as simple full buffer users, we will have a scenario where all cells always have data to transmit on all PRBs (at least for the downlink). Hence, creating a scenario with full load conditions, and there for a different SINR distribution as compared to the XR simulation cases studied during the Rel-17 SI phase with fractional load cases with only moderate number of XR UEs per cell. A case with mixed XR and eMBB traffic will serve to also study how the XR performance is influenced by the present of eMBB, as well as how enhancements for XR UEs may map to higher eMBB throughput (e.g. if being able to serve XR UEs on less radio resources).

As an example, we have simulated a case with 5 XR UEs per cell and besides, some eMBB users are added to each cell (uniformly) to increase the load of the network. The eMBB users generate a Full-buffer type of traffic with maximum PDU size of 1500 Bytes with infinite call duration and use User Datagram Protocol (UDP) transport protocol. In order to assign differentiate the two traffic types, XR UEs are assigned to a high priority XR class and the eMBB UEs are in a low priority Best Effort (BE) class. The time domain (TD) scheduler makes sure to always schedule the XR UEs first and then assign the rest of the resources to the eMBB users. With this scheme, we make sure that the XR services, that have a delay sensitive service with a strict Packet Delay Budget (PDB) of 10 to 15ms, will be served first, before other non-delay critical services can use the rest of abundant resources. Moreover, a similar prioritization exists in the frequency domain (FD) scheduler, after the decision of the TD scheduler. For instance, we compare the performance of two different schemes in FD, namely a Proportional Fair (PF) with no prioritization and an Application-Aware (AA) one with prioritization.

In order to see the effect of coexisting of different services on QoE of the XR users in Dense Urban scenario, Figure 1 presents (a) the ratio of satisfied XR users and (b) the average RB load in the downlink as more eMBB UEs are added to each cell. In part (a), it compares the ratio of satisfied UEs when cells are evenly loaded of the proportional-fair (label ‘PF’) and application-aware (label ‘AA’) scheduling schemes in the FD. Moreover, in part (b) a similar comparison is done, but the main focus is to the change of RB load in this mixed traffic scenario.

| (a) CG in FR1 at 30Mbps with X=99% capacity evaluation. | (b) AR/VR in FR1 at 30Mbps average resource block load in the downlink. |
| --- | --- |

Figure 1 – (a)VR/AR (Video Single-Stream) capacity evaluation of Dense Urban mixed traffic in FR1 with X=99% of frames received within PDB=10ms. The lower bound on the percentage of satisfied users is Y=90% (dashed line). (b) Average RB load in the downlink for the mixed traffic scenario.

Observation 1: Adding more eMBB users deteriorates the QoE of the XR users. This is the key point that should be considered when using more realistic simulation scenarios with multiple types of traffic.

Proposal 5: It is suggested to include one optional simulation case with XR and best effort eMBB users. All the scenario assumptions for DU and InH, as well as the XR traffic models and XR performance KPIs remain the same as in [2], while just adding N full buffer eMBB background users per cell. The default value of N is 1, but other values are also acceptable.

Proposal 6: For the option with XR and best effort eMBB users, we suggest defining additional standard KPIs for eMBB performance such as: average aggregated eMBB cell throughput, 5%-ile, 50%-ile and 95%-ile eMBB user throughput. Other KPIs to reflect the eMBB performance are not excluded.

# SPS/CG and dynamic grant scheduling

## Downlink scheduling

Scheduling in DL can be realized by employing SPS or dynamic grant. SPS is configured via RRC message and activated/deactivated by sending DCI. The main characteristics of NR Rel-16 DL SPS are:

•	DL radio resources for sending one transport block with a regular time-periodicity is configured for a UE.

•	Periodicity of any integer of a slot (N*14). Minimum periodicity in Rel-15 is 10 ms.

•	Up to 8 SPS configurations per BWP (configured through RRC signaling).

The XR video traffic can be characterized as follows:

- Non-integer periodicity (e.g., 16.6 ms for 60 fps);

- non-negligible and varying jitter (e.g., following Truncated Gaussian distribution in a range [-4,4] ms as per Rel17 XR SI agreements);

- Large and varying video frame size (e.g., following Truncated Gaussian distribution in a range [31; 93] kB for 60 fps and 30 Mbit/s);

- Strict PDB requirements (10, 15 ms for AR/VR and CG)

As can be observed from the main XR video traffic characteristics, due to the non-negligible jitter, non-integer periodicity, and quite large size of one video frame SPS is not directly applicable for XR use case. As reported by several companies in RAN1 Rel17 XR over NR SI, it is evident that current SPS (with integer periodicity) does not support configurations that match the considered values of 60 fps, 90 fps, and 120 fps. Secondly, it is evident that the payloads coming with XR services are rather large; if the average source data rate e.g., equals 45 Mbps with 60 fps, the average payload size per frame equals 750 kbit. This is so large that in many cases will need to be transmitted to the UE in multiple transport blocks. Furthermore, it should be recognized that XR applications may use adaptive encoding schemes where the source data rate (and potentially the number of frames per second) may be occasionally changed.

All of the above need to be carefully considered when introducing SPS enhancements and clear benefit of enhanced SPS (eSPS) for XR use cases should be provided to motivate the changes.

Proposal 7: When introducing SPS enhancements, a clear benefit of SPS/eSPS for XR use cases should be provided.

## Uplink scheduling

Scheduling in UL can be realized by employing configured (CG) or dynamic grant. For CG based scheduling, the parameters are configured via RRC message. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2). The main characteristics of NR Rel-16 UL CG are:

•	UL radio resources for sending one transport block with a regular time-periodicity is configured for a UE.

•	The periodicities are: 2, 7, n*14 symbols, where the range for n depends on configured SCS.

•	Up to 12 configured grant configurations per BWP (configured through RRC signaling).

There is a pose update/control information delivered by UE for various XR applications. The main characteristics of UL pose/control traffic can be summarized as follows:

- Packet with size of 100 bytes (as per Rel17 XR SI agreements);

- Data rate of 0.2 Mbit/s (as per Rel17 XR SI agreements);

- Packet inter-arrival time is 4 ms with no jitter (as per Rel17 XR SI agreements);

- PDB of 10 ms.

The traffic characteristics of UL pose/update make CG more applicable to be applied in UL in its current form. It should be carefully studied whether any CG enhancements are needed and a clear benefit of enhanced CG (eCG) should be shown to motivate any changes.

Proposal 8: When introducing CG enhancements, a clear benefit of CG/eCG for XR use cases should be provided.

# Other enhancements

In this section, we illustrate enhancements to other features than SPS and CG for the improvement of the system capacity. In particular, we illustrate enhancements to the link adaptation when CBG-based transmission is used.

## Enhanced link adaptation for CBG-based transmission

For XR cases with large incoming frames, we typically schedule the user with really large transport blocks. The packet delay budget (PDB) for XR video transmissions is in order of 10-15 ms for the RAN part [2], which means that it allows for at least one HARQ transmission, and in many cases, two retransmissions can be afforded without jeopardizing the PDB. However, due to the large TB sizes for XR video frames, it becomes costly from a memory point of view if we start to have too many retransmissions.

Using code block group (CBG) based transmissions is therefore an attractive choice for XR use cases. CBG-based transmissions is specified in 3GPP TS 38.214. Details of HARQ feedback for CBG based transmissions are specified in 3GPP TS 38.213. Aspects of CBG-based transmissions also appear in the MAC specification (3GPP TS 38.321). The basic principle is that the TB is organized as into multiple Code Blocks (CBs). The maximum size of a CB is 8448 bits. The CBs are grouped into CBGs. For each received TB, the receiver provides feedback to indicate which CBGs are in error, and only the erroneously received CBGs are thereafter retransmitted by the transmitter. For transmission of large TB for XR use cases as defined in 3GPP TR 38.838, cases with 8 CBGs per TB can be supported by current NR specs. More generally, the maximum number of CBGs per TB is configurable as N ∈ {2, 4, 6, 8} for the PDSCH. If we e.g. consider a TB that consists of 8 CBGs (as supported by the NR specs), one can easily calculate that for a 10% first transmission BLER, only one or two CBGs will be retransmitted. For a 50% first transmission BLER, there is 99.9% probability of retransmitting 1-3 CBGs. These simple numerical examples are under simplified assumptions of identical independent error probabilities for the CBGs. Nevertheless, it shows that even with high initial BLER values for first transmissions, only a modest fraction of CBGs in the TB require retransmission.

The currently defined CQI feedback schemes for NR rely on providing information to guide the gNB on the maximum modulation and coding scheme (MCS) that it can use without exceeding a certain first transmission BLER target. Such CQI solutions are attractive for the legacy HARQ transmission modes, where the full transport block (TB) is retransmitted in case of errors, and hence it makes sense to have CQI feedback that expresses the recommended MCS for a certain BLER of the first transmission of the TB. However, for CBG-based HARQ operation, the full TB is seldom retransmitted as only the potentially erroneously received CBGs are subject to retransmission. The current CQI – and link adaptation scheme – is therefore suboptimal for cases with CBG-based transmissions. It is therefore proposed to further study enhanced CQI scheme that guides the gNB on the maximum MCS scheme it can use while ensuring that only a certain maximum subset of CBGs will need retransmission with a controllable probability. E.g. have a CQI scheme that guides the gNB to use an MCS index such that at most 4 CBGs (out of 8 CBGs) will require retransmission with P=0.1 probability (10%). While some estimations can be achieved on the gNB side, a more accurate assessment is available on the UE side where the channel quality and correlations among CBGs and CBG errors can be monitored in detail.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) AR/VR in FR1 at 30Mbps with X=99%. |
| --- | --- |

Figure 2 – DL Capacity evaluation of the different OLLA schemes for CG traffic (Video Single-Stream) and AR/VR (Video Single-Stream) in Indoor Hotspot deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Figure 2 shows the percentage of satisfied users for Indoor Hotspot scenario in FR1 for different OLLA algorithms. The detailed set of parameters is given in subsection 2.1.2 and Appendix A. For this simulation a maximum rank of 2 is set, while a rank adaptation mechanism is used to assign variable number of streams to UEs. Moreover, the scenarios with data rate of 30Mbps with PDB = 10 ms for AR/VR and 15ms for CG are considered. Three different algorithms are compared: legacy OLLA with TB-based transmission, legacy OLLA with CBG-based transmission and enhanced OLLA (eOLLA) with CBG-based transmission. The first algorithm uses binary HARQ feedback for the reception of the TB, the second one combines all the feedbacks (ACK/NACK from all the CBGs) and converts them into a binary value (i. e, the feedback is an ACK if all the CBGs get an ACK, and it is NACK if at least one CBG gets a NACK). For the third method, multibit feedback is used, where each bit (i.e. one bit for each CBG) has an effect on the OLLA offset adjustment.

For the case of eOLLA, CQI offset variation is proportional to the number of ACKs from the CBGs divided by the total number of CBGs. Moreover, the CQI offset increase is only triggered with both first transmission and first retransmission failure. With this method, eOLLA is controlling the BLER of the second transmission (or first retransmission). It is expected that the resource utilization of such schemes will be higher than the traditional ones. This is due to the fact that the block error rate of the second transmission is much lower than the first one (assuming that a chase combining method is used to increase the probability of decoding in the second transmission). Thus, the gNB will mostly select higher MCS indices and therefore spectral efficiency and radio resource utilization will increase. Such improvements are essential for the XR traffic types that require a vast amount of radio resources.

Based on the presented performance results, we draw the following observations and suggest the following:

Observation 2: Use of CBG-based HARQ transmissions is beneficial for XR use cases, given the large transport block sizes, as well as the PDB that allows for couple of HARQ retransmissions.

Observation 3: The current link adaptation mechanisms, and the corresponding UE CQI feedback designs are suboptimal for CBG-based transmissions.

Observation 4: Application of enhanced CQI scheme can enable enhanced OLLA to improve the performance of the CBG-based transmission. For instance, as shown in Figure 2, the number of satisfied users has increased by 2 times for the case with enhanced CQI and eOLLA.

Proposal 9: Further study enhanced CQI schemes that guides the gNB on the maximum MCS scheme, subject to controlling CBG errors is proposed. Such a mechanism may e.g. ensure that only a certain maximum subset of CBGs will need retransmission with a controllable probability.

Simulation results for the Dense Urban at 30Mbps and Indoor Hotspot at 45Mbps scenarios are included in R1-2204676 Performance results of XR-related enhancements [3].

# Conclusion

In this contribution, we discussed evaluation methodology and XR-related capacity enhancements. The following observations have been made:

Observation 1: Adding more eMBB users deteriorates the QoE of the XR users. This is the key point that should be considered when using more realistic simulation scenarios with multiple types of traffic.

Observation 2: Use of CBG-based HARQ transmissions is beneficial for XR use cases, given the large transport block sizes, as well as the PDB that allows for couple of HARQ retransmissions.

Observation 3: The current link adaptation mechanisms, and the corresponding UE CQI feedback designs are suboptimal for CBG-based transmissions.

Observation 4: Application of enhanced CQI scheme can enable enhanced OLLA to improve the performance of the CBG-based transmission. For instance, as shown in Figure 2, the number of satisfied users has increased by 2 times for the case with enhanced CQI and eOLLA.

The following main proposals have been made:

Proposal 1: Use the capacity evaluation methodology from TR 38.838 as a starting point, when developing the methodology for capacity enhancements studies in Release 18 Study on XR Enhancements for NR .

Proposal 2: To facilitate efficient study of applicable capacity enhancements schemes, identify the minimum set of simulation parameters and deployments. Example areas for potential down scoping: e.g., deployment scenario, frequency range.

Proposal 3: The proposed unified set of traffic model parameters for capacity enhancements evaluations is summarized in Table 1.

Proposal 4: Use the baseline KPIs for capacity evaluation from TR 38.838 and Rel17 XR over NR SI.

Proposal 5: It is suggested to include one optional simulation case with XR and best effort eMBB users. All the scenario assumptions for DU and InH, as well as the XR traffic models and XR performance KPIs remain the same as in [2], while just adding N full buffer eMBB background users per cell. The default value of N is 1, but other values are also acceptable.

Proposal 6: For the option with XR and best effort eMBB users, we suggest defining additional standard KPIs for eMBB performance such as: average aggregated eMBB cell throughput, 5%-ile, 50%-ile and 95%-ile eMBB user throughput. Other KPIs to reflect the eMBB performance are not excluded.

Proposal 7: When introducing SPS enhancements, a clear benefit of SPS/eSPS for XR use cases should be provided.

Proposal 8: When introducing CG enhancements, a clear benefit of CG/eCG for XR use cases should be provided.

Proposal 9: Further study enhanced CQI schemes that guides the gNB on the maximum MCS scheme, subject to controlling CBG errors is proposed. Such a mechanism may e.g. ensure that only a certain maximum subset of CBGs will need retransmission with a controllable probability.

# References

- RP-220285, Revised SID: Study on XR Enhancements for NR, RAN#95.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17)

- R1-2204676, Performance results of XR-related enhancements, Nokia, Nokia Shanghai Bell, RAN1 109-e.

- TS 23.501 System architecture for the 5G System (5GS), Release 17, v17.4.0

- R1-2107657, Performance results in indoor hotspot and dense urban deployments of CG and VR/AR applications, Nokia, Nokia Shanghai Bell, RAN1 106-e.

# Appendix A –Simulation settings for capacity enhancements

In this Appendix A, we summarize the main simulation settings used for capacity enhancements evaluation.

## Indoor Hotspot (InH) scenario

The carrier frequency is set to 4 GHz for FR1. System bandwidth is assumed to be 100 MHz for FR1. Time division duplexing (TDD) is configured as “DDDSU”. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 kHz for FR1. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set as follows: 31dBm with 100MHz (24dBm per 20MHz) in FR1. Table 4 lists the main parameters of the Indoor Hotspot deployment that are considered in this study.

Table 4 – Main parameters for Indoor Hotspot (InH) deployment

| Parameter | Value |
| --- | --- |
| Layout | - 120m x 50m, Single layer (indoor floor, open office) - 12 cells/TRPs - ISD: 20m |
| Channel model | InH |
| Carrier frequency | FR1: 4 GHz |
| Subcarrier spacing | 30 kHz |
| System bandwidth | 100 MHz |
| BS height | 3 m |
| UE height | 1.5 m |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | LMMSE-IRC |
| UE receiver | LMMSE-IRC |
| Channel estimation | Realistic (with ideal CSI) |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS Tx power | 31dBm (24dBm per 20MHz) |
| UE Tx max power | 23 dBm |
| TDD Frame structure | DDDSU |
| Cell Selection | RSRP Slow Fading |
| BS antenna configuration | - Pattern: Ceiling-mount antenna radiation pattern - Gain: 5 dBi - Downtilt: 90° |
|  | - 32 TxRU - (M, N, P, Mg, Ng) = (4,4,2,1,1) - (dH, dV) = (0.5, 0.5)λ - (Mp, Np) = (4,4) |
| UE antenna configuration | - Pattern : Omni-directional, - Gain : 0 dBi, - Configuration : 2T/4R (M, N, P, Mg, Ng) = (1,2,2,1,1) (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,2) |
| Scheduler | SU-MIMO, Proportional Fairness |
| CSI acquisition | Periodic CQI on 2 ms period |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |
| PDCCH overhead | Modelled |
| Target BLER | 10% for first transmission |
| Max HARQ transmission | 3 |
| HARQ scheme | Chase combining |

## Dense Urban (DU) Scenario

The carrier frequency is set to 4 GHz for FR1. System bandwidth has been fixed to 100MHz for FR1. Time division duplexing (TDD) is configured as “DDDSU”. The smallest schedulable radio resource is the physical radio blocks (PRB) of 12 subcarriers, each of 30kHz for FR1. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmissions before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set to 51 dBm (i.e., 44dBm for 20MHz). Table 5 lists the main parameters of the Dense Urban deployment that are considered in this study.

Table 5– Main parameters for Dense Urban (UMa) deployment

| Parameter | Value |  |
| --- | --- | --- |
| Layout | 21 cells with wraparound (ISD: 200m) |  |
| Channel model | Uma |  |
| Carrier frequency | FR1: 4 GHz |  |
| Subcarrier spacing | 30 kHz |  |
| System bandwidth | Option 1: 100 MHz |  |
| BS height | 25m |  |
| UE height | hUT = 3(nfl – 1) + 1.5 |  |
|  | Outdoor: nfl = 1 | Indoor: - nfl ~ uniform(1,Nfl) - Nfl ~ uniform(4,8) |
| BS noise figure | 5 dB |  |
| UE noise figure | 9 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic (with ideal CSI) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS Tx power | 44 dBm per 20 MHz 51 dBm per 100 MHz |  |
| UE Tx max power | 23 dBm |  |
| TDD Frame structure | Option 1: DDDSU |  |
| Mechanical Downtilt | Baseline: 12 degrees |  |
| Cell Selection | RSRP Slow Fading |  |
| BS antenna configuration | 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2), (dH, dV) = (0.5λ, 0.5λ) |  |
| UE antenna configuration | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, -N/Aλ) |  |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |
| Scheduler | SU-MIMO, Proportional Fairness |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |
| PDCCH overhead | Modelled |  |
| Target BLER | 10% for first transmission |  |
| Max HARQ transmission | 3 |  |
| HARQ scheme | Chase combining |  |

# Appendix B – Summary of Rel 17 XR SI simulation parameters

In this Appendix B, we summarize the main simulation settings agreements during Rel 17 XR SI for capacity evaluations.

## TDD configuration

It is up to companies to choose either Option 1 (DDDSU) or Option 2 (DDDUU) for TDD configuration and do the evaluation.

Adopt following update for TDD configuration for XR/CG evaluation

- FR1:

  - Option 1: DDDSU

  - Option 2: DDDUU

- FR2:

  - Option 1: DDDSU

  - Option 2: DDDUU

Detailed S slot format is 10D:2F:2U. Other S slot format(s) can also be optionally evaluated.

There is 2-symbol gap at the end to third “D” slot of DDDUU for FR1/FR2 of Option 2.

## System bandwidth

System bandwidth for XR/CG evaluations are as follows.

- For FR1,

  - Baseline: 100 MHz

  - Optional: 20/40 MHz, 2*100 MHz with CA

- FR2

  - Option 1: 100 MHz

  - Option 2: 400 MHz

Companies should report the CA setting if CA is adopted.

Other system bandwidth can also be optionally evaluated.

## BS Antenna

For outdoor scenarios, the BS antenna parameters are as follows.

- FR1,

  - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

  - Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2)

- Company to report the BS antenna parameters for XR/CG evaluation.

- Other BS antenna parameters can also be optionally evaluated.

(dH, dV) = (0.5λ, 0.5λ)

- FR2:

  - 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

(dH, dV) = (0.5λ, 0.5λ)

Other configurations can be optionally evaluated.

Adopt the following BS antenna parameters for indoor scenario for XR/CG evaluation

- FR1:

  - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4)

  - (dH, dV) = (0.5, 0.5)λ

- FR2:

  - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1)

  - (dH, dV) = (0.5, 0.5)λ

Other BS antenna parameters can be optionally evaluated

For XR/CG evaluation, adopt the following assumptions for downtilt

- Dense Urban in FR1

  - 12 degree

  - Other downtilt can be optionally evaluated.

- Indoor hotspot

  - 90° (pointing to the ground)

Other downtilt can be optionally evaluated

## UE Antenna

UE antenna parameters for XR/CG evaluations are as follows

- FR1:

  - Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ

  - Optional: 4T/4R, 1T/2R, 2T2R

- FR2:

- Option 1 (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151)

  - (M, N, P)=(1, 4, 2), 3 panels (left, right, top)

- Option 2 (from TR 38.802 – developed in Rel-14)

  - 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90°

Company to report the UE antenna parameters for XR/CG evaluation.

Other UE antenna parameters can also be optionally evaluated.

## Deployment

Adopt the following deployment e2e for XR/CG evaluations

- Indoor hotspot: FR1 and FR2

  - Detailed definition of Indoor hotspot refers to TR 38.913.

  - Channel model: InH. Detailed definition of InH refers to TR 38.901.

- Dense urban: FR1 and FR2

  - Detailed deployment refers to TR 38.913, where single layer with Marco layer is assumed.

  - Channel model: UMa. Detailed definition of UMa refers to TR 38.901.

Note 1: When selecting the deployment and evaluation assumptions for XR/CG evaluations, it is up to company to evaluate FR1 or FR2 or both for the frequency range.

Note 2: It does not mean that all applications are evaluated for all the deployment scenarios.

For Dense urban and Urban Macro, the UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873.

|  |  | Urban Micro/Macro cell with high UE density (3D-UMi) /(3D-UMa) |
| --- | --- | --- |
| UE height (hUT) in meters | general equation | hUT=3(nfl – 1) + 1.5 |
|  | nfl for outdoor UEs | 1 |
|  | nfl for indoor UEs | nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |

Adopt the following UE distribution for XR/CG evaluation for outdoor scenario

- For outdoor scenario:

  - FR1: 80% indoor, 20% outdoor

  - FR2: 100% outdoor

Other UE distribution can be evaluated optionally.

Table 6: Simulation assumptions for XR evaluation

| Parameter | Proposed value |  |
| --- | --- | --- |
|  | Indoor hotspot FR1/FR2 | Dense urban FR1/FR2 |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21cells with wraparound ISD: 200m |
| Carrier frequency | FR1: 4 GHz FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz FR2: 120 kHz |  |
| BS height | 3m | 25m |
| UE height | hUT=1.5 m |  |
| BS noise figure | FR1: 5 dB FR2: 7 dB |  |
| UE noise figure | FR1: 9 dB FR2: 13 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic Ideal (optional) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, FR2: UE antenna radiation pattern model 1, 5dBi |  |

Table 7: Simulation assumptions for XR evaluation

| Power control parameter | Companies should report |
| --- | --- |
| Transmission scheme | Companies should report |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report • CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, • Assumptions on SRS: periodicity, processing gain, processing delay, etc • and etc. |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 Optional: UE PDSCH processing Capability #2 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target BLER | Companies should report |
| Max HARQ transmission | Companies should report |

Urban Macro can be reported for XR/CG evaluations only for FR1.

- FFS: whether Uma is optional or not

- Following parameters can be assumed.

| Parameter | Proposed value |
| --- | --- |
|  | Urban Macro (FR1) |
| Layout | 21cells with wraparound ISD = 500 m |
| BS Tx power | FR1: 49 dBm/20 MHz |
| BS height | 25m |