3GPP TSG RAN WG1 #109-e	R1-2204676

e-Meeting, May 09th – May 20th, 2022

Agenda item:      9.11.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Performance results of XR-related enhancements

Document for:     Discussion

# Introduction

RAN#95 approved a Revised SID: Study on XR Enhancements for NR [1] with the following objectives.

### 4.1 Objective of SI or Core part WI or Testing part WI The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In this contribution, we provide additional simulation results for the XR-related enhancements and additional modeling considerations.

# Simulation results

## Additional results for the capacity enhancement of enhanced link adaptation simulations

In this section, we present additional performance results for the enhanced CQI enabling enhanced OLLA simulations. All main simulation parameters are given in Appendix A. Figure 1 shows the ratio of satisfied users for the Indoor Hotspot scenario at 45Mbps for CG and AR/VR applications with rank adaptation enabled. As can be seen, by enabling eOLLA scheme via enhanced CQI increases the number of satisfied users from 3 to 5 UEs for CG and from 2 to 4 UEs for AR/VR applications. As expected, and compared to the case with 30Mbps data rate, the number of satisfied users is lowered for the other cases without enhanced CQI for all the OLLA variants due to larger transport block sizes that cannot be delivered within the delay budget. In addition, Figure 2 presents the results for the Dense Urban case at 30Mbps data rate. There is one additional user that can be satisfied by applying eOLLA scheme compared to legacy OLLA.

| (a) CG in FR1 at 45Mbps with X=99%. | (b) AR/VR in FR1 at 45Mbps with X=99%. |
| --- | --- |

Figure 1 – DL Capacity evaluation of the different OLLA schemes for CG traffic (Video Dual-Stream) and AR/VR (Video Dual-Stream) in Indoor Hotspot deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Figure 2 – DL Capacity evaluation of the different OLLA schemes for AR/VR (Video Single-Stream) in Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Average end-to-end delay evaluation is shown in Figure 3 for InH scenario at 30Mbps data rate comparing the performance of OLLA variants. Two different delay budgets are depicted with dashed lines where a 10ms PDB is for the case of AR/VR applications and the PDB of 15ms is for the CG case. As can be seen, the average delay performance of the eOLLA scheme enabled by the eCQI is much lower than the other OLLA types, making it more favorable for the XR traffic types that are delay sensitive. The main reason for this low delay profile is first, inherit smaller retransmission size of CBG-based transmission compared to full TB-based one. The second reason is due to use of higher MCS indices in eOLLA, packets have lower duration in time and thus, overall delay is shorter for this type of link adaptation technique.

Figure 3 – Average delay evaluation of the different OLLA schemes for AR/VR (Video Single-Stream) in Indoor Hotspot deployment in FR1. Two PDBs are shown for CG (15ms) and AR/VR (10ms) applications each (dashed line).

Observation 1:  Enhanced CQI scheme can enable advanced link adaptation resulting in lower average delay, enhancing the achievable performance for delay sensitive XR traffic types.

## Inter-frame dependency

Application awareness of the PDUs that belong to the same XR frame as well as the interrelation of XR frames can be used to derive more accurate capacity measurements reflected from application perspective. For example, a video stream in an XR application can have different types of frames (e.g., I/P/B frames) within a defined group of pictures (GOP) where the utility and usability of one frame type in the UE application depends on the successful reception of another frame type. To this end, we resort on the solution proposed in [3] to support PDU Set (i.e., XR frame) integrated packet handling in order to identify PDUs that belong to the same XR frame and GOP.

In our evaluation, we illustrate the results on the system capacity obtained for VR/AR applications with a GOP-based two streams traffic model for I-frame and P-frame, with PDB=10ms and PER=1% as agreed in [2]. GOP size is selected as K=8 and the GOP structure is assumed to start with one I-frame followed by 7 P-frames. The average ratio between I-frame and P-frame is selected as α=1.5. In this evaluation, we also define intra-GOP dependency where frames within a GOP can depend on each other for successful reception. It is assumed that if the frame or one of the dependencies of the frame is not received successfully according to the successful reception criteria, that frame is assumed to be not successfully received. According to this, we evaluate two scenarios, namely 1) where each frame in a GOP is assumed to be independent from one another, and 2) where each frame in a GOP is assumed to depend on the predecessor frame in the same GOP.

Figure 4 shows the ratio of satisfied UEs computed with 99% of frames received within the PDB and PER as a function of the average cell load, which is measured in number of UEs; results correspond to a system bandwidth of 100MHz for FR1. All main simulation parameters are given in Appendix A. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we can conclude that in the Indoor Hotspot deployment in FR1 we can support on average 5 UEs per cell when there is no dependency among the frames within GOP and 2 UEs per cell running VR/AR applications at 30Mbps with X=99% when there is dependency among frames within GOP, respectively.

Figure 4 – VR/AR capacity evaluation of InH traffic in FR1 with X=99% of frames received within PDB and PER and 100MHz system bandwidth without and with intra-GOP XR frame dependencies. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Table 1 – System capacity for VR/AR (PDB=10ms, PER=1%) at 30Mbps [#UEs/cell] in an InH deployment in FR1

|  | W/o Dependency | W/ Dependency |
| --- | --- | --- |
| FR1 (100 MHz) X=99% | 5 | 2 |

Observation 2: When considering GOP-based two-stream VR/AR traffic model at 30Mbps for an InH deployment in FR1, one can support the number of UEs per cell as indicated in Table 1.

Observation 3: When considering GOP-based two-stream VR/AR traffic model at 30Mbps for an InH deployment in FR1, supported number of UEs per cell decreases when considering dependency between different frames within GOP.

Observation 4: For certain capacity improvements (e.g., packet discarding) the dependency between frames should be taken into account.

## Sparse PDCCH monitoring

PDCCH decoding is one of the main contributors to UE power consumption, therefore adjusting the PDCCH monitoring periodicity allows for UE power saving; following the procedure in TS 38.213 (sec. 10.1: UE procedure for determining physical downlink control channel assignment)  and explained more in detail in [4], Figure 5 shows the system capacity and UE power saving for three values of PDCCH monitoring frequency, ks,  in an FR1 InH XR deployment assuming the parameters shown in Appendix A and a single search space per UE.

The results show a UE power saving between 36-39% for  ks=2 and between 50-53% for  ks=4, when compared to the baseline case with  ks=1 (i.e., PDCCH monitored in every slot), at the expenses of a high-capacity loss. For CG, when comparing to the baseline scenario at system capacity, i.e., 5 UEs/cell, only 40% of the UEs were satisfied when  ks=2 , with this number decreasing to 18% when  ks=4. For VR/AR, the capacity loss is even higher due to the lower PDB, with 19% and 13% satisfied UEs for  ks=2 and   ks=4, respectively.

| - Capacity for CG in FR1 at 30Mbps. | - Capacity for VR/AR in FR1 at 30Mbps. |
| --- | --- |
| (c) UE power saving for CG and VR/AR in FR at 30Mbps. |  |

Figure 5 – DL capacity and UE power saving in FR1 InH for CG and VR/AR considering different values of PDCCH monitoring periodicity,  ks.

Observation 5: Using a single PDCCH monitoring periodicity  ks>1  for XR services results in significant UE power saving at the expenses of high-capacity loss.

Observation 6: For both CG and VR/AR, results show a UE power saving between 36-39% for  ks=2 and between 50-53% for  ks=4, when compared to the baseline case with  ks=1.

Observation 7:  For CG, when evaluating at system capacity and comparing to the baseline scenario with  ks=1, only 40% and 18% satisfied UEs were observed for  ks=2 and  ks=4, respectively.

Observation 8: For VR/AR, when evaluating at system capacity and comparing to the baseline scenario with  ks=1, only 19% and 13% satisfied UEs were observed for  ks=2 and  ks=4, respectively.

These results show that configuring a  ks value higher than 1 for XR services might result in an undesirable system performance; a solution then is to rely of PDCCH monitoring adaptation according to traffic behaviour to have e.g.,     ks=1 during the activity periods and  ks> 1 during the inactivity periods.

Figure 6 shows the simulation results for the same scenario as in Figure 5 with 5 UEs/cell deployed (system capacity), but considering two values of  ks, with  ks=1 being applied during the data activity periods and until the last packet from the XR frame has been delivered and with  ks= 2  or  ks=4 being applied during the data inactivity periods and for a period equal to “PDCCH monitoring adaptation window length”. The results show that adapting the PDCCH monitoring periodicity to the data behaviour can offer a good trade-off between system capacity and UE power consumption; if we consider a PDCCH monitoring periodicity adaption period of 10 ms or less, the same number of UEs could still be satisfied (i.e., 5 UEs/cell) while getting a UE power saving between 10-24% depending on the configuration and the XR application. From the results it can also be seen that an appropriate selection of the adaptation period could set the difference between losing or not system capacity, which is mainly due to the XR traffic jitter. However, these results do not consider possible delays or interruptions associated with changing the value of  ks, which could cause a higher capacity loss than that shown in the results depending on the approach used to change  ks (e.g., via BWP switch or search space sets group switching, as explained in [4]. Nevertheless, adjusting the PDCCH monitoring periodicity could be considered to increase the UE power saving without heavily compromising the system capacity for XR services.

| - Capacity for CG in FR1 at 30Mbps. | - Capacity for VR/AR in FR1 at 30Mbps. |
| --- | --- |
| (c) UE power saving for CG and VR/AR in FR1 at 30Mbps. |  |

Figure 6  –   DL capacity and UE power saving in FR1 InH for CG and VR/AR considering 5 UEs/cell and adaptation of PDCCH monitoring periodicity,  ks.

Observation 9: XR services might benefit from PDCCH monitoring adaptation according to traffic behaviour to have e.g.,  ks=1 during the activity periods and  ks> 1 during the inactivity periods.

Observation 10: For both CG and VR/AR, when adapting the PDCCH monitoring frequency during the inactivity periods, the results show a UE power saving gain between 10-24% without significant capacity loss for adaptation periods lower than 10ms.

Observation 11: Further evaluation of the impact of adapting the PDCCH monitoring frequency on the system performance are recommended, especially considering the possible delays associated with changing the value  ks.

# Conclusion

In this contribution we have discussed additional simulation results related to capacity and power saving evaluations.

Particularly, the following observations have been made:

Observation 1:  Enhanced CQI scheme can enable advanced link adaptation resulting in lower average delay, enhancing the achievable performance for delay sensitive XR traffic types.

Observation 2: When considering GOP-based two-stream VR/AR traffic model at 30Mbps for an InH deployment in FR1, one can support the number of UEs per cell as indicated in Table 1.

Observation 3: When considering GOP-based two-stream VR/AR traffic model at 30Mbps for an InH deployment in FR1, supported number of UEs per cell decreases when considering dependency between different frames within GOP.

Observation 4: For certain capacity improvements (e.g., packet discarding) the dependency between frames should be taken into account.

Observation 5: Using a single PDCCH monitoring periodicity  ks>1  for XR services results in significant UE power saving at the expenses of high-capacity loss.

Observation 6: For both CG and VR/AR, results show a UE power saving between 36-39% for  ks=2 and between 50-53% for  ks=4, when compared to the baseline case with  ks=1.

Observation 7:  For CG, when evaluating at system capacity and comparing to the baseline scenario with  ks=1, only 40% and 18% satisfied UEs were observed for  ks=2 and  ks=4, respectively.

Observation 8: For VR/AR, when evaluating at system capacity and comparing to the baseline scenario with  ks=1, only 19% and 13% satisfied UEs were observed for  ks=2 and  ks=4, respectively.

Observation 9: XR services might benefit from PDCCH monitoring adaptation according to traffic behaviour to have e.g.,  ks=1 during the activity periods and  ks> 1 during the inactivity periods.

Observation 10: For both CG and VR/AR, when adapting the PDCCH monitoring frequency during the inactivity periods, the results show a UE power saving gain between 10-24% without significant capacity loss for adaptation periods lower than 10ms.

Observation 11: Further evaluation of the impact of adapting the PDCCH monitoring frequency on the system performance are recommended, especially considering the possible delays associated with changing the value  ks.

# References

- RP-220285, Revised SID: Study on XR Enhancements for NR, RAN#95.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17).

- TR 23.700-60, Study on XR (Extended Reality) and media services (Release 18).

- R1-2204674, Discussion on XR-specific power saving enhancements, Nokia, Nokia Shanghai Bell, RAN1 109-e.

# Appendix A –Simulation settings for capacity and power saving evaluations

In this Appendix, we summarize the main simulation settings used for capacity and power saving evaluation.

## Traffic models for system-level evaluation

Below, we discuss the parameters for the statistical model for XR traffic we used in the performance evaluation of our proposed capacity and power saving evaluations. The parameters have been set according to the discussion in [2]. Table 2  illustrates the parameters of the traffic models used for generating DL traffic. Table 3  illustrates the traffic models of XR applications as well as their QoS constraints used to evaluate user satisfaction and system capacity. Note that the maximum packet size of the transport layer is fixed to the maximum frame size to avoid segmentation at transport layer.

Table 2 – Parameters of DL traffic models for XR applications

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Type of content | Video (DL) | Video (DL) | Video (DL) |  |
| Traffic model | Single stream for dual-eye buffer | Single stream for dual-eye buffer | Two streams of I-frame and P-frame (GOP-based) (see NOTE 1) |  |
| Number of streams | 1 | 1 | 2 |  |
| Packet size distribution | Truncated Gaussian |  |  |  |
| Bitrate | 30 Mbps | 45 Mbps | 30 Mbps |  |
| Packet size (avg) | 62500 Bytes | 93750 Bytes | I-Frame | P-Frame |
|  |  |  | 88235 Bytes | 58823 Bytes |
| Packet size (std) | 6563 Bytes (10.5% of mean value) | 9844 Bytes (10.5% of mean value) | 9264 Bytes (10.5% of mean value) | 6176 Bytes (10.5% of mean value) |
| Packet size (max) | 93750 Bytes (150% of mean value) | 140625 Bytes (150% of mean value) | 132352 Bytes (150% of mean value) | 88234 Bytes (150% of mean value) |
| Packet size (min) | 31250 Bytes (50% of mean value) | 46875 Bytes (50% of mean value) | 44117 Bytes (50% of mean value) | 29411 Bytes (50% of mean value) |
| Transport protocol | UDP |  |  |  |
| Maximum transport packet size | 93750 Bytes 1 packet = 1 PDU | 140625Bytes 1 packet = 1 PDU | 132352 Bytes 1 packet = 1 PDU |  |
| Packet Rate | 60 fps | 60 fps | 60 fps |  |
| Inter-arrival time distribution | Truncated Gaussian |  |  |  |
| Interarrival time (avg) | 16.67 ms |  |  |  |
| Interarrival time (std) | 2 ms |  |  |  |
| Interarrival time (max) | 20.67 ms |  |  |  |
| Interarrival time (min) | 12.67 ms |  |  |  |

NOTE 1: For the video DL traffic with two streams of I-Frame and P-frame is GOP-based with GOP size selected as K=8 and follows the GOP structure of one I-frame followed by seven P-frames. The average size ratio between I-frame and P-frame is selected as α=1.5.

Table 3 – Traffic models and QoS constraints used to evaluate XR applications in DL direction.

| Application | CG | VR | AR | VR |
| --- | --- | --- | --- | --- |
| Traffic model | Video single-stream | Video single-stream | Video single-stream | Video two-stream of I-frame and P-frame (GOP-based) |
| Bitrate | 30 Mbps | {30, 45} Mbps | {30, 45} Mbps | 30 Mbps |
| Packet rate | 60 fps | 60 fps | 60 fps | 60 fps |
| Packet Delay Budget (PDB) | 15 ms | 10 ms | 10 ms | I-frame |
|  |  |  |  | 10 ms |
| Packet Error Rate (PER) |  |  |  | 1% |
| Number of streams | 1 | 1 | 1 | 2 |

## Indoor Hotspot (InH) scenario

The carrier frequency is set to 4 GHz for FR1. System bandwidth is assumed to be 100 MHz for FR1. Time division duplexing (TDD) is configured as “DDDSU”. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 kHz for FR1. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set as follows: 31dBm with 100MHz (24dBm per 20MHz) in FR1. Table 4 lists the main parameters of the Indoor Hotspot deployment that are considered in this study.

Table 4 – Main parameters for Indoor Hotspot (InH) deployment

| Parameter | Value |
| --- | --- |
| Layout | 120m x 50m, Single layer (indoor floor, open office) 12 cells/TRPs ISD: 20m |
| Channel model | InH |
| Carrier frequency | FR1: 4 GHz |
| Subcarrier spacing | 30 kHz |
| System bandwidth | 100 MHz |
| BS height | 3 m |
| UE height | 1.5 m |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | LMMSE-IRC |
| UE receiver | LMMSE-IRC |
| Channel estimation | Realistic (with ideal CSI) |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS Tx power | 31dBm (24dBm per 20MHz) |
| UE Tx max power | 23 dBm |
| TDD Frame structure | DDDSU |
| Cell Selection | RSRP Slow Fading |
| BS antenna configuration | Pattern: Ceiling-mount antenna radiation pattern Gain: 5 dBi Downtilt: 90° |
|  | 32 TxRU (M, N, P, Mg, Ng) = (4,4,2,1,1) (dH, dV) = (0.5, 0.5)λ (Mp, Np) = (4,4) |
| UE antenna configuration | Pattern : Omni-directional, Gain : 0 dBi, Configuration : 2T/4R (M, N, P, Mg, Ng) = (1,2,2,1,1) (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,2) |
| Scheduler | SU-MIMO, Proportional Fairness |
| CSI acquisition | Periodic CQI on 2 ms period |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |
| PDCCH overhead | Modelled |
| Target BLER | 10% for first transmission |
| Max HARQ transmission | 3 |
| HARQ scheme | Chase combining |

## Dense Urban (DU) Scenario

The carrier frequency is set to 4 GHz for FR1. System bandwidth has been fixed to 100MHz for FR1. Time division duplexing (TDD) is configured as “DDDSU”. The smallest schedulable radio resource is the physical radio blocks (PRB) of 12 subcarriers, each of 30kHz for FR1. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmissions before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set to 51 dBm (i.e., 44dBm for 20MHz). Table 5 lists the main parameters of the Dense Urban deployment that are considered in this study.

Table 5– Main parameters for Dense Urban (UMa) deployment

| Parameter | Value |  |
| --- | --- | --- |
| Layout | 21 cells with wraparound (ISD: 200m) |  |
| Channel model | Uma |  |
| Carrier frequency | FR1: 4 GHz |  |
| Subcarrier spacing | 30 kHz |  |
| System bandwidth | Option 1: 100 MHz |  |
| BS height | 25m |  |
| UE height | hUT = 3(nfl – 1) + 1.5 |  |
|  | Outdoor: nfl = 1 | Indoor: - nfl ~ uniform(1,Nfl) - Nfl ~ uniform(4,8) |
| BS noise figure | 5 dB |  |
| UE noise figure | 9 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic (with ideal CSI) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS Tx power | 44 dBm per 20 MHz 51 dBm per 100 MHz |  |
| UE Tx max power | 23 dBm |  |
| TDD Frame structure | Option 1: DDDSU |  |
| Mechanical Downtilt | Baseline: 12 degrees |  |
| Cell Selection | RSRP Slow Fading |  |
| BS antenna configuration | 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2), (dH, dV) = (0.5λ, 0.5λ) |  |
| UE antenna configuration | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, -N/Aλ) |  |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |
| Scheduler | SU-MIMO, Proportional Fairness |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |
| PDCCH overhead | Modelled |  |
| Target BLER | 10% for first transmission |  |
| Max HARQ transmission | 3 |  |
| HARQ scheme | Chase combining |  |