3GPP TSG RAN WG1 #109-e			R1-2204680

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.1.1

Source:	Google Inc.

Title:	Maintenance of enhancements on multi-beam operation

Document for: 	Discussion

- Introduction

In this contribution, we discuss remaining issue(s) of multi-beam enhancements. We also discuss an agreement made in RAN1#103(e), which is used for MAC-CE/DCI based beam indication of unified TCI. Most of the agreement has been captured into the current specification. However, one portion of the agreement was missed and we propose a text proposal to bring it back.

- Discussion

  - Unified TCI framework

In RAN1#108(e), one heated issue is related to cross-carrier DCI-based TCI state indication. From discussions, a slight majority supported the latest proposed proposal. Most of companies against the proposal for cross-carrier beam indication considered that the intended behavior has already been supported, that is, which serving cell to apply TCI or beam indication naturally follows the scheduled cell indicated by CIF (Carrier Indicator Field).

In our views, since CIF normally points to which serving cell to receive the scheduled DL assignment (based on TS 38.213 and 38.331), it is beneficial to clarify at least in Chair’s notes. A clarification would be useful especially for DCI without DL assignment. Therefore, we propose the following proposal.

Proposal 1: RAN1 to make an agreement or conclusion to clarify the following:

- Unified TCI indicated by a DCI is applied to a serving cell indicated by the CIF, when UE is configured with CrossCarrierSchedulingConfig.

  - Signaling medium

In RAN1#103(e), RAN1 has made the following agreement to support MAC-CE based and DCI based beam indication under Rel-17 unified TCI framework [1].

Agreement On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework: - Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states - The existing DCI formats 1_1 and 1_2 are reused for beam indication - Support a mechanism for UE to acknowledge successful decoding of beam indication - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI - FFS: Whether any additional specification support is needed - Support activation of one or more TCI states via MAC CE analogous to Rel.15/16: - At least for the single activated TCI state, the activated TCI state is applied - The content for the MAC CE is determined based on the outcome of issue 1 - FFS: If supported, default TCI state when more than one TCI states are activated by MAC CE - Note: There is no implications on the support of single TRP or multi-TRP - FFS: Additional enhancement such as L1-based beam indication with group-common DCI - FFS: Whether the Rel.17 beam indication can also apply to beam indication for single channel (e.g. PDSCH only, single CORESET) or a subset of channels - FFS: Additional details on extending the support of L1-based beam indication when separate UL (from DL) common beam indication is configured

The DCI based beam indication method has been captured in TS 38.214. On other hand, the MAC-CE based beam indication was missed. As shown in the yellow-highlight, RAN1 has agreed that when only one TCI state is activated by activation MAC-CE, this TCI state is directly applied without need of DCI indication to select one. However, current specification doesn’t reflect such portion. Hence, we provide the following text proposal and propose to agree it.

Text Proposal 1 (TS 38.214 V17.1.0): Reason for change: Capture previous agreement, which is missed in current specification Summary for change: Add one sentence to reflect that “At least for the single activated TCI state, the activated TCI state is applied” in the agreement in RAN1#103(e) Consequences if not approved: Specification and RAN1 agreement are not aligned ### 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321] or 6.1.3.x of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs. When a set of TCI state IDs are activated for a set of CCs/DL BWPs and if applicable, for a set of CCs/UL BWPs, where the applicable list of CCs is determined by the indicated CC in the activation command, the same set of TCI state IDs are applied for all DL and/or UL BWPs in the indicated CCs. If the activation command activates only one TCI state, the activated TCI state is an indicated TCI state. When the bwp-id or cell for QCL-TypeA/D source RS in a QCL-Info of the TCI state configured with DLorJointTCIState is not configured, the UE assumes that QCL-TypeA/D source RS is configured in the CC/DL BWP where TCI state applies. < Unchanged parts are omitted >

Proposal 2: RAN1 adopts Text Proposal 1 for capturing agreement of beam indication

- Conclusion

According to the above discussion(s), we have the following proposal(s).

Proposal 1: RAN1 to make an agreement or conclusion to clarify the following:

- Unified TCI indicated by a DCI is applied to a serving cell indicated by the CIF, when UE is configured with CrossCarrierSchedulingConfig.

Proposal 2: RAN1 adopts Text Proposal 1 for capturing agreement of beam indication

# Reference

- R1-2100001 Final Report of 3GPP TSG RAN WG1 #103-e v1.0.0 (Online meeting, 26th October - 13th November 2020)