3GPP TSG RAN WG1 #109-e			        					R1- 2204685

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.1.4.1

Source: 	MediaTek Inc.

Title:	UL precoding indication for multi-panel transmission

Document for: 	Discussion and Decision

Introduction

The detailed RAN1 objectives for Rel-18 MIMO WID (RP-213598) are listed as follows:

RAN1: - Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32 - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios. Check RAN1 workload situation at RAN#98 (Dec 2022).

According to above objectives, we will share our view on simultaneous multi-panel UL transmission to MTRP for the following aspects:

- Potential PUSCH transmission schemes

- Potential PUCCH transmission schemes

- Beam measurement/reporting enhancement

Discussion

Potential PUSCH transmission schemes

In our view, at least the following PUSCH transmission schemes can be considered:

FDM Scheme A: Two non-overlapping FDRAs of a single PUSCH transmission occasion are transmitted by two UE panels to two TRPs, respectively

In Rel-16, S-DCI based 'fdmSchemeA' supports UE reception on two non-overlapping frequency domain resource allocations of a single PDSCH transmission occasion transmitted from MTRP. Similarly, in Rel-18 simultaneous multi-panel UL transmission, two non-overlapping FDRAs of a single PUSCH transmission occasion can be transmitted by two UE panels to two TRPs, respectively. However, whether this scheme can bring any reliability improvement when blockage happens in one of TRPs should be justified.

Figure 1. FDM scheme A for PUSCH transmission

FDM Scheme B: Two FDMed PUSCH transmission occasions of a same TB with a same or different RVs are transmitted by two UE panels to two TRPs, respectively

In Rel-17, PUSCH occasions/repetitions of a TB transmitted towards MTRP on more than one PUSCH transmission occasions are supported in TDM manner. In Rel-18 simultaneous multi-panel UL transmission, FDM manner can be considered to improve PUSCH reliability, where two PUSCH repetitions of a same TB with a same of different RVs are transmitted with two non-overlapping frequency domain resource allocations at the same time towards MTRP.

Figure 2. FDM scheme B for PUSCH transmission

SDM scheme A: Two DMRS CDM groups are transmitted by two UE panels to two TRPs, respectively

In Rel-16, S-DCI based SDM scheme supports UE reception on two DMRS CDM groups transmitted from two TRPs, respectively. Similarly, in Rel-18 simultaneous multi-panel UL transmission, two DMRS CDM groups can be transmitted by two UE panels to two TRPs, respectively, for better UL throughput.

Figure 3. SDM scheme A for PUSCH transmission

SDM scheme B: Two individual CWs are transmitted by two UE panels to two TRPs, respectively

Another protentional SDM scheme is transmission of two individual CWs by two UE panels to two TRPs, respectively. The two individual CWs are generated from two individual TBs and allocated to the same time and frequency resources, but each CW can be allocated with individual MCS/NDI/RV, which has potential benefit of better link adaptation when the difference of link qualities between TRPs are quite large. In case of a decoding error on one CW, only that CW needs to be retransmitted. However, in current NR, even for DL, two CWs are supported only when the number of layers is more than four. If two CWs are supported for the number of layers is equal or less than four, the current CW-to-layer mapping needs to be extended to support two CWs for 2, 3 and 4 layers. Meanwhile, some new DCIs fields are needed for the 2nd CW/TB in DCI format 0_1/0_2. Considering the potential larger specification effort and change on UE hardware structure, whether this scheme can bring the significant gain should be justified. Note that when the difference of link qualities between TRPs are large, it’s better to transmit all layers to the best TRP.

Figure 4. SDM scheme B for PUSCH transmission

SFN scheme: The same PUSCH/PUSCH-DMRS are transmitted by two UE panels to two TRPs, respectively

In Re-17, PDSCH/PDCCH-SFN was introduced for improving reliability for HST, where UE can perform PDSCH/PDCCH reception from two TPRs, where two TRPs transmits the same PDCCH/PDSCH to UE. Since vehicle is one of the target devices in Rel-18 simultaneous multi-panel UL transmission, PUSCH-SFN for HST should be considered as well.

M-DCI scheme: Two scheduled PUSCHs with fully/partially/non-overlapping in time and frequency domain and transmitted by two UE panels to two TRPs, respectively

In Rel-16, when M-DCI based scheme is configured (i.e., when UE is configured with two different values of coresetPoolIndex in CORESETs), UE can receive two DCIs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain, where each PDCCH and the scheduled PDSCH are transmitted from one certain TRP. Similarly, in Rel-18 simultaneous multi-panel UL transmission, when M-DCI based scheme is configured, UE can be scheduled with two PUSCHs that are transmitted by two UE panels to two TRPs, respectively, with fully/partial/non-overlapping in time and frequency domain.

Figure 5. M-DCI scheme for PUSCH transmission

Once the supported PUSCH transmission schemes are identified, the following potential issues can be further studied and discussed:

- UL precoding for each of the supported PUSCH transmission scheme

- Frequency domain resource allocation for FDM schemes, if supported

Proposal 1: For simultaneous multi-panel UL transmission to MTRP targeting CPE/FWA/vehicle/industrial devices, consider the following PUSCH transmission schemes:

- FDM scheme B: Two FDMed PUSCH transmission occasions of a same TB with a same or different RVs are transmitted by two UE panels to two TRPs, respectively

- SDM scheme A: Two DMRS CDM groups are transmitted by two UE panels to two TRPs, respectively

- SFN scheme: The same PUSCH/PUSCH-DMRS are transmitted by two UE panels to two TRPs, respectively

- M-DCI scheme: Two scheduled PUSCHs with fully/partial/non-overlapping in time and frequency domain and transmitted by two UE panels to two TRPs, respectively

Proposal 2: For simultaneous multi-panel UL transmission to MTRP targeting CPE/FWA/vehicle/industrial devices, whether to support the following PUSCH transmission scheme need to be further justified:

- FDM scheme A: Two non-overlapping FDRAs of a single PUSCH transmission occasion are transmitted by two UE panels to two TRPs, respectively

- SDM scheme B: Two individual CWs are transmitted by two UE panels to two TRPs, respectively

Proposal 3: Further study the UL precoding once the supported PUSCH transmission schemes are identified

Proposal 4: Further study the frequency domain resource allocation of PUSCH for FDM scheme(s), if supported

Potential PUCCH transmission schemes

In our view, at least the following PUCCH transmission schemes can be considered:

FDM scheme A: Two non-overlapping FDRAs of a single PUCCH transmission occasion are transmitted by two UE panels to two TRPs, respectively

FDM scheme B: Two FDMed PUCCH transmission occasions are transmitted by two UE panels to two TRPs, respectively

These two FDM schemes are similar to the potential FDM schemes for PUSCH transmission. However, whether FDM scheme A can bring any reliability improvement when blockage happens in one of TRPs should be also justified.

Figure 6. FDM schemes for PUSCH transmission (a) FDM scheme A (b) FDM scheme B

SFN scheme: The same PUCCH/PUCCH-DMRS are transmitted by two UE panels to two TRPs, respectively

In Re-17, PDSCH/PDCCH-SFN was introduced for improving reliability for HST, where UE can perform PDSCH/PDCCH reception from two TPRs, where two TRPs transmits the same PDCCH/PDSCH to UE. Since vehicle is one of the target devices in Rel-18 simultaneous multi-panel UL transmission, PUCCH-SFN for HST should be considered as well.

M-DCI scheme: Two PUCCHs with fully/partial/non-overlapping in time and frequency domain and transmitted by two UE panels to two TRPs, respectively

Similar to PUSCH, in M-DCI based MTPR, different PUCCH transmissions may be allowed to be fully/partially overlapped in time and frequency domain due to individual scheduling on each TRP. Even PUCCH cannot use UL precoding to reduce interference between two overlapped PUCCH transmissions, interference still can be reduced by different two UL spatial filters on different panels for the two PUCCH transmissions in FR2.

Figure 7. M-DCI schemes for PUCCH transmission

Once the supported PUCCH transmission schemes are identified, the following potential issues can be further studied and discussed:

- Frequency domain resource allocation and PUCCH format(s) for FDM schemes, if supported

- Change of multiplexing rule for PUCCH+PUCCH and PUSCH+PUCCH overlapping in time domain

Proposal 5: For simultaneous multi-panel UL transmission to MTRP targeting CPE/FWA/vehicle/industrial devices, consider the following PUCCH transmission schemes:

- FDM scheme B: Two FDMed PUCCH transmission occasions are transmitted by two UE panels to two TRPs, respectively

- SFN scheme: The same PUCCH/PUCCH-DMRS are transmitted by two UE panels to two TRPs, respectively

- M-DCI scheme: Two PUCCHs with fully/partial/non-overlapping in time domain and transmitted by two UE panels to two TRPs, respectively

Proposal 6: For simultaneous multi-panel UL transmission to MTRP targeting CPE/FWA/vehicle/industrial devices, whether to support the following PUCCH transmission scheme need to be further justified:

- FDM scheme A: Two non-overlapping FDRAs of a single PUCCH transmission occasion are transmitted by two UE panels to two TRPs, respectively

Proposal 7: Further study the frequency domain resource allocation of PUCCH for FDM scheme, if supported

Proposal 8: Further study the PUCCH format(s) for FDM scheme, if supported

Proposal 9: Further study the change of multiplexing rule for PUCCH+PUCCH and PUSCH+PUCCH overlapping in time domain

Beam measurement/reporting enhancement

To support simultaneous UL transmission across two panels, NW needs to know which two references can be indicated to UE for determining two spatial transmission filters that can be applied simultaneously. We see thee are two potential mechanisms to achieve this goal.

Alt1: Extension of group-based beam reporting to simultaneous UL transmission

Since group-based beam reporting in current NR (including Rel-15 and Rel17) is specified for simultaneous DL reception but not for simultaneous UL transmission. However, considering beam correspondence is generally hold for NR UE, it could be straightforward to extend group-based beam reporting to simultaneous UL transmission. For example, UE can simultaneously apply the UL transmission filter(s) determined from the two SSB/CSI-RS resources in the same beam group.

Alt2: Enhancement to beam reporting with UE capability value set index(s)

In Rel-17, UE capability value set index(s) can be reported along with beam report to indicate, where each UE capability value set index is associated a maximum number of SRS ports that UE can support. We see UE capability value set index can implicitly represent the UE panel. Based on this assumption, the two SSB/CSI-RS resources associated with different UE capability value set indexes shall correspond to different UE panels, and UE can simultaneously apply the UL transmission filter(s) determined from the two SSB/CSI-RS resources. However, in Rel-17, there is one restriction that different UE capability value set indexes cannot be associated with the same maximum number of SRS ports, which means same UE capability value set index may be shared by multiple UE panels with the same capability and cannot be used for panel differentiation. Thus, this restriction should be removed if Alt2 is used for simultaneous UL transmission.

Proposal 10: On beam measurement/reporting enhancement to support simultaneous UL transmission, consider the following two alternatives:

- Alt1: Extension of group-based beam reporting to simultaneous UL transmission

- Alt2: Enhancement to beam reporting with UE capability value set index(s)

Conclusion

In this contribution, simultaneous multi-panel UL transmission to MTRP was discussed. Based on the discussion in the previous sections, we made the following proposals and observations:

Proposal 1: For simultaneous multi-panel UL transmission to MTRP targeting CPE/FWA/vehicle/industrial devices, consider the following PUSCH transmission schemes:

- FDM scheme B: Two FDMed PUSCH transmission occasions of a same TB with a same or different RVs are transmitted by two UE panels to two TRPs, respectively

- SDM scheme A: Two DMRS CDM groups are transmitted by two UE panels to two TRPs, respectively

- SFN scheme: The same PUSCH/PUSCH-DMRS are transmitted by two UE panels to two TRPs, respectively

- M-DCI scheme: Two scheduled PUSCHs with fully/partial/non-overlapping in time and frequency domain and transmitted by two UE panels to two TRPs, respectively

Proposal 2: For simultaneous multi-panel UL transmission to MTRP targeting CPE/FWA/vehicle/industrial devices, whether to support the following PUSCH transmission scheme need to be further justified:

- FDM scheme A: Two non-overlapping FDRAs of a single PUSCH transmission occasion are transmitted by two UE panels to two TRPs, respectively

- SDM scheme B: Two individual CWs are transmitted by two UE panels to two TRPs, respectively

Proposal 3: Further study the UL precoding once the supported PUSCH transmission schemes are identified

Proposal 4: Further study the frequency domain resource allocation of PUSCH for FDM scheme(s), if supported

Proposal 5: For simultaneous multi-panel UL transmission to MTRP targeting CPE/FWA/vehicle/industrial devices, consider the following PUCCH transmission schemes:

- FDM scheme B: Two FDMed PUCCH transmission occasions are transmitted by two UE panels to two TRPs, respectively

- SFN scheme: The same PUCCH/PUCCH-DMRS are transmitted by two UE panels to two TRPs, respectively

- M-DCI scheme: Two PUCCHs with fully/partial/non-overlapping in time domain and transmitted by two UE panels to two TRPs, respectively

Proposal 6: For simultaneous multi-panel UL transmission to MTRP targeting CPE/FWA/vehicle/industrial devices, whether to support the following PUCCH transmission scheme need to be further justified:

- FDM scheme A: Two non-overlapping FDRAs of a single PUCCH transmission occasion are transmitted by two UE panels to two TRPs, respectively

Proposal 7: Further study the frequency domain resource allocation of PUCCH for FDM scheme, if supported

Proposal 8: Further study the PUCCH format(s) for FDM scheme, if supported

Proposal 9: Further study the change of multiplexing rule for PUCCH+PUCCH and PUSCH+PUCCH overlapping in time domain

Proposal 10: On beam measurement/reporting enhancement to support simultaneous UL transmission, consider the following two alternatives:

- Alt1: Extension of group-based beam reporting to simultaneous UL transmission

- Alt2: Enhancement to beam reporting with UE capability value set index(s)

Reference

- RP-213598, “WID: MIMO Evolution for Downlink and Uplink”, RAN#94e