R1-2204693

3GPP TSG RAN WG1 Meeting #109-e

E-Meeting, May 9-20, 2022

| Agenda Item: | 9.1.3.1 |  |
| --- | --- | --- |
| AI Description: | Increased number of orthogonal DMRS ports Including increasing orthogonal DMRS ports for UL/DL MU-MIMO and 8 Tx UL SU-MIMO. |  |
| Source: | MediaTek Inc. |  |
| Title: | Expanding DMRS System Capacity: Seamless Extensions of Existing Patterns |  |
| Document for: | Discussion and Decision |

- Introduction

In the RAN (Plenary) Meeting #94e, a new Work Item (WI) was approved targeting MIMO evolution for Downlink (DL) and Uplink (UL). The 3rd objective of the aformentioned WI is:

“3. Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

- Striving for a common design between DL and UL DMRS

- Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS”

The Demodulation Reference Signal (DMRS) is crucial for receiver operations where it enables the receiver to aquire accurate channel state inforation so that it can eliminate the channel’s distortion effect on transmitted symbols (i.e., channel equalization) prior to symbol demodulation.

The current maximum numbers of orthognoal DMRS ports are 4, 6, 8 and 12, which can be obtained using Type 1 and Type 2 DMRS with single and double OFDM symbols, repsectively. These values also dictate the maximum number of simultaneous transmission of data streams since data is always accompanied by DMRS for coherent reception. From this short description, it becomes clear that the current DMRS design limits the number of MU-MIMO data streams to ≤12.

In this document, we discuss various alternatives to increaseing the DMRS system capacity (i.e., the maximum number of orthogonal DMRS ports), without increasing the DMRS overhead, following the aformentioned WID objective. We also put a special emphasis on seamless coexistence of newly proposed DMRS patterns with the legacy design.

- Higher DMRS Capacity

Increasing the number of orthogonal DMRS ports without increasing its overhead imposes limitations on how this capacity expansion can be ahcieved. Briefly, we believe two main alternatives present themselves, namely:

  - Frequency Division Multiplexing (FDM)

  - Code Division Multiplexing (CDM), by extending

    - Frequency Domain Orthogonal Cover Code (FD-OCC)

    - Time Domain Orthogonal Cover Code (TD-OCC)

Next, we will discuss those alternatives in detail.

- FDM

In an FDM approach, the DMRS patterns, for all ports, occupy fewer Resource Elements (RE) in the frequency domain. Recall that the objective in this WID is to double the number of DMRS ports. This can be achieved by splitting the frequency resources of every legacy DMRS port, for each type, across two new DMRS ports. An example is shown in Table I and Table II below. In Table I, the legacy DMRS Type 1 with single OFDM symbol is shown, where, for each port, each two consecutive REs belong to one RE group and are used for CDM multiplxing with another DMRS port. To expand this legacy Type 1 DMRS using FDM, we split the REs mapped to each port among two new ports. Each one of the new DMRS ports occupy just half the number of REs comapred to the legacy one, all while keeping the CDM multiplexing capability of both the new and legacy designs identical. The new pattern is shown in Table II.

Coexistence with Legacy Design:

Increasing the DMRS system capacity via FDM results in ports that can seamlessly coexist with the legacy counterpart. For example, notice that the same REs and CDM code of port #0 of the legacy in Table I are split across port#0 and port#8 of the new FDMed Type 1 in Table II. This means that if legacy port #0 is configured, then both new ports #0 and #8 cannot be configured since they introduce interference to the legacy port #0. However, all other ports of the new FDMed Type 1 (besides #0 and #8) are orthogonal to the legacy port #0, and can be configured for simultaneous use.

| TABLE I: Type 1 DMRS with Single-Symbol |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Single Symbol |  |  |  |  |  |  |  |  |  |  |  |
| Port | #0 |  |  | #1 |  |  | #2 |  |  | #3 |  |
| SC/OFDM | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |
| 11 |  |  |  |  |  |  | 1 |  |  | -1 |  |
| 10 | 1 |  |  | -1 |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  | 1 |  |  | 1 |  |
| 8 | 1 |  |  | 1 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  | 1 |  |  | -1 |  |
| 6 | 1 |  |  | -1 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  | 1 |  |  | 1 |  |
| 4 | 1 |  |  | 1 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  | 1 |  |  | -1 |  |
| 2 | 1 |  |  | -1 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  | 1 |  |  | 1 |  |
| 0 | 1 |  |  | 1 |  |  |  |  |  |  |  |

| TABLE II: Type 1 FDMed DMRS with Single-Symbol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Single Symbol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Port | #0 |  |  | #1 |  |  | #2 |  |  | #3 |  |  | #8 |  |  | #9 |  |  | #10 |  |  | #11 |  |
| SC/OFDM | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |
| 23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | -1 |  |
| 22 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | -1 |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  | 1 |  |  | -1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | 1 |  |  | -1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 1 |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | -1 |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | -1 |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  | 1 |  |  | -1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 1 |  |  | -1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 1 |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | -1 |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | -1 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  | 1 |  |  | -1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 1 |  |  | -1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 1 |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 1: Increasing the number of orthogonal DMRS ports via FDM results in DMRS ports that seamlessly coexist with legacy DMRS ports.

Proposal 1: Prioritize seamless coexistence of new DMRS ports with legacy DMRS over small performance degradation.

Pattern Repeatability:

Notice that in Table II, we list the DMRS pattern for two consecutive Resource Blocks (RB), unlike the case in Table I. The reason lies in the nature of the Type 1 DMRS RE mapping in the frequency domain. Specifically, the DMRS pattern of Type 1 occupies 6 REs in groups of 2 each (for CDM multiplexing). Since it is desirbale to keep the REs beloning to the same group as close possible to avoid interference to the other CDMed port, we treat the REs as groups. In Type 1, 3 groups per RB exist, which does not make for a repeatable pattern across all RBs. However, if we treat the DMRS RE mapping in RB bundles of size 2 (i.e., pairs of RB), then it is possible to repeat the pattern produces by 6 RE groups every 2 RBs. This particular RB bundling issue is going to show up for any DMRS capacity enhancement that is based on Type 1. This is not the case, however, with FDM based extension of Type 2 DMRS since in Type 2 each legacy DMRS port contains just 2 RE groups. Hence, dividing the 2 available RE groups among two new Type 2 FDMed ports still gives a repeatable DMRS pattern every RB.

Observation 2: Increasing the number of orthogonal DMRS ports via FDM, based on the legacy Type 1, results in RE patterns that are repeatable every two RBs. Specifically, all even or all odd RBs have the same RE pattern, however, RE mapping of even and odd RBs for the same DMRS port is different.

- CDM

CDM based DMRS capacity enhancement relies on increasing the size of CDM groups. The current maximum value of CDM multiplexing capability per group is 2 and 4 for single and double symbol DMRS, respectively. To double the number of orthogonal DMRS ports, CDM based increase require increasing the CDM group size to 4 and 8 for single and double symbol DMRS. This can be achieved in either the Frequency Domain (FD) using a longer FD-OCC code, or in the Time Domain (TD) using a longer TD-OCC code.

  - FD-OCC

Recall that an RE group for both legacy DMRS types has a size of 2 REs in the frequency domain. FD-OCC requires an increase of this number up to 4. This can be achieved by merging each two consective RE groups to form one larger RE group, making it possible to use a length 4 OCC in the frequency domain. An example DMRS pattern based on this method is shown in Table IV using RE mapping of Type 2 as a base pattern.

| TABLE IV: Type 2 FD-CDMed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port | #0 |  |  | #12 |  |  | #1 |  |  | #13 |  |  | #2 |  |  | #14 |  |  | #3 |  |  | #15 |  |
| SC/Sym | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |
| 6 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |
| 0 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |
| Port | #4 |  |  | #16 |  |  | #5 |  |  | #17 |  |  | #6 |  |  | #18 |  |  | #7 |  |  | #19 |  |
| SC/Sym | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |
| 8 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |
| 2 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Port | #8 |  |  | #20 |  |  | #9 |  |  | #21 |  |  | #10 |  |  | #22 |  |  | #11 |  |  | #23 |  |
| SC/Sym | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |
| 11 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |
| 10 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |
| 4 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Applying the same concept to Type 1 is also possible, however, since each RB contains 3 RE groups per RB, one CDM group would have REs split across 2 RBs. This means that a Type 1 FD-CDMed enhancement cannot be configured with an odd number of RBs. An illustration is depicted in TABLE V. This could also have implications on implementaions of channel estimation solutions, which cannot be done on one RB level.

| TABLE V: Type 1 FD-CDMed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port | #0 |  |  | #12 |  |  | #1 |  |  | #13 |  |  | #2 |  |  | #14 |  |  | #3 |  |  | #15 |  |
| SC/Sym | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |
| 23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |
| 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |
| 17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |
| 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |

Proposal 2: Study the implications of splitting RE groups across 2 RBs.

Effect of larger FD-OCC length:

Longer OCCs in FD could have a detrimental effect on channel estimation performance as delay spread increases. That is becasue the orthogonality provided by OCC becomes questionable as the underlying channel gain across the shared REs become more distinct. This is made even worse when RE mapping similar to both Types 1 and 2 is assumed since the 4 subcarriers hosting an RE group are not even adjacent, but are rather spread out across a span of 7 or 8 subcarriers. Moreover, a larger CDM group size means that up to 7 other ports can now share the same REs, making interference between ports a potentially serious drawback to FD-OCC.

One alternative to try to mitigate the effect of longer FD-OCC is to consider a different RE mapping variant where all 4 subcarriers constituting one RE group are adjacent. An example for CDM group is shown in Table VI.

| TABLE VI: Length 4 FD-OCC with Clustered REs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port | #0 |  |  | #12 |  |  | #1 |  |  | #13 |  |  | #2 |  |  | #14 |  |  | #3 |  |  | #15 |  |
| SC/Sym | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |  | 0 | 1 |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |
| 2 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | -1 | -1 |  | -1 | 1 |
| 1 | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |  | 1 | 1 |  | 1 | -1 |  | -1 | -1 |  | -1 | 1 |
| 0 | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |  | 1 | 1 |  | 1 | -1 |

  - TD-OCC

Using additional DMRS symbols (instances), we can also increase the size of each CDM group in the time dimension. This method, however, is only valid if an even number of DMRS instances/symbols is available. An example is shown in Table VII where a Type 1 double-symbol DMRS is used, with a shared channel duration of 14 symbols, one additional DMRS symbol, and no intra-slot frequency hopping. In this example the first double-symbol DMRS instance occurs in OFDM symbol #2, while the additional one is in symbol #10. An OCC code that spans all 4 OFDM symbols can increase the multiplexing capability of each CDM group from 4 to 8 (and similarly for single-symbol DMRS from 2 to 4).

Just like the FD-OCC is succeptability to bad performance under large delay spread, TD-OCC is also succeptible to bad performance under large doppler spread.

| TABLE VII: Type 1 TD-CDMed DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Single Symbol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| port #0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | One OFDM Symbol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sc idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  | 1 | 1 |  |  |  |  |  |  | 1 | 1 |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 1 | 1 |  |  |  |  |  |  | 1 | 1 |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  | 1 | 1 |  |  |  |  |  |  | 1 | 1 |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | 1 | 1 |  |  |  |  |  |  | 1 | 1 |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | 1 | 1 |  |  |  |  |  |  | 1 | 1 |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  | 1 | 1 |  |  |  |  |  |  | 1 | 1 |  |  |  |
| port #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | One OFDM Symbol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sc idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  | 1 | 1 |  |  |  |  |  |  | -1 | -1 |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | 1 | 1 |  |  |  |  |  |  | -1 | -1 |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  | 1 | 1 |  |  |  |  |  |  | -1 | -1 |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | 1 | 1 |  |  |  |  |  |  | -1 | -1 |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | 1 | 1 |  |  |  |  |  |  | -1 | -1 |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  | 1 | 1 |  |  |  |  |  |  | -1 | -1 |  |  |  |

Proposal 3: One of the following alternatives for increasing the number of orthogonal DMRS should be considered:

- Splitting RE groups of  each legacy DMRS port across two new DMRS ports, using Frequency Division Multiplexing (FDM).

- Merging concecutive pairs of RE groups in the frequency dimension of legacy DMRS to form larger CDM groups via longer FD-OCC.

- Merging concecutive RE groups in the time dimension of legacy DMRS to form larger CDM groups via longer TD-OCC, when an even number of DMRS symbols is configured.

- EVM

Supporting more DMRS ports while maintaining the overhead at the current limits will likely result in degradation of channel estimation performance. The main objective is then to keep any performance degradation to a minimum for scenarios that would benefit the most from more DMRS ports.

Our proposed evaluation assumptions are summarized in Table VIII for DL MU-MIMO and in Table IX for UL SU-MIMO with 8 TX antennas.

More over, since the use of additional DMRS symbols is mainly targeted for channels with medium and large doppler spreads, a fair assessment of TD-OCC based DMRS capacity enhancement should consider no less than medium doppler scenarios where additional DMRS symbols is expected to be configured.

Proposal 4: EVM for methods of increasing the number of orthogonal DMRS ports via exploiting additional DMRS symbols should consider at least medium doppler spread scenarios.

Proposal 5: EVM should include a study of the effect of new DMRS port coexistence with legacy ports.

Table VIII: Simulation assumptions for DL MU-MIMO

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Scenario |  | Dense Urban (macro only) | UMa |
| Duplex, Waveform |  | TDD, OFDM |  |
| Multiple access |  | OFDMA |  |
| Frequency Range |  | FR1 only. |  |
| Inter-BS distance |  | 200 m | 500 m |
| Channel model |  | According to the TR 38.901 |  |
| Antenna setup and port layouts at gNB |  | 8 TX: (2,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8) λ |  |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2) (dH,dV) = (0.5, 0.5) λ |  |
| BS Tx power |  | 41 dBm | 46 dBm |
| BS antenna height |  | 25 m | 25 m |
| BS noise figure |  | 5 dB |  |
| UE noise figure |  | 7 dB |  |
| UE antenna height & gain |  | Follow TR36.873 |  |
| Modulation |  | Up to 256 QAM |  |
| Numerology | Slot/non-slot | 14 OFDM symbols per slot |  |
|  | SCS | 30 kHz |  |
| Simulation bandwidth |  | 40 MHz |  |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |  |
| Traffic model |  | Full-buffer, FTP1 with 50% Resource Utilization |  |
| UE distribution |  | [80%] indoor (3km/h), [20%] outdoor (30km/h) | [80%] indoor (3km/h), [20%] outdoor (30km/h) |

| Table IX: Simulation assumptions for UL SU-MIMO |  |  |  |
| --- | --- | --- | --- |
| Parameter |  | Value |  |
| Scenario |  | Dense Urban (macro only) | UMa |
| Duplex, Waveform |  | TDD, OFDM |  |
| Multiple access |  | OFDMA |  |
| Frequency Range |  | FR1 only. |  |
| Inter-BS distance |  | 200 m | 500 m |
| Channel model |  | According to the TR 38.901 |  |
| Antenna setup and port layouts at gNB |  | 8 TX: (2,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8) λ |  |
| Antenna setup and port layouts at UE |  | 8 TX: (2,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5) λ |  |
| BS Tx power |  | 41 dBm | 46 dBm |
| BS antenna height |  | 25 m | 25 m |
| BS noise figure |  | 5 dB |  |
| UE noise figure |  | 7 dB |  |
| UE antenna height & gain |  | Follow TR36.873 |  |
| Modulation |  | Up to 256 QAM |  |
| Numerology | Slot/non-slot | 14 OFDM symbols per slot |  |
|  | SCS | 30 kHz |  |
| Simulation bandwidth |  | 40 MHz |  |
| Frame structure |  | Slot Format 1 (all uplink) for all slots |  |
| Traffic model |  | Full-buffer, FTP1 with 50% Resource Utilization |  |
| UE distribution |  | [80%] indoor (3km/h), [20%] outdoor (30km/h) | [80%] indoor (3km/h), [20%] outdoor (30km/h) |

- Conclusion

Observation 1: Increasing the number of orthogonal DMRS ports via FDM results in DMRS ports that seamlessly coexist with legacy DMRS ports.

Proposal 1: Prioritize seamless coexistence of new DMRS ports with legacy DMRS over small performance degradation.

Observation 2: Increasing the number of orthogonal DMRS ports via FDM, based on the legacy Type 1, results in RE patterns that are repeatable every two RBs. Specifically, all even or all odd RBs have the same RE pattern, however, RE mapping of even and odd RBs for the same DMRS port is different.

Proposal 2: Study the implications of splitting RE groups across 2 RBs.

Proposal 3: One of the following alternatives for increasing the number of orthogonal DMRS should be considered:

- Splitting RE groups of  each legacy DMRS port across two new DMRS ports, using Frequency Division Multiplexing (FDM).

- Merging concecutive pairs of RE groups in the frequency dimension of legacy DMRS to form larger CDM groups via longer FD-OCC.

- Merging concecutive RE groups in the time dimension of legacy DMRS to form larger CDM groups via longer TD-OCC, when an even number of DMRS symbols is configured.

Proposal 4: EVM for methods of increasing the number of orthogonal DMRS ports via exploiting additional DMRS symbols should consider at least medium doppler spread scenarios.

Proposal 5: EVM should include a study of the effect of new DMRS port coexistence with legacy ports.

- References

  - 3GPP RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021