3GPP TSG RAN WG1 Meeting #109-e  	                          R1-2204694

e-Meeting, May 9th – May 20th, 2022

Agenda Item: 8.13

Source: MediaTek Inc.

Title: On Cross-Carrier Scheduling from sSCell to P(S)Cell

Document for: Discussion and decision

- Introduction

In this contribution, we focus on the discussions for Rel-17 remaining issues of cross-carrier scheduling from SCell to P(S)Cell for NR.

- Application delay of the scaling factor α

In RAN1 #108e [1], it is agreed that

Agreement

- For a UE configured for CCS from sSCell to P(S)Cell, scaling factor α is not applied for PDCCH overbooking/BD/CCE limit computation when sSCell is deactivated, or when an activated sSCell is switched to dormant BWP; otherwise scaling factor α is applied

  - Timing for disabling scaling factor α when sSCell is deactivated follows sSCell deactivation timing in current specifications, i.e., no later than the minimum requirement defined in TS 38.133 as captured in 38.213 subclause 4.3

  - Timing for disabling scaling factor α follows the non-dormant to dormant BWP switching delay in current specifications (TS 38.133).

  - Introduce separate FG to indicate UE support for disabling scaling factor α when sSCell is deactivated

  - Introduce separate FG to indicate UE support for disabling scaling factor α when activated sSCell is switched to dormant BWP

  - Note: It is up to UE implementation whether/when to apply the scaling factor α during sSCell activation and during sSCell BWP switch

In 38.213 [2] 4.3, it is stated that:

- With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot n + k

Observation 1: According to 38.213 [2] 4.3, UE activates the sSCell no earlier than slot n + k, which is not considered in RAN1 #108e agreement [1]. UE also applies α no earlier than slot n + k. However, RAN1 #108e agreement seems fine since this can already be common understanding.

- Summary

In this contribution, we focus on the discussions for Rel-17 cross-carrier scheduling from SCell to P(S)Cell for NR and have the following observations and proposals:

Observation 1: According to 38.213 [2] 4.3, UE activates the sSCell no earlier than slot n + k, which is not considered in RAN1 #108e agreement [1]. UE also applies α no earlier than slot n + k. However, RAN1 #108e agreement seems fine since this can already be common understanding.

- Reference

Chairman’s Notes, RAN1 #108-e (Rel-17 8.13 DSS)

3GPP TS 38.213 V16.6.0, ”NR; Physical layer procedures for control”