3GPP TSG RAN WG1 #109-e			R1-2204695

e-Meeting, May 9th – May 20th, 2022

Agenda item:	8.16.7

Source: 	MediaTek Inc.

Title: 	On UE features for UE power saving enhancements

Document for:	Discussion and decision

- Introduction

This paper provides our views on Rel-17 UE features in R1-2202928 (Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e) for UE power saving enhancements. Detailed proposals are described in the following sections.

- Views on UE features for connected mode UE power saving enhancements

In RAN1 #108-e, RAN1 agrees the following draft UE features for connected mode UE power saving enhancements, which includes search space set group (SSSG) switching and PDCCH skipping:

In Rel-16, SSSG switching is also defined from the NR-U agenda, and the UE supported switch delay P is defined with the two following UE capabilities (cited from 38.306 V16.8.0):

| searchSpaceSwitchWithDCI-r16 Indicates whether the UE supports switching between two groups of search space sets with DCI 2_0 monitoring that comprises of the following functional components: - Monitor DCI 2_0 with a search space set switching field; - Support switching the search space set group with PDCCH decoding in group 1; - Support a timer to switch back to original search space set group; - Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group. The UE can switch search space set groups for different cells independently, unless the UE supports jointSearchSpaceSwitchAcrossCells-r16. The UE supports search space set group switching capability-1: P=25/25/25 symbols for µ=0/1/2, unless the UE supports searchSpaceSwitchCapability2-r16. The UE supports search space switching triggers to be configured for up to 4 cells or 4 cell groups. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| searchSpaceSwitchCapability2-r16 Indicates whether the UE supports search space set group switching Capability-2: P=10/12/22 symbols for µ = 0/1/2 SCS. If the UE supports this feature, the UE needs to report searchSpaceSwitchWithDCI-r16 or searchSpaceSwitchWithoutDCI-r16. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Similar switch dealy P value should be reported for R17 SSSG switching features.

Proposal 3:  Include the following candidate values for SSSG switch delay in the “Components” column of FG 29-3 b/c/d:

- Value 1: search space set group switching delay P=25/25/25/50 symbols for µ=0/1/2/3

- Value 2: search space set group switching delay P=10/12/22/44 symbols for µ=0/1/2/3

# Proposed changes on current RAN1 UE features table

# NR_UE_pow_sav_enh (modified contents in red text)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB [2. Support receiving L1 indication for TRS availability] |  | N |  | UE cannot receive TRS resources for idle/inactive mode | N/A | N/A | N/A | N/A | [Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7] | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based SSSG switching, if PDCCHSkippingDurationList is not configured Candidate values for SSSG switching delay are: {Value 1: switching delay P=25/25/25/50 symbols for µ=0/1/2/3, Value 2: switching delay P=10/12/22/44 symbols for µ=0/1/2/3} |  | Y |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based SSSG switching, if PDCCHSkippingDurationList is not configured Candidate values for SSSG switching delay are: {Value 1: switching delay P=25/25/25/50 symbols for µ=0/1/2/3, Value 2: switching delay P=10/12/22/44 symbols for µ=0/1/2/3} | 29-3b | Y |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs, PDCCH skipping by scheduling DCI, and timer based SSSG switching Candidate values for SSSG switching delay are: {Value 1: switching delay P=25/25/25/50 symbols for µ=0/1/2/3, Value 2: switching delay P=10/12/22/44 symbols for µ=0/1/2/3} | 29-3a, 29-3b | Y |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |