3GPP TSG RAN WG1 Meeting #109-e  	                          R1-2204697

e-Meeting, May 9th – May 20th, 2022

Agenda Item: 9.10.1

Source: MediaTek Inc.

Title: On multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for: Discussion and decision

- Introduction

In the revised WID on Rel-18 Multi-carrier enhancements [1], the objectives of this working item for multi-cell PUSCH/PDSCH scheduling with a single DCI are listed as follows:

Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1]

- Identify the maximum number of cells that can be scheduled simultaneously

- Consider both intra-band and inter-band CA operation

- Consider both FR1 and FR2

- The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

This paper provides our views on multi-cell PUSCH/PDSCH scheduling with a single DCI.

- On DCI payload limitation and maximum number of cells that can be scheduled simultaneously

According to 38.212 [2], the 5G NR polar code supports a maximum DCI size of 164 bits due to the interleaver design. Equivalently, DCI without CRC can have at most 140 payload bits.

Observation 1: According to 38.212 [2], the 5G NR polar code supports a maximum DCI size of 164 bits due to the interleaver design. Equivalently, DCI without CRC can have at most 140 payload bits.

For multi-cell PUSCH/PDSCH scheduling with a single DCI, there would be common bit fields which are shared (same value) among the multiple scheduled cells. At the same time, there would be designated bit fields which would be assigned independently for each scheduled cell. Take one example from Huawei’s contribution from Rel-17 [3], the common and designated bit fields allocation for DL scheduling (DCI format 1_1) is shown below as in Figure 1.

Figure 1. One exemplary common and designated bit fields allocation for DCI format 1_1 [3]

From Figure 1, it can be seen that the number of bits for common bit fields is 24 (82-34-24), and the number of bits for designated bit fields is 34.

In RAN1 #103e [4], the following assumption of DCI payload size on two-cell PDSCH scheduling via a single DCI is agreed:

Agreement

| DCI payload size (excluding CRC) | Single PDSCH scheduling: 60 bits as baseline payload size Two-cell PDSCH scheduling: 72/84/96/104 bits |
| --- | --- |

It can be seen that the number of bits for common bit fields is assumed to be 48/36/24/12, and the number of bits for designated bit fields is assumed to be 12/24/36/44.

Observation 2: For multi-cell PUSCH/PDSCH scheduling with a single DCI, there would be common bit fields which are shared (same value) among the multiple scheduled cells. At the same time, there would be designated bit fields which would be assigned independently for each scheduled cell.

- According to one example shown in Figure 1 from [3, Huawei], for PDSCH scheduling, the number of bits for common bit fields is 24, and the number of bits for designated bit fields is 34.

- According to RAN1 #103e [4], the number of bits for common bit fields is assumed to be 48/36/24/12, and the number of bits for designated bit fields is assumed to be 12/24/36/44.

With Observation 2, we first assume the number of bits for common bit fields to be 24, and the number of bits for designated bit fields to be 36. In Figure 2, it can be seen that the 140 bits DCI payload limit can only accommodate up to 3 cells if one DCI segment is used (b).

Observation 3: Assuming the number of bits for common bit fields is 24, and the number of bits for designated bit fields is 36 (using information from [3, 4]). In Figure 2, it can be seen that the 140 bits DCI payload limit can only accommodate up to 3 cells if one DCI segment is used (b).

Since the 140 bits DCI payload limit is hard to be increased in 5G NR, one potential way to increase the maximum number of cells that can be scheduled simultaneously is to divide a single DCI into two segments, as shown in Figure 2(c).

Figure 2. Comparison of legacy scheme v.s. one/two segment DCI aggregation, assuming 4 cells scheduled, number of bits for common bit fields is 24, number of bits for designated bit fields is 36

Observation 4: The 140 bits DCI payload limit is hard to be increased in 5G NR; hence, one potential way to increase the maximum number of cells that can be scheduled simultaneously is to divide a single DCI into two segments, as shown in Figure 2(c).

According to the revised WID on Rel-18 Multi-carrier enhancements [1], one key point of RAN1 discussion is to

- Identify the maximum number of cells that can be scheduled simultaneously

  - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

Observation 5: According to the revised WID on Rel-18 Multi-carrier enhancements [1], one key point of RAN1 discussion is to

- Identify the maximum number of cells that can be scheduled simultaneously

  - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

Hence, we have the following proposals:

Proposal 1: For R18 multi-cell PUSCH/PDSCH scheduling with a single DCI, RAN1 aims to support the maximum number of cells that can be scheduled simultaneously to be 4, or more.

Proposal 2: RAN1 to adopt a 2-segment DCI structure (as shown in Figure 2(c)) to support R18 multi-cell PUSCH/PDSCH scheduling with a single DCI.

- On 2-segment aggregated DCI design

For which DCI fields should be common bit fields and which DCI fields should be designated bit fields (which would be assigned independently for each scheduled cell), this has gone through a long discussion during R17 two-cell PDSCH scheduling agenda.

Observation 6: For which DCI fields should be common bit fields and which DCI fields should be designated bit fields (which would be assigned independently for each scheduled cell), this has gone through a long discussion during R17 two-cell PDSCH scheduling agenda.

To move the discussion forward and keep good flexibility on DCI fields usage, we have the following proposal:

Proposal 3: For R18 multi-cell PUSCH/PDSCH scheduling with a single DCI, it is up to network RRC configuration to assign which DCI fields to be common bit fields and which DCI fields to be designated bit fields (which would be assigned independently for each scheduled cell)

- For example, through a bitmap to determine each DCI bit field is a common bit field or a designated bit field with one bit

For the 2-segement aggregated DCI, to avoid UE implementation impact, the 2 segments can be decoded separately on the same scheduling cell The 1st and 2nd segment DCI are then linked together to form one multi-cell scheduling DCI with some link procedure.

Proposal 4: For the 2-segment aggregated DCI, the 1st and 2nd segment DCI are decoded separately on the same scheduling cell. The 1st and 2nd segment DCI are then linked together to form one multi-cell scheduling DCI. The link procedure of 1st and 2nd segment DCI can be based on some designated DCI bit values of the 1st or 2nd segment DCI

- The linked 1st segment and 2nd segment DCI should be “both DL scheduling DCIs” or “both UL scheduling DCIs”

- Summary

In this contribution, we focus on the discussions for R18 multi-cell PUSCH/PDSCH scheduling and have the following observations and proposals:

Observation 1: According to 38.212 [2], the 5G NR polar code supports a maximum DCI size of 164 bits due to the interleaver design. Equivalently, DCI without CRC can have at most 140 payload bits.

Observation 2: For multi-cell PUSCH/PDSCH scheduling with a single DCI, there would be common bit fields which are shared (same value) among the multiple scheduled cells. At the same time, there would be designated bit fields which would be assigned independently for each scheduled cell.

- According to one example shown in Figure 1 from [3, Huawei], for PDSCH scheduling, the number of bits for common bit fields is 24, and the number of bits for designated bit fields is 34.

- According to RAN1 #103e [4], the number of bits for common bit fields is assumed to be 48/36/24/12, and the number of bits for designated bit fields is assumed to be 12/24/36/44.

Observation 3: Assuming the number of bits for common bit fields is 24, and the number of bits for designated bit fields is 36 (using information from [3, 4]). In Figure 2, it can be seen that the 140 bits DCI payload limit can only accommodate up to 3 cells if one DCI segment is used (b).

Observation 4: The 140 bits DCI payload limit is hard to be increased in 5G NR; hence, one potential way to increase the maximum number of cells that can be scheduled simultaneously is to divide a single DCI into two segments, as shown in Figure 2(c).

Observation 5: According to the revised WID on Rel-18 Multi-carrier enhancements [1], one key point of RAN1 discussion is to

- Identify the maximum number of cells that can be scheduled simultaneously

  - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

Proposal 1: For R18 multi-cell PUSCH/PDSCH scheduling with a single DCI, RAN1 aims to support the maximum number of cells that can be scheduled simultaneously to be 4, or more.

Proposal 2: RAN1 to adopt a 2-segment DCI structure (as shown in Figure 2(c)) to support R18 multi-cell PUSCH/PDSCH scheduling with a single DCI.

Observation 6: For which DCI fields should be common bit fields and which DCI fields should be designated bit fields (which would be assigned independently for each scheduled cell), this has gone through a long discussion during R17 two-cell PDSCH scheduling agenda.

Proposal 3: For R18 multi-cell PUSCH/PDSCH scheduling with a single DCI, it is up to network RRC configuration to assign which DCI fields to be common bit fields and which DCI fields to be designated bit fields (which would be assigned independently for each scheduled cell)

- For example, through a bitmap to determine each DCI bit field is a common bit field or a designated bit field with one bit

Proposal 4: For the 2-segment aggregated DCI, the 1st and 2nd segment DCI are decoded separately on the same scheduling cell. The 1st and 2nd segment DCI are then linked together to form one multi-cell scheduling DCI. The link procedure of 1st and 2nd segment DCI can be based on some designated DCI bit values of the 1st or 2nd segment DCI.

- The linked 1st segment and 2nd segment DCI should be “both DL scheduling DCIs” or “both UL scheduling DCIs”

- Reference

RP-220834, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., RAN #95-e

3GPP TS 38.212 V16.9.0, ”NR; Multiplexing and channel coding”

R1-2100194, “Discussion on multi-carrier scheduling using single PDCCH”, Huawei, HiSilicon, RAN1 #104e

[4]    Chairman’s Notes, RAN1 #103e (8.13 DSS)