3GPP TSG RAN WG1 #109-e		R1-2204703

e-Meeting, May 9th – May 20th, 2022

Agenda Item: 8.16.17

Source: MediaTek Inc.

Title: On UE capability of maximum number of layers for multi-TRP

Document for: Discussion & Decision

- Introduction

In the RAN1#108e meeting, we raised the issue of maximum number of layers for multi-TRP [1]. Although the issue was generally recognized, some companies commented that it would be too late for Rel-16 to introduce new FG and some further text check is needed. Thus, it was concluded “no consensus to introduce such a new FG for Rel-16” for the reported issue [2]. However, most companies seem fine to introduce the new FG to Rel-17, and we believe addressing this underreporting issue is important. Hence, here we revisit this topic and propose to have a new FG for Rel-17.

In this document, we first present the fundamental physical limitation and explain the shortcoming of current UE capability reporting. Next, we provide reasons why it is needed to address the mentioned issue. Finally, we present the proposed FG for Rel-17.

- Fundamental limitation of multi-TRP in FR2

Since the main benefit of multi-TRP is to have spatial diversity, the two TRPs are in general not co-located. In FR2, highly directional beams are required to combat pathloss and thus different Rx beams need to be formed for reception from different TRPs. It is less likely that one RxU can receive signals with good RSRP from both TRPs simultaneously. In short, in FR2 one RxU can only beamform to one TRP at a time. The above phenomenon is illustrated in Figure 1.

Figure 1: Fundamental limitation of multi-TRP in FR2.

Due to the above fact, the number of layers per TRP is limited by the number of associated RxUs. More specifically, for a UE equipped with N RxUs to be served by two TRPs simultaneously, the number of layers per TRPs is limited to N/2 RxUs assuming equal number of associated RxUs for each TRP.

Observation 1: In FR2, the maximum number of layers per TRP should be upper bounded by the number of associated RxUs, instead of total number of RxUs.

- Interpretation of maxNumberMIMO-LayersPDSCH

Consider a UE with N RxUs. In the conventional single-TRP operation, all available RxUs can be used for reception from this TRP and thus UE can report maxNumberMIMO-LayersPDSCH = N. In the current NR, this field is shared among all multi-TRP schemes, possibly with a different interpretation. For the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and for the FDM schemes of single-DCI based multi-TRP, the maximum number of layers per PDSCH can only be N/2 in FR2, according to the discussion in Section 2. However, the current UE capability reporting assumes the same number of layers as single-TRP, which would force the UE to underreport its capability for single-TRP and other multi-TRP schemes, i.e., single-DCI based SDM and TDM schemes and multi-DCI based multi-TRP with overlap in time and frequency.

Observation 2: For a UE intending to support the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and the single-DCI based FDM schemes, according to the current UE capability reporting, in FR2 the maximum number of layers needs to be underreported for single-TRP and other multi-TRP schemes, i.e., multi-DCI based multi-TRP with overlap in time and frequency, and single-DCI based SDM and TDM schemes.

Observation 3: In the RAN1#108e meeting, we raised this issue of maximum number of layers for multi-TRP [1]. Although the issue was generally recognized, some companies commented that it would be too late for Rel-16 to introduce new FG and some further text check is needed. Thus, it was concluded “no consensus to introduce such a new FG for Rel-16” for the reported issue [2]. However, most companies seem fine to introduce the new FG to Rel-17, and we believe addressing this underreporting issue is important.

One natural question is whether support of the mentioned schemes is needed for FR2. For multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, it is the baseline of the multi-DCI based multi-TRP schemes. Thus, from the perspective of UE capability reporting, even if a UE only wants to support the case of full overlap in time and in frequency, the UE still needs to support the case of non-overlap in frequency.

As for the single-DCI based FDM schemes, it has the following advantages for FR2:

- More reliable than the single-TRP scheme.

- The FDM scheme B is more robust than the SDM scheme when considering the blockage effect, as the current design cannot guarantee that the layers from the same TRP are self-decodable.

- Higher system throughput and better power saving than the TDM schemes.

For 3), the reason for higher system throughput is that it is challenging to serve multiple users simultaneously in FR2 as different analog beams are required in general. In this case, it is preferable to serve each UE using as less time as possible. Better power saving is achievable because UE can enter the sleeping mode earlier. Although the maximum UE throughput would be lower for the FDM schemes in FR2 as the number of layers is halved, the FDM schemes can still be beneficial for URLLC or low-power applications.

Based on the above discussions, we find it necessary to introduce a new UE capability. Following the valuable comments in the RAN1#108e meeting, we propose the following unified UE feature applicable both for the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and for the FDM schemes of single-DCI based multi-TRP. In the meantime, we add a restriction to avoid conflict with the existing field maxMIMO-LayersForMulti-DCI-mTRP-r16 [3].

Proposal 1: Introduce the following new UE feature for Rel-17:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 If this field is included, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. If this field is included, for DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. The UE that indicates support of this feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16. | Band | No | N/A | FR2 only |

Observation 4: For multi-DCI based multi-TRP, the following table summarizes the UE behaviors considering maxMIMO-LayersForMulti-DCI-mTRP-r16 and maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17. We note that if maxMIMO-LayersForMulti-DCI-mTRP-r16 is not included, the two FGs cover independent cases: maxMIMO-LayersForMulti-DCI-mTRP-r16 only covers the case of overlap in time AND in frequency; maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 only covers the case of overlap in time BUT NOT in frequency. It is noted that maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 is the newly proposed UE capability, while maxNumberMIMO-LayersPDSCH and maxMIMO-LayersForMulti-DCI-mTRP-r16 are existing 5G NR UE capabilities.

| maxMIMO-LayersForMulti-DCI-mTRP-r16 | maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 | Interpretation of maximum number of layers from maxNumberMIMO-LayersPDSCH (N) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Overlap in time and in frequency | Overlap in time but not in frequency | Not overlap in time |
| Not include | Not include | N layers across two PDSCHs | N layers per PDSCH | N layers per PDSCH |
| Not include | Include | N layers across two PDSCHs | N/2 layers per PDSCH | N layers per PDSCH |
| Include | Not include | N layers per PDSCH | N layers per PDSCH | N layers per PDSCH |
| Include | Include | Error case |  |  |

- Conclusion

Observation 1: In FR2, the maximum number of layers per TRP should be upper bounded by the number of associated RxUs, instead of total number of RxUs.

Observation 2: For a UE intending to support the multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency and the single-DCI based FDM schemes, according to the current UE capability reporting, in FR2 the maximum number of layers needs to be underreported for single-TRP and other multi-TRP schemes, i.e., multi-DCI based multi-TRP with overlap in time and frequency, and single-DCI based SDM and TDM schemes.

Observation 3: In the RAN1#108e meeting, we raised this issue of maximum number of layers for multi-TRP [1]. Although the issue was generally recognized, some companies commented that it would be too late for Rel-16 to introduce new FG and some further text check is needed. Thus, it was concluded “no consensus to introduce such a new FG for Rel-16” for the reported issue [2]. However, most companies seem fine to introduce the new FG to Rel-17, and we believe addressing this underreporting issue is important.

Proposal 1: Introduce the following new UE feature for Rel-17:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 If this field is included, for DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. If this field is included, for DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH. The UE that indicates support of this feature shall not indicate support maxMIMO-LayersForMulti-DCI-mTRP-r16. | Band | No | N/A | FR2 only |

Observation 4: For multi-DCI based multi-TRP, the following table summarizes the UE behaviors considering maxMIMO-LayersForMulti-DCI-mTRP-r16 and maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17. We note that if maxMIMO-LayersForMulti-DCI-mTRP-r16 is not included, the two FGs cover independent cases: maxMIMO-LayersForMulti-DCI-mTRP-r16 only covers the case of overlap in time AND in frequency; maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 only covers the case of overlap in time BUT NOT in frequency. It is noted that maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 is the newly proposed UE capability, while maxNumberMIMO-LayersPDSCH and maxMIMO-LayersForMulti-DCI-mTRP-r16 are existing 5G NR UE capabilities.

| maxMIMO-LayersForMulti-DCI-mTRP-r16 | maxNumberMIMO-LayersPDSCH-For-mTRP-FDM-r17 | Interpretation of maximum number of layers from maxNumberMIMO-LayersPDSCH (N) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Overlap in time and in frequency | Overlap in time but not in frequency | Not overlap in time |
| Not include | Not include | N layers across two PDSCHs | N layers per PDSCH | N layers per PDSCH |
| Not include | Include | N layers across two PDSCHs | N/2 layers per PDSCH | N layers per PDSCH |
| Include | Not include | N layers per PDSCH | N layers per PDSCH | N layers per PDSCH |
| Include | Include | Error case |  |  |

- References

- R1-2202084, “On UE capability of maximum number of layers for multi-TRP,” MediaTek.

- R1-2202511, “Summary on [108-e-R16-UE-features-MIMO-02]”

- 3GPP TS 38.306: “User Equipment (UE) radio access capabilities (Release 16)”.