3GPP TSG RAN WG1 #109-e			            R1-2204704

e-Meeting, May 9th – 20th, 2022

Agenda Item: 8.7.1.

Source: MediaTek Inc.

Title: Maintenance on idle/inactive-mode UE power saving

Document for: Discussion and Decision

# Introduction

In this contribution, the following identified issues for Rel-17 idle/inactive-mode UE power saving enhancements after RAN1#108-e meeting [1]-[7] will be discussed and resolved:

- Paging Early Indication

  - Alignment with RAN2 agreement – removing the case PEI is applied without subgroupsNumPerPO

  - Whether to clarify joint PDCCH monitoring behaviour when both SDT and PEI are applied

- TRS information for idle/inactive UEs

  - TP for resolving reference start ambiguity for TRS availability indication

# Maintenance on Paging Early Indication Design

## Alignment with RAN2 agreement

In RAN2#117-e meeting, the following on bundled functionalities of PEI and UE subgrouping are agreed [8]:

PEI + UEID subgrouping is one capability A PEI-capable UE must support at least UEID-based subgrouping method. RAN2 confirms that “PEI without subgrouping” can be implemented by configuring PEI plus UEID subgrouping with one subgroup. “PEI without subgrouping” can be configured by only one method.

Accordingly, there is no such case PEI is applied without UE subgroup number provided, and the following TP is suggested for the alignment with the RAN2 agreements.

Proposal 1: Adopt the following TPs to TS 38.212 and TS 38.213 for removing the case PEI is applied without subgroupsNumPerPO provided, as per the RAN2 agreements in RAN2#117-e.

##### Section 7.3.1.3.8 (Format 2_7) of TS 38.212 <Unchanged text omitted> DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO > 0 is configured; otherwise each bit in the field indicates the UE group of a paging occasion. <Unchanged text omitted>

Section 10.4A of TS 38.213 <Unchanged text omitted> A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. <Unchanged text omitted>

## Whether to clarify joint PDCCH monitoring behaviour when both SDT and PEI are applied

In RAN1#108-e meeting, there is discussion on whether/how to include TP for clarifying UE monitoring behaviour when both SDT and PEI are applied. The necessity depends on whether the same CSS is utilized for the PDCCH of STD and PEI. In [10], there include the following quotation that shows separated CSS is agreed for SDT. On the other hand, PEI applies one of up to 4 common SS sets configured by commonSearchSpaceList. Accordingly, we have the following observation and conclusion:

Observation 1: The common search space sets for SDT and PEI are different.

Conclusion 1: No TP is needed for clarifying the joint PDCCH monitoring for both SDT and PEI.

RAN1 LS to RAN2 (R1-2102125) For RA-SDT RAN1 has discussed CORESET and search space for monitoring the PDCCH addressed to the C-RNTI after successful completion of the RACH procedure during RA-SDT. From RAN1 perspective, the existing type-1 PDCCH CSS used for random access can be reused but it may impose higher PDCCH blocking rate and impact to the legacy UE performing random access, so at least a new search space that is different from the existing CSS should be supported for RA-SDT. It is feasible from RAN1 perspective to use either a new common search space or a UE-specific search space, thus it can be up to RAN2 to make the decision. The configuration of CORESET will be further discussed, basically following the same design logic for search space. Based on the above, the following conclusions have been made. Based on the above, the following conclusions have been made. - From RAN1 perspective, at least a separate SearchSpace that is different from the existing common SearchSpace should be supported for monitoring the PDCCH addressed to the C-RNTI after successful completion of the RACH procedure during RA-SDT - It is up to RAN2 decision if the separate SearchSpace is UE-specific or common to the UEs performing RA-SDT - If the separate SearchSpace is not configured, type-1 PDCCH CSS can be reused. - FFS UE-specific CORESET or common CORESET

Agreements in RAN2#114-e Agreement 1. CFRA is not supported for RA-SDT 2. The separate search space is common to the UEs performing RA-SDT. Inform RAN1 of this agreement 3. Working assumption: UE-specific search space is configured for UEs performing CG-SDT. RAN2 asks RAN1 whether this working assumption can be confirmed 4. The UE needs to monitor paging after UE initiates SDT for system information change, PWS. FFS for other cases 5. CG-SDT resource can be configured on either initial BWP or separate SDT BWP. Ask RAN1 to confirm 6. FFS CS-RNTI based dynamic retransmission is reused for CG-SDT

Agreement in RAN1#107-e Agreement ‘peiSearchSpace’ can be configured to one of up to 4 common SS sets configured by commonSearchSpaceList with SearchSpaceId > 0

Section 6.3.2 of TS 38.331 (Rel-17) pei-SearchSpace ID of dedicated search space for PEI. It can be configured to one of up to 4 common SS sets configured by commonSearchSpaceList with SearchSpaceId > 0. The CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level follows Table 10.1-1 of TS38.213 [13]. SearchSpaceId = 0 can be configured for the case of SS/PBCH block and CORESET multiplexing pattern 2 or 3.

# Maintenance on TRS Information for Idle/Inactive UEs

For NR, PO or PEI-O corresponds a set of PDCCH MOs which can be allocated in different frames. In [9], it is identified that existing specification can cause ambiguous reference points for the indicated validity duration if PO or PEI-O spreads over two frames that correspond to different default paging cycles.

Figure 1: Illustration of ambiguity in reference point (quoted from [9])

In RAN1#108-e [1], the following is agreed with the intention of selecting one of the two TPs to resolve the ambiguity. If spec change is intended, TP2 looks a proposal that can avoid such ambiguity, where TP1 doesn’t resolve the issue. Consequently, the following proposal is suggested:

Proposal 2: According to the agreement for determining the reference point for the indicated validity duration, adopt TP#2 to resolve the ambiguity issue when PO or PEI-O spreads over two frames that correspond to different paging cycles.

Agreement Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication - Alt1: Adopt TP#1 - Alt2: Adopt TP#2 - Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same. - Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value. ============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#2==========================================.

# Summary

In this contribution, the identified issues for Rel-17 idle/inactive-mode UE power saving enhancements after RAN1#108-e meeting [1]-[7] are discussed and resolved. In particular, the following are suggested:

Proposal 1: Adopt the following TPs to TS 38.212 and TS 38.213 for removing the case PEI is applied without subgroupsNumPerPO provided, as per the RAN2 agreements in RAN2#117-e.

##### Section 7.3.1.3.8 (Format 2_7) of TS 38.212 <Unchanged text omitted> DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO, if subgroupsNumPerPO is configured; otherwise NSGPO is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPO > 0 is configured; otherwise each bit in the field indicates the UE group of a paging occasion. <Unchanged text omitted>

Section 10.4A of TS 38.213 <Unchanged text omitted> A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. <Unchanged text omitted>

Observation 1: The common search space sets for SDT and PEI are different.

Conclusion 1: No TP is needed for clarifying the joint PDCCH monitoring for both SDT and PEI.

Proposal 2: According to the agreement for determining the reference point for the indicated validity duration, adopt TP#2 to resolve the ambiguity issue when PO or PEI-O spreads over two frames that correspond to different paging cycles.

# Reference

“Draft Report of 3GPP TSG RAN WG1 #108-e”, MCC support, RAN1#108-e

R1-2202067, “Summary#1 of Maintenance on Paging Enhancements”, Moderator (MediaTek), RAN1#108-e

R1-2202933, “Collection of RAN1-related agreements for UE power saving enhancements for NR (post RAN1 #108-e)”, Rapporteur (MediaTek), RAN1#108-e

R1-2202975, “Corrections on NR UE Power Saving Enhancements (38.202)”, Qualcomm, RAN1#108-e

R1-2202969, “Corrections on UE power saving enhancements in 38.212”, Huawei, RAN1#108-e

R1-2202953, “Corrections on UE power savings enhancements in NR (38.213)”, Samsung, RAN1#108-e

R1-2202964, “Corrections on NR UE Power Saving Enhancements (38.214)” Nokia, RAN1#108-e

RP-220747, “Status report for WI UE Power Saving Enhancements for NR”, Rapporteur (MediaTek), RAN #95-e

R1-2200945, “Remaining issue on assistance TRS occasions for IDLE/inactive mode”, Huawei, HiSilicon, RAN1#108-e

“[108-e-R17-PowSav-01] Email discussion for maintenance on potential paging enhancements”, online available @ https://list.etsi.org/scripts/wa.exe?A2=3GPP_TSG_RAN_WG1;27a3a6c9.2203A_1&S=