3GPP TSG RAN WG1 #109-e		    	R1-2204709

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.9.1

Title:	Discussion on NR PDCCH reception in symbols with LTE CRS REs

Source:	MediaTek Inc.

Document for:	Discussion

- Introduction

In current specification, UE is not required to monitor NR PDCCH in the resources overlapped with LTE CRS. Whether to lift such restriction in Rel-18 DSS operation by allowing puncturing NR PDCCH with LTE CRS when the resources are overlapped was discussed in RAN #94-e meeting. On one hand, allowing NR PDCCH reception overlapping with LTE CRS can increase the resource utilization for NR PDCCH, especially for the UE monitoring first 3 symbols of a slot under 4 ports LTE CRS. On the other hand, the punctured PDCCH content and the PDCCH DMRS degrade the BLER performance and increase complexity on channel estimation, respectively. To investigate the overall impact, further study on the benefit and necessity of supporting NR PDCCH reception in symbols with LTE CRS REs is captured in WID [1] as follows

- Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1]

  - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources.

Our views on simulation assumption for further performance evaluation, performance analysis, and specification impact are as follows.

- Simulation assumption for performance evaluation

PDCCH monitoring symbols

During the discussion in RAN #94 meeting, one of the motivations to support puncturing NR PDCCH by LTE CRS is to improve NR PDCCH resource utilization when UE monitors PDCCH in the first 3 symbols of a slot under 4 LTE CRS ports, where only one symbol is available for NR PDCCH due to the current restriction. However, the study scope is not only confined in such configuration based on WID. That is, monitoring PDCCH other than the first 3 symbols is also covered in the study scope from WID, e.g., monitoring PDCCH in the first 4 symbols introduced in RAN1 #108e meeting [2]. Without a clear definition on the baseline(s), the alignment between company evaluation results can be challenging and the conclusion can be misleading. To facilitate the study, we suggest including following two UE PDCCH monitoring capabilities as baselines for further evaluation on the performance of punctured PDCCH:

- Monitoring the first 3 symbols (FG 3-1)

- Monitoring the first 4 symbols [2]

In our view, both UE capabilities are commonly used in DSS operation. In fact, the capability of monitoring the first 4 symbols is proposed as a practical extension of FG 3-1 in DSS operation.

To investigate the impact of puncturing, the above baselines together with the configuration that symbol 0 and symbol 1 are occupied by 4 ports LTE CRS should be considered. In particular, allowing NR PDCCH in symbol 1 to be punctured by LTE CRS on top of the two baseline PDCCH monitoring behaviors should be considered for the respective performance gain evaluation, which is illustrated in Figure 1 and Figure 2. Note that the performance comparison between puncturing and non-puncturing PDCCH under the UE capability of monitoring PDCCH in the first 4 symbols can be used as a reference for the performance evaluation of the common NR PDCCH monitoring configuration where UE monitors PDCCH in a three symbols CORESET and one of the symbols is punctured by LTE CRS.

Figure 1: Illustration of monitoring NR PDCCH in the first 3 symbols with and without puncturing PDCCH in symbol 1 by LTE CRS

Figure 2: Illustration of monitoring NR PDCCH in the first 4 symbols with and without puncturing PDCCH in symbol 1 by LTE CRS

Channel bandwidth

To simplify the simulation and obtain a clear picture on the impact of PDCCH puncturing, it is desirable to assume NR PDCCH bandwidth is fully overlapped with LTE CRS transmission bandwidth, i.e., partially overlapped in frequency domain between LTE CRS and NR PDCCH should be avoided in performance evaluation.

To specify the candidate values of channel bandwidth for performance evaluation, we suggest considering at least 5M and 10MHz bandwidth for further study to accommodate LTE bandwidth and the traffic. Although 20 MHz is the maximum LTE channel bandwidth and we are open to evaluate the configuration, it is expected that LTE usage should be lower in practical Rel-18 DSS operation and the evaluation results from smaller bandwidth should be prioritized.

Other parameters

To justify the performance evaluation results, we suggest considering typical simulation assumption parameters of PDCCH demodulation performance requirement and we summarized in Table 1 together with our suggestions on channel bandwidth and PDCCH monitoring symbols. Note that how to handle the punctured PDCCH DMRS in CE should be discussed first to align simulation setup across different companies, which can also benefit CE complexity discussion later on.

Proposal 1: RAN1 adopts Table 1 as Rel-18 DSS simulation assumption for evaluation on the performance of puncturing PDCCH by LTE CRS

Table 1: Rel-18 DSS simulation assumption for evaluation on the performance of puncturing PDCCH by LTE CRS

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz / 3 GHz |
| SCS | 15 kHz |
| PDCCH monitoring symbols | first 3 symbols / first 4 symbols |
| Bandwidth | 5 MHz / 10 MHz /20 MHz |
| Channel model | TDL-A 30-10 / TDL-C 300-100 |
| Correlation | low |
| Number of BS antennas | 1 Tx / 2 Tx |
| Number of UE antennas | 2 Rx / 4Rx |
| Channel estimation | TBD |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 40 / 70 bits |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBS |

- Performance analysis, specification, and CE implementation impact

Performance analysis

In this section, we provide our initial assessment on the performance comparison between puncturing and non-puncturing PDCCH scheme based on the simulation assumption in Table 2. The simulation results are shown in Figure 3 and Figure 4.

Table 2 Simulation assumption for evaluation on the performance of puncturing PDCCH by LTE CRS

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| PDCCH monitoring symbols | first 3 symbols / first 4 symbols |
| Bandwidth | 20 MHz |
| Channel model | TDL-A 30-10 |
| Correlation | Low |
| Number of BS antennas | 1 Tx |
| Number of UE antennas | 2 Rx |
| Channel estimation | Ideal CE |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 70 bits |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBS |

In Figure 3, BLER curves for 1 symbol PDCCH without puncturing (solid curves) and 2 symbols PDCCH with one symbol punctured by LTE CRS (dash curves) are listed. It can be observed that, for the same AL, puncturing PDCCH shows at least 2 dB BLER degradation from non-puncturing PDCCH. Based on the SINR distribution of low-band UMA scenario [3], the average CCE number of 1 symbol PDCCH is around 6 and the average CCE number of 2 symbol PDCCH with puncturing is around 7.3, which is around 20% increase to no puncturing case. On the other hand, the extra punctured symbol for PDCCH provide 67% resource increase for PDCCH transmission. Consequently, the PDCCH capacity gain provided by 2 symbols PDCCH with one punctured symbol over 1 symbol PDCCH is around 37%. Similarly, it can be derived that the PDCCH capacity gain provided by 3 symbols PDCCH with 1 punctured symbol over 2 symbol PDCCH is around 15% based on the BLER curves between 2 symbol PDCCH without puncturing (solid curves) and 3 symbols PDCCH with one symbol punctured by LTE CRS (dash curves) listed in Figure 4.

From the above analysis on PDCCH capacity gain under different configurations of PDCCH monitoring symbols, it is noticed that the PDCCH capacity gain of enabling extra punctured PDCCH symbol largely decreases for the UEs monitoring first 4 PDCCH symbols (symbol 0 is LTE PDCCH, symbol 1 is NR PDCCH punctured by LTE CRS, symbol 2 and 3 are NR PDCCH). Note that, the above analysis is based on the ideal channel estimation and the PDCCH capacity gain from puncturing one extra symbol is expected to be further decreased when the impact of punctured PDCCH DMRS is considered. Moreover, monitoring PDCCH in the first 4 symbols without puncturing by LTE CRS on symbol 0 and 1 provides around 20% PDCCH capacity gain over monitoring PDCCH in the first 3 symbols with puncturing on symbol due to the additional non-punctured resource on symbol 3. Therefore, we have the following observations:

Observation 1: PDCCH capacity gain from extra one NR PDCCH symbol punctured by LTE CRS is not significant for UEs monitoring PDCCH in the first 4 symbols of a slot.

Observation 2: Monitoring PDCCH in the first 4 symbols without puncturing provides better and consistent PDCCH capacity gain over monitoring PDCCH in the first 3 symbols with and without out puncturing.

Figure 3: 1 symbol PDCCH without puncturing (solid curves) and 2 symbols PDCCH with one puncturing symbol (dash curves) BLER performance curves

Figure 4: 2 symbol PDCCH without puncturing (solid curves) and 3 symbols PDCCH with one puncturing symbol (dash curves) BLER performance curves

Specification and CE implementation impact

One identified concern of allowing NR PDCCH reception in symbols with LTE CRS REs is the impact on CE implementation considering punctured PDCCH DMRS. In current specification, there is no precedent example of allowing puncturing DMRS due to the significant impact on channel estimation. Consequently, implementing CE to handle adaptive and irregular PDCCH DMRS patterns can be challenging to UE vendors and a whole new design might be needed. Furthermore, it is possible to configure UE more than one LTE CRS patterns, which adds complexity for CE design to another level.

In addition to CE implementation impact, another related impact is on specification efforts. In RAN #94, it was argued that the specification update is limited to allowing UEs to monitor PDCCH candidates overlapped with LTE CRS by simply removing the related restriction in RAN1 specification. However, due to the existence of legacy UEs, the specification update might not be negligible. For example, whether to allow puncturing on cell-specific PDCCHs and if allowed, specification change requires careful design to accommodate the coexistence of legacy UEs and advanced UEs. Also, RAN4 specification impact is significant, e.g., PDCCH demodulation requirements and other BM procedures performance requirements using PDCCH BLER performance as reference.

On the other hand, the UE capability of monitoring PDCCH in the first 4 symbols of a slot has been introduced in RAN1 #108-e meeting, which requires less specification impact and CE implementation complexity compared to enabling puncturing NR PDCCH by LTE CRS. Moreover, the associated PDCCH capacity gain is consistent even more than 1 LTE CRS pattern is configured, which is a clear advantage over the puncturing schemes depended on configured CRS patterns.

Observation 3: Enabling LTE CRS to puncture NR PDCCH and NR PDCCH DMRS has non-negligible impact on RAN1 and RAN4 specification and channel estimation implementation.

Observation 4: Enabling monitoring PDCCH in the first 4 symbols achieves better trade-off than enabling LTE CRS to puncture NR PDCCH in terms of PDCCH capacity gain, spec impact, and CE implementation complexity.

- Conclusion

Based on the PDCCH capacity analysis example, specification impact, and CE complexity, we summarize the comparison between 1 symbol PDCCH without puncturing, 2 symbols PDCCH with one punctured symbol, and 2 symbol PDCCH without puncturing into Table 3. It can be concluded that enabling monitoring PDCCH in the first 4 symbols achieves better PDCCH capacity gain than enabling LTE CRS to puncture NR PDCCH without spec impact and extra CE implementation complexity. Furthermore, based on the outcome of performance analysis, the PDCCH capacity gain offered from extra punctured PDCCH symbol is not significant for UE monitoring PDCCH in the first 4 symbols, which is summarized in Table 4. Therefore, the overall benefit of supporting enabling LTE CRS to puncture NR PDCCH is not clear compared to the existing capability of monitoring PDCCH in the first 4 symbols without puncturing based on our analysis.

Table 3: Comparison between 1 symbol PDCCH, 2 symbols PDCCH, and 2 symbol PDCCH with 1 punctured symbol

| UEs | R-15 | R-16/17 | R-18 |
| --- | --- | --- | --- |
| PDCCH monitoring capability | First 3 symbols | First 4 symbols | First 3 symbols |
| PDCCH monitoring symbol | Symbol 2 | Symbol 2+Symbol 3 | Symbol 1(punctured)+Symbol 2 |
| Normalized PDCCH capacity | 1 | 1.6 | 1.4 |
| Spec impact |  |  | X |
| CE impact |  |  | X |

Table 4 Comparison between 2 symbol PDCCH and 3 symbol PDCCH with 1 punctured symbol

| UEs | R-16/17 | R-18 |
| --- | --- | --- |
| PDCCH monitoring capability | First 4 symbols | First 4 symbols |
| PDCCH monitoring symbol | Symbol 2+Symbol 3 | Symbol 1(punctured)+Symbol 2+symbol 3 |
| Normalized PDCCH capacity | 1 | 1.15 |
| Spec impact |  | X |
| CE impact |  | X |

In summary, we have the following proposals:

Proposal 1: RAN1 adopts Table 1 as Rel-18 DSS simulation assumption for evaluation on the performance of puncturing PDCCH by LTE CRS

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz / 3 GHz |
| SCS | 15 kHz |
| PDCCH monitoring symbols | first 3 symbols / first 4 symbols |
| Bandwidth | 5 MHz / 10 MHz /20 MHz |
| Channel model | TDL-A 30-10 / TDL-C 300-100 |
| Correlation | low |
| Number of BS antennas | 1 Tx / 2 Tx |
| Number of UE antennas | 2 Rx / 4Rx |
| Channel estimation | TBD |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 40 / 70 bits |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBS |

Observation 1: PDCCH capacity gain from extra one NR PDCCH symbol punctured by LTE CRS is not significant for UEs monitoring PDCCH in the first 4 symbols of a slot.

Observation 2: Monitoring PDCCH in the first 4 symbols without puncturing provides better and consistent PDCCH capacity gain over monitoring PDCCH in the first 3 symbols with and without out puncturing.

Observation 3: Enabling LTE CRS to puncture NR PDCCH and NR PDCCH DMRS has non-negligible impact on RAN1 and RAN4 specification and channel estimation implementation.

Observation 4: Enabling monitoring PDCCH in the first 4 symbols achieves better trade-off than enabling LTE CRS to puncture NR PDCCH in terms of PDCCH capacity gain, spec impact, and CE implementation complexity.

- References

- RP-213575, “New WI: Enhancement of NR Dynamic spectrum sharing (DSS)”, December, 2021

- Chairman's Notes RAN1#108-e, Feburary 2022

- RP-212946, “On DSS enhancements in Rel-18”, Ericsson, December, 2021