3GPP TSG RAN WG1 #109-e         	                                               R1-2204715 e-Meeting, May 9th – 20th, 2022

Agenda item:	9.6.2

Source: 	MediaTek Inc.

Title: 	On Simulation Needs and Assumptions for RedCap UEs

Document for:	Discussion and decision

# Introduction

A new study item on further NR UE complexity reduction was approved with the following objectives [1]:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

# Discussion

In our companion contribution [2], we identify that both SSB and CORESET#0 cannot be confined within 5MHz at SCS=30kHz. To minimize impact on specification and existing network and UEs, we think redesign of SSB, particularly PBCH, and CORESET#0 should be avoided. On the other hand, if the support for 5MHz with 30kHz SCS relies on UE implementation, we need to evaluate the corresponding performance loss.

Proposal 1: Evaluate PBCH BLER performance with PBCH punctured to 5MHz at 30kHz SCS.

Proposal 2: Evaluate Type0-PDCCH BLER performance with CORESET#0 punctured to 5MHz at 30kHz SCS.

Proposal 3: Evaluate SIB1 PDSCH BLER performance with SIB1 punctured to 5MHz at 30kHz SCS.

# Conclusions

In this contribution, we have the following proposals.

Proposal 1: Evaluate PBCH BLER performance with PBCH punctured to 5MHz at 30kHz SCS.

Proposal 2: Evaluate Type0-PDCCH BLER performance with CORESET#0 punctured to 5MHz at 30kHz SCS.

Proposal 3: Evaluate SIB1 PDSCH BLER performance with SIB1 punctured to 5MHz at 30kHz SCS.

# References

- RP-213661, “New SID on Study on further NR RedCap UE complexity reduction,” Ericsson, 3GPP TSG RAN meeting #94e, December 2021

- R1-2204714, “On Potential Solutions to Further Reduce UE Complexity,” MediaTek Inc., 3GPP TSG RAN WG1 meeting #109e, May 2022