3GPP TSG RAN WG1 #109-e		R1-2204716

e-Meeting, May 9th – 20th, 2022

Source: 	MediaTek Inc.

Title:	Discussion on RAN2 LS on HARQ process for MCCH and Broadcast MTCH

Agenda item:	5

Document for:	Discussion & Decision

## Introduction

In last RAN2 meeting, they have discussed dedicated HARQ process IDs for MCCH and Broadcast MTCH(s) and want to capture the related information in the running CR of TS38.321. Regarding whether the single HARQ process for MCCH and single HARQ process for MTCH is same of different, they didn’t get consensus even if majority views thought it is up to UE implementation. Finally, RAN2 sent an LS to RAN1 [1] and ask RAN1 whether can confirm the following assumption with an agreement that there are no dedicated HARQ process IDs for MCCH and Broadcast MTCH:

There are no dedicated HARQ process IDs for MCCH and Broadcast MTCH (assumption: single HARQ process for MCCH and single HARQ process for MTCH, not clear whether they can share the same, details would be RAN1 scope)

In this contribution, we will discuss the issue and answer above question from RAN1’s perspective.

## Discussion

Regarding the HARQ process for Broadcast MCCH and MTCH, RAN1 has achieved the following conclusion and agreement in RAN1#107bis-e [2]:

Conclusion Additional HARQ process(es) is(are) not introduced for Rel-17 MBS broadcast reception on serving cell. - Note: The UE is not expected to support hardware for more HARQ processes for receiving broadcast in Rel-17 in addition to the maximum number of HARQ processes supported for receiving unicast in Rel-16, i.e. the HARQ process resources are shared between broadcast, unicast and multicast Agreement: HARQ process ID is not indicated in DCI format 4_0 for both MCCH and MTCH.

Based on the above agreement and conclusion, we can see that there is no dedicated HARQ process ID for MCCH and MTCH, which also aligns with RAN2’s agreement. Regarding UE how to allocate the HARQ buffer for receiving the MCCH and MTCH, from our perspective, it is up to UE implementation, and it does not need to make any restriction from the UE side, e.g., whether the same or different HARQ process for MCCH and MTCH. Besides, in the approved TS 38.321 CR [3], the following text marked with blue was agreed.

The MAC entity shall: 1> if a downlink assignment has been indicated: 2> allocate the TB(s) received from the physical layer and the associated HARQ information to the HARQ process indicated by the associated HARQ information. 1> if a downlink assignment has been indicated for the broadcast HARQ process: 2> allocate the received TB to the broadcast HARQ process. NOTE: It is up to UE impletentation to allocate the received TB for MCCH or broadcast MTCH to one HARQ process.

From our understanding, the added note is sufficient to describe the UE’s behaviour and there is no need to make any further description/restriction for UE receiving the MCCH and MTCH from HARQ buffer perspective.

Proposal 1: It is up to UE implementation on how to receive the MCCH and MTCH from HARQ buffer perspective, and the corresponding description in current Spec of TS38.321 is sufficient.

## Conclusion

In this contribution, we discuss the LS from RAN2 on HARQ process for MCCH and Broadcast MTCH, and the following proposal is suggested:

Proposal 1: It is up to UE implementation on how to receive the MCCH and MTCH from HARQ buffer perspective, and the corresponding description in current Spec of TS38.321 is sufficient.

## Reference

- R2-2204017, LS on HARQ process for MCCH and Broadcast MTCH(s), Samsung, RAN2#117-e, 21st February – 3rd March 2022

- RAN1 Chairman’s Notes, RAN1 #107bis-e, February 2022.

- R2-2203818, Introduction of NR MBS, OPPO, RAN2#117-e, 21st February – 3rd March 2022