3GPP TSG RAN WG1 #109-e		    R1-2204717

e-Meeting, May 9th – 20th, 2022

Source: 	MediaTek Inc.

Title:	Remaining issues on improve multicast reliability for RRC_CONNECTED UEs

Agenda item:	8.12.1

Document for:	Discussion & Decision

## Introduction

In last previous e-meeting [1], the mechanisms on improve the multicast reliability were discussed and some agreements were achieved at the end of meeting. However, there are still some remaining issues that need to be further discussed to finalize the Rel-17 MBS. In this contribution, we will further discuss the detailed design about the remaining issues to improve multicast reliability for RRC_CONNECTED UEs.

## Discussion

  - More than one NACK only bits

When more than one NACK-only based feedback bits are transmitted in the same PUCCH slot, it was discussed in last several meetings and the following agreements were achieved in last e-meeting.

Agreement: For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability Agreement: Regarding RRC configuring Alt1 or Alt4 (from the previous agreement) for multiplexing more than one NACK-only in the same PUCCH transmission, the configuration is per CFR.

Based on the above agreement, both Alt1 and Alt4 is supported for more than one NACK-only feedback, and which solution to be configured is based on the RRC signaling. However, considering the different UE device type and fast commercial deployment of the MBS feature, it is better to define a UE capability for UE to report which solution is preferred if more than one NACK only based feedback are occurred in the same PUCCH slot.

Proposal 1: Define a UE capability for UE to report whether to support Alt1(i.e., Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits) or Alt4(i.e., o	define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback) if more than one NACK-only feedback bits are transmitted in the same PUCCH slot.

For the Alt1, one remaining issue is how to determine the PUCCH resource for transmitting the HARQ-ACK bits. Regarding the PUCCH resource for the ACK/NACK and NACK-only feedback, the following agreement was achieved in RAN1#106bis-e meeting:

Agreement: For UE supporting both ACK/NACK based and NACK-only based feedback for multicast, for the same G-RNTI, support the following - UE can be configured with either ACK/NACK based or NACK-only feedback for a single G-RNTI. - Note: Case1-1: if configured with ACK/NACK based feedback, UE can be optionally configured a separate PUCCH-Config/PUCCH-ConfigurationList for multicast. Otherwise, PUCCH-Config/PUCCH-ConfigurationList for unicast applies (This has been agreed.) - Case 1-2: if configured with NACK-only based feedback, when separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is not configured, PUCCH-Config/PUCCH-ConfigurationList for unicast applies.

It means that the NW will not configure the separate PUCCH-Config/PUCCH-ConfigurationList for ACK/NACK based HARQ feedback for the same G-RNTI if the configured HARQ ACK feedback option is NACK-only based feedback, which implied that there is no specific ACK/NACK based PUCCH resource for reporting the ACK/NACK bits transformed by the NACK-only bits. In RAN1#107bis-e meeting, it also has agreed that the separate PUCCH-Config/ PUCCH-ConfigurationList can be configured as following for NACK-only based feedback.

Agreement: For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback, - 1 PUCCH resource set in each PUCCH-Config. - up to 32 PUCCH resources in PUCCH resource set - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback.

If the separate PUCCH resource is not configured, the PUCCH-Config/PUCCH-ConfigurationList for unicast applies as agreed in RAN1#106bis-e meeting. Thus, we suggest the PUCCH resource for Alt1 use that of for NACK-only based feedback, which is configured by RRC signaling (e.g., for unicast or multicast). Since each ACK/NACK bit (or TB) corresponds to one PUCCH resource indicator within the corresponding DCI format, another question is how to determine which PUCCH resource ID is used for the Alt1. The simplest solution is to use the last DCI scheduling multicast corresponding to the more than one NACK-only feedback bits.

Proposal 2: The PUCCH resource of NACK-only based feedback configured via RRC signalling is reused for the Alt 1, and the “last DCI” scheduling multicast is used to determine the PUCCH resource ID.

For the Alt4 as agreed, from our understanding, up to 15 orthogonal PUCCH resource will be configured by RRC signaling (e.g., separate PUCCH-Config/ PUCCH-ConfigurationList), and the UE can determine which PUCCH ID is used based on the pre-defined mapping relationship between the number of TBs and PUCCH ID. For example, the 3TBs (TB1, TB2, TB3) will correspond to the 7 PUCCH resources (e.g., PUCCH ID0, …, PUCCH ID6), and the mapping relationship can de pre-defined as: (NACK, /, /) ---> PUCCH ID0, (NACK, /, NACK) ---> PUCCH ID1, (NACK, NACK, /) ---> PUCCH ID2, (NACK, NACK, NACK) ---> PUCCH ID3, (/, NACK, /) ---> PUCCH ID4, (/, NACK, NACK) ---> PUCCH ID5, (/, /, NACK) ---> PUCCH ID6. If the TB1 and TB3 are received successfully and the TB2 is decoded failed, the UE will use the PUCCH ID 4 to transmit the NACK information following the above pre-defined mapping relationship.

Proposal 3: For Alt4, the PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the pre-defined mapping relationship between number of TBs and PUCCH resource ID.

For the Alt4, there is another remaining issue is that whether this applies to a single G-RNTI and multiple G-RNTIs. From our perspective, if the Alt4 is applied to a single G-RNTI, the gNB need to configure multiple times up to PUCCH resources for the multiple G-RNTIs case. However, as we discussed in previous meeting, we usually assumed and agreed that the PUCCH resources (e.g., for NACK only feedback or ACK/NACK feedback) are used for all G-RNTIs corresponding NACK only feedback or ACK/NACK feedback, respectively. E.g., the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback, and the PUCCH-Config is applied to all G-RNTIs with ACK/NACK based feedback.

Proposal 4: Alt4 applies to the all G-RNTIs for supporting more than one NACK-only feedback in the same PUCCH transmission.

  - HARQ-ACK disabling for Type-1 CB construction

As we discussed and agreed in previous meeting, the HARQ-ACK feedback can be disabled/enabled by RRC configuration or DCI indication per G-RNTI, and multiple G-RNTIs also can be configured for the same UE. One question is that how to construct the Type-1 CB with HARQ-ACK enabling/disabling, especially for the HARQ-ACK disable case, it was extensively discussed in the last e-meeting, and the following agreements were achieved:

Agreement: For Type-1 codebook generation, - if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot; - For other cases, FFS between the 3 alternatives below: - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs. - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI - Other alternatives are not precluded

Based on the agreement, the UE will not report the HARQ-ACK information if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling. The remaining issue is that if at least one configured G-RNTI is with HARQ-ACK enabled, whether the UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH for the Type-1 codebook generation. From our understanding, since the HARQ-ACK is disabled by the RRC signalling, it does not make sense to enforce the UE to report any HARQ-ACK information. However, if the UE does not report the disabled HARQ-ACK CB bits, the Type-1 CB size will be changed, which against the design principle that Type-1 CB generation is used for all the G-RNTIs as commented by some companies. Considering that which HARQ-ACK CB type to be used and the HARQ-ACK enabling/disabling are configured by the gNB via RRC signalling, we prefer that the case mentioned above can be avoided by the NW scheduling, e.g., prefer Alt3.

Proposal 5: For Type-1 codebook generation, all the configured G-RNTIs are with the same HARQ enabling/disabling behaviour via RRC signalling configuration, i.e., UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI.

Regarding the ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, the following consensus was achieved in last meeting:

Agreement: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, - FFS between the 3 4 alternatives below: - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling. - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC. - Other alternatives are not precluded.

From our perspective, we share the similar view with all configured G-RNTIs corresponding only dynamic scheduling and UE is not expected to be configured with G-RNTI with HARQ-ACK enabled (or disabled) by RRC signalling and G-CS-RNTI with HARQ-ACK disabled (or enabled) by RRC signalling.

Proposal 6: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, UE is not expected to be configured with G-RNTI with HARQ-ACK enabled (or disabled) by RRC signalling and G-CS-RNTI with HARQ-ACK disabled (or enabled) by RRC signalling.

  - NACK-only feedback collides with SR

Regarding the NACK only collide with SR in the same slot, the following proposal was discussed in previous meeting:

Proposal 3.1.1: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following: - For up to two TBs/HARQ-ACK bits of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16. - PUCCH resources configured for unicast will be used subject to legacy procedure as Rel-15/16. - For the case of more than two TBs of NACK-only, SR is dropped.

From our perspective, the proposal is basically reasonable except for the last sub-bullet. As discussed in previous meeting, the NACK-only feedback can be transformed into ACK/NACK based feedback for more than 2 TBs if configured by RRC signaling. And it will use the PF 2/3/4 resource for more than two TB case. Therefore, it is no need to drop SR since the PF 2/3/4 can transit multiple bits. Thus, we suggest the proposal as following:

Proposal 7: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

  - Enable/disable HARQ-ACK feedback

Regarding enable/disable HARQ-ACK feedback mode for multicast, the following agreements were achieved in previous RAN1 meeting:

Agreement: Confirm the WA made in RAN1#106-e meeting regarding enabling/disabling HARQ-ACK feedback. Agreement (WA made in RAN1#106-e): Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows: Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback. - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling. - FFS details on RRC signaling and group-common DCI indicating. - FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases. - FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction. - FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission. - FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback. - FFS whether/how to apply it to SPS group-common PDSCH. - UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details. Note: It is up to network implementation to avoid any potential HARQ ACK mismatch between different UEs in the same multicast group.

Regarding whether/how to allow UE not to react to the DCI signaling, some companies proposed that some UEs within multicast group have low reliability without HARQ-ACK feedback and other UEs within the same multicast group have higher reliability with HARQ-ACK feedback. However, from our understanding, the HARQ feedback mechanism is the same for all UEs within in the same multicast group. Otherwise, the scheduling processing is more complicated and is not preferred. Besides, the service priority of the same group cast services is the same for the UEs within the same group. Therefore, it is better to define the same HARQ-ACK feedback behaviour for all the UEs in the same group.

Proposal 8: The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

As agreed, if RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback. In this scenario, whether enabling or disabling the feedback as the default mode is FFS. From our understanding, HARQ ACK is an enhancement for NR MBS, and no HARQ feedback in LTE SC-PTM. Thus, if the RRC signaling doesn’t configure the function, disabling the HARQ feedback can be regarded as a default mode. If the MBS services need the high reliability quality, the RRC signaling can enable the HARQ ACK feedback mechanism.

Proposal 9: Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

## Conclusion

In this contribution, it further discusses the multicast HARQ reliability issues for RRC_CONNECTED UEs with following proposals:

Proposal 1: Define a UE capability for UE to report whether to support Alt1(i.e., Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits) or Alt4(i.e., o	define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback) if more than one NACK-only feedback bits are transmitted in the same PUCCH slot.

Proposal 2: The PUCCH resource of NACK-only based feedback configured via RRC signalling is reused for the Alt 1, and the “last DCI” scheduling multicast is used to determine the PUCCH resource ID.

Proposal 3: For Alt4, the PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the pre-defined mapping relationship between number of TBs and PUCCH resource ID.

Proposal 4: Alt4 applies to the all G-RNTIs for supporting more than one NACK-only feedback in the same PUCCH transmission.

Proposal 5: For Type-1 codebook generation, all the configured G-RNTIs are with the same HARQ enabling/disabling behaviour via RRC signalling configuration, i.e., UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI.

Proposal 6: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, UE is not expected to be configured with G-RNTI with HARQ-ACK enabled (or disabled) by RRC signalling and G-CS-RNTI with HARQ-ACK disabled (or enabled) by RRC signalling.

Proposal 7: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

Proposal 8: The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

Proposal 9: Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

## Reference

- RAN1 Chairman’s Notes, RAN1 #108-e, March 2022.