3GPP TSG RAN WG1 #109-e		    R1-2204718

e-Meeting, May 9th – 20th, 2022

Source: 	MediaTek Inc.

Title:	Views on UE features for NR MBS

Agenda item:	8.16.12

Document for:	Discussion & Decision

## Introduction

According to the guidance of agreed timeline and planning in RAN#92-e [1], the discussion for Rel-17 UE features in RAN1 has been started. The UE features have been discussed in last several meetings, and the latest updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e can be found in [2]. In last RAN1 meeting [3], the MBS UE features were further discussed, and the corresponding conclusion can be found in [4]. In this contribution, we will further discuss the NR MBS UE features based on the agreements achieved in several last RAN1 meetings.

## Discussion

In the latest approved TS 38.212 spec, the three DCI formats were defined for MBS broadcast and multicast that DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively. In order to align the MBS DCI format naming, we suggest the following proposal:

Proposal 1: The MBS multicast broadcast DCI format naming in UE feature discussion should be aligned with that of latest approved TS 38.212 spec, i.e., MBS broadcast and multicast DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively.

Considering the concept of group-common PDCCH/PDSCH is used for broadcast and multicast and it only state “group-common PDCCH/PDSCH” in some FG, it may cause some reader confused, e.g., which means for broadcast or multicast? Therefore, we want to clarify the concept more clearly with some constrains, e.g., group-common PDCCH/PDSCH for multicast or broadcast, respectively.

Proposal 2: Clarify the wording of group-common PDCCH/PDSCH more clearly for broadcast and multicast respectively, e.g., group-common PDCCH/PDSCH for multicast or broadcast.

Regarding the discussion of which granularity type definition should be defined for MBS UE feature, from our view, the different carriers within the same band will have different characteristics or usage purpose, especially for considering the band n28 is a shared band by operators. What’s more, considering the IODT testing cost, we prefer the type definition for MBS UE features should be based on the granularity per FSPC for the remaining feature group.

Proposal 3: The type definition for MBS UE features should be based on the granularity per FSPC for the remaining feature group.

  - 33-1: Broadcast

Regarding the number of MCCH-RNTI and G-RNTI for broadcast, there has not been discussed for now and need to make it clearer. For the MCCH-RNTI, we think only one MCCH-RNTI is sufficient for UE obtaining the MBS control information as similar with the legacy LTE SC-PTM mechanism. For the G-RNTI issue, RAN2 has achieved an agreement that “one-to-many mapping between G-RNTI and MBS sessions is supported” in previous meeting, which means the multiple services if existed can use only for G-RNTI for services reception. In addition, MBS UE has introduced the multiple RNTIs for MBS feature (e.g., multiple multicast G-RNTIs/G-CS-RNTIs, broadcast MCCH-RNTI) however, a total number RNTI supported by UE is limited. Considering the above reason and broadcast is a best-effort reception, we suggest only one G-RNTI is supported for broadcast reception.

Proposal 4: For FG 33-1, adding a note that “For component 1, only one MCCH-RNTI is supported for broadcast”.

Proposal 5: For FG 33-1, adding a note that “For component 2, only one G-RNTI is supported for broadcast”.

Regarding the CFR number for broadcast reception, it is no clear motivation to support multiple CFR. We had agreed that the number of CFRs for multicast is no more than one, and the corresponding agreement is copied as following. We suggest the similar mechanism can be reused for broadcast reception since we also have agreed that the CFR frequency for MCCH and MTCH can be configured by SIBx.

Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17. Agreement For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs: - The CFR frequency resources used for MCCH and MTCH are configured by SIBx;

Proposal 6: For FG 33-1, adding a note that “For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx”.

Regarding slot-level repetition for MTCH, the following agreement was achieved in last meeting:

Agreement: The capability for support of dynamic slot-level repetition for MTCH is separated from FG 33-1. - maximum number of dynamic slot-level repetitions is 16 - FFS whether to merge with FG 33-3-1

Regarding the FFS whether to merge with FG 33-3-1, from our understanding, broadcast and multicast are two different FGs, what’s more, some detailed information has not decided yet for now, e.g., the whether the same or different reporting type for broadcast and multicast. So, at current stage, we slightly prefer using a separate FG to describe the capability for supporting of dynamic slot-level repetitio for MTCH.

Proposal 7: Adding a new separate FG 33-1-1 to capture the dynamic slot-level repetition for broadcast MTCH.

| 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH DCI indicated slot-level repetition up to 16 for MTCH is defined as another FG, FFS whether to merge with FG 33-3-1 |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN |  | For component 1, only one MCCH-RNTI is supported for broadcast For component 2, only one G-RNTI is supported for broadcast For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx | Up to RAN2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-1-1 | Dynamic slot-level repetition for broadcast MTCH | - Support up to X times dynamic slot-level repetition for broadcast MTCH. | 33-1 |  |  |  | Per FSPC |  |  |  | Candidate values for X is: {8, 16} | Optional with capability signalling |

  - 33-2: Dynamic scheduling for multicast

Regarding the FG 33-2, the following agreements were achieved in last meeting:

Agreement - FG 33-2c is merged into FG 33-2a and FG 33-2d is kept as follows 33. NR_MBS; 33-2a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast FFS shared/separate PUCCH resource configurations with/from unicast At least 33-2c or 33-2d is merged, FFS which one or both; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2c; PTM retransmission for multicast; Support of PTM retransmission for multicast FFS whether to merge with 33-2a; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2d; PTP retransmission for multicast; Support of PTP retransmission for multicast FFS whether to merge with 33-2a; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. Agreement: FG 33-2 is separated to two FGs: One for PCell case, the other for SCell case - Type of FG for PCell case is [per band or per FSPC] - Type of FG for SCell case is per FSPC - FG for PCell is added as a Prerequisite FG for FG for SCell

Regarding the multicast service reception on SCell, the following agreement was achieved in last meeting:

Agreement: If UE supports carrier aggregation for unicast, multicast reception on an activated SCell with self-scheduling is supported subject to UE capability in Rel-17. - UE is not expected to be configured simultaneously with more than one component carrier for multicast reception. - Cross-carrier scheduling for multicast reception is not supported in Rel-17. - The capability of supporting MBS multicast on SCell is a separate capability from the CA capability for unicast. - The granularity of UE reporting the capability of supporting MBS multicast reception is per FSPC

The part of the agreements has been reflected in the updated UE features after RAN1#108-e, however, some descriptions are not clear in [2]. For example, there is no any sentence to reflect this restriction that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception.” Thus, we suggest adding a note to reflect the agreement for both FG 33-2 and FG 33-2h.

Proposal 8: For FG 33-2, adding a note that “for component 2, up to one CFR is supported for multicast reception”.

Regarding the CFR number for multicast reception, the following agreement was achieved in previous RAN1 meeting:

Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17.

Thus, we prefer to update the 2nd component based on the latest agreement.

Proposal 9: For FG 33-2, adding a note that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception”.

Proposal 10: For FG 33-2h, adding a note that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception”.

Since more than one component carrier for multicast reception is not supported, it implies that the UE will not support the FG 33-2 and FG 33-2h simultaneously, so, the current prerequisite FG for the FG 33-2h is not correct, which is needed to be removed.

Proposal 11: For FG 33-2h, delating the prerequisite feature group description.

Regarding the reporting type for the FG 33-2, the issue was discussed in last meeting and needed to be further decided between per band or per FSPC in this meeting. As we agreed before, only one CC is used for multicast reception and the reporting type for SCell CC is per FSPC, from our understanding, the similar reporting type can be used for the PCell CC. Besides, considering band 28 is shared band by operator, which means that the entire band may not only be used for MBS, and it is not very clear which carrier will be used the MBS scheduling for now, so, the reporting type of FSPC is more flexible for MBS fast commercial deployment in current stage.

Proposal 12: The reporting type of FG 33-2 is per FSPC.

| 33-2 | Dynamic scheduling for multicast for PCell | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast | FFS | Yes |  |  | [Per band or per FSPC] Per FSPC | [N/A] | [N/A] |  | Note 1: UE is not expected to be configured simultaneously with more than one component carrier for multicast reception Note2: for component 2, up to one CFR is supported for multicast reception | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-2h | Dynamic scheduling for multicast for SCell | Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Note 1: UE is not expected to be configured simultaneously with more than one component carrier for multicast reception | Optional with capability signalling |

  - 33-2e: Multiple G-RNTIs for group-common PDSCHs

In the RAN2#116-e meeting, one-to-many mapping between G-RNTI and MBS sessions/services was agreed as following.

one-to-many mapping between G-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work.

Since the multiple MBS sessions associated with one G-RNTI is supported, the motivation for supporting multiple G-RNTIs for group-common PDSCH is not clear, and supporting multiple G-RNTIs will increase the probability of false alarm for PDCCH detection, which will degrade the system performance. In addition, MBS UE has introduced the multiple RNTIs for MBS feature (e.g., multicast G-RNTI/G-CS-RNTI, broadcast MCCH-RNTI/G-RNTI), however, a total number RNTI supported by UE is limited. Considering the reason above, we think up to 2 G-NRTIs are sufficient for Rel-17 UE supporting multicast services if FG 33-2e is supported.

Proposal 13: For FG 33-2e, adding a note that “For component 1, the candidate value for M is: {0, 1, 2}”.

| 33-2e | Multiple G-RNTIs for group-common PDSCHs | - Capability on number of Support up to M G-RNTI for multicast groupcast FFS details. | 33-2 | Yes |  |  | Per UE | No | No |  | For component 1, the candidate value for M is: {0, 1,2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 33-3-2: FDM-ed unicast PDSCH and group-common PDSCH

Regarding the prerequisite feature groups 33-1, we think the broadcast reception is “best-effort” reception and the UE in RRC IDLE/INACTIVE cannot report capability. So, we think FDMed reception for MBS is supported for multicast and not broadcast.

Proposal 14: For FG 33-3-2, the prerequisite feature groups 33-1 should be deleted.

| 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. | 33-1, 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 33-3-3: Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

Regarding the FG 33-3-3, we share the similar view as that of FG 33-3-2 and has the following proposal:

Proposal 15: For FG 33-3-3, the prerequisite feature groups 33-1 should be deleted.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH for multicast in a slot per CC - Support TDM among N (N>1) group-common PDSCHs for multicast in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs for multicast in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) for multicast are counted as unicast PDSCH(s). | 33-1, 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 33-5-1h: Multiple G-CS-RNTIs for SPS group-common PDSCHs

Regarding the multiple G-CS-RNTIs for SPS group-common PDSCHs, it is similar with FG 33-2e (i.g., multiple G-RNTIs for group-common PDSCHs). Based on the same reason, we suggest that up to 2 G-CS-RNTIs are supported for SPS scheduling if FG 33-5-1 is supported.

Proposal 16: For FG 33-2e, adding a note that “the candidate value for SPS multicast G-CS-RNTI is: {0, 1, 2}”.

Another remaining issue is that whether the multiple G-CS-RNTI is per slot per CC, actually, RAN2 has agreed that the reporting type is per UE in the approved TS 38.306 [5] as copied following.

RAN2’s agreements: maxNumberRNTIs-MBS-r17 Indicates the maximum number of simultaneous reception of PDCCH scrambled with G-RNTIs/G-CS-RNTIs for MBS multicast.; UE; No; No; No.

Thus, the wording of “per slot per CC” in the component should be deleted, and the reporting type should be updated based on the RAN2’s approved CR, e.g., per UE.

Proposal 17: For FG 33-2e, the wording of “per slot per CC” in the component should be deleted, and the reporting type should be updated based on the RAN2’s approved CR, e.g., per UE.

| 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast [per slot per CC] | 33-5-1 | Yes |  |  | Per UE | [No] | [No] |  | Note 1: The candidate value for SPS multicast G-CS-RNTI is: {0, 1, 2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Conclusion

In this contribution, it discusses the NR MBS UE features, and the following proposal are suggested:

Proposal 1: The MBS multicast broadcast DCI format naming in UE feature discussion should be aligned with that of latest approved TS 38.212 spec, i.e., MBS broadcast and multicast DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively.

Proposal 2: Clarify the wording of group-common PDCCH/PDSCH more clearly for broadcast and multicast respectively, e.g., group-common PDCCH/PDSCH for multicast or broadcast.

Proposal 3: The type definition for MBS UE features should be based on the granularity per FSPC for the remaining feature group.

### 33-1: Broadcast

Proposal 4: For FG 33-1, adding a note that “For component 1, only one MCCH-RNTI is supported for broadcast”.

Proposal 5: For FG 33-1, adding a note that “For component 2, only one G-RNTI is supported for broadcast”.

Proposal 6: For FG 33-1, adding a note that “For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx”.

Proposal 7: Adding a new separate FG 33-1-1 to capture the dynamic slot-level repetition for broadcast MTCH.

| 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH DCI indicated slot-level repetition up to 16 for MTCH is defined as another FG, FFS whether to merge with FG 33-3-1 |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN |  | For component 1, only one MCCH-RNTI is supported for broadcast For component 2, only one G-RNTI is supported for broadcast For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx | Up to RAN2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-1-1 | Dynamic slot-level repetition for broadcast MTCH | - Support up to X times dynamic slot-level repetition for broadcast MTCH. | 33-1 |  |  |  | Per FSPC |  |  |  | Candidate values for X is: {8, 16} | Optional with capability signalling |

### 33-2: Dynamic scheduling for multicast

Proposal 8: For FG 33-2, adding a note that “for component 2, up to one CFR is supported for multicast reception”.

Proposal 9: For FG 33-2, adding a note that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception”.

Proposal 10: For FG 33-2h, adding a note that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception”.

Proposal 11: For FG 33-2h, delating the prerequisite feature group description.

Proposal 12: The reporting type of FG 33-2 is per FSPC.

| 33-2 | Dynamic scheduling for multicast for PCell | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast | FFS | Yes |  |  | [Per band or per FSPC] Per FSPC | [N/A] | [N/A] |  | Note 1: UE is not expected to be configured simultaneously with more than one component carrier for multicast reception Note2: for component 2, up to one CFR is supported for multicast reception | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-2h | Dynamic scheduling for multicast for SCell | Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Note 1: UE is not expected to be configured simultaneously with more than one component carrier for multicast reception | Optional with capability signalling |

### 33-2e: Multiple G-RNTIs for group-common PDSCHs

Proposal 13: For FG 33-2e, adding a note that “For component 1, the candidate value for M is: {0, 1, 2}”.

| 33-2e | Multiple G-RNTIs for group-common PDSCHs | - Capability on number of Support up to M G-RNTI for multicast groupcast FFS details. | 33-2 | Yes |  |  | Per UE | No | No |  | For component 1, the candidate value for M is: {0, 1,2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 33-3-2: FDM-ed unicast PDSCH and group-common PDSCH

Proposal 14: For FG 33-3-2, the prerequisite feature groups 33-1 should be deleted.

| 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. | 33-1, 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 33-3-3: Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

Proposal 15: For FG 33-3-3, the prerequisite feature groups 33-1 should be deleted.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH for multicast in a slot per CC - Support TDM among N (N>1) group-common PDSCHs for multicast in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs for multicast in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) for multicast are counted as unicast PDSCH(s). | 33-1, 33-2 | Yes |  |  | Per UE Per FSPC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 33-5-1h: Multiple G-CS-RNTIs for SPS group-common PDSCHs

Proposal 16: For FG 33-2e, adding a note that “the candidate value for SPS multicast G-CS-RNTI is: {0, 1, 2}”.

Proposal 17: For FG 33-2e, the wording of “per slot per CC” in the component should be deleted, and the reporting type should be updated based on the RAN2’s approved CR, e.g., per UE.

| 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast [per slot per CC] | 33-5-1 | Yes |  |  | Per UE | [No] | [No] |  | Note 1: The candidate value for SPS multicast G-CS-RNTI is: {0, 1, 2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Reference

- RP-211582, “RAN Timeline and Meeting Planning”, RAN#92-e, RAN Chair, RAN1 Chair, RAN2 Chair, RAN3 Chair, RAN4 Chair, RAN5 Chair, June 14th -18th, 2021.

- R1-2202928, “Updated RAN1 UE features list for Rel-17 NR after RAN1 108-e”, RAN1#108-e, Moderators (AT&T, NTT DOCOMO, INC.), February 21st – March 3rd, 2022

- RAN1 Chairman’s Notes, RAN1 #108-e, March 2022.

- R1-2201520, “Summary on UE features for NR MBS”, RAN1#108-e, NTT DOCOMO, INC, February 21st – March 3rd, 2022

- R2-2203969, 38.306 CR for MBS UE capabilities, MediaTek, 21st February– 3rd March 2022