3GPP TSG RAN WG1 Meeting #109-e		R1-2204721

e-Meeting, May 9th – 20th, 2022

Agenda Item: 9.3.1

Source: MediaTek Inc.

Title: Deployment scenarios and evaluation methodology for NR duplex evolution

Document for: Discussion and Decision

# Introduction

A study item on evolution of NR duplex operation was approved in RAN#94e with the following objective ‎[1]:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss the deployment scenarios and evaluations methodology/assumptions for SBFD and DTDD, and we provide a set of proposals on how to evaluate the performance and feasibility of these schemes. In addition, we provide preliminary results for the performance of SBFD scheme.

# Interactions between RAN1 and RAN4 on the SID

There seems to be some potential overlap in RAN1 and RAN4 scope from the following SID objectives:

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

In Rel-16 RAN4 performed system level simulation analysis for adjacent channel scenarios and RAN1 for co-channel scenarios. However, the scenarios described below suggest that there is not such a clear partition between adjacent and co-channel operation at least for the UE side when considering SBFD, and actually the impacting requirements for CLI are all defined by RAN4.

We would therefore suggest that system level simulations are performed by RAN1, but the RF characterization is under full control and dependency on information from RAN4. This may need confirmation at RAN#96.

- RAN1 to work on the assumption that full SLS will be performed by RAN1 for SBFD, but dependent on information from RAN4 to characterise CLI modelling at UE Tx and Rx. Confirm the way of working between RAN1 and RAN4 with a SID revision at RAN#96.

# UE RF characterization aspects

In this section, we discuss the UE RF characterization that need to be considered for SBFD and DTDD feasibility and performance evaluations. It should be noted that similar aspects should be addressed for the gNB as well.

## Possible legacy UE configuration approaches for SBFD operation

In order to understand further the impact to the legacy UE from SBFD operation in the cell/network, some consideration of how the UE configured would be useful.

Several strategies may be adopted in the configuration of each legacy UE (per-UE):

- The DL and UL subbands used by UE1 (victim receiving DL) and UE2 (aggressor transmitting UL) respectively are narrower than the configured UE BWP and UE channel bandwidths configured for UE1 and UE2. This means that UE1 channel bandwidth and DL BWP may fully or partially overlap with UL sub-band used by UE2. UE2 channel bandwidth and UL BWP may fully or partially overlap with the DL sub-band received by UE1.

- Scheduler confines DL RBs to be received by UE1 and UL RBs to be transmitted by UE2 to be fully within their own sub-bands in the same slot, and therefore non-overlapping but possibly directly adjacent with no guard-band.

- UE channel bandwidth and BWPs are confined to the bandwidth of each DL and UL sub-band configured by the network.

Figure 1: a) UE channel and BWPs confined within sub-band b) BWPs overlap across sub-bands.

## UE channel BWs/BWPs overlap across sub-bands

In terms of inter-sub-band power emissions from UE2 transmitted sub-band into UE1 receive sub-band, the following would need to be considered:

- In-band Emissions (IBE) requirements from TS 38.101-1/2, in terms of General IBE, Carrier Leakage, and IQ images. These would be relevant not only for adjacent DL sub-bands but also any other DL sub-band within the UE2 UL channel bandwidth/BWP.

In addition, the ability of the reject a signal transmitted in another sub-band needs to be considered. From a receiver sub-band selectivity perspective, the following would need to be considered:

- Unlike the BS, there are NO “in-channel selectivity” RF requirements defined for the UE today in 38.101-1/2. However, in considering any UE receiver “in-channel” impact the following would need to be considered:

  - The impact to UE AGC operation due to large difference between UE2 Tx interferer power and UE1 Rx wanted signal power, and the impact of changes in interferer or interferer power over time on effective AGC operation in the UE.

The impact of sub-carrier interference due to any symbol-level timing misalignment between UE1 DL reception and UE2 UL transmission (as observed at UE1) with respect to CP duration, or due to UE frequency error (specified as 0.1ppm within TS 38.101-1/2).

- A model that is similar to in-channel selectivity need to be considered for the impact of inter-UE CLI on reception performance in the scenario when UE channel BWs/BWPs overlap across sub-bands. This model is within RAN4 expertise.

## UE channel BW/BWP confined within a sub-band

In terms of inter-sub-band power emissions from UE2 transmitted sub-band into UE1 receive sub-band, the following would need to be considered:

- ACLR, Spectrum Emission Mask, and Spurious Emissions into adjacent and other sub-bands within the network operated channel bandwidth, and to adjacent channels potentially operated by other operators. These requirements are all defined in TS 38.101-1/2.

- The impact of potential IQ images caused by RB transmissions within a UE channel bandwidth that are not centred around the LO frequency.

In addition, the ability to reject a signal transmitted in another sub-band needs to be considered. From a receiver sub-band selectivity perspective, the following would need to be considered:

- ACS, In-band blocking, Out-of-band blocking, all defined in TS 38.101-1/2.

- The impact from changes in interferer or interference power on effective AGC operation in the UE.

There is some implementation freedom in the 3GPP specifications as to the UE channel bandwidth required to be configured by the UE when it is in idle mode. Configuration of the UE channel bandwidth via dedicated-UE-channel bandwidths signaling once the UE moves into connected mode can remove this ambiguity. However, in idle mode it is of course possible that the SSB and common channels experience interference from an UL sub-band, so this needs to be taken into account. It is worth nothing that the UE channel bandwidth is limited to those defined in 38.101-1 and depends on the UE capability reporting.

Main challenge with this option for BWP configuration is that the mandatory UE features do not support reception (including CSI, SSB), UE-UE CLI measurements and transmission (including SRS) outside the BWP bandwidth, which makes such configuration not suitable for network operation for legacy UEs.

- The characteristics/requirements relating to Tx & Rx channel selectivity are within RAN4 expertise.

- Different RAN4 RF requirements exist for the different potential UE configuration approaches, and the different requirements may have different impacts on the ability of legacy UEs to be protected from interference in a SBFD operating scenario.

- RAN1 making “assumptions” on how these requirements should be characterized in the context of SBFD does not seem appropriate.

- RAN1 to liaise RAN4 to request feedback on the characterization of the relevant UE Tx/Rx requirements for the different UE configuration scenarios: 1) UE channel BWs/BWPs overlap across sub-bands, 2) UE channel BW/BWP confined within a sub-band.

## Possible questions to RAN4

The following questions are proposed to RAN4:

- Please can RAN4 provide input to enable RAN1 to characterize CLI (Cross-Link Interference) between UEs in system-level simulations for the following UE operating scenarios for FR1 and FR2:

  - UE Channel BW and BWP in DL or UL is not confined within a sub-band for SBFD operation.

  - UE channel BW and BWP in DL and UL are confined within a sub-band for SBFD operation.

Note: RAN4 should take into account that interference may vary across slots, and that Tx and Rx between UEs in different sub-bands may not be fully time-aligned.

- Does RAN4 see any RAN4 specific pros and cons with either configuration a or b?

- Include the above questions in the liaison to RAN4.

# Deployment scenarios and evaluation assumptions

The performance for SBFD and DTDD schemes can be conducted via several approaches, including system-level simulations, link-level simulations, and analysis. In the following, we provide some of the assumptions that could be considered in evaluating the SBFD and DTDD schemes.

## Deployment scenarios

The feasibility and the performance of SBFD and DTDD schemes will highly depend on the deployment scenarios where these features enabled. Thus, it is essential that RAN1 consider realistic deployment scenarios that helps in understanding:

- Under which conditions SBFD or DTDD is feasible.

- The impact to legacy operation (legacy operators & legacy UEs).

- The possible gains & drawbacks of SBFD or DTDD.

In the following, we provide a list of the deployment scenarios that should be considered in RAN1 for the evaluation of SBFD and DTDD schemes.

Scenarios for SBFD Scheme:

- Single-operator SBFD, single SBFD pattern: in this scenario, a single operator is considered and the SBFD configuration is aligned among all the cells belong to the operator. This scenario could represent a case where a specific NR band is owned by a single operator in a given geographical area.

- Single-operator SBFD, multiple SBFD patterns: in this scenario, a single operator is considered and different SBFD configurations are used among cells belong to the operator. The motivation for this scenario is when different SBFD configurations are needed due to different service requirements. As an example, one SBFD configuration can be used for the Macro/outdoor layer for smartphone services, while another SBFD configuration is used for small or indoor cells that is optimized for different type of services. This scenario will also serve to evaluate the co-existence with legacy TDD operation within the same operator.

- Two operators, 1st operator SBFD 2nd operator legacy TDD: in this scenario, one operator implements SBFD while the second operator uses legacy TDD on adjacent carrier. This main objective of this scenario is to evaluate the co-existence with legacy operator by evaluating any impact from operator-1 to (legacy) operator-2.

Scenarios for DTDD Scheme:

- Macro-Macro DTDD: two operators deploying Macro cells with different TDD patterns. Different grid shifts could be assumed between the two operators as explained in the following sections. The 0% grid shift could be the most challenging case for such deployment because of the high transmission power of the Macro cells and the proximity between the gNBs, which results in high inter-gNB CLI.

- Macro-SmallCell DTDD: The Macro cells of the two operators use aligned TDD patterns, while the small cells of one operator use a TDD pattern that is misaligned with the Macro cells. In this scenario, there is no inter-gNB CLI between the Macro cells (of the same or different operators). However, there will be inter-gNB CLI between Macro cells and small cells.

- Indoor-Indoor DTDD: two operators deploying indoor/small cells with different TDD patterns.

The deployment scenarios and the corresponding topologies are listed in Table 1 and Table 2.

Table 1: Deployment scenarios and topologies for SBFD.

| Scenario No. | Deployment Scenario | Topology |
| --- | --- | --- |
| 1 | Single-operator SBFD Same SBFD pattern in all cells | Single Layer: Urban, Dens Urban, Indoor |
| 2 | Single-operator SBFD Different SBFD patterns among cells | Single Layer: Urban, Dens Urban HetNet: Macro-Indoor |
| 3 | Two operators: 1st operator SBFD 2nd operator legacy TDD | Single Layer: Urban, Dens Urban, Indoor HetNet: Macro-Indoor Grid shift: 0% and 100% |

Table 2: Deployment scenarios and topologies for DTDD.

| Scenario No. | Operator#1 | Operator#2 | Notes |
| --- | --- | --- | --- |
| 1 | Macro | Macro | Grid shift: 0%, 100% |
| 2 | HetNet: Macro-Small | Macro | Macro cells: aligned TDD pattern Indoor cells: misaligned with Macro cells |
| 3 | Indoor | Indoor | Misaligned TDD patterns, Grid shift: between 0% and 100% |

- The feasibility and the performance of SBFD and DTDD schemes highly depend on the deployment scenarios considered for these schemes.

- For the evaluations of SBFD and DTDD schemes, RAN1 should consider the deployment scenarios listed in Table 1 and Table 2.

## UEs deployment

In the legacy TDD deployment, both UL and DL occurred during separate time slots hence there is no probability that a UE UL signal will cause interference with the DL signal of a neighbouring UE. However, with SBFD, both UL and DL occur during the same time slot at the gNB. The simultaneous DL and UL transmissions will introduce Inter-UE CLI as the UL transmission signal of one UE will negatively impact the reception of DL signal at another UE that is near it.

In conventional simulations, UEs are deployed in the simulation environment by being uniformly and randomly distributed. However, this results in a scenario whereby the separation distance between the UEs is relatively large. As an example, for a DU deployment scenario with 10 UEs per cell, there are no UEs with proximity to each other. For such scenario, the smallest separation distance between UEs was 5.5m. As shown in Figure 2 most of the inter-UE distances (~67.43%) lie between 40m to 70m (we ignore the distances between far UEs). Using the IMT2020 ChannelA_UMa pathloss model, this would result in a pathloss figure between the range of 75.3 dB to 80.6 dB. Consequently, under these conventional simulation settings, the effect of inter-UE CLI will not be captured as the separation distance between the UEs is too large to account for this.

Figure 2: UEs’ separation distance ranges of uniformly and randomly distributed UEs.

This uniform random distribution also does not depict a real-world scenario whereby users who may be in possession of one or more mobile devices congregate to form groups in settings such as in meeting rooms, restaurants and in public transportation. The separation distances in such settings are much smaller than those depicted in a uniformly and randomly distributed scenario. In order to account for this phenomenon within a simulation environment, a certain proportion of the UEs should be deployed to form groups/clusters.

Clustering is an important aspect which should be considered when evaluating SBFD as the simultaneous DL and UL transmissions taking place will cause a significant amount of interference especially when the UEs are located close to each other. This interference therefore needs to be accounted for in the simulations. This can be achieved by a subset of the UEs being uniformly and randomly distributed while the remaining UEs can be closely grouped together within a certain radius to form multiple UE clusters as shown in Figure 3.

Figure 3: Illustration of UEs clustering.

In order to demonstrate the effect clustering has on the overall system performance, the inter-UE CLI measurements from a uniformly and randomly distributed UEs were compared to a deployment with clustered UE (Figure 3). The results presented in Figure 4 represent the separation distance ranges of the clustered deployment and Figure 5 the inter-UE CLI CDFs for both clustered and uniformly and randomly distributed deployments.

Figure 4: UEs’ separation distance ranges with clustered UEs deployment.

Figure 5: CDFs of inter-UE CLI with and without UEs’ clustering.

From the two CDF plots, it is clear to see that the addition of the clusters causes the CLI to increase by values of up to 17.2 dBs in the upper percentiles. This increase in the CLI values can be mainly attributed to the reduction in the Euclidian distances between the UEs within the clusters. These results therefore underpin the need to have a clustered deployment scenario as it presents a better representation of the interference levels UEs will experience with SBFD being implemented.

- This uniform random distribution does not depict a real-world scenario whereby users congregate to form groups/clusters.

- The uniform random distribution results in separation distance between the UEs that is relatively large. Consequently, the effect of inter-UE CLI will not be captured in the evaluations.

- For the evaluations of SBFD and DTDD schemes, RAN1 should consider clustered UEs deployments to accurately capture the impact of inter-UE CLI.

## Grid shift between operators

For the deployment scenarios that assumes two operators, different grid shifts could be considered between the two operators. RAN1 could start with evaluating 0% and 100% grid shifts. For the 0% grid shift, some isolation or coupling loss may be considered between the co-located gNBs of the two operators. This to account for the fact that there is some separation between the two gNBs even if they deployed on the same location (e.g., same building rooftop, same mast).

For outdoor deployment, the 100% grid shift can be implemented by shifting gNBs of the second operator, relative to the gNBs’ locations of the first operator, by ±ISD/2 on one axis and ±ISD/(2*sqrt(3)) on the other axis, as illustrated in Figure 6. This will give the maximum distance between the two operators gNBs. For indoor deployment, multiple grid shifts between 0% and 100% can be assumed to reflect and uncoordinated deployments for such scenario.

Figure 6: Illustration of two operators deployment with 100% grid shift.

- For deployment scenarios with two operators, as starting point, 0% and 100% grid shift are assumed between the two operators’ gNBs.

- For the 100% grid shift in Macro deployment, gNBs of the second operator are shifted, relative to the gNBs’ locations of the first operator, by ±ISD/2 on one axis and ±ISD/(2*sqrt(3)) on the other axis.

# SBFD configurations

For SBFD, several configurations could be considered for the evaluations. For a given baseline TDD pattern, the gNB can introduce an UL subband in a DL slot. The UL subband could be in the middle of the channel bandwidth to provide extra protection from inter-gNB CLI for the UL transmissions, and inter-UE CLI for the DL transmissions on adjacent carriers/channels. Another possible configuration is that the UL subband located at the edge of the channel bandwidth. Such configurations are illustrated in Figure 7.

Figure 7: SBFD configurations with UL subbands in the centre or the edge channel bandwidth.

Another possible configuration is to have DL subband in UL slots. However, this could cause co-existence issues with legacy TDD operators due to the inter-gNB CLI. Unlike having UL subband in DL slot where the victim gNB/operator will be the one that deploy SBFD, for DL subband in the UL slots, the victim gNB will be the gNB/operator with legacy TDD operation. Also, as the objective of SBFD is to increase the UL opportunities rather than DL opportunities, there is no technical justification to have of DL subband in an UL slot. Hence, the case of having DL subband in an UL slot shouldn’t be considered for SBFD.

- For SBFD, UL subbands are assumed in DL slots. The UL subbands can be in the centre of the channel bandwidth or at the edge channel bandwidth.

- DL subbands in UL slots are not considered for SBFD scheme.

# Interference types in SBFD/DTDD

In DTDD/SBFD, on a given time slot the transmission directions of different gNBs and/or UEs may not be aligned. In the case of DTDD, this implies that different TDD patterns may be used at each gNB, whereas in the case of SBFD, subbands of the same slot could be assigned for UL and DL transmissions. Consequently, gNBs and UEs experience CLI and, for SBFD, there will self-interference (SI) at the gNB as illustrated in Figure 8.

Figure 8: Interferences in DTDD and SBFD scenarios.

## Interferences at the gNB

In DTDD/SBFD, the DL transmission from a gNB may cause severe interference to the UL reception of the same or neighbouring gNBs, i.e., DL-to-UL interference. Interference between gNBs may be high due to the high probability of line-of-sight between neighbouring gNBs. The different types of interference at the gNB are:

Inter-gNB adjacent channel CLI: In both DTDD and SBFD, inter-gNB adjacent channel CLI occurs when neighbouring gNBs operating on adjacent frequency bands uses different TDD configurations. Considering slot#1 in the SBFD patterns in Figure 9, the UL reception of gNB1 on frequency F1 may experience interference from the DL transmission of gNB5 on frequency F2.

Inter-gNB co-channel CLI: In the case of DTDD, inter-gNB co-channel CLI exists if neighbouring gNBs operating on the same frequency band use opposing transmission directions on the same time-frequency resources. This is illustrated in Figure 9 for the DTDD scenario, where the DL transmission of gNB3 interferes with the UL reception of gNB4.

Inter-gNB intra-subband CLI: In SBFD, inter-gNB intra-subband CLI occurs when gNBs operating on the same frequency band uses different SBFD/TDD configuration such that a gNB uses specific resources for UL while another gNB uses the same resources for DL (e.g., slots 1 in SBFD/TDD configuration for gNB2 and gNB3 illustrated in Figure 9). The UL reception of gNB2 on the UL subband of this slot will experience interference from the DL transmission of gNB3 on the same slot.

Inter-gNB inter-subband CLI: In SBFD, inter-gNB inter-subband CLI occurs between adjacent subbands as neighbouring gNBs operating on the same frequency band transmit and receive simultaneously (e.g., slots#1 for gNB1 and gNB2 illustrated in Figure 9). The transmission of gNB2 on the DL subbands of this slot will interfere with the UL reception of gNB1 on the UL subband of the same slot, and vice versa. This is caused by emissions/leakage outside the DL subbands.

Self-interference: In SBFD the DL transmission of a gNB will interfere with the UL reception of the same gNB (as illustrated in Figure 9 for gNB1).

Figure 9: Interference types at the gNB.

## Interferences at the UE

Like the interference at the gNB, the UL transmission of a UE may cause interference to the DL reception of UE in SBFD/DTDD, i.e., UL-to-DL interference. If two UEs are very close to each other, the interference from the UL transmission of the aggressor UE to the DL reception of the victim UE can be severe. The different types of interference at the UE are:

Inter-UE adjacent channel CLI: In both DTDD and SBFD, inter-UE adjacent channel CLI occurs when nearby UEs operating on adjacent frequency bands transmit and receive simultaneously. Considering the SBFD/TDD patterns in Figure 10, the UL transmission of a UE1 on frequency F1 may cause interference to the DL reception of a UE5 on frequency F2.

Inter-UE co-channel CLI: In the case of DTDD, inter-UE co-channel CLI exists if nearby UEs operating on the same frequency band are scheduled on different transmission directions on the same time-frequency resources. This is shown in Figure 10 for the DTDD scenario, where the UL transmission of UE4 interferes with the DL reception of UE3.

Inter-UE intra-subband CLI: In SBFD, inter-UE intra-subband CLI occurs between overlapping subbands when a UE receives in the DL while a nearby UE operating in the same frequency band transmits in the UL on the same time-frequency resources (as illustrated in Figure 10 for the CLI between UE1 and UE3). This interference can only occur in inter-cell scenarios.

Inter-UE inter-subband CLI: In SBFD, inter-UE inter-subband CLI occurs between adjacent subbands when a UE receives in a DL subband while a nearby UE operating in the same frequency band transmits in nonoverlapping UL subband at the same time (as illustrated in Figure 10 for the CLI between UE1 and UE2). This is caused by emissions/leakage outside the UL subbands and can occur as both inter-cell and intra-cell interference.

Figure 10: Interference types at the UE.

- For DTDD and SBFD evaluations, RAN1 considers the following interference types:

- At the gNB: Inter-gNB, adjacent channel CLI, Inter-gNB co-channel CLI, Inter-gNB intra-subband CLI, Inter-gNB inter-subband CLI, and Self-interference.

- At the UE: Inter-UE adjacent channel CLI, Inter-UE co-channel CLI, Inter-UE intra-subband CLI, Inter-UE inter-subband CLI.

# Initial evaluation results

In this section, we provide some initial results for SBFD scheme based on some preliminary assumptions, focusing on the impact of inter-gNB and inter-UE CLIs on the system performance. We consider a basic scenario with synchronized SBFD and TDD patterns among cells. Three cases are considered for our initial evaluation:

- Fixed TDD patterns without SBFD slots.

- Fixed TDD patterns where all downlink slots are SBFD slots; assume there is no CLI.

- Fixed TDD patterns where all downlink slots are SBFD slots; impact of CLI is captured according to the presented inter-subband interference models.

Following TDD and SBFD pattern is assumed in this work:

In SBFD slots, UL PRBs are the 16 PRBs around center carrier frequency. Guard band size between DL PRBs and UL PRBs is 2 PRBs. The rest of parameter settings mainly follow IMT-2020 simulation assumptions and are concluded in Table 3 in the Appendix.

The following assumptions are adopted for inter-gNB CLI modeling:

- Inter-gNB CLI = 45 dB. For simplicity, we assume CBW is the same as all the DL allocated PRBs.

- Inter-UE CLI is modeled based on the UE in-band emissions [TS 38.101-1].

- For DU scenario, we assume sectors located at the same site are perfectly isolated so that no inter-sector CLI among cells at the same site.

We used this subband leakage modelling as a starting point. However, this modelling approach needs verification according to the operating scenario assumed (Section ‎3 above on RF modelling).

The self-interference is not modeled in these simulations. The evaluations will be updated once RAN4/RAN1 adopt more accurate interference models.

For a DL Rx-UE, received interference consists of 1) co-channel interference (CCI) from base-stations in neighboring cells and 2) inter-SB CLI from UL UEs in serving and neighboring cells. In UL direction, received interference consists of 1) inter-SB CLI from gNBs in neighboring cells and 2) CCI from UEs served in the neighboring cells.

## Evaluation results for inter-gNB CLI

Figure 11 presents SLS evaluation UL spectral efficiency for dense-urban and InH scenarios. As it can be seen from the figure, the inter-gNB CLI causes about 15% reduction in UL spectral efficiency in DU deployment. The distribution of CCI and CLI in the dense-urban scenario and the InH scenario are also analyzed in Figure 12. Negative values indicate that the UE observes inter-gNB CLI higher than the co-channel interference (CCI) from other UEs. Overall, as shown from in the results, inter-gNB CLI leads to significant impact to system performance in UL direction. Also, it is expected that once self-interference and inter-sector interference are considered, the UL performance will be further degraded.

In summary, the inter-subband CLI from neighboring gNBs is very critical, especially for high Tx power gNB. To reduce the CLI impact from base stations, SBFD may be more suitable for small cells where less Tx power is required.

- The inter-gNB CLI degrades the UL performance, where about 15% SE loss is observed in FR1 dense urban scenario. Less impact from inter-gNB CLI can be observed for indoor hotspot scenario.

Figure 11: Average UL spetral efficency for the conisdered cases: 1) Legacy TDD, 2) SBFD without CLI, 3) SBFD with CLI.

Figure 12: Distribution of the difference between CCI and inter-gNB CLI (for DU and InH).

## Evaluation results for inter-UE CLI

In this section, initial results will be presented for impact of inter-UE CLI on the DL performance. To evaluate the impact of inter-UE CLI on the DL performance, the concept of UEs clustering is used (see Section ‎4.2). For a dense urban deployment, subset of the UEs are grouped to form a cluster, and one cluster of UEs is assumed in each sector. Figure 13 shows the DL spectral efficiency for the DL. As expected, the DL SE is impacted by SBFD due to allocating some of the resources to UL. Also, it can be observed that there is about 4% reduction in the DL SE due to the inter-UE CLI.

- Inter-UE CLI has negative impact to the DL performance.

Figure 13: Average DL spetral efficency for the conisdered cases: 1) Legacy TDD, 2) SBFD without CLI, 3) SBFD with CLI.

# Conclusion

In this contribution, we discussed the deployment scenarios and evaluations methodology/assumptions for SBFD and DTDD, and presented preliminary results for the performance of SBFD scheme. We have the following observations and proposals:

- A model that is similar to in-channel selectivity need to be considered for the impact of inter-UE CLI on reception performance in the scenario when UE channel BWs/BWPs overlap across sub-bands. This model is within RAN4 expertise.

- The characteristics/requirements relating to Tx & Rx channel selectivity are within RAN4 expertise.

- Different RAN4 RF requirements exist for the different potential UE configuration approaches, and the different requirements may have different impacts on the ability of legacy UEs to be protected from interference in a SBFD operating scenario.

- RAN1 making “assumptions” on how these requirements should be characterized in the context of SBFD does not seem appropriate.

- The feasibility and the performance of SBFD and DTDD schemes highly depend on the deployment scenarios considered for these schemes.

- This uniform random distribution does not depict a real-world scenario whereby users congregate to form groups/clusters.

- The uniform random distribution results in separation distance between the UEs that is relatively large. Consequently, the effect of inter-UE CLI will not be captured in the evaluations.

- The inter-gNB CLI degrades the UL performance, where about 15% SE loss is observed in FR1 dense urban scenario. Less impact from inter-gNB CLI can be observed for indoor hotspot scenario.

- Inter-UE CLI has negative impact to the DL performance.

- RAN1 to work on the assumption that full SLS will be performed by RAN1 for SBFD, but dependent on information from RAN4 to characterise CLI modelling at UE Tx and Rx. Confirm the way of working between RAN1 and RAN4 with a SID revision at RAN#96.

- RAN1 to liaise RAN4 to request feedback on the characterization of the relevant UE Tx/Rx requirements for the different UE configuration scenarios: 1) UE channel BWs/BWPs overlap across sub-bands, 2) UE channel BW/BWP confined within a sub-band.

- Include the following questions in the liaison to RAN4:

- Please can RAN4 provide input to enable RAN1 to characterize CLI (Cross-Link Interference) between UEs in system-level simulations for the following UE operating scenarios for FR1 and FR2:

    - UE Channel BW and BWP in DL or UL is not confined within a sub-band for SBFD operation.

    - UE channel BW and BWP in DL and UL are confined within a sub-band for SBFD operation.

Note: RAN4 should take into account that interference may vary across slots, and that Tx and Rx between UEs in different sub-bands may not be fully time-aligned.

- Does RAN4 see any RAN4 specific pros and cons with either configuration a or b?

- For the evaluations of SBFD and DTDD schemes, RAN1 should consider the deployment scenarios listed in Table 1 and Table 2.

- For the evaluations of SBFD and DTDD schemes, RAN1 should consider clustered UEs deployments to accurately capture the impact of inter-UE CLI.

- For deployment scenarios with two operators, as starting point, 0% and 100% grid shift are assumed between the two operators’ gNBs:

- For the 100% grid shift in Macro deployment, gNBs of the second operator are shifted, relative to the gNBs’ locations of the first operator, by ±ISD/2 on one axis and ±ISD/(2*sqrt(3)) on the other axis.

- For SBFD, UL subbands are assumed in DL slots. The UL subbands can be in the centre of the channel bandwidth or at the edge channel bandwidth.

- DL subbands in UL slots are not considered for SBFD scheme.

- For DTDD and SBFD evaluations, RAN1 considers the following interference types:

- At the gNB: Inter-gNB, adjacent channel CLI, Inter-gNB co-channel CLI, Inter-gNB intra-subband CLI, Inter-gNB inter-subband CLI, and Self-interference.

- At the UE: Inter-UE adjacent channel CLI, Inter-UE co-channel CLI, Inter-UE intra-subband CLI, Inter-UE inter-subband CLI.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC, RAN#94e, December 2021.

# Appendix: Simulation assumptions in SLS evaluation

Table 3: Simulation assumptions in SLS evaluation.

| Simulation assumptions |  |  |  |
| --- | --- | --- | --- |
| Scenario |  | FR1 Dense Urban | FR1 Indoor HotSpot |
| Evaluation configuration |  | Configuration A | Configuration A |
| Channel model |  | UMa_A | InH_A |
| ISD |  | 200 m | 20 m (12TRxP) |
| Traffic Type |  | Full buffer | Full buffer |
| Carrier Frequency |  | 4 GHz | 4 GHz |
| System bandwidth |  | 10MHz | 10MHz |
| Subcarrier spacing |  | 15 KHz | 15 KHz |
| Symbols number per slot |  | 14 | 14 |
| TDD Pattern |  | DDDSUDDSUU | DDDSUDDSUU |
| Number of antenna elements per TRxP |  | 128Tx cross-polarized antenna (M,N,P,Mg,Ng,Mp,Np) = (8,8,2,1,1,2,8) | 32Tx cross-polarized antenna (M,N,P,Mg,Ng,Mp,Np) = (4,4,2,1,1,4,4) |
| Number of TXRU per TRxP |  | 32TXRU | 32TXRU |
| Number of antenna elements per UE |  | 4Rx 0°,90° polarization antenna (M,N,P,Mg,Ng,Mp,Np) = (1,2,2,1,1,1,2) 2Tx 0°,90° polarization antenna (M,N,P,Mg,Ng,Mp,Np) = (1,1,2,1,1,1,1) | 4Rx 0°,90° polarization antenna (M,N,P,Mg,Ng,Mp,Np) = (1,2,2,1,1,1,2) 2Tx 0°,90° polarization antenna (M,N,P,Mg,Ng,Mp,Np) = (1,1,2,1,1,1,1) |
| Number of TXRU per UE |  | 4TXRU and 2TXRU for DL and UL | 4TXRU and 2TXRU for DL and UL |
| Transmit power per TRxP |  | 41dBm | 21dBm |
| UE max Tx power |  | 23dBm | 23dBm |
| TRxP number per site |  | 3 | 1 |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) | 180deg in GCS (pointing to the ground) |
| Electronic tilt |  | 105deg in LCS | 90deg in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-PF in DL and SU-PF in UL | MU-PF in DL and SU-PF in UL |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | MU-MIMO with max rank 2 adaptation per user, Maximum MU layer = 12 SU-MIMO with max rank 2 adaptation per user | MU-MIMO with max rank 2 adaptation per user, Maximum MU layer = 12 SU-MIMO with max rank 2 adaptation per user |
| Receiver type |  | MMSE-IRC | MMSE-IRC |
| CSI feedback |  | 5 slots period based on CSI-RS with delay | 5 slots period based on CSI-RS with delay |
| Precoder derivation |  | NR Type II codebook based | NR Type II codebook based |
| Power control |  | α= 0.6, P0 = -60dBm | α= 0.9, P0 = -86dBm |
| Power backoff model |  | Continuous RB allocation: follow TS 38.101; Non-continuous RB allocation: additional 2 RB reduction | Continuous RB allocation: follow TS 38.101; Non-continuous RB allocation: additional 2 RB reduction |
| Overhead (DL) | PDCCH | 2 complete symbols | 2 complete symbols |
|  | DMRS | Type II, based on MU-layer (dynamic in simulation) | Type II, based on MU-layer (dynamic in simulation) |
|  | CSI-RS | 32 ports per 5 slots | 32 ports per 5 slots |
|  | SSB | 1 SSB per 20 ms | 1 SSB per 20 ms |
|  | TRS | 2 consecutive slots per 20ms, 1 port, 50PRB | 2 consecutive slots per 20ms, 1 port, 50PRB |
| Overhead (UL) | PUCCH | 2 RBs, 14 OFDM symbols | 2 RBs, 14 OFDM symbols |
|  | DMRS | Type II, 1 additional DMRS symbol, and no FDM with PUSCH | Type II, 1 additional DMRS symbol, and no FDM with PUSCH |
|  | SRS | 2 symbols per 5 slots, 8 RBs per symbol | 2 symbols per 5 slots, 8 RBs per symbol |
| Channel estimation |  | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM |