3GPP TSG RAN WG1 Meeting #109-e		R1-2204722

e-Meeting, May 9th – 20th, 2022

Agenda Item: 9.3.2

Source: MediaTek Inc.

Title: Discussion on subband non-overlapping full duplex for NR

Document for: Discussion and Decision

# Introduction

A study item on evolution of NR duplex operation was approved in RAN#94e with the following objective [1]:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping - full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss the relevant sub-band layouts and SBFD specific issues that arise in addition to the ones that are common with DTDD ([2]), and provide a set of proposals on UE configuration and UE-UE CLI handling.

# Motivation and scope for SBFD operation

Current TDD systems have the following limitations:

- Latency – due to TDD alignment delay

- Capacity – due to the rigid UL-DL split limiting uplink capacity; and latency potentially preventing retransmission for low latency, high data rate (DL or UL) traffic, such as XR

- UL Coverage extension by repetition is limited by the scarcity of uplink slots

Dynamic TDD partially offers some flexibility to overcome these but:

- does not offer the same practical time-domain availability of uplink resources – impact on user experience KPIs from this limitation should be studied further

- inter-operator coexistence issues may exist between non-aligned TDD deployments on adjacent channels

SBFD may further support improvements to user experience by:

- providing more UL resources in time-domain

- potentially limiting adjacent channel interferences by aligning link directions (per slot) near the channel edge, whilst allowing non-alignment away from the channel edge (Figure 1)

Figure 1: Interferences with UL sub-bands in DL slots in example sub-band layout

In accordance with these motivations, only UL sub-bands in DL slots should be considered. These increase the number of slots with UL transmission opportunity while do not produce inter-operator BS-BS adjacent channel interference. Inter-operator UE-UE cross-link interference caused to the adjacent cell as well as BS-BS CLI suffered from the adjacent cell could be mitigated by the possibility to enable “additional frequency guard band” to the BS channel edge. Sub-band layouts with the UL sub-band at the centre of the network channel and on the edge can be both considered.

- SBFD may further support improvements to user experience beyond dynamic TDD by:

- providing additional practical UL resources in time-domain

- potentially limiting adjacent channel interferences by aligning link directions (per slot) near the channel edge, whilst allowing non-alignment away from the channel edge

- For SBFD, UL sub-bands are assumed in DL slots. Sub-band layouts with the UL sub-band at the centre of the network channel and on the edge can be both considered.

- DL sub-bands in UL slots are not considered for SBFD scheme.

# UE-UE CLI handling schemes

## Pros and Cons of different UE configurations

We consider the following UE bandwidth configuration approaches (Figure 2):

- The DL and UL sub-bands used by UE1 (victim receiving DL) and UE2 (aggressor transmitting UL) respectively are narrower than the configured UE BWP and UE channel bandwidths configured for UE1 and UE2. This means that UE1 channel bandwidth and DL BWP may fully or partially overlap with UL sub-band used by UE2. UE2 channel bandwidth and UL BWP may fully or partially overlap with the DL sub-band received by UE1.

- Scheduler confines DL RBs to be received by UE1 and UL RBs to be transmitted by UE2 to be fully within their own sub-bands in the same slot, and therefore non-overlapping but possibly directly adjacent with no guard-band.

- UE channel bandwidth and BWPs are confined to the bandwidth of each DL and UL sub-band configured by the network.

Figure 2 a) BWPs overlap across sub-bands b) UE channel bandwidths and BWPs confined within sub-bands

Option-1: BWP overlapping across sub-bands

Benefits:

A single BWP-pair can be used to schedule flexibly and efficiently over multiple sub-bands. PUCCH and PUSCH frequency hopping offset can span the whole channel. Resource overhead from SSB is minimized.

Drawbacks:

Potential for more UL-DL interference issues (see [3] on RF modelling). The interference issue may be lower in denser deployment scenarios – due to lower UE Tx power, higher DL-DL inter-cell interference and smaller UE TA and delay spreads.

As a countermeasure for issues with idle mode camping and initial access in deployment scenarios (e.g. macro and maybe others) of high UL-DL interferences, protection of common channels should be considered, e.g. by not placing them in dynamic or partitioned slots. However, maintaining some non-SBFD slots may limit the potential benefits of SBFD due to limiting the additional amount of possible UL resource.

Option-2: UE channel bandwidths and BWPs confined within sub-bands

According to current specifications, per each numerology and per uplink or downlink a single UE channel bandwidth and location can be configured per serving cell. Following initial access, the UE channel can be reconfigured using the fields downlinkChannelBW-PerSCS-List and uplinkChannelBW-PerSCS-List in dedicated signalling.

Benefits: Inter-sub-band interferences are controlled by well-defined RF requirements. (see [3] on RF modelling)

Drawbacks: Each DL sub-band needs to be configured with SSB according to mandatory UE capability FG6.1. Potential inefficiencies of scheduling.

- The study of SBFD operation should consider the following two UE configuration options:

- BWP overlapping across sub-bands

- UE channel bandwidths and BWPs confined within sub-bands

## Identification of nearby potential interferers

CLI-RSSI and SRS-RSRP measurements

Release-16 supports CLI-RSSI and SRS-RSRP UE measurements and reporting to enable UE-UE CLI aware scheduling in the BS.

CLI-RSSI is simply an RSSI measurement over selected UE configurable frequency-time resources of the radio frame, which can characterize the total amount of interference power without making distinction between its sources or components. The dominant source of significant levels of interference may potentially be identified by the scheduler by comparing the results of multiple measurements and the information on the set of scheduled UEs. On the downside, CLI-RSSI measurements are not selective with respect to a specific UE aggressor; on the positive side, they can be performed without extra network resources (reference signals).

An SRS-RSRP measurement, on the other hand, is selective and measures the signal from a particular aggressor configured with the same SRS resource as the one configured in the measurement object of the victim performing the measurement. Although SRS-RSRP requires the transmission of SRS resources, however, multiple SRS transmissions and measurements can be frequency multiplexed and both used in channel- and CLI measurement simultaneously. Since SRS-RSRP characterizes a UE-UE pair, the reported value can be used in two scenarios with reciprocal roles of victim and aggressor between these two UEs.

- Since SRS-RSRP characterizes the coupling between a UE-UE pair, the reported value can be used in two scenarios with reciprocal roles of victim and aggressor between these two UEs.

UE-UE CLI aware scheduling and coordination

If the network is aware of which UEs are close to each other (strongly coupled) and not close to each other then it may optimally allocate sub-bands to avoid UE-to-UE interference. This applies to intra-site and inter-site scenarios.

For legacy UEs not supporting CLI measurements from Rel-16 it may be a challenge for them to help with this on their own. However, considering a mix of UEs with and without this measurement capability, the former ones (i.e., those having the capability) can measure the SRS-RSRP metric to any other UE(s) that the scheduler deems relevant. As a result, their distance / coupling to other UEs can be established and used in the reciprocal scenario where they act as transmitters. Because the level of interference is under the scheduler’s control when a UE having CLI-measurement capability is transmitting, the performance of legacy UEs does not get jeopardized inadvertently.

The same cannot be said about the scenario where two legacy UEs without CLI-measurement capability are scheduled as aggressor-victim pairs in adjacent sub-bands of the same slot: their distance (coupling) is unknown. Therefore, the safe solution is to schedule them in separate slots. However, in scenarios, where the UE distribution can be modelled by UE clusters [3], a single member of such a cluster supporting CLI measurement may be able to identify other members of the cluster and represent the cluster in UE-UE CLI measurement with respect to measuring other UEs.

- When a UE having CLI-measurement capability is transmitting, the performance of legacy UEs does not get jeopardized inadvertently, because the scheduler is able to gather respective SRS-RSRP data from the aggressor beforehand.

- In scenarios, where the UE distribution can be modelled by UE clusters [3], a single member of such a cluster supporting CLI measurement may be able to identify other members of the cluster and represent the cluster in UE-UE CLI measurement with respect to measuring other UEs.

- Assess the ability for the network to be aware of the proximity between “legacy” UE1 and “CLI-measurement capable” UE2, in order to minimise inter-UE interference from UE2 to UE1.

# Conclusions

In this contribution, we discussed the relevant sub-band layouts and SBFD specific issues that arise in addition to the ones that are common with DTDD ([2]), and provided a set of proposals on UE configuration and UE-UE CLI handling; and we have the following observations and proposals:

- SBFD may further support improvements to user experience beyond dynamic TDD by:

- providing additional practical UL resources in time-domain

- potentially limiting adjacent channel interferences by aligning link directions (per slot) near the channel edge, whilst allowing non-alignment away from the channel edge

- For SBFD, UL sub-bands are assumed in DL slots. Sub-band layouts with the UL sub-band at the centre of the network channel and on the edge can be both considered.

- DL sub-bands in UL slots are not considered for SBFD scheme.

- The study of SBFD operation should consider the following two UE configuration options:

- BWP overlapping across sub-bands

- UE channel bandwidths and BWPs confined within sub-bands

- Since SRS-RSRP characterizes the coupling between a UE-UE pair, the reported value can be used in two scenarios with reciprocal roles of victim and aggressor between these two UEs.

- When a UE having CLI-measurement capability is transmitting, the performance of legacy UEs does not get jeopardized inadvertently, because the scheduler is able to gather respective SRS-RSRP data from the aggressor beforehand.

- In scenarios, where the UE distribution can be modelled by UE clusters [3], a single member of such a cluster supporting CLI measurement may be able to identify other members of the cluster and represent the cluster in UE-UE CLI measurement with respect to measuring other UEs.

- Assess the ability for the network to be aware of the proximity between “legacy” UE1 and “CLI-measurement capable” UE2, in order to minimise inter-UE interference from UE2 to UE1.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC, RAN#94e, December 2021.

- R1-2204723, “Discussion on potential enhancements on dynamic/flexible TDD”, Mediatek, RAN1#109, May 2022.

- R1-2204721, Deployment scenarios and evaluation methodology for NR duplex evolution”, Mediatek, RAN1#109, May 2022.