3GPP TSG RAN WG1 #109-e																			R1-2204729

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.16.8

Source: 	MediaTek Inc.

Title: 	Discussion on UE features for NR coverage enhancement

Document for:	Discussion and Decision

# Introduction

This document is to discuss the questions in FL summary for discussion in RAN1-108_e [1].

# Overview on FGs for Coverage Enhancements

Considering the test cost and the applicability/support of the band (e.g., licensed band and unlicensed band), per band support is more preferred for all features.

Proposal 1: All features are per band.

All features of coverage enhancements are to improve performance rather than the basic features to build a new function. All features should be optional with capability signalling.

Proposal 2: All UE features are optional with capability signalling

In case UE indicates the support of feature A and feature B, it is unclear whether UE should automatically support feature A+B. So it is suggested to have the following conclusion, i.e., by default UE is not expected to support feature A+B if feature A and feature B are indicated as supported.

Proposal 3: By default, UE is not expected to support the combined feature A+B if feature A and feature B are indicated and supported separately.

For 30-4h, the yellow part seems stable without any comments in the last meeting. So it can be confirmed if there is no more concern.

| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) | 30-4a, 30-4b, 30-4c, or 30-4d |
| --- | --- | --- | --- | --- |

Proposal 4: Confirm the components and perquisite FGs for 30-4h.

# References

[1] R1-2201517, Summary on UE features for NR coverage enhancement, Moderator (NTT DOCOMO, INC.)