3GPP TSG RAN WG1 #109-e			R1-2204736

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.11.1

Source:	Ericsson

Title:	Remaining open issues on resource allocation procedures for power saving

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss and provide text proposal on the remaining open aspects and critical issues related to the power saving mechanism in resource allocation for SL including corrections to the spefication text.

# 2	Text Proposals related to resource allocation procedure for power saving UEs

## 2.1	Resource Selection procedures in a resource pool

During RAN1#103-e, the following resource allocation mechanisms were agreed to be possible in a resource pool based on (pre-)configuration:

Agreements: - In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

Based on this agreement, the following text has been included in TS 38.214 to cover all the different possible pre-configured resource allocation mechanisms in a resource pool.

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> Optionally, the indication of resource selection mechanism(s), as allowedResourceSelectionConfig, which may comprise of full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof. <Unchanged parts omitted>

However, based on the text included in the specification any of the following configurations could be possible for a resource pool:

- Resource pool configuration: {full sensing only and partial sensing only}

- Resource pool configuration: {full sensing only, partial sensing only and random resource selection only}

- Resource pool configuration: {full sensing only and random resource selection only}

- Resource pool configuration: {partial sensing only and random resource selection only}

In our view, it is clear that having a resource pool configuration with any of the previous configuration leads to an uncertain behaviour due to the inclusion of the word “only” for each of the possible resource selection procedures. Therefore, we propose the following text proposal in order to align the specification with the intention of the agreement reached in RAN1#103-e and avoid any misleading configuration of the resource pool.

### 2.1.1 Text proposal

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> Optionally, the indication of resource selection mechanism(s), as allowedResourceSelectionConfig, which may comprise of full sensing, partial sensing, random resource selection, or any combination(s) thereof, including allowing a single resource allocation mechanism. <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.214, 8.1.4]

- Reason for change: the text is not aligned with the intention of the RAN1 agreement.

- Summary of change: remove the wording “only” on each of the resource selection mechanism to avoid misconfiguration of the resource pool. Include wording to allow a single resource selection mechanism to be configured per resource pool.

- Consequences if not approved: the resource pool configuration is not correct and leads to imprecise implementation of the resource pool configuration.

## 2.2	Contiguous sensing window definition

The following text has been included in TS 38.214 for the definition of the contiguous partial sensing for both periodic and aperiodic transmissions:

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> Optionally, indication of the size in logical slots of contiguous partial sensing window as contiguousSensingWindowPeriodic. Optionally, indication of the size in logical slots of contiguous partial sensing window as contiguousSensingWindowAperiodic. <Unchanged parts omitted>

In our view, the text is not clear about the situation where either one contiguous sensing window or the other, i.e., the periodic or the aperiodic version, is used. Therefore, we propose the following modification to make the text clearer.

### 2.2.1 Text proposal

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> Optionally, indication of the size in logical slots of contiguous partial sensing window for periodic transmissions as defined by the parameter contiguousSensingWindowPeriodic. Optionally, indication of the size in logical slots of contiguous partial sensing window for aperiodic transmissions as defined by the parameter contiguousSensingWindowAperiodic. <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.214, 8.1.4]

- Reason for change: the text is not clear under which circumstances each of the parameters, i.e., contiguousSensingWindowAperiodic and contiguousSensingWindowPeriodic, is used.

- Summary of change: Include text to indicate the type of transmission, i.e., periodic or aperiodic, which triggers the used of each of the associated parameters for the contiguous sensing window.

- Consequences if not approved: the size and range of the contiguous sensing window for each of the different transmission types is not correct.

## 2.3	Definition of sl-ResourceReservedPeriodList

The parameter sl-ResourceReservedPeriodList is included as one of the parameters provided by higher layers which is used in the resource allocation procedure for mode 2. However, in the current version of the specification there is no definition of the value and/or intention of this parameters as shown in the following text:

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> - sl-RS-ForSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1. - sl-ResourceReservePeriodList - sl-SensingWindow: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindow msec <Unchanged parts omitted>

Therefore, we propose to include the definition agreed in RAN1 for the possible values of the parameter sl-ResourceReservedPeriodList as follows:

### 2.3.1 Text proposal

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> - sl-RS-ForSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1. - sl-ResourceReservePeriodList is the configured set of possible resource reservation periods allowed in the resource pool. - sl-SensingWindow: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindow msec <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.214, 8.1.4]

- Reason for change: the definition of the parameter sl-ResourceReservePeriodList is missing in the specification

- Summary of change: Include the definition of the parameter sl-ResourceReservePeriodList as per RAN1 agreements.

- Consequences if not approved: missing definition of a relevant parameter for resource allocation procedure.

## 2.4	Minimum number of slots in the candidate resource set

The following text in TS 38.214 indicates the minimum number of slots which should be reported to higher layer within the candidate resource set so that higher layers can select a resource among them. However, in our view the current text in the specification is not clear and misses a critical word, i.e., candidate, when defining the resources where the minimum number of slots can be reported.

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> - Optionally, minimum number of Y slots as Y_min (minNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the resources corresponding to periodic-based partial sensing. - Optionally, minimum number of Y' slots as Y'min (minNumCandidaateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the resources corresponding to contiguous partial sensing. <Unchanged parts omitted>

Therefore, we propose the following text proposal in order to align the specification with the intended RAN1 agreements and terminology in addition to give more clarity to the specification text.

### 2.4.1 Text proposal

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> - Optionally, minimum number of Y slots as Y_min (minNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing operation. - Optionally, minimum number of Y' slots as Y'min (minNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to contiguous partial sensing operation. <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.214, 8.1.4]

- Reason for change: the actual text is not clear enough nor aligned with the current terminology in 3GPP SL.

- Summary of change: Include the wording “candidate” to clarify the set of resources where the minimum number of slots shall be indicated

- Consequences if not approved: the text is not clear nor aligned with the current SL terminology.

## 2.5	Contiguous partial sensing operation

The following text has been included for defining the operation of the UE performing contiguous partial sensing including the minimum number of resources to be selected by the UE and whether in case the number of resources obtained by the sensing operation is not fulfilling the minimum number, the decision to include the remaining ones is up to UE implementation. Moreover, the definition of the resource selection window for each of the different resource selection mechanisms, i.e., full-sensing, periodic partial sensing and contiguous partial sensing, is provided.

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> - A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). - Y is selected by UE where Y≥Ymin. - Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, if the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots. <Unchanged parts omitted>

Based on the agreements reached in RAN1#107-bis-e, we have the following definition of the resource selection window for the case of a UE performing contiguous partial sensing:

Agreement When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n, - T1 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4. - No update to specification is necessary due to this agreement - Note: The selected Y’ slots do not overlap with the sensing window

In our view, the current text does not consider the issue that the selected Y’ slots, i.e., the minimum number of slots to be reported, shall not overlap with the partial sensing window of the UE performing contiguous partial sensing. Therefore, we propose to include this indication in order to align the specification with the agreements reached in RAN1.

### 2.5.1 Text proposal

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> - A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] that do not overlap with the sensing window for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). - Y is selected by UE where Y≥Ymin. - Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, if the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots. <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.214, 8.1.4]

- Reason for change: the set Y’ of candidate slots is not restricted to avoid overlapping with the sensing window

- Summary of change: Include text to limit the set Y’ of candidate slots and avoid overlapping with the sensing window as per RAN1 agreement

- Consequences if not approved: unclear UE behavior and potential misfunctioning of the resource allocation procedure

## 2.6	Alignment of resource selection and SL DRX active of the Rx UE

During last RAN plenary, it was agreed that based on the UE implementation the Tx UE selects resources which are within the SL DRX active time of the Rx UE as the following text indicates:

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> 7a) If sidelink DRX active time of RX UE is provided by the higher layer and there is no candidate single-slot resource remained within the sidelink DRX active time in the set SA, the UE based on its implementation additionally selects and includes at least one candidate single-slot resources within the sidelink DRX active time in the set SA. <Unchanged parts omitted>

### 2.6.1 Text proposal

In our view, the text needs to be clarified indicating that the SL DRX active time is the one defined for the Rx UE in order to avoid any misunderstanding between the SL DRX of the Rx and the Tx UE. Therefore, we propose the following text proposal to solve this issue.

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> 7a) If sidelink DRX active time of RX UE is provided by the higher layer and there is no candidate single-slot resource remained within the sidelink DRX active time of the RX UE in the set SA, the UE based on its implementation additionally selects and includes at least one candidate single-slot resources within the sidelink DRX active time of the RX UE in the set SA. <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.214, 8.1.4]

- Reason for change: clarify that the SL DRX active used to select the resources is the one of the Rx UE

- Summary of change: add wording to clarify the intention of the text

- Consequences if not approved: unclear UE behavior and potential misfunctioning of the resource allocation procedure.

## 2.7	Sensing windows with PBPS and CPS for re-evaluation/re-selection of resources

In the current specification text, the sensing window for the contiguous partial sensing operation is defined as M. It has been defined that the UE shall perform sensing at least during M slots, however, as stated in the current specification form, it could be possible that perfoming the sensing operation M slots prior to the first resource within the associtaed resource selection window is not enough to fulfil the agreement of performing sensing during M slots – due to the processing times of the UE. Therefore, we propose to include an indication to clarify the intention of the text.

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0, - Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot tyiSL and ending at the last slot of the Y' candidate slots, where tyiSL is the first candidate slot after slot n+T3. - It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL - For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by contiguousSensingWindowAperiodic. <Unchanged parts omitted>

### 2.7.1 Text proposal

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0, - Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot tyiSL and ending at the last slot of the Y' candidate slots, where tyiSL is the first candidate slot after slot n+T3. - It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL taking into consideration associated processing times. - For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by contiguousSensingWindowAperiodic. <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.214, 8.1.4]

- Reason for change: the current text does not take into consideration the processing times associated to the sensing and resource selection operation

- Summary of change: Include text to consider the processing times in order to ensure the minimum size M of the contiguous sensing window is fulfilled

- Consequences if not approved: the minimum size M might not be fulfilled and the UE operation does not follow the RAN1 agreements

# 3	Conclusion

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.214, 8.1.4]

	Reason for change: the text is not aligned with the intention of the RAN1 agreement.

	Summary of change: remove the wording “only” on each of the resource selection mechanism to avoid misconfiguration of the resource pool. Include wording to allow a single resource selection mechanism to be configured per resource pool.

	Consequences if not approved: the resource pool configuration is not correct and leads to imprecise implementation of the resource pool configuration.

Proposal 2	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.214, 8.1.4]

	Reason for change: the text is not clear under which circumstances each of the parameters, i.e., contiguousSensingWindowAperiodic and contiguousSensingWindowPeriodic, is used.

	Summary of change: Include text to indicate the type of transmission, i.e., periodic or aperiodic, which triggers the used of each of the associated parameters for the contiguous sensing window.

	Consequences if not approved: the size and range of the contiguous sensing window for each of the different transmission types is not correct.

Proposal 3	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.214, 8.1.4]

	Reason for change: the definition of the parameter sl-ResourceReservePeriodList is missing in the specification

	Summary of change: Include the definition of the parameter sl-ResourceReservePeriodList as per RAN1 agreements.

	Consequences if not approved: missing definition of a relevant parameter for resource allocation procedure.

Proposal 4	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.214, 8.1.4]

	Reason for change: the actual text is not clear enough nor aligned with the current terminology in 3GPP SL.

	Summary of change: Include the wording “candidate” to clarify the set of resources where the minimum number of slots shall be indicated

	Consequences if not approved: the text is not clear nor aligned with the current SL terminology.

Proposal 5	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.214, 8.1.4]

	Reason for change: the set Y’ of candidate slots is not restricted to avoid overlapping with the sensing window

	Summary of change: Include text to limit the set Y’ of candidate slots and avoid overlapping with the sensing window as per RAN1 agreement

	Consequences if not approved: unclear UE behavior and potential misfunctioning of the resource allocation procedure

Proposal 6	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.214, 8.1.4]

	Reason for change: clarify that the SL DRX active used to select the resources is the one of the Rx UE

	Summary of change: add wording to clarify the intention of the text

	Consequences if not approved: unclear UE behavior and potential misfunctioning of the resource allocation procedure.

Proposal 7	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.214, 8.1.4]

	Reason for change: the current text does not take into consideration the processing times associated to the sensing and resource selection operation

	Summary of change: Include text to consider the processing times in order to ensure the minimum size M of the contiguous sensing window is fulfilled

	Consequences if not approved: the minimum size M might not be fulfilled and the UE operation does not follow the RAN1 agreements