3GPP TSG-RAN WG1 Meeting #109-e	R1-2204737

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.11.2

Source:	Ericsson

Title:		Remaining open issues on mode 2 enhancements for inter-UE coordination and related text proposals

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we provide proposals and solutions for the remaining open issues for the inter-UE coordination agenda item. Moreover, the paper includes text proposals for both inter-UE coordination schemes.

# 2	Remaining open issues in agreements

During the last meetings in RAN1 several agreements were left without further discussions, and they include some aspects that are for further study. In this section, we provide proposals to solve these issues in order to finalize the feature.

## 2.1 UE-B behavior upon receiving multiple scheme 1 coordination messages

One open issue is the UE-B behavior upon receiving several inter-UE coordination information from a single UE-A or from various UE-As as indicated in the following agreement:

Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As, - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As. - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline) - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

For this case, we propose to differentiate between the inter-UE coordination message which are received based on a previous explicit request or when the inter-UE coordination message has been triggered by other condition rather than the explicit request. Therefore, we propose that if multiple inter-UE coordination messages are triggered by the same condition, i.e., either by explicit request or by any other condition, the use of them is based on the UE-B’s implementation.

For the case when the inter-UE coordination messages have been triggered by the same condition, i.e., either by explicit request or by any other condition, it is up to UE implementation how to make use of them.

On the case when the multiple inter-UE coordination messages have been triggered by different mechanism – either from the same UE-A or from several UE-A(s) – we propose to prioritize the inter-UE coordination messages which have been triggered by an explicit request since the information of the inter-UE coordination message since based on the information contianed in the request the coordination message is better suited for the specific UE-B’s transmission.

For the case when the received inter-UE coordination messages have been triggered by different conditions, i.e., either by explicit request or by any other condition, UE-B prioritizes the inter-UE coordination message triggered by an explicit request.

## 2.2 Triggering timing of inter-UE coordination scheme 1

One remaining issue from RAN1#108-e meeting is connected to the timeline for scheduling the transmission of the sidelink transmission carrying the inter-UE coordination information. The agreement related to this issue is as follows:

Agreement - Notations: - (n+T_1) – Start slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement - (n+T_2) – End slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered - For inter-UE coordination information triggered by UE-B’s explicit request - Alt 1-1: - X1 ≤ (n’+T’_1) - (n’+T’_2) ≤ X2 - For inter-UE coordination information triggered by a condition other than explicit request reception, - Alt 2-2: - (n’+T’_2) < X3 - FFS: Values for X1, X2, X3

Based on this agreement, there are two different alternatives that need to be completed:

- First, the timeline for scheduling the transmission of the sidelink transmission carrying the inter-UE coordination information when an explicit request has been received.

- Second, the timeline for scheduling the transmission of the sidelink transmission carrying the inter-UE coordination information when it has been triggered by a condition other than explicit request reception

On this regard, we propose that for the case when an explicit request has been received, the timeline for scheduling the transmission of the sidelink transmission carrying the inter-UE coordination information has to be restricted by the resource selection window included in the explicit request.

For inter-UE coordination information triggered by UE-B’s explicit request

- Alt 1-1:

  - X1 ≤ (n’+T’_1)

  - (n’+T’_2) ≤ X2

    - The value X1 = n+T_1 and the value X2 = n+T_2

For the case when the inter-UE coordination message is triggered by another condition rather than an explicit request, we propose that the timeline for scheduling the transmission of the sidelink transmission carrying the inter-UE coordination information is restricted by the value n+T_2 as selected by UE-A’s implementation.

For inter-UE coordination information triggered by a condition other than explicit request reception

  - Alt 2-2:

    - (n’+T’_2) < X3

      - The value X3 = n+T_2

## 2.3	Inter-UE coordination for power saving UEs

In our view, it is important to consider all the scenarios where the inter-UE coordination mechanism could be useful and pursue as much as possible a unified mechanism. During RAN1#103-e, it was agreed in the power saving AI to consider UEs which can only receive PSFCH and S-SSB for evaluation and designing of the SL features for Rel-17. Consequently, such UEs cannot perform sensing. The presence of such UEs degrades the performance for all the UEs in the system. Moreover, it was agreed in the power saving agenda that the re-evaluation and pre-emption checking can be triggered by the inter-UE coordination scheme 2 regardless of the UE operation.

Conclusion - SL reception Type A and Type D should be used as the reference for evaluation and designing of SL power saving features in R17. - Type A: UE is not capable of performing reception of any SL signals and channels, FFS with exception of performing PSFCH and S-SSB reception (aim to conclude in RAN1#104-e) - Type D: UE is capable of performing reception of all SL signals and channels defined in R16. It does not preclude UE to perform reception of a subset of SL signals/channels - If there are evaluations with assumptions other than the above reference, the detailed assumptions need to be reported - Note: the types and the associated capability defined here are not intended to be defined as Rel-17 UE features as is. Conclusion: - PSFCH reception is not included for Type A UE - S-SSB reception is not included for Type A UE - SL reception Type B is additionally added - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception - Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B) From the power saving agenda: Conclusion: No additional triggering enhancement on top of existing Rel-16 mechanism in re-evaluation and pre-emption checking for partial sensing UEs in Rel-17, including enabling / disabling re-evaluation by (pre-)configuration.. - This does not restrict the triggering of re-evaluation and pre-emption checking due to inter-UE coordination message in scheme 2 (if agreed).

It is desirable that such UEs can make use of the inter-UE coordination framework to reduce the degradation as much as possible. Of course, this should not play against their original goal of saving power. In our view, inter-UE coordination using Scheme 2 is very suited for this purpose as it does not require a big increase in RX time while Scheme 1 is not suitable to be used with power saving UEs.

Regarding Scheme 1, there are two potential cases: the case of Option A where UE-B combines the information from the coordination message along with its own sensing information. Since we are discussing non-sensing UEs, Option A is not relevant since there is no sensing information gathered by UE-B. In the case of Option B in scheme 1, UE-B does not have any sensing information of its own and it might use the coordination information sent by UE-A to perform the resource selection. This coordination information may arguably improve the performance of the resource selection since it includes information about the available resources, however, based on the power saving operation from UE-B the following behavior is expected:

- UE-B is not able to receive any SL transmission, i.e., Type-A UE; or

- UE-B is only able to receive S-SSB transmissions for synchronization purposes and PSFCH transmissions.

Therefore, based on these two behaviors it is not feasible that UE-B is able to receive the coordination message with the structure that is defined for scheme 1. The size of the inter-UE coordination message for scheme 1 for both preferred and non-preferred resources is too large to be carried by PSFCH.

Moreover, using PSSCH for conveying the coordination message in scheme 1 requires that the transmission is scheduled following the existing procedures, which makes the arrival time at UE-B unknown. Consequently, UE-B has to be awake/ready to receive the coordination message during a certain period, i.e., has to be sensing for a certain period of time, which defies the idea of performing a power saving operation where the sensing time has to be reduced as much as possible.

It is not suitable to use the Inter-UE coordination Scheme 1 for non-sensing UEs, i.e., type-A and type-B, due to the format and the non-deterministic time of arrival of the coordination message in Scheme 1.

For the case of scheme 2, we analyze the benefits of inter-UE coordination in a scenario where some of the UEs do not perform sensing (e.g., Type-A UEs in Partial Sensing). We consider the following two types of UEs that do not perform sensing. The simulation assumptions can be found in Appendix A-2.

- O1 – Full Sensing UEs using the Rel-16 sensing and resource allocation procedure. This is the baseline.

- O2 – Full Sensing UEs using collision prevention (Scheme 2)

- O3 – No Sensing UE

  - The UE selects resources at random, without any sensing information.

- O4 – No Sensing with inter-UE coordination with single bit messages

  - The UE selects resources at random, without any sensing information.

  - After every transmission, the UE checks if it has received an inter-UE coordination message (single bit). If received, the UE drops the concerned reservation and reselects resources.

Figure 2 includes results from two simulations. In the first simulation, inter-UE coordination was not used (the resulting curves are those for O1 and O3). In the second simulation, inter-UE coordination was used (the resulting curves are those for O2 and O4). The results are presented for two distributions of Full Sensing and No Sensing UEs: the first one (solid curves) is approximately 90%/10%; the second one (dash-dotted curves) is 50%/50%.

We note that the use of inter-UE coordination not only significantly improves the performance of ‘Full Sensing’ UEs (O1 vs O2) but also that of ‘No Sensing’ UEs (O3 vs O4). Indeed ‘No Sensing’ UEs obtain the biggest advantage using inter-UE coordination.

Figure 1: PRR comparison for full-sensing and non-sensing UEs with and without Inter-UE coordination

For the case where a UE is capable of performing sensing but performs random resource selection mechanism, i.e., there is no sensing for the initial reservation of resources, we propose based on the simulation results the following procedure:

- UE-B that performs random resource selection, i.e., no sensing, received the coordination message from UE-A which is the intended receiver of the transmission.

  - UE-B perform resource exclusion at the time of receiving the coordination message based only on the information from UE-A and creates a set of resources S_A1.

  - Upon creating a set of candidate resources based only on the coordination information from UE-A, UE-B performs re-evaluation/re-selection of the set of resources which results in a candidate set of resources composed of a subset of the resources in S_A1, i.e., S_A2  S_A1.

  - In case the transmission is not successful, i.e., a HARQ-NACK is received for the transmission based only on the coordination message, UE-B performs sensing for the re-transmission, i.e., it creates the candidate set of resources based on its own information and coordination message from UE-A.

A UE capable of sensing triggers re-evaluation/pre-emption procedure after receiving a set of resources within the scheme 1 Inter-UE coordination framework for both Option A and Option B if supported.

For a UE capable of sensing operating under Option B if the initial transmission based only on the coordination information is not successful, e.g., received HARQ-NACK, the UE performs sensing for resource selection for the re-transmission.

# 3	Text Proposals related to Inter-UE coordination Scheme 2

## 3.1 Timeline for transmitting the conflict indication

For the transmission of the conflict indication in the PSFCH resources, the distance in number of slots shall satisfy the constraint provided by sl-MinTimeGapPSFCH as indicated in the following agreement:

Agreement - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value - X = sl-MinTimeGapPSFCH - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied

In the current specification text this constraint is not included in the procedure for transmitting the PSFCH indicating a conflict to the other UE. In our view, this should be included in this section to avoid any conflicting behaviour, i.e., transmitting the conflict indication without taking into consideration the requirement provided by sl-MinTimeGapPSFCH, when transmitting the conflict indication.

### 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged parts omitted> A UE can be enabled, by inter-UECoordinationScheme2, to transmit a PSFCH with conflict information in a resource pool. The UE can determine, based on an indication by a SCI format 1-A, a set of resources that includes one or more slots and resource blocks that are reserved for PSSCH transmission. If the UE determines a conflict for a reserved resource for PSSCH transmission, the UE provides conflict information in a PSFCH. <Unchanged parts omitted>

Therefore, the following text proposal solves the aforementioned potential misbehavior of the UE transmitting the conflict indication.

### 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged parts omitted> A UE can be enabled, by inter-UECoordinationScheme2, to transmit a PSFCH with conflict information in a resource pool. The UE can determine, based on an indication by a SCI format 1-A, a set of resources that includes one or more slots and resource blocks that are reserved for PSSCH transmission. If the UE determines a conflict for a reserved resource for PSSCH transmission, the UE provides conflict information in a PSFCH: - If PSFCHOccasionScheme2 = 'followSCI', the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If PSFCHOccasionScheme2 = 'followReservedResource', the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.213, 16.3.0]

- Reason for change: the timeline for the transmission of the conflict information in PSFCH is missing.

- Summary of change: Include the restriction and requirements to when the conflict information can be transmitted in PSFCH resources.

- Consequences if not approved: the conflict information might not be transmitted at a time that fulfills the agreed timeline restrictions

## 3.2 Receiver UE behavior if timeline for conflict indication is not fulfilled

The following specification text indicates the UE behavior when receiving a PSFCH resources containing conflict information.

### 16.3.1 UE procedure for receiving PSFCH with control information <Unchanged parts omitted> If a UE receives a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE receives the PSFCH in the resource pool in a slot determined based on PSFCHOccasionScheme2 - If PSFCHOccasionScheme2 = 'followSCI', the UE receives the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not receive the PSFCH with conflict information - If PSFCHOccasionScheme2 = 'followReservedResource', the UE receives the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE does not receive the PSFCH with conflict information <Unchanged parts omitted>

In our view, the text regarding the UE behavior shall not be defined as “does not receive” since this can be understood as something to be mandated at the UE transmitting the conflict indication, i.e., the other UE do not transmit the conflict indication. However, this part of the specification is written from the receiver UE perspective based on the case that the conflict indication has been transmitted not following the constraint provided by sl-MinTimeGapPSFCH. Therefore, in our view, a better way to indicate this behaviour is to change the wording to “discard” meaning that a upon receiving the conflict indication the receiving UE does not trigger the re-selection of resources. Note that given the specified transmitter behaviour, the receiver cannot be sure about the contents of the PSFCH transmission, if it exists. The only reasonable behaviour is to discard the contents.

### 16.3.1 UE procedure for receiving PSFCH with control information <Unchanged parts omitted> If a UE receives a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE receives the PSFCH in the resource pool in a slot determined based on PSFCHOccasionScheme2 - If PSFCHOccasionScheme2 = 'followSCI', the UE receives the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE discards the PSFCH with conflict information - If PSFCHOccasionScheme2 = 'followReservedResource', the UE receives the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE discards the PSFCH with conflict information <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.213, 16.3.1]

- Reason for change: the specification text does not include the proper UE behavior

- Summary of change: Change wording from “does not receive” to “discards”

- Consequences if not approved: the specification text is not clear regarding the UE behavior upon receiving conflict information not fulfilling the timeline requirements.

## 3.3 Understanding of reserved bits in SCI format 1-A

In the following specification text, the reserved bits in the SCI format 1-A are indicated to be able to have any value.

## 16.4 UE procedure for transmitting PSCCH <Unchanged parts omitted> For decoding of a SCI format 1-A, a UE may assume that a number of bits provided by sl-NumReservedBits can have any value. <Unchanged parts omitted>

However, in the current specification the value of the reserved bits have been modified to indicate whether the inter-UE coordination scheme 2 is configured, and therefore, can be applied based on the information contained in SCI format 1-A as follows:

#### 8.3.1.1 SCI format 1-A <Unchanged parts omitted> Reserved – a number of bits as determined by the following: - Nreserved bits as configured by higher layer parameter sl-NumReservedBits, with value set to zero, if higher layer parameter indicationUEBScheme2 is not configured, or if higher layer parameter indicationUEBScheme2 is configured to 'Disabled'; - (Nreserved-1) bits otherwise, with value set to zero. <Unchanged parts omitted>

Therefore, we propose the following text proposal to avoid any confusion with the newly added fields in the SCI format 1-A during Rel-17:

## 16.4 UE procedure for transmitting PSCCH <Unchanged parts omitted> For decoding of a SCI format 1-A, a UE may assume that a number of bits provided by sl-NumReservedBits can have any value following the defined fields in [8.3.1, 38.212] <Unchanged parts omitted>

Include the aforementioned TP for the allowed resource selection mechanism:

- Impacted specification: [TS 38.213, 16.4]

- Reason for change: the specification text might be misleading on the understanding of the reserved bits in SCI format 1-A

- Summary of change: Explicitly indicate the definition of the reserved bits for SCI format 1-A in TS 38.212.

- Consequences if not approved: the specification text is not clear regarding the reserved bits in SCI format 1-A.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	It is not suitable to use the Inter-UE coordination Scheme 1 for non-sensing UEs, i.e., type-A and type-B, due to the format and the non-deterministic time of arrival of the coordination message in Scheme 1.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the case when the inter-UE coordination messages have been triggered by the same condition, i.e., either by explicit request or by any other condition, it is up to UE implementation how to make use of them.

Proposal 2	For the case when the received inter-UE coordination messages have been triggered by different conditions, i.e., either by explicit request or by any other condition, UE-B prioritizes the inter-UE coordination message triggered by an explicit request.

Proposal 3	For inter-UE coordination information triggered by UE-B’s explicit request

	Alt 1-1:

	X1 ≤ (n’+T’_1)

	(n’+T’_2) ≤ X2

i.	The value X1 = n+T_1 and the value X2 = n+T_2

Proposal 4	For inter-UE coordination information triggered by a condition other than explicit request reception

o	Alt 2-2:

	(n’+T’_2) < X3

	The value X3 = n+T_2

Proposal 5	A UE capable of sensing triggers re-evaluation/pre-emption procedure after receiving a set of resources within the scheme 1 Inter-UE coordination framework for both Option A and Option B if supported.

Proposal 6	For a UE capable of sensing operating under Option B if the initial transmission based only on the coordination information is not successful, e.g., received HARQ-NACK, the UE performs sensing for resource selection for the re-transmission.

Proposal 7	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.213, 16.3.0]

	Reason for change: the timeline for the transmission of the conflict information in PSFCH is missing

	Summary of change: Include the restriction and requirements to when the conflict information can be transmitted in PSFCH resources.

	Consequences if not approved: the conflict information might not be transmitted at a time that fulfills the agreed timeline restrictions

Proposal 8	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.213, 16.3.1]

	Reason for change: the specification text does not include the proper UE behavior

	Summary of change: Change wording from “does not receive” to “discards”

	Consequences if not approved: the specification text is not clear regarding the UE behavior upon receiving conflict information not fulfilling the timeline requirements.

Proposal 9	Include the aforementioned TP for the allowed resource selection mechanism:

	Impacted specification: [TS 38.213, 16.4]

	Reason for change: the specification text might be misleading on the understanding of the reserved bits in SCI format 1-A

	Summary of change: Explicitly indicate the definition of the reserved bits for SCI format 1-A in TS 38.212.

	Consequences if not approved: the specification text is not clear regarding the reserved bits in SCI format 1-A.

# Appendix – Simulation Assumptions

## A-1	Analysis of Inter-UE coordination and power saving UEs

Table 1 contains the different simulations assumptions used for generating the results on the use of inter-UE coordination together with power-saving UEs. Other assumptions and models follow TR 37.885 and TR 38.885.

Table 1: Simulation assumptions

|  | Parameter | Value |
| --- | --- | --- |
| Scenario | Deployment | Highway Option A |
|  | Number of UEs | 155 (As determined by TR 37.885) |
|  | Channel models | See TR 37.885 |
| Traffic | Model | Aperiodic medium intensity with fixed packet size 800 bytes |
|  | PDB | 50 ms |
|  | Cast Mode | Groupcast Option 2 with group distance = 500 m |
| RF | Carrier frequency | 6 GHz |
|  | Bandwidth | 40 MHz |
|  | SCS | 30 kHz |
|  | Antenna configuration | 2 TX / 2 RX |
| Pool configuration | Sub-channels | 4 |
| Scheduling | Max. transmissions per TB | 4 |
|  | Reservations per SCI | 1 |
|  | Gap between retransmissions | 2 slots |
|  | MCS | 16QAM with CR=1/2 |
| Sensing | RSRP threshold | -80 dBm |