3GPP TSG RAN WG1 #109-e			R1- 2204739

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.16.11

Source:	Ericsson

Title:	UE features for NR sidelink enhancements

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we provide some comments and modifications to the feature groups considered for NR SL Rel-17 after the RAN1#108-e meeting and included in R1-2202929 [1]. Moreover, further details on the splitted feature group from the original 32-2 – as decided by RAN plenary – are given.

# 2	Feature group discussion

## 2.1	Feature group 32-2

During RAN plenary#95-e, it was discussed and agreed to split the FG 32-2 into two different sub-FGs:

- One sub-FG for support of only S-SSB reception

- Another sub-FG for support of PSFCH reception with pre-requisite of S-SSB reception

Based on this agreement, we propose the following definition of the sub-FGs:

### 2.1.1	Feature group 32-2-1

This feature group is related to the reception of S-SSB as the only SL transmission that can be received by the SL UE.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2-1 | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB. | None | No | No | The UE does not receive synchronization signalling from other SL UEs | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

FG 32-2-1 has the following components:

- Prerequisites: None

- Need for gNB to know the feature: No

- Capability signalling between the UEs: No

- UE feature granularity: Per band

- Consequence if the feature is not supported: The UE does not receive synchronization signalling from other UEs.

### 2.1.2	Feature group 32-2-2

The FG 32-2-2 uses as pre-requisite the FG32-2-1 meaning that if the UE is capable of receiving PSFCH from other UEs it should also be able to receive synchronization signalling from other UEs, i.e., S-SSB. Moreover, since both FGs deal with specific signalling from the receiver UE perspective, i.e., there is no need for the gNB to allocate resources for transmission, there is no need to signal these capabilities to the gNB.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2-2 | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH | 32-2-1 | No | No | The UE cannot receive feedback signalling (PSFCH) from other UEs | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

FG 32-2-2 has the following components:

- Prerequisites: FG 32-2-1

- Need for gNB to know the feature: No

- Capability signalling between the UEs: No

- UE feature granularity: Per band

- Consequence if the features is not supported: The UE cannot receive PSFCH from other UEs.

## 2.2	Feature group 32-4

In RAN1#108-e, many details for feature group FG 32-4 were decided, however, some open aspects are still open aspect to be finalized.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu | [TBD] | Yes | No | UE does not support transmission according to the partial sensing and resource allocation | Per FS | N.A. | N.A. | N.A. | Note: Random selection [according to Rel-16] in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs. Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

There are two remaining issues for the FG 32-4. The first of them is to define the pre-requisite features groups (if any). In our view, a UE that performs partial sensing, i.e., both periodic and contiguous partial sensing, needs to be able to synchronize with the rest of UEs and/or the network/GNSS system. Therefore, we propose to include as pre-requisite the FG which are related to the acquisition of synchronization signalling from UEs and from the network, i.e., FG 32-4b, 32-4c and 32-2-1.

- A UE performing partial sensing in mode 2 is required to be able to synchronize to other UEs and/or the network/GNSS.

Another issue with the pre-requisite feature groups is that a UE performing partial sensing operation shall be able to perform random resource allocation procedures as it is one of the potential options to be used when performing partial sensing operation as given by the following RAN1 agreement:

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - If the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min, - How UE includes other candidate slots is up to UE implementation - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results starting at M consecutive logical slots before ty0 and ending at Tproc,0 + Tproc,1 slots earlier than ty0. - FFS: By default, M is 31 unless (pre-)configured with another value, where M is (pre-)configured based on transmission priority - FFS: The range of (pre-)configured M from a TBD lowest value up to 30 - When the minimum M slots for CPS cannot be guaranteed, support both - Option A, the UE ensures the Y’min criterion is fulfilled - Option B: UE performs random resource selection - When the UE performs Option A or Option B is up to UE implementation

Moreover, regarding whether we should include the wording “according to Rel-16” in the note for the random resource selection operation of the UE, in our view this remark is not needed. We think it is fine to keep the note as it is, without highlighting the wording “according to Rel-16”. Given the lack of agreements stating anything else, Rel-16 random resource selection is applied in the exceptional pool.

Remove the highlighted part “according to Rel-16” in the note related to random resource selection.

For the FG 32-4, the remaining unstable fields are completed as follows:

- Pre-requisite FGs: 32-4b;32-4c and/or 32-2-1

- Remove the highlighted wording “according to Rel-16” from the note field.

## 2.3	Feature group 32-4b

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-4b | Synchronization sources for NR sidelink transmission | 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a [6) UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1.] |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

For the FG 32-4b, there is only one open issue regarding whether to include the support of the SyncRefUE as a synchronization reference. In our view, if the UE supports the FG 15-1 as defined in Rel-16, it should also be able to support the SyncRefUE as a synchronization source. However, based on the current wording is not clear whether the UE will support both operations, i.e., supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink, therefore, we propose to change the wording to clarify the oepration.

For the FG 32-4b, include the following component:

- 6) If the UE supports 15-1, it supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink.

## 2.4	Feature group 32-5a-1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a-1 | Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set. | [At least one of 32-5a-2 and 32-5a-3] | Yes | Yes | UE does not support transmitting inter-UE coordination scheme 1 in NR sidelink mode 2. [UE does not receive an explicit request for inter-UE coordination information] | [Per band] | [N.A.] | [N.A.] | [N.A.] | FFS | Optional with capability signalling. |

For the FG32-5a-1, there are several open issues to be decided in RAN1. The first issue corresponds to the pre-requisite feature groups. In our view, a UE that is capable of transmitting inter-UE coordination scheme 1 needs to support the feature groups which are related to the transmissions and reception of SL transmissions. Therefore, we propose to include the FG 15-1 and FG 15-3 as defined for Rel-16 SL UEs. Moreover, it is also possible that a UE involved in the transmission of the inter-UE coordination message for scheme 1 can be performing partial sensing mechanism, and therefore, we propose to include it too.

Add in the pre-requisite feature groups field for the FG 32-5a-1 the FGs related to SL transmission, reception and sensing, i.e., 15-1, 15-3 and 32-4.

Additionally, it was discussed during the last RAN1 meeting the inclusion as pre-requisite FGs of reception of inter-UE coordination scheme 1 (both for preferred and non-preferred resource set). We think that this is a reasonable manner to ensure that the inter-UE coordination mechanism is useful, i.e., there are UEs which assist other UEs while also being able to receive this assistance.

Include as pre-requisite FGs for the FG 32-5a-1, the feature groups related to the reception of inter-UE coordination scheme 1 for both preferred and non-preferred resource set.

For the FG 32-5a, the remaining unstable fields are completed as follows:

- Pre-requisite FGs: 15-1, 15-3, 32-4 and at least one of 32-5a-2 and 32-5a-3

- Consequence if the feature is not supported by the UE: Keep the highlighted text “UE does not receive an explicit request for inter-UE coordination information”

- Granularity of the feature: Per band

- Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1

## 2.5	Feature group 32-5a-2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a-2 | Receiving Inter-UE coordination information of preferred resource set in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of preferred resource set only. | [TBD] | Yes | Yes | UE does not support receiving inter-UE coordination of preferred resource set in NR sidelink mode 2. [UE does not transmit an explicit request for inter-UE coordination information of preferred resource set only] | Per band | N.A. | N.A. | N.A. | FFS | Optional with capability signalling. |

For the FG32-5a-2, the main remaining issue is to decide on the pre-requisite FGs that are needed for this feature. In our view, since this feature is related to the reception of the inter-UE coordination message, the UE is required to support as pre-requisite the FGs which are related to the reception of SL transmissions. Therefore, we propose to include as pre-requisite FG, the FG 15-1.

Include as pre-requisite FG for 32-5a-2, the FGs related to the reception of SL transmission, i.e., FG 15-1.

Additionally, the UE supports the transmission of an explicit request to a peer UE in order to request a set of preferred resources. On this regard, the UE should be able to perform sensing of the resource pool in order to obtain the free resources to send its request to the other UE. Therefore, we propose to include as pre-requisite FGs the following ones: 15-3, 32-4.

For FG 32-5a-2, the UE is able to transmit a request to a peer UE(s). Therefore, it is required to have as pre-requisite the FGs related to the transmission in mode 2, i.e., FGs 15-3, 32-4.

For the FG 32-5a-2, the remaining unstable fields are completed as follows:

- Pre-requisite FGs: 15-1, 15-3, 32-4.

- Consequence if the feature is not supported by the UE: Keep the highlighted text “UE does not transmit an explicit request for inter-UE coordination information of preferred resource set only”

- Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1

## 2.6	Feature group 32-5a-3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a-3 | Receiving Inter-UE coordination information of non-preferred resource set in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of non-preferred resource set only. | [TBD] | Yes | Yes | UE does not support receiving inter-UE coordination of non-preferred resource set in NR sidelink mode 2. [UE does not transmit an explicit request for inter-UE coordination information of non-preferred resource set only] | Per band | N.A. | N.A. | N.A. | FFS | Optional with capability signalling. |

For the FG32-5a-3, the main remaining issue is to decide on the pre-requisite FGs that are needed for this feature. In our view, since this feature is related to the reception of the inter-UE coordination message, the UE is required to support as pre-requisite the FGs which are related to the reception of SL transmissions. Therefore, we propose to include as pre-requisite FG, the FG 15-1 as defined in NR SL Rel-16. Moreover, since this features group is related to the reception of non-preferred resources, the UE needs to be capable of creating its own sensing results. Consequently, we propose to add the FGs related to transmitting in SL using sensing information, i.e., 32-4 (partial sensing) and 15-3 (full sensing).

Include as pre-requisite FG for 32-5a-3, the FGs related to the reception of SL transmission and performing SL transmission including sensing, i.e., FG 15-1, 15-3 and 32-4.

Additionally, the UE supports the transmission of an explicit request to a peer UE in order to request a set of preferred resources. On this regard, the UE should be able to perform sensing of the resource pool in order to obtain the free resources to send its request to the other UE. Therefore, we propose to include as pre-requisite FGs the following ones: 15-3, 32-4.

For FG 32-5a-3, the UE is able to transmit a request to a peer UE(s). Therefore, it is required to have as pre-requisite the FGs related to the transmission in mode 2, i.e., FGs 15-3, 32-4.

For the FG 32-5a-3, the remaining unstable fields are completed as follows:

- Pre-requisite FGs: 15-1, 15-3, 32-4.

- Consequence if the feature is not supported by the UE: Keep the highlighted text “UE does not transmit an explicit request for inter-UE coordination information of preferred resource set only”

- Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1.

## 2.7	Feature group 32-5b-1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. 2) UE can transmit up to M PSFCH(s) resources in a slot | 32-5b-2, TBD | Yes | Yes | UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | [N.A.] | [N.A.] | [N.A.] | [Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1] Candidate values for M are {4, 8, 16} If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number and the same among those FGs. | Optional with capability signalling. |

For the FG32-5b-1, the main remaining issue is to decide on the pre-requisite FGs that are needed for this feature. In our view, the UE should be able to perform sensing of the resource pool in order to obtain the free resources to send its request to the other UE. Therefore, we propose to include as pre-requisite FGs the following ones: 15-3, 32-4.

For FG 32-5b-1, the UE is able to transmit a request to a peer UE(s). Therefore, it is required to have as pre-requisite the FGs related to the transmission in mode 2, i.e., FGs 15-3, 32-4.

For the FG 32-5b-1, the remaining unstable fields are completed as follows:

- Pre-requisite FGs: 15-3, 32-4.

- Granularity of the feature: Per band

- Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1.

## 2.8	Feature group 32-5b-2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 2) UE can receive up to N PSFCH(s) resources in a slot. | [TBD] | Yes | Yes | UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2. | Per band | N.A. | N.A. | N.A. | [Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1] Candidate values for N are {5, 15, 25, 32, 35, 45, 50, 64} If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs. | Optional with capability signalling. |

Moreover, once the UE receives the conflict indication, the resource re-evaluation and/or re-selection is triggered. On this regard, the UE shall be able to perform sensing in order to do these operations. Therefore, we propose to include as pre-requisite FGs the ones related to the sensing operation that is performed for re-evaluation and re-selection of resources upon receiving the conflict indication from a peer UE.

For FG 32-5b-2, include as pre-requisite the FGs related to the re-evaluation and re-selection operation upon receiving the conflict indication, i.e., FGs 15-3 and 32-4.

For the FG 32-5b-2, the remaining unstable fields are completed as follows:

- Pre-requisite FGs: 15-3, 32-4

- Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1.

## 2.9	Feature group 32-6-1 and 32-6-2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-6-1 | Reception of Scheme 1 inter-UE coordination information over 2nd SCI | 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. | FFS | Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-6-2 | Reception of Scheme 1 explicit request over 2nd SCI | 1) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission | FFS | Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH | [Per band] | [N.A.] | [N.A.] | [N.A.] | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |

For the FGs 32-6-1 and 32-6-2, both have similar requirements in terms of the pre-requisite FGs that need to be supported. Based on the agreements reached in RAN1, the UE shall be able to support simultaneously the reception of inter-UE coordination message for preferred and non-preferred. Therefore, we propose to include as pre-requisite the FGs related to the reception of the inter-UE coordination message. Moreover, for the case of FG 32-6-2, since it is related to the reception of an explicit request, the UE shall support additionally the transmission of the inter-UE coordination message both for preferred and non-preferred resources.

For the FG 32-6-1, the remaining unstable fields are completed as follows:

- Pre-requisite FGs: 32-5a-2 and 32-5a-3

For the FG 32-6-2, the remaining unstable fields are completed as follows:

- Pre-requisite FGs: 32-5a-2, 32-5a-3 and 32-5a-1

- Granularity of the feature: Per band

## 2.10	Feature group 32-7

The only remaining issue for the FG 32-7 is the granularity of the FG. In our view, following a similar approach as the one taken for similar feature groups and the one taken in previous releases, the granularity of the feature should be per band.

For the FG 32-7, the remaining unstable fields are completed as follows:

- Granularity of the feature: Per band

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A UE performing partial sensing in mode 2 is required to be able to synchronize to other UEs and/or the network/GNSS system.

Based on the discussion in the previous sections we propose the following:

Proposal 1	FG 32-2-1 has the following components:

	Prerequisites: None

	Need for gNB to know the feature: No

	Capability signalling between the UEs: No

	UE feature granularity: Per band

	Consequence if the feature is not supported: The UE does not receive synchronization signalling from other UEs.

Proposal 2	FG 32-2-2 has the following components:

	Prerequisites: FG 32-2-1

	Need for gNB to know the feature: No

	Capability signalling between the UEs: No

	UE feature granularity: Per band

	Consequence if the features is not supported: The UE cannot receive PSFCH from other UEs.

Proposal 3	Remove the highlighted part “according to Rel-16” in the note related to random resource selection.

Proposal 4	For the FG 32-4, the remaining unstable fields are completed as follows:

	Pre-requisite FGs: 32-4b;32-4c and/or 32-2-1

	Remove the highlighted wording “according to Rel-16” from the note field.

Proposal 5	For the FG 32-4b, include the following component:

	6) If the UE supports 15-1, it supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink.

Proposal 6	Add in the pre-requisite feature groups field for the FG 32-5a-1 the FGs related to SL transmission, reception and sensing, i.e., 15-1, 15-3 and 32-4.

Proposal 7	Include as pre-requisite FGs for the FG 32-5a-1, the feature groups related to the reception of inter-UE coordination scheme 1 for both preferred and non-preferred resource set.

Proposal 8	For the FG 32-5a, the remaining unstable fields are completed as follows:

	Pre-requisite FGs: 15-1, 15-3, 32-4 and at least one of 32-5a-2 and 32-5a-3

	Consequence if the feature is not supported by the UE: Keep the highlighted text “UE does not receive an explicit request for inter-UE coordination information”

	Granularity of the feature: Per band

	Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1

Proposal 9	Include as pre-requisite FG for 32-5a-2, the FGs related to the reception of SL transmission, i.e., FG 15-1.

Proposal 10	For FG 32-5a-2, the UE is able to transmit a request to a peer UE(s). Therefore, it is required to have as pre-requisite the FGs related to the transmission in mode 2, i.e., FGs 15-3, 32-4.

Proposal 11	For the FG 32-5a-2, the remaining unstable fields are completed as follows:

	Pre-requisite FGs: 15-1, 15-3, 32-4.

	Consequence if the feature is not supported by the UE: Keep the highlighted text “UE does not transmit an explicit request for inter-UE coordination information of preferred resource set only”

	Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1

Proposal 12	Include as pre-requisite FG for 32-5a-3, the FGs related to the reception of SL transmission and performing SL transmission including sensing, i.e., FG 15-1, 15-3 and 32-4.

Proposal 13	For FG 32-5a-3, the UE is able to transmit a request to a peer UE(s). Therefore, it is required to have as pre-requisite the FGs related to the transmission in mode 2, i.e., FGs 15-3, 32-4.

Proposal 14	For the FG 32-5a-3, the remaining unstable fields are completed as follows:

	Pre-requisite FGs: 15-1, 15-3, 32-4.

	Consequence if the feature is not supported by the UE: Keep the highlighted text “UE does not transmit an explicit request for inter-UE coordination information of preferred resource set only”

	Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1.

Proposal 15	For FG 32-5b-1, the UE is able to transmit a request to a peer UE(s). Therefore, it is required to have as pre-requisite the FGs related to the transmission in mode 2, i.e., FGs 15-3, 32-4.

Proposal 16	For the FG 32-5b-1, the remaining unstable fields are completed as follows:

	Pre-requisite FGs: 15-3, 32-4.

	Granularity of the feature: Per band

	Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1.

Proposal 17	For FG 32-5b-2, include as pre-requisite the FGs related to the re-evaluation and re-selection operation upon receiving the conflict indication, i.e., FGs 15-3 and 32-4.

Proposal 18	For the FG 32-5b-2, the remaining unstable fields are completed as follows:

	Pre-requisite FGs: 15-3, 32-4

	Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1.

Proposal 19	For the FG 32-6-1, the remaining unstable fields are completed as follows:

	Pre-requisite FGs: 32-5a-2 and 32-5a-3

Proposal 20	For the FG 32-6-2, the remaining unstable fields are completed as follows:

	Pre-requisite FGs: 32-5a-2, 32-5a-3 and 32-5a-1

	Granularity of the feature: Per band

Proposal 21	For the FG 32-7, the remaining unstable fields are completed as follows:

	Granularity of the feature: Per band

# 4	References

R1-2202929, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#108-e eMeeting, February 21st – March 3rd, 2022, 2022.