3GPP TSG-RAN WG1 Meeting #109-e	R1- 2204741

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.1.1

Source:	Ericsson

Title:	Channel access mechanism for SL-U

Document for:	Discussion, Decision

# 1	Introduction

In RAN#94e, a new WID on NR sidelink evolution was agreed [1]. Objective # 2 of the agreed WID aims to study and specify the operation of sidelink on unlicensed spectrum. The details of the objective as agreed are stated below.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this paper, we discuss the channel access mechanism for SL-U and explain our thoughts on the use of existing SL functionalities together with the NR-U channel access mechanisms. We also discuss the evaluation methodology to be used for SL-U.

# 2		Channel access

For NR-U two different types of channel access mechanisms were specified: dynamic channel access and semi-static channel access. Dynamic channel access is mainly designed for load-based equipment and is a natural way of accessing the channel in the unlicensed spectrum i.e., a device performs CCA according to LBT type 1 or type 2 whenever it has some data to transmit. In contrast, semi-static channel access is designed for frame-based equipment. In semi-static channel access, CCA is performed on a single observation slot (9 s) per fixed frame period. According to our observation, the need of specifying the semi-static channel access for SL-U is not clear. This is mainly because SL-U UE operating in mode-2 for resource allocation will anyway have to perform sensing by decoding SCI. Therefore, the benefits of energy saving and low complexity are not prominent.

Dynamic channel access of NR-U is the baseline for SL-U.

In the following, we discuss the different channel access (LBT) rules for each of the different SL transmissions. From the channel access perspective, the transmission by the SL UE is quite similar to the gNB in case of NR-U. For example, in mode 2 the SL UE determines which resources to use for transmission and other parameters. Therefore, it is recommended to reuse the downlink channel access rules for the transmission by SL UE.

From channel access perspective, SL transmission in SL-U is similar to DL transmissions in NR-U.

Reuse channel access rules (including COT sharing) for gNB (DL) transmissions in NR-U for transmissions in SL-U.

In SL-U, the baseline channel access mechanisms for PSCCH/PSSCH must be standalone (i.e., not depend on COT sharing, etc.). Type 1 LBT is the appropriate channel access in this case. The applicability of Type 2 LBT should be studied further, especially in the context of COT sharing (e.g., between a PSFCH transmission and the corresponding PSCCH/PSSCH retransmission, etc.).

SL-U supports type 1 LBT for PSCCH/PSSCH transmissions.

Study further the applicability of type 2 (2A/2B/2C) LBT for PSCCH/PSSCH transmissions.

When it comes to COT sharing mechanisms, our view is that the COT should not be shared for PSCCH/PSSCH transmissions between two or more UEs. This is because a UE initiating COT cannot have the full view of the channel occupancy in contrary to gNB and will not be able to avoid collisions with other UEs in the system.

COT is not shared for PSCCH/PSSCH transmissions between different UEs.

On the other hand, transmissions of HARQ feedback by the receiver UE(s) can happen within the COT shared by the transmitter UE. Therefore, it is imperative to support type 2 LBT (mainly type 2A and type 2B) for PSFCH transmission.

SL-U supports type 2 LBT for PSFCH transmissions.

COT sharing is allowed between the transmitter UE and the receiver UE for PSCCH/PSSCH and corresponding PSFCH transmissions, respectively.

For S-SSB transmissions, channel access for NR-U discovery burst can be used as a baseline. Meaning, type 2A LBT is supported for S-SSB transmissions.

SL-U supports type 2A LBT for S-SSB transmissions.

## 2.1	Channel access for wideband mode

As stated above, DL channel access procedures are the most relevant for SL transmissions. Therefore, for SL multi-channel operation, DL multi-channel approaches remain applicable. According to DL multi-channel access, there are two approaches. In the first approach, gNB runs parallel independent backoff procedures on each channel and each channel needs to complete the Type 1 LBT individually before performing simultaneous transmissions. Whereas, in the second approach, gNB runs a single random backoff procedure on one of the channels and CCA check is required on other channels just before the transmission. In our opinion, both the approaches are applicable to SL-U and should be studied.

DL multichannel approaches (Type A and Type B) are applicable to SL multichannel transmission.

Support both Type A and Type B DL multichannel approaches for SL multichannel access.

# 3	Mode-2 based resource allocation procedure

In this section, we discuss how the functionalities of mode-2 based resource allocation procedure of SL can be used together with CCA/LBT procedures mandated by regulations. To comply with CCA regulations of unlicensed spectrum and be able to reuse most of mode-2 based resource allocation procedure for SL, we believe that LBT is to be seen as a procedure that is applied on top. For example, CCA/LBT is performed before a transmission on the resources selected based on mode-2 SL resource allocation.

CCA/LBT procedure is applied on top of mode-2 based resource allocation procedure of SL.

One straightforward approach to apply CCA/LBT on top of mode-2 based resource allocation can be describe by the following steps:

- Step 1: A UE performs sensing and resource selection based on the resource selection procedures specified in SL Rel-16 (or Rel-17), to select resources for an initial transmission and possibly some retransmissions of a TB.

- Step 2: Before each selected resource, the UE performs CCA/LBT to determine whether it can transmit on the selected resource or not.

However, there are a few potential issues with such approach:

- A UE is bound to select/reserve a few resources and it may not be able to use some of them due to LBT failure.

- It combines the restrictions of both schemes without providing solutions to the new issues that arise. For example, it provides no mechanism to deal with the situation that the combination of LBT and Mode-2 resource exclusion prevent transmission altogether.

Our view is that LBT should not only be seen as an additional constraint, but as a new mechanism that can be advantageously used by SL-U. For example, to perform early transmissions when the channel is free. Unlike in Rel-16, skipping a slot in unlicensed spectrum may result in a long delay for the transmission (e.g., because a WiFi UE accesses the channel, etc.).

Applying LBT procedure on top of mode-2 resource allocation without any changes does not address new issues and does not exploit any possible advantages.

A second way to combine CCA/LBT and mode-2 is by not having a selection of the resource for the initial transmission at all (i.e., a selection based on mode-2 resource allocation and independent of LBT). Instead, a UE selects the resource for initial transmission as soon as it is available according to both mode-2 sensing and LBT procedures. Once the initial resource is selected, the UE can reserve more resources from the set of available resources and indicate the reservation as part of initial transmission. Such procedure would consist of the following steps:

- Step 1: A UE performs sensing based on the procedure specified in SL Rel-16 (or Rel -17) to identify a set of available resources. This set of resources is essentially the one determined by Clause 8.1.4 in TS 38.214 [2].

- Step 2: The UE starts performing CCA/LBT as soon as the packet arrives at the buffer and select the first available resource (from the set of available resources obtained in Step 1) when the channel is found to be available by the LBT procedure.

- Step 3: Following the Rel-16 procedure, the UE selects further resources (for retransmissions) from the set of available resources and sends the corresponding reservations as part of the transmission in the first selected resource (i.e., the one determined in Step 2).

The drawback of this approach is that since the resource selection for initial transmission is determined by LBT when applied on top of resource exclusions, the UE may have trouble to find resources, especially in congested channels. That is, resource exclusion (mode 2) may discard some resources that should have been considered as available once the restrictions introduced by LBT are considered too.

Selecting resources for initial transmission using a combination of LBT and resource exclusion using Mode-2 may result in excessive latency

Therefore, to reuse as much as Rel. 16 mode-2 based resource allocation procedure and exploit the advantage of early transmissions in unlicensed spectrum, we propose the following procedure.

- Step 1: A UE performs sensing and resource selection based on the resource selection procedures specified in SL Rel-16 (or Rel-17), to select resources for an initial transmission and possibly for some retransmissions of a TB.

- Step 2: The UE starts performing CCA/LBT as soon as the packet arrives at the buffer and in addition also selects the first available resource (from the set of available resources) when the channel is found to be available by LBT procedure. We call this as opportunistic transmission. In case the channel is not found to be available by LBT procedure before the initially selected resource, the UE waits to transmit on the initially selected resource.

With this procedure, we circumvent the drawbacks of the above-described alternative procedures and combine the benefits of both (i.e., early channel access, low specification impact and reusing mode-2 resource allocation procedure).

We illustrate this procedure in Figure 1 below.

Figure 1. Resource allocation procedure with LBT on top and opportunistic transmission. At time t=n, the UE selects 4 resources (in green) for transmission of a TB. The selection is made using the sensing results based on the legacy procedures. At the same time, the UE starts performing LBT. Prior to the first selected resource (leftmost green box), the UE completes the LBT procedure and finds an earlier resource (in orange) that is available for transmission according to the sensing results. This allows the UE to perform the first transmission earlier, in an opportunistic manner.

Based on the explanation above, we have the following proposal.

NR SL-U Mode 2 supports opportunistic transmission (i.e., early transmission) based on LBT success.

Furthermore, we observe that in Rel. 16 mode-2 based resource allocation procedure, the selection of resources from the set of available resources is done in a random fashion. Such procedure is problematic for the LBT procedure as the LBT counter (N) is frozen as soon as channel is identified to be busy by energy detection. It does not matter if the channel is completely occupied or partially occupied which is possible if subchannel size is smaller than the RBset (channel bandwidth). In other words, spreading different transmissions over time instead of concentrating them on a reduced number of slots is a bigger issue for LBT based on energy detection and it should be avoided. This is illustrated in Figure 2.

Having different transmissions spread over time instead of concentrating them in a reduced number of slots is problematic for energy detection based CCA procedure (LBT Type 1).

Figure 2. Left: With a 50% load, every slot has occupied resources (in gray) and LBT counter N can only be decreased during the silent GPs. Right: Transmissions are grouped in a few slots. Although the load is still 50%, having some unused slots (in white) allows for decreasing the LBT counter quicker.

To reduce the spread of different transmissions over time, we propose to adopt ‘frequency-first’ selection instead of random selection during resource selection procedure (step 1 above).

Specify ‘frequency-first selection’ during resource selection from the candidate of available resources.

The Rel-16 specifications include a re-evaluation procedure that allow a UE to reselect the already selected resources based on the updated sensing information when a collision is detected. We believe that this procedure should be extended to concentrate the transmissions over time.

The resource re-evaluation procedure is used to reduce the spread of transmissions in time prior to sending a reservation.

As mentioned above, LBT procedure is performed by the UE before transmitting on the selected resource. In this case, there are two possible cases: (1) LBT is successful before the selected resource, or (2) LBT is unsuccessful before the selected resource. In case (1), it is obvious that the UE will use the selected resource for transmission. However, in case (2), UE behaviour is not clear. That is, a UE can either skip its transmission and do not perform reselection of resource or a UE perform reselection of the resource.

In case of LBT failure before the selected resource, UE behaviour is unclear.

In our view, LBT failure before the selected resource can be seen like re-evaluation (for non-reserved resources, e.g., for initial transmission) and pre-emption (for already reserved resource). That means, whenever LBT failure occurs before the selected resource, resource reselection is performed by the UE.

LBT failure before the selected resource triggers resource re-selection.

## 3.1	Resource allocation modifications for wideband mode

In case of wideband operation, there is a mode where the carrier bandwidth is greater than the channel bandwidth (i.e., wideband mode 2). To operate SL-U in wideband mode 2, SL bandwidth part and SL resource pool can be configured to consist of multiple RBsets or channels. The organization of resources in unlicensed spectrum for SL operation is described in details in our companion contribution [2].

SL bandwidth part and SL resource pool can be (pre-)configured to be an integer multiple of RBsets or channels.

According to the existing SL mode 2 resource allocation, a set of resources is determined by a transmitting UE to be available within a resource selection window based on previous reservation signals by other UEs which are received during the sensing window. The set of available resources are confined within the resource pool configured to the UE. Once the set of available resources is determined, the UE selects resources to be used for (re)transmission in a random fashion. Therefore, it is possible that the resource is selected spans multiple channels. However, such way of resource allocation is not very efficient in wideband operation because of the following reasons:

- Transmission can only be performed on the selected resource when the LBT is successful in all the channels occupied by the resource. This increases the likelihood of LBT failure because the transmission cannot be performed on the selected resource if LBT fails in one of the occupied channels.

- If the selected resource spans on multiple LBT channels, it produces bandwidth fragmentation since CCA is performed on per channel and the whole sub-band will be determined to be as unavailable even if one subchannel is occupied.

Random resource selection increases the chances of LBT failures in case of wideband operation.

In our opinion, it is important to specify enhancements to Rel-16 resource selection procedure to operate on wideband mode of operation on unlicensed spectrum. One possible solution to address the problems described above is to specify an additional restriction that limits UE to select resources within one of the channels in the configured resource pool and avoid selecting resources spanning multiple channels unless needed for large TB sizes.

RAN1 specifies enhancements to resource selection for wideband mode such that the selected resources are confined within a single channel unless TB size demands otherwise.

# 4	Mode-1 based resource allocation procedure

The Mode-1 procedures introduced in Rel-16 include providing SL grants (DG, CG) and reporting of SL HARQ FB information to the gNB. These features may require modifications to accommodate potential changes to the SL procedures, but it is difficult to assess at this point what the impact is. Moreover, given that, per WID, Uu signalling for Mode 1 is exchanged on a licensed carrier, the changes will likely be small. Our proposal is to defer work on Mode 1 until the main changes of the SL interface are defined, especially for the SL HARQ protocol.

RAN1 to postpone the work on Mode 1 until the main changes of the SL interface are defined (e.g., SL HARQ protocol, SCI contents, etc.).

# 5	Evaluation methodology

RAN1 has defined an evaluation methodology for V2X UCs (in TR 37.885 [4]) with some additions for studying power saving aspects in Rel-17. Studying SL operation in unlicensed spectrum will require new evaluation assumptions covering at least:

- Deployment aspects

- Traffic model

- Interference model

Besides some of the material in TR 37.885, we believe that the NR-U assumptions in TR 38.889 [5] are a good starting point in some cases. In Table 1, we have collected some simulation parameters to be used for SL-U system level evaluations. In the coming sections we provide further details and justifications.

Table 1. System-level simulation parameters for SL-U.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 5GHz / 6 GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of UEs | 5x2 (5 pairs) Drop model: See Section 5.1 |
| Number of Interferers | 10 Drop model: See Section 5.1 |
| SCS | To be reported together with simulation results |
| Channel Model | NR InH Mixed Office model |
| UE/STA Tx Power | 23dBm for 5 GHz (total across all TX antennas) with PSD constraint 10 dBm/MHz |
|  | 14dBm for 6 GHz (total across all TX antennas) with PSD constraint 1 dBm/MHz |
| UE/STA Antenna gain | 0dBi |
| UE/STA Receiver Noise Figure | 9dB |
| UE receiver | MMSE-IRC as the baseline receiver |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | See Section 5.2 and Section 5.3 |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

The parameters in Table 1 are the baseline for system level evaluation of SL-U.

## 5.1	Deployment and UE drop models

The RAN1 deployment and UE drop models defined so far target V2X or PS. Our view is that most SL-U UCs will be indoor. For this reason, we think new deployment and UE drop models are necessary. We propose to reuse the indoor sub-7 GHz indoor scenario from TR 38.889, but without gNBs. This scenario is illustrated in Figure 3.

Figure 3. Indoor sub7 simulation office layout from [5].

We propose to randomly drop:

- 5 SL-U UE pairs (10 UEs in total) in the scenario. Each pair is separated by Uniform[10,25] m

- 10 interfering devices, randomly distributed Uniform in x and y ranges of the scenario

For SL-U evaluations the indoor model sub-7 GHz is used as the baseline:

- 5 SL-U UE pairs (10 UEs in total) in the scenario. Each pair is separated by Uniform[10,25] m

- 10 interfering devices, randomly distributed Uniform in x and y ranges of the scenario

- D2D channel model is InH office pathloss model with proper d_3D with indoor mixed office LOS probability

## 5.2	Traffic model

It is clear from the discussion in [3], that operation in 20 MHz channels is the baseline for Rel-18 work.

A single 20 MHz unlicensed channel is baseline for evaluations in Rel-18.

When defining evaluation assumptions, it is important to consider the best performance that the NR SL (Rel-16) can deliver, especially in terms of peak rates. We summarize this in Table 2. Note that in practice the achievable rates will be significantly lower due to several factors: retransmissions, collisions, inefficient utilization of the resources in Mode 2, gap between retransmissions, etc.

Table 2. Peak SL data rates with 20 MHz (50 RBs with SCS=30kHz)

|  | Peak rate (Mbps) |  |
| --- | --- | --- |
|  | With PSFCH overhead | Without PSFCH overhead |
| 64 QAM table | 49.1 | 69.632 |
| 256 QAM table | 65.5 | 92.208 |

Regarding traffic, we have summarized the main characteristics of some traffic models agreed in 3GPP in Table 3, including NR V2X traffic models from TR 37.885 [4], NR-U traffic models from [5](which are based on LTE-LAA traffic models in [6], which in turn are modified version of the models in [7]), and XR traffic models [8]. We note that:

- The Periodic 1 and Periodic 2 models and the Aperiodic 1 model agreed for NR V2X are not appropriate for modelling for the typical UCs that would be deployed over unlicensed spectrum.

- The Periodic 3 and Aperiodic 2 models, on the other hand, have some similarities with the VR/AR DL models from TR 38.838, although they present somewhat longer inter-arrival times, smaller packets, and lower rates.

- The AR/VR models were most often used in licensed bands (UL/DL) with channels of 100 MHz or more.

Table 3. Some traffic models agreed by 3GPP that are relevant for SL-U evaluation.

|  | Inter-arrival time |  | Packet size |  | Rate (avg) |
| --- | --- | --- | --- | --- | --- |
| Model | Model | Average | Model | Average |  |
| TR 37.885 Periodic 1 | Fixed | 100 ms | Fixed seq. 300+4*190 | 212 byte | 0.017 Mbps |
| TR 37.885 Periodic 2 | Fixed | 10 ms | Rand (0.8/0.2) 800, 1200 byte | 880 byte | 0.704 Mbps |
| TR 37.885 Periodic 3 | Fixed | 30 ms | Rand 30:10:60 kbyte | 45000 byte | 12 Mbps |
| TR 37.885 Aperiodic 1 | Fixed + Exp. RV | 100 ms | Rand 200:200:2000 byte | 1100 byte | 0.088 Mbps |
| TR 37.885 Aperiodic 2 | Fixed + Exp. RV | 20 ms | Rand 10:4:30 kbyte | 24000 byte | 9.6 Mbps |
| TR 38.838 VR/AR DL stream | Fixed + Truncated Norm. | 16.67 ms | Truncated Norm (50%-150% avg) | 62500, 93750 byte | 30, 45 Mbps |
| TR 38.838 CG DL stream | Fixed + Truncated Norm. | 16.67 ms | Truncated Norm (50%-150% avg) | 16667, 62500 byte | 8, 30 Mbps |
| TR 38.889 FTP Model 1 | Poisson arrivals | [2, 1, 0.67, 0.5, 0.4] s | Fixed | 0.5 Mbyte | [2, 4, 6, 8, 10] Mbps |
| TR 38.889 FTP Model 3 | Poisson arrivals | [2, 1, 0.67, 0.5, 0.4] s | Fixed | 0.5 Mbyte | [2, 4, 6, 8, 10] Mbps |

Our view is that the Periodic 3 and Aperiodic 2 models are suitable for SL-U evaluations. However, it is necessary to consider lower loads as well.

The Periodic 3 and Aperiodic 2 models in TR 37.885 are the baseline for SL-U evaluations. In addition to the parameters in TR 37.885 a low-load version is defined

- Periodic 3 with low load has packet size reduced by a factor of 5.

- Aperiodic 2 with low load has packet size reduced by a factor of 5.

## 5.3	Interference model

We think that modelling asynchronous interference is important to obtain good conclusions that drive system design. There exist multiple technologies that can operate in unlicensed spectrum (e.g., WiFi, BT, SL-U). Given that this WI does not have an objective on coexistence, we believe that each company can choose the interfering technology. What is crucial is that:

- the interference is asynchronous and that the different simulations consider the same distribution of load between SL-U and the interfering technology.

- similar load (SL-U/interfering) and low-load (interfering) situations are studied.

- the number of devices of both technologies is comparable (e.g., to avoid having a single interfering device that impacts a small part of the scenario).

Asynchronous interference is modelled. The choice of technology is up to each company.

For evaluating the impact of interference, two cases are studied:

- SL-U and interfering devices with the same traffic model.

- The traffic for SL-U UEs is 5 times bigger than for the interfering devices (e.g., normal load vs low load model).

For evaluating the impact of interference, at least the case that the number of SL-U devices equals that of interfering devices is considered.

# 6	Conclusion

In the previous sections we made the following observations:

Observation 1	From channel access perspective, SL transmission in SL-U is similar to DL transmissions in NR-U.

Observation 2	DL multichannel approaches (Type A and Type B) are applicable to SL multichannel transmission.

Observation 3	Applying LBT procedure on top of mode-2 resource allocation without any changes does not address new issues and does not exploit any possible advantages.

Observation 4	Selecting resources for initial transmission using a combination of LBT and resource exclusion using Mode-2 may result in excessive latency

Observation 5	Having different transmissions spread over time instead of concentrating them in a reduced number of slots is problematic for energy detection based CCA procedure (LBT Type 1).

Observation 6	In case of LBT failure before the selected resource, UE behaviour is unclear.

Observation 7	Random resource selection increases the chances of LBT failures in case of wideband operation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Dynamic channel access of NR-U is the baseline for SL-U.

Proposal 2	Reuse channel access rules (including COT sharing) for gNB (DL) transmissions in NR-U for transmissions in SL-U.

Proposal 3	SL-U supports type 1 LBT for PSCCH/PSSCH transmissions.

Proposal 4	Study further the applicability of type 2 (2A/2B/2C) LBT for PSCCH/PSSCH transmissions.

Proposal 5	COT is not shared for PSCCH/PSSCH transmissions between different UEs.

Proposal 6	SL-U supports type 2 LBT for PSFCH transmissions.

Proposal 7	COT sharing is allowed between the transmitter UE and the receiver UE for PSCCH/PSSCH and corresponding PSFCH transmissions, respectively.

Proposal 8	SL-U supports type 2A LBT for S-SSB transmissions.

Proposal 9	Support both Type A and Type B DL multichannel approaches for SL multichannel access.

Proposal 10	CCA/LBT procedure is applied on top of mode-2 based resource allocation procedure of SL.

Proposal 11	NR SL-U Mode 2 supports opportunistic transmission (i.e., early transmission) based on LBT success.

Proposal 12	Specify ‘frequency-first selection’ during resource selection from the candidate of available resources.

Proposal 13	The resource re-evaluation procedure is used to reduce the spread of transmissions in time prior to sending a reservation.

Proposal 14	LBT failure before the selected resource triggers resource re-selection.

Proposal 15	SL bandwidth part and SL resource pool can be (pre-)configured to be an integer multiple of RBsets or channels.

Proposal 16	RAN1 specifies enhancements to resource selection for wideband mode such that the selected resources are confined within a single channel unless TB size demands otherwise.

Proposal 17	RAN1 to postpone the work on Mode 1 until the main changes of the SL interface are defined (e.g., SL HARQ protocol, SCI contents, etc.).

Proposal 18	The parameters in Table 1 are the baseline for system level evaluation of SL-U.

Proposal 19	For SL-U evaluations the indoor model sub-7 GHz is used as the baseline:

	5 SL-U UE pairs (10 UEs in total) in the scenario. Each pair is separated by Uniform[10,25] m

	10 interfering devices, randomly distributed Uniform in x and y ranges of the scenario

	D2D channel model is InH office pathloss model with proper d_3D with indoor mixed office LOS probability

Proposal 20	A single 20 MHz unlicensed channel is baseline for evaluations in Rel-18.

Proposal 21	The Periodic 3 and Aperiodic 2 models in TR 37.885 are the baseline for SL-U evaluations. In addition to the parameters in TR 37.885 a low-load version is defined

	Periodic 3 with low load has packet size reduced by a factor of 5.

	Aperiodic 2 with low load has packet size reduced by a factor of 5.

Proposal 22	Asynchronous interference is modelled. The choice of technology is up to each company.

Proposal 23	For evaluating the impact of interference, two cases are studied:

	SL-U and interfering devices with the same traffic model.

	The traffic for SL-U UEs is 5 times bigger than for the interfering devices (e.g., normal load vs low load model).

Proposal 24	For evaluating the impact of interference, at least the case that the number of SL-U devices equals that of interfering devices is considered.

# 7	References

RP-213678, New WID on NR sidelink evolution, OPPO and LG Electronics, RAN#94e, Dec.6-17, 2021.

3GPP, “TS 38.214: Physical layer procedures for data”

R1-2204742, PHY channel design framework for SL-U, Ericsson, RAN1#109e, May 9-20, 2022.

TR 37.885, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR”

TR 38.889, “Study on NR-based access to unlicensed spectrum”

TR 36.889, “Study on Licensed-Assisted Access to Unlicensed Spectrum”

TR 36.814, “Further advancements for E-UTRA physical layer aspects”

TR 38.838, “Study on XR (Extended Reality) Evaluations for NR”