3GPP TSG-RAN WG1 Meeting #109-e	R1- 2204742

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.1.2

Source:	Ericsson

Title:	PHY channel design framework for SL-U

Document for:	Discussion, Decision

# 1	Introduction

In RAN#94e, a new WID on NR sidelink evolution was agreed [1]. Objective # 2 of the agreed WID aims to study and specify the operation of sidelink on unlicensed spectrum. The details of the objective as agreed is stated below.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this paper, we discuss the physical channel design framework including system aspects, physical channel structures and physical layer procedures.

# 2		System aspects

## 2.1	Organization of resources

In NR SL, time and frequency resources are configured to the UE in the form of resource pool configuration. Furthermore, subchannel is defined as part of resource pool configuration and determines the minimum scheduling granularity for PSSCH transmission in the frequency domain. Per WID, existing SL features should be used as a baseline. Therefore, we propose to reuse the definition of resource pools and subchannels as specified for NR SL to a large extent. However, it is not clear how the subchannels are defined in case of interlaced transmissions. We propose to further study the impact of interlacing on the definition of subchannel.

RAN1 reuses NR SL definition of resource pools and subchannels for the operation of SL in unlicensed spectrum. FFS impact of interlacing and relationship with the definition of subchannel.

However, there are a couple of configuration restrictions that must be assumed for SL-U.

- The size of the resource pool in the frequency domain cannot be smaller than a channel size (e.g., 20 MHz) on an unlicensed band. This is because LBT is performed per channel (e.g. 20 MHz) basis and there is no advantage of configuring a resource pool smaller than a channel size.

- The subchannel size cannot exceed a channel size (e.g., 20 MHz) on an unlicensed band. This is because subchannel size exceeding a channel size will restrict the coexistence of UEs supporting different bandwidths and produce bandwidth fragmentation because LBT is performed per channel.

On the other hand, the size of resource pool in frequency domain can exceed a channel size and can be configured to span integer multiple channels in case of wideband mode-2 SL-U operation (see Section 2.2). Similarly, the subchannel size can be smaller than a channel size of unlicensed band. In this case, UE may not use all the resources within the channel (or RBset defined in NR-U). This can be useful to enable resource efficient operation of SL-U and reduce hidden node problems by randomizing the selection of different subchannels in the same slot. Consequently, we propose to support FDM of SL transmissions from system perspective.

SL-U transmissions from different UEs can be FDMed within the same channel of unlicensed band.

## 2.2	Modes of operation

Besides 20 MHz operation, NR-U also supports wideband operation (> 20 MHz) which is based on either aggregation of multiple carriers (aka. wideband mode 1) or single carrier (aka. wideband mode 2). However, listen-before-talk (LBT) or clear channel access (CCA) is performed for each 20 MHz channel.

CCA (aka. LBT) is performed for each channel (i.e., 20 MHz).

In wideband mode 1, each channel is as an individual carrier and aggregation of multiple carriers is performed following the carrier aggregation (CA) procedures of NR Uu. In our opinion, this is the simplest way of enabling wideband operation in SL-U and should be supported. However, there are two aspects that must be considered:

- The specification of NR SL CA can only begin after further checking the progress of the WI by RAN plenary in RAN#97.

- The objective on SL-CA is restricted to FR1 licensed spectrum and the ITS band in FR1. It is important that RAN WGs specify the SL CA operation first and then check its applicability for the SL-U operation of wideband mode-1.

Given this, we propose to postpone the work on SL-U wideband mode 1.

Wideband mode 1 (based on CA procedures) is the simplest mode to support wideband (>20 MHz) SL-U operation.

RAN WGs postpone the work on SL-U wideband mode 1 until SL CA (objective 1 of WID) is specified.

For wideband mode 2, it is considered that the carrier has bandwidth greater than the channel (20 MHz) as defined for unlicensed bands (i.e., carrier bandwidth > channel bandwidth). To support this mode of operation for SL-U, at least the following aspects need to be considered:

- Interoperability of SL UEs supporting different bandwidths. In SL-U, it is possible that not all the UEs support wideband operation. Therefore, mechanisms so that different UEs can interpret and decode each other correctly should be considered. This is further discussed in Section 4.2.

- Modifications to resource allocation procedures. As mentioned above, CCA is performed per channel. Therefore, it should be considered while studying and specifying the resource allocation mechanism. This is further discussed in our companion contribution [2].

Based on this, we propose the following:

NR SL-U supports wideband mode-2.

- RAN1 to study the interoperability of UEs supporting different bandwidths and enhancements to the resource allocation procedures for wideband operation.

Discussions on these aspects are in the corresponding sections.

# 3	Physical layer structure

In this section, we describe the physical layer structures for the operation of SL-U.

## 3.1 	SL slot length

For NR operation in unlicensed spectrum, it is desirable to use short slots. This not only allows for reduced transmission latency but also increases the number of transmission opportunities.

One way to realize this is by using mini slots. That is, having PHY transmission structures with different durations (i.e., number of symbols). However, having transmissions of different lengths (by different TX UEs) within a slot creates sudden changes in RX power which results in need for retuning the AGC. For these reasons, mini-slot operation is not supported in SL. Therefore, we propose not to support mini-slot for SL-U either.

Mini-slot is not supported in SL-U

The other way to reduce transmission latency and increase the number of transmission opportunities in SL-U is to configure shorter SL slot lengths. Since this is configured at system level, the starting and ending times of simultaneous SL transmissions are the same, avoiding additional AGC issues. Note that in practice, the only feasible short SL slot length is 7 symbols. A similar effect can be achieved by using a higher subcarrier spacing (SCS) without any extra specification impact. In fact, operation with higher SCS would be more efficient as the fixed overhead (AGC, GP) would be reduced from 2/7 (short SL slot) to 2/14 (increased SCS).

For having shorter SL-U transmissions, using higher SCS is preferable over configuring a shorter SL slot length (i.e., 7 OFDM symbols).

Based on this, we propose the following:

SL-U uses a SL slot with 14 OFDM symbols (including AGC, GP, etc., if supported).

## 3.2	Multi-slot transmissions

In unlicensed spectrum, a transmission is associated with a prior channel access mechanism, i.e., LBT or CCA procedure. If the channel is busy, the transmission will not take place. Therefore, it is desirable to try to minimize the number of independent transmissions. For example, by making longer transmissions conveying larger payloads.

In unlicensed spectrum, it is desirable to minimize as much as possible the number of channel access procedures when performing multiple transmissions.

On this regard, there are two potential approaches that can be done reusing as much as possible – per WID instructions – the legacy SL procedures and channel structures:

- Transmission of the same TB in consecutive slots: Using this scheme the same TB is transmitted using a blind retransmission scheme in consecutive slots to reduce the likelihood of having LBT failure for that particular transmission.

- Transmission of different TBs in consecutive slots: Using this scheme several transmissions can be grouped and instead of performing independent channel access mechanisms, i.e., one channel access mechanism for each individual transmission, it is possible to transmit several transmissions where only one channel access mechanism is needed, i.e., for the first transmission of the group.

In our view it is important to study the transmission in consecutive slots of the same TB and/or different TBs from the same UE to assess the potential gains for SL-U operation.

Study the transmission of multiple or a single TB by one UE in consecutive slots to reduce the number of channel access attempts.

To enable the transmission of several TBs or the same TB in consecutive slots, it is desirable to reuse as much as possible the structure and format used for resource allocation in the legacy NR SL in Rel-16 and Rel-17. In our view, it is possible to reuse the legacy fields in SCI format 1-A used for selection and reservation of resources to perform multiple transmissions in consecutive slots.

For consecutive transmission of the same TB or multiple TBs, it is possible to reuse the fields used for selection and reservation of resources in legacy SCI format 1-A.

## 3.3	Physical channels

In this section, we describe our view on the impact and enhancements to physical channel design for SL operation in unlicensed spectrum.

In general terms, the structure of the PHY channels introduced in Rel-16 (PSCCH/PSSCH, PSFCH, S-SSB) is also suitable for operation in unlicensed spectrum. However, the impact of the regulations (esp. channel access) must be studied. We discuss several aspects in the following sections.

The legacy structure of the PHY channels and the S-SSB is suitable as a baseline for SL operation in unlicensed spectrum.

### 3.3.1	Interlacing

To maintain the high output power and at the same time comply to local regulations on maximum EIRP, power spectral density, and occupied channel bandwidth, transmission on a block-interlaced structure is supported in NR-U for uplink in addition to the legacy non-interlaced structure. According to our analysis, the performance of SL is also affected due to limitations on power spectral density, especially for transmission on a small bandwidth. Figure 1 below shows the decrease in packet reception ratio (PRR) when applying transmit power limitations in comparison to transmissions with the total transmit power of the UE. In the simulations, we consider a 5 MHz allocation, which will result in 16.99 dBm transmit power for 5GHz operation (n46) and 7.99 dBm transmit power for 6GHz (n96/n102) operation.

Figure 1: Performance impact due to transmit power limitation on unlicensed bands

Power spectral density limitation for unlicensed bands (n46/n96/n102) results in significant decrease in performance.

Based on this, it is quite clear that interlacing needs to be considered for the operation of SL on unlicensed spectrum, at least for PSCCH/PSSCH transmission.

SL-U supports interlaced transmissions for PSCCH and PSSCH.

For PSFCH/S-SSB, supporting interlaced transmissions is the most straightforward way to satisfy PSD and OCB requirements. However, the gains of interlacing PSFCH and S-SSB in terms of performance require careful investigation. Therefore, we propose to study further the need of supporting interlacing for PSFCH and S-SSB.

Study further the need of supporting interlacing for PSFCH and S-SSB.

### 3.3.2	Channel access

In our contribution [2], we explained the channel access rules that are applicable to different physical channels for sidelink. In summary, we observe that SL transmissions are very similar to DL transmissions from the channel access perspective and the DL channel access rules of NR-U are the baseline to enable SL-U.

Reuse channel access rules for DL transmissions in NR-U for transmissions in SL-U.

Furthermore, we believe that the COT sharing between UEs for data transmissions should not be allowed. However, COT sharing can be a useful operation when it is done between UEs for data and the corresponding HARQ feedback transmissions.

COT is not shared for PSCCH/PSSCH transmissions between UEs.

COT is shared for PSCCH/PSSCH and the related PSFCH transmissions between UEs.

Based on the above, it is evident that the use of LBT type 1 and LBT type 2 (2A/2B) is rather a straightforward operation for the SL data transmission and the corresponding HARQ feedback, respectively. However, the need of LBT type 2 SL data transmission and LBT type 1 for HARQ feedback is not obvious and we propose to study their need.

LBT type 1 and LBT type 2 are the baseline CCA mechanism for SL data transmissions and the corresponding HARQ feedback, respectively. FFS the need of LBT type 2 for SL data transmission and LBT type 1 for HARQ feedback transmission.

### 3.3.3	Impact on AGC

In SL, the first symbol of a transmission is always used as an AGC training symbol. For SL-U, we propose to reuse this structure i.e., first symbol of a PSCCH/PSSCH or a PSFCH is a repetition of the next symbol and is used for training the ACG of the RX UE..

First symbol of a PSCCH/PSSCH or a PSFCH is duplicated and used as an AGC symbol.

Given the AGC training happens to be in the beginning of reception, it is not desirable to have changing receive signal power during the reception. Therefore, transmissions by all the devices operating on the unlicensed band must be aligned to avoid AGC saturation or quantization noise. In case LBT Type 1 is performed before the transmission (in compliance to unlicensed band regulation), a UE can choose not to decrement the LBT counter (N) to ensure that the transmission can only start at the slot boundary. For LBT Type 2, there is no issue to ensure that a transmission can only start at a designated time.

Regulation allows a UE to choose not to decrement the LBT counter N and this can be used to ensure the start of the transmission at the appropriate times.

Within an NR slot, SL-U transmissions can only start at the predefined times from Rel-16 (e.g., start of the slot for PSCCH/PSSCH, first OFDM symbol for PSFCH, etc.).

### 3.3.4	Impact on GP

In SL, a guard period is used to allow TX-RX switching. However, in case of transmission burst spanning multiple slots, the presence of the GP at the end of each slot will require UE to perform CCA/LBT before each transmission. Note that in unlicensed band, transmission burst is defined as a set of transmissions (from UE or gNB) without any gaps greater than 16 ms and transmission after a gap (< 16 ms) within the transmission burst can be performed without performing additional CCA. With the presence of GP of 1 OFDM symbol (66.67 ms for 15 kHz, 33.3 ms for 30kHz, and 16.67 kHz for 60 kHz plus their corresponding CPs), it is not possible to consider two or more consecutive transmissions from the same UE as a transmission burst.

SL transmissions in consecutive slots from the same UE cannot be considered as a transmission burst due to the presence of the GP.

To avoid unnecessary CCA, we propose to specify enhancements on the usage of GP depending on the scenario. For example, the GP is not used to maintain an uninterrupted channel occupancy by the UE over consecutive slots as shown in Figure 2. Given that the presence/absence of GP needs to be considered by receiving UE, it should be indicated as part of SCI.

Specify enhancements to dynamically use the GP symbol.

Figure 2: GP is not used while performing SL transmission over consecutive slots.

# 4	 Physical layer procedures

## 4.1	Transmission of HARQ feedback

In NR SL, dedicated resources for HARQ FB transmissions are configured in a resource pool. In the slot domain, if HARQ FB is allowed by the configuration, the PSFCH resources can be configured in every Nth slot, where N = 1,2,4. These time resources are often referred to as PSFCH occasions. In the PRB domain, a set of contiguous PRBs in the PSFCH occasions are used for PSFCH (the set can be the all the PRBs of the resource pool).

The Rel-16 approach for configuring PSFCH resources achieves at least two goals: 1) ensuring that all SL-U UEs have a common understanding of HARQ FB resources, therefore simplifying the HARQ FB protocol, and 2) avoiding interference from non-HARQ FB SL transmissions to HARQ FB transmissions, thereby improving HARQ FB reliability. Given those benefits, in our view, there is no need to change the existing way of configuring PSFCH resources in the slot domain. In the PRB domain, due to the potential need of interlacing or other mechanism to meet the OCB requirement, how to configure PSFCH resources can be further studied.

Resources for HARQ FB transmissions in SL-U are configured in pre-determined slots as in NR SL Rel-16. FFS configuration in the PRB domain.

While having dedicated PSFCH resources in a resource pool helps minimize interference from other (non-PSFCH) SL transmissions to PSFCH, it cannot prevent non-SL devices from using the resources. As a matter of fact, a SL UE may fail to grab the channel (i.e., LBT failure) to transmit a PSFCH if there is already an ongoing non-SL transmission. In this case, if we reuse the PSFCH procedure in SL Rel-16, the PSFCH that cannot be transmitted due to LBT failure will never be transmitted. This is because in SL Rel-16, there is only one PSFCH occasion associated with each PSCCH/PSSCH transmission. Such situation can be detrimental to SL performance for several reasons:

- At system level, the load may be increased because the lack of HARQ FB is often assumed as a NACK by the UE. This will result in an increase in PRR.

- At UE level, the absence of NACK will trigger an adaptation (i.e., increase of the contention window). This will result in an increase in latency.

Therefore, changes to the PSFCH procedure are needed. In our view, the most efficient way to mitigate the impact of LBT failure on PSFCH transmissions is to allow a more flexible timing relation between a PSSCH and its associated PSFCH. Specifically, each PSSCH may be allowed to be acknowledged by more than one PSFCH occasion. An example is given in Figure 3, where there are up to 3 PSFCH occasions to acknowledge one PSCCH/PSSCH. In this example, the first two PSFCH occasions cannot be used due to LBT failures, only the last occasion is used. Details on how many PSFCH occasions are allowed and how to configure/signal the number of PSFCH occasions are subject to further study. Another benefit of having multiple chances to transmit a PSFCH is that if a HARQ FB cannot be transmitted in one PSFCH occasion (e.g., due to lower priority compared to another HARQ FB), it can be transmitted in a later occasion. Such possibility is not allowed by the legacy Rel-16 design.

The Rel-16 SL HARQ FB procedure is vulnerable to LBT failure prior to PSFCH transmissions and may force some SL HARQ FB to be dropped unnecessarily.

For SL-U, there are multiple PSFCH occasions to acknowledge a PSCCH/PSSCH transmission. FFS details of signalling, configuration, UE behaviours, etc.

Figure 3: Example of multiple (three) PSFCH occasions associated with one PSCCH/PSSCH. In this example, the Rx UE can only send the HARQ FB in the last PSFCH occasion due to LBT failure in the first two PSFCH occasions.

As presented in Section 3.3.2, we envision the use of COT sharing between a PSSCH and its associated PSFCH. Such COT sharing operation is illustrated in Figure 4.

For a pair of Tx UE – Rx UE:

- The Tx UE can share a COT (by means of PSCCH/PSSCH transmission) to the Rx UE for HARQ FB transmission to the Tx UE.

Figure 4: COT shared from PSCCH/PSSCH to the associated PSFCH. The solid arrows denote resource reservation.

Alternatively, the COT may be shared by the UE transmitting the PSFCH to the UE performing the retransmission. This is illustrated in Figure 5.

The UE transmitting a PSFCH can share the COT to speed up channel access for the retransmission.

Figure 5. The UE sending the HARQ FB on PSFCH shares the COT with the UE to allow for efficient retransmission.

As for the physical format of the HARQ FB channel, we believe the same PSFCH format as in NR SL (i.e., sequence based, derived from NR PUCCH format 0) can be reused. Using two consecutive OFDM symbols in a PSFCH occasion for PSFCH also suffices. The only issue that needs to be studied is whether interlacing should be used for PSFCH or any other mechanism to meet the OCB requirement is specified.

SL-U reuses the PSFCH format of NR SL. FFS the need of interlacing or any other change to meet OCB requirements.

## 4.2	Wideband procedures

Different UEs may support different maximum bandwidth. For example, some high capability UEs can support wideband operation (e.g., 80 MHz) and some UEs can support only single channel operation (e.g., 20 MHz). Existence of different capability UEs may result in interoperability problem due to distributed nature of the SL operation. Therefore, it is important that different capability UEs can coexist in the same resource resources pool without causing any performance degradation.

Different capability UEs must coexist in the same resource pool without causing performance degradation.

As mentioned in Section 2.1, a resource pool is configured to a UE such that it is within the SL bandwidth part and eventually within the bandwidth supported by the UE. Therefore, a resource pool configured for wideband UE can be larger as compared to the resource pool configured for a narrowband UE. For example, in Figure 5 below, a wideband UE supporting 80 MHz can be configured with a multichannel resource pool, whereas a narrowband UE supporting 20MHz can only be configured with a single channel resource pool.

Figure 6: Multichannel and single channel resource pool

According to Rel. 16 NR SL procedure, many SCI fields (e.g., FDRA field) are determined based on the number of subchannels in the resource pool. Therefore, if different UEs have different resource pool sizes, it will result in increased blind decoding as the SCI sizes used by the UEs will be different. Hence, we propose RAN1 to study mechanisms to cater issues caused due to different resource pool sizes used by different UEs.

RAN1 studies and specifies mechanisms to tackle issues caused by different overlapping resource pool sizes used by different UEs.

Also, when the transmission of a TB spans multiple channels, there exist a problem that Rx UE may not be able to receive the SCI indicating the occupied resource allocation if it happens to be in the different channel than the one on which the Rx UE is operating. Therefore, it is important that an enhancement to resource indication via SCI is studied and specified.

Rel-16 resource indication via SCI has limitation when operating in wideband mode 2 for transmissions spanning multiple channels.

To address this issue, we propose RAN1 to specify procedures that allow repetition of (partial or complete) SCI in case a transmission spans more than one channel.

RAN1 studies and specifies procedure to support SCI repetition over multiple channels in case a transmission spans more than one channel.

Furthermore, to enable communication between UEs with different bandwidth capabilities, it is important that the Tx UE receives the radio access capability of the Rx UE (in a unicast pair) and configures the logical channels associated with the Rx UE with a set of channels, whereas the set of channels is determined from the Rx UE capability information. In other words, data transmission from the Tx UE should be confined within the bandwidth (or channels) supported by the Rx UE. Therefore, we believe that such procedure should be specified. In case of groupcast (and broadcast, if considered for SL-U) mode, the use of TX profile framework specified in NR SL Rel. 16 is a relevant framework to resolve the interoperability issue in wideband mode.

RAN2 procedures for the logical channel restriction and TX profile framework are needed to address the interoperability issue in wideband operation.

## 4.3	Synchronization procedure

The Rel-16 procedure for synchronization has two main components:

- The definition of a top priority synchronization reference (i.e., GNSS or Network) along with a list of priorities for other synchronization references.

- SL signaling (i.e., S-SSB) used for propagating synchronization information between UEs.

SL operating in unlicensed bands targets many use cases, most of them in environments without GNSS coverage (e.g., indoor). Thus, it is not reasonable to expect that GNSS is a widely available synchronization reference. Similarly, we do not expect that SL in unlicensed spectrum is always deployed or used in connection with a network. In some cases, SL-U will be operated by a UEs with a network connection, but in many other cases it will not. Thus, it is not reasonable to expect that network is a widely available synchronization reference.

In many practical situations, the UEs may not have access to GNSS or network as synchronization references.

Although the existing procedures allow for operation when neither GNSS nor network are available as synchronization references at all, they are not designed for this purpose. Thus, we believe that RAN1 should study potential changes to the SLSS procedure to improve its efficiency in terms of energy and stability.

RAN1 to study enhancements to the SLSS procedure to improve its energy efficiency and stability for operation without access to GNSS/network synchronization references.

Besides this, RAN1 should consider the impact of operating on unlicensed spectrum on the SLSS procedure. More specifically:

- A transmitter may not gain access to the channel at pre-defined times.

- A receiver may have to relax the assumptions regarding the arrival of S-SSB.

Our view is that some of the tools used for NR-U can be considered for dealing with the channel access issues such as failed LBT outcomes. In particular, the use of discovery bursts (i.e., repetitions of S-SSB within a 160 ms period) and defining a S-SSB transmission window (i.e., a set of slots in which S-SSB may be transmitted), provided channel access is granted. We illustrate these two enhancements in Figure 6.

Figure 7. Top: legacy SLSS procedure. Middle: A single S-SSB transmission happens every 160 ms, but in one within a window. Bottom: A UE is expected to transmit S-SSB twice every 160 ms in two consecutive slots within a window.

Regarding channel access, we believe that COT sharing between users, for transmission of S-SSB on slots is useful. That is, UE1 gets access to the channel in slot k and transmits S-SSB. This transmission shares the COT with UE2, so that UE transmits S-SSB in a following slot (e.g., in slot k+1). Other channel access aspects related to S-SSB are discussed in [2].

Figure 8. COT sharing between transmissions of S-SSB. UE1 gets access to the channel and transmits S-SSB in the first slot. The COT is shared with a second UE that transmits S-SSB (to be received by UE1) and returns the COT. A repetition of each transmission takes places in the third and fourth slots.

To adapt the SLSS procedure to unlicensed channel access the following options are introduced:

- Repetitions of S-SSB within a S-SSB period (160 ms).

- S-SSB transmission within a window (i.e., S-SSB window) if LBT is successful.

- COT sharing between users for transmission of S-SSB.

One further issue to be discussed is whether S-SSB can be multiplexed with PSCCH/PSSCH transmissions. In the past, S-SSB was transmitted on dedicated time resources, meaning that S-SSB was not multiplexed in frequency with other transmissions. In our view, there are strong reasons to keep operation this way:

- During S-SSB slots, UEs are expected to transmit or receive S-SSB. Given that synchronization is essential for proper operation, we expect that UEs will not drop S-SSB reception when there are RX/TX conflicts. Thus, multiplexing of S-SSB with PSCCH/PSSCH is only useful if the TX UE is transmitting S-SSB and the intended RX UEs(s) are receiving S-SSB in the same slot. However, this cannot be guaranteed because the choice of slot for transmitting S-SSB depends on the synchronization procedure.

- Power sharing between S-SSB and PSCCH/PSSCH is not desirable given the importance and characteristics of the former (e.g., no HARQ retransmissions, etc.).

- S-SSB transmissions spanning a large part of the 20 MHz channel will have to be discussed (e.g., by means of interlacing or repetitions). A common solution is required for all UEs, regardless of whether they have data to transmit PSCCH/PSSCH multiplexed with S-SSB or not.

From a UE perspective, S-SSB is not FDMed with other SL transmissions.

### 4.3.1	Wideband aspects

The unlicensed bands comprise multiple channels. In principle, it is possible to access each of them independently, in an asynchronous manner. However, this is not desirable for a highly synchronized system like NR. Thus, we believe that RAN1 should only consider the case that, from SL-U perspective the different channels are synchronized. This would focus the work in RAN1 and, although it would not prevent having asynchronous SL-U in different channels in theory, it would result in a specification that is designed for synchronous operations.

RAN1 assumes that SL-U operates synchronously on all the channels of an unlicensed band.

As discussed earlier, we believe that the SLSS procedure will play a central role in SL-U operation. Thus, it is critical that all UEs are synchronized, regardless of their TX/RX capabilities. This means that synchronization should be available for UEs that are receiving on a single channel as well as on UEs that are receiving on multiple/all channels. There are two possibilities to achieve this: i) restrict S-SSB transmission to a given channel; ii) require a UE to transmit S-SSB on all channels in which it can do so (i.e., based on TX capabilities, LBT outcome, etc.). In our view, having a predefined channel for S-SSB transmission (and potentially many other pieces of signalling) is not desirable as it would result in an asymmetric utilization of the channel. We think that option ii) works better.

A UE transmits S-SSB on all channels if its capabilities support it and channel access is granted.

# 5	Conclusion

In the previous sections we made the following observations:

Observation 1	CCA (aka. LBT) is performed for each channel (i.e., 20 MHz).

Observation 2	Wideband mode 1 (based on CA procedures) is the simplest mode to support wideband (>20 MHz) SL-U operation.

Observation 3	For having shorter SL-U transmissions, using higher SCS is preferable over configuring a shorter SL slot length (i.e., 7 OFDM symbols).

Observation 4	In unlicensed spectrum, it is desirable to minimize as much as possible the number of channel access procedures when performing multiple transmissions.

Observation 5	For consecutive transmission of the same TB or multiple TBs, it is possible to reuse the fields used for selection and reservation of resources in legacy SCI format 1-A.

Observation 6	The legacy structure of the PHY channels and the S-SSB is suitable as a baseline for SL operation in unlicensed spectrum.

Observation 7	Power spectral density limitation for unlicensed bands (n46/n96/n102) results in significant decrease in performance.

Observation 8	First symbol of a PSCCH/PSSCH or a PSFCH is duplicated and used as an AGC symbol.

Observation 9	Regulation allows a UE to choose not to decrement the LBT counter N and this can be used to ensure the start of the transmission at the appropriate times.

Observation 10	SL transmissions in consecutive slots from the same UE cannot be considered as a transmission burst due to the presence of the GP.

Observation 11	The Rel-16 SL HARQ FB procedure is vulnerable to LBT failure prior to PSFCH transmissions and may force some SL HARQ FB to be dropped unnecessarily.

Observation 12	The UE transmitting a PSFCH can share the COT to speed up channel access for the retransmission.

Observation 13	Different capability UEs must coexist in the same resource pool without causing performance degradation.

Observation 14	Rel-16 resource indication via SCI has limitation when operating in wideband mode 2 for transmissions spanning multiple channels.

Observation 15	RAN2 procedures for the logical channel restriction and TX profile framework are needed to address the interoperability issue in wideband operation.

Observation 16	In many practical situations, the UEs may not have access to GNSS or network as synchronization references.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN1 reuses NR SL definition of resource pools and subchannels for the operation of SL in unlicensed spectrum. FFS impact of interlacing and relationship with the definition of subchannel.

Proposal 2	SL-U transmissions from different UEs can be FDMed within the same channel of unlicensed band.

Proposal 3	RAN WGs postpone the work on SL-U wideband mode 1 until SL CA (objective 1 of WID) is specified.

Proposal 4	NR SL-U supports wideband mode-2.

	RAN1 to study the interoperability of UEs supporting different bandwidths and enhancements to the resource allocation procedures for wideband operation.

Proposal 5	Mini-slot is not supported in SL-U

Proposal 6	SL-U uses a SL slot with 14 OFDM symbols (including AGC, GP, etc., if supported).

Proposal 7	Study the transmission of multiple or a single TB by one UE in consecutive slots to reduce the number of channel access attempts.

Proposal 8	SL-U supports interlaced transmissions for PSCCH and PSSCH.

Proposal 9	Study further the need of supporting interlacing for PSFCH and S-SSB.

Proposal 10	Reuse channel access rules for DL transmissions in NR-U for transmissions in SL-U.

Proposal 11	COT is not shared for PSCCH/PSSCH transmissions between UEs.

Proposal 12	COT is shared for PSCCH/PSSCH and the related PSFCH transmissions between UEs.

Proposal 13	LBT type 1 and LBT type 2 are the baseline CCA mechanism for SL data transmissions and the corresponding HARQ feedback, respectively. FFS the need of LBT type 2 for SL data transmission and LBT type 1 for HARQ feedback transmission.

Proposal 14	Within an NR slot, SL-U transmissions can only start at the predefined times from Rel-16 (e.g., start of the slot for PSCCH/PSSCH, first OFDM symbol for PSFCH, etc.).

Proposal 15	Specify enhancements to dynamically use the GP symbol.

Proposal 16	Resources for HARQ FB transmissions in SL-U are configured in pre-determined slots as in NR SL Rel-16. FFS configuration in the PRB domain.

Proposal 17	For SL-U, there are multiple PSFCH occasions to acknowledge a PSCCH/PSSCH transmission. FFS details of signalling, configuration, UE behaviours, etc.

Proposal 18	For a pair of Tx UE – Rx UE:

	The Tx UE can share a COT (by means of PSCCH/PSSCH transmission) to the Rx UE for HARQ FB transmission to the Tx UE.

Proposal 19	SL-U reuses the PSFCH format of NR SL. FFS the need of interlacing or any other change to meet OCB requirements.

Proposal 20	RAN1 studies and specifies mechanisms to tackle issues caused by different overlapping resource pool sizes used by different UEs.

Proposal 21	RAN1 studies and specifies procedure to support SCI repetition over multiple channels in case a transmission spans more than one channel.

Proposal 22	RAN1 to study enhancements to the SLSS procedure to improve its energy efficiency and stability for operation without access to GNSS/network synchronization references.

Proposal 23	To adapt the SLSS procedure to unlicensed channel access the following options are introduced:

	Repetitions of S-SSB within a S-SSB period (160 ms).

	S-SSB transmission within a window (i.e., S-SSB window) if LBT is successful.

	COT sharing between users for transmission of S-SSB.

Proposal 24	From a UE perspective, S-SSB is not FDMed with other SL transmissions.

Proposal 25	RAN1 assumes that SL-U operates synchronously on all the channels of an unlicensed band.

Proposal 26	A UE transmits S-SSB on all channels if its capabilities support it and channel access is granted.

# 6	References

RP-213678, New WID on NR sidelink evolution, OPPO and LG Electronics, RAN#94e, Dec.6-17, 2021.

R1-2204741, Channel access mechanism for SL-U, Ericsson, RAN1#109e, May 9-20, 2022.

3GPP “TS 37.213: Physical layer procedures for shared spectrum channel access”