3GPP TSG-RAN WG1 Meeting #109-e	R1-2204743

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.4.5

Source:	Ericsson

Title:	Other aspects related to SL-U

Document for:	Discussion, Decision

# 1	Introduction

In RAN#94e, a new WID on NR sidelink evolution was agreed [1]. Objective # 2 of the agreed WID aims to study and specify the operation of sidelink on unlicensed spectrum. The details of the objective as agreed is stated below.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this paper, we discuss other aspect of NR SL in unlicensed spectrum, including the applicability of certain NR SL features.

# 2	Casting types

In NR SL Rel-16 and Rel-17, different casting types (i.e., unicast, groupcast, and broadcast) are supported catering variety of use cases originating mainly from automotive and public safety. In Rel-18, for operation of sidelink in unlicensed bands, the focus of the WID is mainly on commercial services (e.g., XR) that require high data rate and can benefit from the use of unlicensed spectrum.

Main use cases for SL-U originates from commercial services such as XR.

For such commercial services, unicast (on physical layer) is the main transmission type that should be considered as a baseline for the design of SL-U. Therefore, we propose that RAN WGs focus on unicast transmissions to enable SL on unlicensed spectrum. Furthermore, there can be some commercial use cases which can justify the use of groupcast/multicast communication [2]. However, it is not clear what is the advantage of using groupcast option 1 for commercial use cases. Besides, information about location can be difficult to obtain and/or not very useful in many non-V2X scenarios. Therefore, we propose to focus only on groupcast option 2.

RAN WGs consider unicast and groupcast option 2 as the design baseline for SL-U.

From [2], the use of SL broadcast for commercial services is not well justified. Therefore, we propose the following.

Further study the need of supporting broadcast for SL operation on unlicensed spectrum.

# 3 	RLF/RLM procedure

In NR SL Rel. 16, no RLM procedure exists as there are no periodic reference signals that can be used for measurement. Therefore, there is no explicit indication to higher layers regarding IS/OOS. In SL-U, we believe that this conclusion remains valid for the same reasons (i.e., absence of periodic reference signals).

RLM procedure is not specified in NR SL due to absence of periodic measurement signals.

RLM procedure is not specified for SL-U.

However, RAN2 specified in Rel. 16 different triggers for declaring RLF and those are: (1) if RLC retransmissions reach maximum number, (2) Absence of HARQ feedback (DTX), and (3) T400 expiry. In case of SL-U, as described in Section 2.3.1, there is a possibility that the HARQ feedback is not transmitted due to LBT failure. However, currently, there is no procedure for Tx UE to differentiate between the absence of HARQ feedback (due to non-reception of the corresponding SCI/TB or HARQ feedback) and the absence of HARQ feedback due to LBT failure. In other words, absence of HARQ feedback due to LBT failure at the Rx UE will be counted as a DTX at the Tx UE side and is used to trigger RLF.

Absence of HARQ feedback due to LBT failure at Rx UE for transmitting the feedback is also counted as a DTX at the TX UE side and will be used to trigger RLF.

Moreover, in a congested channel, chances of dropping the HARQ feedback are high which will lead to incorrect and frequent RLF at the Tx UE. Therefore, we propose to study mechanisms to avoid frequent RLF due to LBT failures.

RAN1/RAN2 to study mechanisms to avoid frequent/incorrect RLF due to LBT failures.

# 4	Congestion control

Congestion control has been part of all 3GPP SL specifications (LTE, NR) for V2X. This feature was introduced in Rel-14 to ensure that 3GPP specifications would be suitable for operating in a spectrum that had been regulated for a different technology (DSRC/ITS-G5) and for a very specific use case (V2X/ITS). These regulations are quite different from the ones that are applicable in FR1 unlicensed spectrum (e.g., bands n46, n96/n102) and do not require separate congestion control procedures.

The congestion control procedure was introduced for operation in ITS spectrum and is not relevant for operation in unlicensed bands.

We believe that, for SL-U, RAN1 should skip the discussions on congestion control altogether.

Discussions on congestion control for SL operation in unlicensed spectrum are down-prioritized in Rel-18.

# 5	In-device coexistence

In-device coexistence features were introduced in Rel-16 to ensure a good interaction between LTE SL and NR SL within a device that would implement both technologies. Given that LTE only supports ProSe and V2X and that neither of them are meant to be deployed in unlicensed spectrum, we believe that, for SL-U, RAN1 should skip the discussions on in-device coexistence altogether

LTE/NR SL in-device coexistence is not relevant for SL operation in unlicensed spectrum.

Discussions on in-device coexistence for SL operation in unlicensed spectrum are down-prioritized in Rel-18.

# 6	Coexistence with NR-U

In unlicensed spectrum, different technologies contend for the channel based on some regulations that define some channel sharing mechanism. This mechanism is typically less efficient than coordinated transmissions on licensed spectrum. Our view is that, notwithstanding the corresponding regulations, 3GPP technologies for should be designed in a way to improve the coexistence between 3GPP UEs in unlicensed spectrum. That is, the coexistence between LTE-LAA/NR-U UL/DL transmissions and SL transmissions should be more efficient than between LTE-LAA/NR-U UL/DL and other D2D technologies (e.g., WiFi). This would make 3GPP technologies as a whole more attractive from a commercial point of view.

In licensed spectrum, coexistence between users participating in UL/DL transmissions and SL users is achieved by means of:

- Design restrictions (e.g., restricting SL operation to cell-specific UL resources, etc.).

- Synchronization and pool configuration.

- Mode 1 scheduling.

- Power control.

For operation in unlicensed spectrum, this refers to interference between SL-U and NR-U users. We distinguish two cases:

- For SL-U users under network coverage (Mode 1 and/or Mode 2) the corresponding Uu signaling (e.g., SIB and RRC for configuration, mode 1 control information) uses licensed spectrum.

- For SL-U users under network coverage (Mode 1 and/or Mode 2) the corresponding Uu signaling (e.g., SIB and RRC for configuration, mode 1 control information) uses unlicensed spectrum (i.e., NR-U).

Coexistence between SL-U and NR-U for case 2) resembles the licensed case. The WID states “2. Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only”. Our understanding is that this should be understood more broadly, meaning that all Uu signaling for SL is limited to licensed spectrum only. It would otherwise be quite strange to convey RRC configurations, SIB, etc. over NR-U and DCI plus corresponding SL HARQ FB reports over licensed spectrum.

RAN1 to consider only the case that SL-related Uu signaling is conveyed over licensed spectrum.

Given this clarification, only case 1) above is in scope for Rel-18. This case is somewhat different from Uu/SL coexistence in licensed bands. RAN1 should study the differences and specify new coexistence mechanisms, if necessary.

Coexistence between SL-U and NR-U when SL-related Uu signaling is conveyed in licensed spectrum is different from SL/Uu coexistence in licensed spectrum.

RAN1 to discuss coexistence between SL-U and NR-U within the limitations of the WID and specify mechanisms if necessary.

Figure 1. Two scenarios for coexistence between SL-U and NR-U. Left: Uu for controlling SL-U UEs uses licensed spectrum. Right: Uu for controlling SL-U UEs uses unlicensed spectrum (i.e., NR-U)

# 7	Conclusion

In the previous sections we made the following observations:

Observation 1	Main use cases for SL-U originates from commercial services such as XR.

Observation 2	RLM procedure is not specified in NR SL due to absence of periodic measurement signals.

Observation 3	Absence of HARQ feedback due to LBT failure at Rx UE for transmitting the feedback is also counted as a DTX at the TX UE side and will be used to trigger RLF.

Observation 4	The congestion control procedure was introduced for operation in ITS spectrum and is not relevant for operation in unlicensed bands.

Observation 5	LTE/NR SL in-device coexistence is not relevant for SL operation in unlicensed spectrum.

Observation 6	Coexistence between SL-U and NR-U when SL-related Uu signaling is conveyed in licensed spectrum is different from SL/Uu coexistence in licensed spectrum.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN WGs consider unicast and groupcast option 2 as the design baseline for SL-U.

Proposal 2	Further study the need of supporting broadcast for SL operation on unlicensed spectrum.

Proposal 3	RLM procedure is not specified for SL-U.

Proposal 4	RAN1/RAN2 to study mechanisms to avoid frequent/incorrect RLF due to LBT failures.

Proposal 5	Discussions on congestion control for SL operation in unlicensed spectrum are down-prioritized in Rel-18.

Proposal 6	Discussions on in-device coexistence for SL operation in unlicensed spectrum are down-prioritized in Rel-18.

Proposal 7	RAN1 to consider only the case that SL-related Uu signaling is conveyed over licensed spectrum.

Proposal 8	RAN1 to discuss coexistence between SL-U and NR-U within the limitations of the WID and specify mechanisms if necessary.

# 8	References

RP-213678, New WID on NR sidelink evolution, OPPO and LG Electronics, RAN#94e, Dec.6-17, 2021.

3GPP TR 22.842, Study on Network Controlled Interactive Services, Release 17, V17.2.0, 2019-12.