3GPP TSG RAN WG1 #109	R1-2204745

e-Meeting, May 9th – May 20th, 2022

Agenda item:		8.7.1

Source:	Nordic Semiconductor ASA

Title:					On remaining issues of TRS design for idle/inactive UEs

Document for:		Discussion and Decision

# Introduction

In RAN1#108 most of the open issues has been resolved for the topic, however few things remained open, which we will discuss in the following sections.

# Reference point for validity indication

Agreement Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication - Alt1: Adopt TP#1 - Alt2: Adopt TP#2 - Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same. Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value. ============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#2==========================================

Our preference would be to adopt TP#2, this because it unifies validity periods for the case when PDCCH MO and PEI MO happen to be physically in different DRX cycles. As a consequence, this allows for simpler feature implementation at both gNB and UE.

Proposal-1: Adopt TP#2 as clarification for the iTRS validity reference point.

# iTRS PDSCH rate-matching

Agreement TRS-ResourceSetConfig IE is not accounted in determination of available resource elements for PDSCH.

Agreement was made in RAN1#108, but not captured in the specifications, therefore, it is an essential to capture such agreement in RAN1#109. iTRS can be handled the same way as NZP-CSI-RS for mobility.

Porposal-2: Adopt TP for TS38.211 sub-clause 7.3.1.5 to clarify idle TRS rate-matching.

#### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE, or CSI-RS configured by the higher-layer parameter TRS-ResourceSetConfig, or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l.

# Configuration of iTRS

Agreement For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 - FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

Based on RAN1#107 agreement, FFS is still open regarding further limitation on number of TRS resource sets. It is a bit unclear why a UE in lower bands, where e.g. max 4beams are allowed, should support up to 64 resource-sets of Idle TRS, it would be beneficial for UE implementation to specify max number of TRS resources as function of Lmax. For example, 2 or 3 times Lmax.

Furthermore, even if UE is not required to monitor all of those at given time, UE still must comprehend/store all the TRS configurations, because gNB can dynamically validate/invalidate the preferred one “on fly”.

Proposal-3: The maximum number of TRS resource sets per cell within a band is function of Lmax and upper-bounded by 64.

# Conclusions

In this contribution we discussed remaining aspect of iTRS design and we had the following proposals:

Proposal-1: Adopt TP#2 as clarification for the iTRS validity reference point.

Porposal-2: Adopt TP for TS38.211 sub-clause 7.3.1.5 to clarify idle TRS rate-matching.

Proposal-3: The maximum number of TRS resource sets per cell within a band is function of Lmax and upper-bounded by 64.

# References

- R1-2202651, 	Moderator summary#3 on TRS/CSI-RS occasion(s) for idle/inactive UEs	, Moderator (Samsung)